3GPP TSG SA WG3 (Security) Meeting #76	S3-142326

25-29 August 2014 Sophia Antipolis (France)	revision of S3-142172

Source:	HUAWEI, Hisilicon, Deutsche Telekom AG

Title:	Resolving Editor’s Note for the synchronization of the PC3 interface

Document for:	Approval

Agenda Item:	7.11

Work Item / Release:	ProSe / Rel-12

Abstract of the contribution:

1, Introduction

This document solves one editor’s note in solution #6.2 in the ProSe security TR 33.833.

It is proposed to delete the following editor’s note in section 6.1.2.3 of the ProSe security TR 33.833，

Editor’s note: The Synchronization issue of the SQN and the HSS load are FFS.

And the editor’s note in section 6.1.2.5 of the ProSe security TR 33.833,

Editor's note: For AKA solution, the impact of synchronization errors between UICC and HSS is FFS

2, Discussion

For simplicity, we denote the AKA-based solution for PC3 interface by prose AKA-based solution.

The synchronization issue of the SQN is originated from the synchronization procedure of AKA-based mechanisms.

The following AKA–based authentication mechanisms are defined in 3GPP specs:

- EPS AKA, for LTE authentication

- UMTS AKA, for 3G authentication

- GBA AKA, for GBA authentication

- IMS AKA, for IMS authentication

- EAP AKA/EAP AKA’, for WLAN authentication

Therefore, SQN synchronization is a common issue for all AKA-based authentication domains. The existing solution to minimize synchronisation failures due to interleaving authentications can also be applied to this solution proposal.

3, PCR

It is suggested that the following changes are agreed by SA3:

*****************Change 1 Begins********************

6.1.2.3	Solution description: keys establishment procedure for PC3

The ProSe function can interact with the HSS to retrieve the authentication data in order to authenticate the UE for ProSe service. The ProSe function retrieves the ProSe AV from the HSS and performs AKA with ProSe UE to share IK', CK' with the Prose UE, and uses the IK'||CK' as the integrity protection and encryption keys needed for PC3 security protection. The content of the ProSe AV includes {RAND, XRES, IK', CK', AUTN}.

ProSe UE

ProSe Function

HSS

1

,

Registration request

(

IMSI

)

4

,

P

r

o

S

e

a

u

t

h

e

n

t

i

c

a

t

i

o

n

d

a

t

a

r

e

s

p

o

n

s

e

(

P

r

o

S

e

A

V

,

i

n

c

l

u

d

i

n

g

I

K

’

,

C

K

’

)

5

,

P

r

o

S

e

a

u

t

h

e

n

t

i

c

a

t

i

o

n

r

e

q

u

e

s

t

3

,

Derive IK’

,

CK’ using

ProSe Function ID

7

,

ProSe authentication response

8

,

Registration response

2

,

ProSe Authentication data request

(

IMSI

,

ProSe Function ID

)

6

,

Derive IK’

,

CK’ using

ProSe Function ID

Figure 6.1.2.3-1: PC3 security protection keys establishment procedure

1. The ProSe UE sends its IMSI to the ProSe Function in the registration request message.

2. The ProSe Function includes UE's IMSI and the ProSe Function's ID in the ProSe authentication data request message to the HSS to retrieve the ProSe authentication vector.

3. After the HSS derives CK, IK, the HSS derives CK'||IK' =KDF (CK||IK, Prose Function ID,SQN AK).

4. The HSS sends the AV including CK', IK' to the ProSe Function.

5. The ProSe function sends ProSe authentication request to the ProSe UE,

6. The ProSe UE authenticates the network, derives CK, IK, and then the UE derives CK'||IK'=KDF(CK||IK, Prose Function ID,SQN AK)

7. The ProSe UE sends ProSe authentication response to the ProSe function, which authenticates the UE.

8. The ProSe Function sends the registration response message.

*****************Change 1 Ends********************

*****************Change 2 Begins********************

6.1.2.5	Solution Analysis

This solution adds a new authentication domain to the existing domains CS, PS, IMS, GBA, and WLAN. In order to minimize synchronisation failures due to interleaving authentications with existing authentication domains, the HSS should be adapted to treat authentication requests via the PC4a interface as a new authentication domain.

Editor's note: For AKA solution, since for each authentication request a request to the HLR will be made, load on HSS is FFS

Editor's note: For AKA solution, how will roaming be done, when the ProSe server is residing in another operator network, is FFS

The benefits of this solution are analyzed as the following:

1. No extra network entity is required for the AKA-based solution

Editor's note: For AKA solution, the security risks by adding an additional interface to HLR/HSS are FFS.

As shown in the Fig. 6.1.2.5-1, on the network side, for the AKA-based solution, the key distribution is from the HSS to the ProSe Function utilizing the PC4a interface, but for the GBA-based solution, the BSF is needed for the purpose of the key distribution.

*****************Change 2 Ends********************