3GPP TR 33.863 V14.2.0 (2017-06)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Study on battery efficient security for very low throughput Machine Type Communication (MTC) devices

(Release 14)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

LTE, UMTS, Security, Architecture, CIoT, Battery efficient, MTC

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

1	Scope	8

2	References	8

3	Definitions, symbols and abbreviations	10

3.1	Definitions	10

3.2	Symbols	10

3.3	Abbreviations	10

4	Background and key objectives	12

4.0 	Motivation	12

4.1	Architectural assumptions	12

4.2	Clarification of "device to enterprise security" term	13

4.3	"Device to enterprise" user plane protection	14

4.4	"Device to HPLMN" user plane protection	14

4.5	Battery usage challenges	15

4.6	Cellular IoT traffic model	15

5	Key issues	16

5.1	Issue 1: N-PDU data tampering and eavesdropping	16

5.1.1	Issue description	16

5.1.2	Threat description	16

5.1.3	Security requirements	16

5.2	Issue 2: Efficient user data protection challenges	16

5.2.1	Issue description	16

5.2.2	Threat description	17

5.2.3	Security requirements	17

5.3	Issue 3: "VPLMN Specific Needs"	17

5.3.1	Issue description	17

5.3.2	Threat description	17

5.3.3	Security requirements	17

5.4	Issue 4: End-to-end security	17

5.4.1	Issue description	17

5.4.2	Threat description	18

5.4.3	Security requirements	18

6	Candidate solutions	18

6.0	General	18

6.1	Solution #1: "UE to HPLMN" security solutions based on UMTS/EPS AKA enhancements.	18

6.1.1	Introduction	18

6.1.2	Solution description	19

6.1.2.1	"UE to HPLMN" security solution with HSE context establishment procedure	19

6.1.2.2	"UE to HPLMN" security solution with HLR push procedure – Alternative	21

6.1.2.3	"UE to HPLMN" security solution with HSE pull procedure	22

6.1.2.4	Key derivation rules	24

6.1.2.5	Solution variant: End to Middle Key Server	25

6.1.2.6	Solution variant: key derivation on the ME (EPS AKA only)	25

6.1.3	Solution evaluation	26

6.2	Solution #2: "End-to-middle security based on AKA"	26

6.2.1	Introduction	26

6.2.2	Solution description	26

6.2.2.1	End-to-middle security solution based on AKA	26

6.2.2.2	Key derivation rules	27

6.2.2.3	Usage of e2m security	27

6.2.3	Solution evaluation	28

6.3	Solution #3: "Independent VPLMN and e2m security associations"	28

6.3.1	Introduction	28

6.3.2	Solution description	28

6.3.2.1	Independent VPLMN and e2m security associations	28

6.3.3	Solution evaluation	29

6.4	Solution #4: "Security policies"	29

6.4.1	Introduction	29

6.4.2	Solution description	30

6.4.2.1	Authentication and key usage policy	30

6.4.2.2	Algorithm policy	30

6.4.2.3	VPLMN Specific Algorithm policies	30

6.4.3	Solution evaluation	30

6.5	Solution #5: "End-to-end security solution"	31

6.5.1	Introduction	31

6.5.2	Solution Description	31

6.5.2.1	Specific e2e security association	31

6.5.2.2	Derivation of e2eKEYSET	31

6.5.2.3	Triggering the key derivation	31

6.5.2.4	Setting the timer	32

6.5.2.5	Interfaces of the EESE	32

6.5.3	Solution Evaluation	32

6.6	Solution #6: Bearer protection	32

6.6.1	Introduction	32

6.6.2	Solution description	32

6.6.3	Solution evaluation	33

6.7	Solution #X: "End-to-end" for solutions 1 and 2	33

6.7.1	Introduction	33

6.7.2	Solution #1 and #2 in End-to-End case	33

6.7.3	Solution Evaluation	33

6.8	Solution #8: Complete end to middle solution	34

6.8.1	Introduction	34

6.8.2	Solution description	35

6.8.2.1	Proposed Architecture	35

6.8.2.2	Service Discovery and Negotiation	35

6.8.2.3	Ability to enable and disable the BEST service	39

6.8.2.4	End to Middle Security User Plane and Signalling Plane	40

6.8.2.4.1	Data transport	40

6.8.2.4.2	End to Middle Secured Data Protocol (EMSDP)	40

6.8.2.4.3	EMSDP general structure	40

6.8.2.4.3	EMSDP Counter Schemes	42

6.8.2.4.4	EMSDP Integrity protection	42

6.8.2.4.5	EMSDP Encryption	43

6.8.2.4.6	EMSDP Commands	44

6.8.2.7	Key Agreement and Refreshing	45

6.8.2.7.1	Overview	45

6.8.2.7.2	Key setup messaging between HSE and UE	45

6.8.2.7.3	BEST key derivation mechanism	48

6.8.2.8	Starting a BEST service session	49

6.8.2.8.1	UE initiated BEST session	49

6.8.2.8.2	HSE initiated BEST session	50

6.8.2.9	Resuming a BEST session following a power cycle at the UE or a re-attach	50

6.8.2.10	BEST service session operation	50

6.8.2.11	Ending a BEST service session	50

6.8.3	Solution Evaluation	50

6.9	Solution #9: Complete end to end solution	51

6.9.1	Introduction	51

6.9.2	Solution description	52

6.9.2.1	Proposed Architecture	52

6.9.2.2	Service Discovery and Negotiation	53

6.9.2.3	Ability to Enable and Disable the BEST service	53

6.9.2.4	End to Middle Security User Plane and Signalling Plane	53

6.9.2.4.1	Data transport	53

6.9.2.4.2	End to Middle Secured Data Protocol (EMSDP)	53

6.9.2.4.3	EMSDP general structure	54

6.9.2.4.3A	EMSDP Counter Schemes	54

6.9.2.4.4	EMSDP Integrity protection	54

6.9.2.4.5	EMSDP Encryption	54

6.9.2.4.6	EMSDP Commands	54

6.9.2.7	Key Agreement and Refreshing	57

6.9.2.7.1	Overview	57

6.9.2.7.2	Key setup messaging between HSE and UE	57

6.9.2.7.3	BEST key derivation mechanism	60

6.9.2.8	Starting a BEST service session	62

6.9.2.8.1	UE initiated BEST session	62

6.9.2.8.2	HSE initiated BEST session	63

6.9.2.9	Resuming a BEST session following a power cycle at the UE or a re-attach	63

6.9.2.10	BEST service session operation	64

6.9.2.11	Ending a BEST service session	64

6.9.3	Solution Evaluation	64

6.10	Solution #10: "AKA-based session key generation for application protocols"	64

6.10.1	Introduction	64

6.10.2	Solution description	65

6.10.2.1	Features	65

6.10.2.2	Interface between EMKS and EMSE	68

6.10.2.2.1	Introduction	68

6.10.2.2.2	Procedures over the RESTful HTTP reference point	68

6.10.2.3	Example use of solution 10: information flow using pre-shared key DTLS	69

6.10.3	Solution evaluation	71

6.11	Solution #11: A method for IoT service layer security bootstrapping solution	71

6.11.1	Introduction	71

6.11.2	Solution description	72

6.11.2.1	Proposed architecture	72

6.11.2.2	Security boostrapping and key refreshing	72

6.11.2.2.1 	Overview	72

6.11.2.2.2	Key agreement and boostrapping with HSS deriving master session key	72

6.11.2.2.3	Key refreshing	75

6.11.3	Solution evaluation	76

6.12	Solution #12: A method for IoT service layer security bootstrapping solution	76

6.12.1	Introduction	76

6.12.2	Solution description	76

6.12.2.1	Proposed architecture	76

6.12.2.2	Security boostrapping and key refreshing	77

6.12.2.2.1	Overview	77

6.12.2.2.2	Key agreement and boostrapping	77

6.12.2.2.3	Key refreshing	80

6.12.3	Solution evaluation	81

7	Conclusions	82

7.1	Issues identified	82

7.2	Solution evaluation summary	82

7.3	Recommendation for normative work	86

Annex A:	 AKA procedures assessment in very low data throughput environment	87

A.1	Introduction	87

A.2	UMTS AKA	87

A.3	DTLS handshake for ECDHE-ECDSA configuration	89

A.3.1	DTLS handshake procedure measurement	89

A.3.2	TLS Record and Handshake message measurement	91

A.4	DTLS record header overhead description for ciphered data	95

A.4.1	DTLS record header measurement	95

A.5	TLS handshake session resumption	96

A.5.1	TLS handshake session resumption procedure measurement	96

A.5.2	TLS Record and Handshake message measurement	96

A.6	GBA bootstrapping procedure	99

A.6.1	Bootstrapping procedure description measurement	99

A.6.2	PSK-TLS procedure measurement in GBA case.	101

Annex B:	 Review of security standardization efforts in other SDOs	105

B.0	Introduction	105

B.1	(D)TLS optimization efforts in IETF	105

B.1.1	Background	105

B.1.2	Existing and evolving TLS optimizations	105

B.1.3	Making the full handshake lighter	106

B.1.4	Resuming existing connection	106

Annex C:	Proposed normative changes	107

C.1	Introduction	107

C.2	Proposed changes to 3GPP TS 33.401 [28]	107

C.2.1	Overview of changes	107

C.3	Proposed changes to 3GPP TS 43.020 [47]	108

Annex D:	Change history	109

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document's objectives are to:

a)	Investigate whether battery efficient 'device to enterprise' mechanisms to provide sufficient security exist.

b)	Investigate potential enhancements to 3GPP's security architecture(s) that enable the Home Operator to be able to offer well guaranteed security to enterprises e.g. to provide security between the UE and a node in the home operator domain.

c)	Both S1 and Gb based architectures should be considered when undertaking b, above.

The impacts of relevant authentication and key agreement procedures are also presented.

The following should be taken into account for the study:

-	The potential solutions should aim at minimising the energy consumption of devices to help meet the objective of a ten year battery life in the extended coverage situation and take into consideration the very low data rate capability of the radio interface when using the Energy Consumption Evaluation Methodology described in TR 45.820.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 41.001: "GSM Release specifications".

[3]	Void

[4] 	IETF Transport Layer Security (tls) Working Group, Charter, http://datatracker.ietf.org/wg/tls/charter/

[5] 	IETF RFC 7252: "The Constrained Application Protocol (CoAP)".

[6] 	I-D draft-ypoeluev-tls-m2mcertificate-00 "Transport Layer Security (TLS) and Datagram Transport Layer Security (DTLS) Authentication Using M2M Certificate".

[7] 	I-D draft-ietf-tls-cached-info-19 "Transport Layer Security (TLS) Cached Information Extension".

[8] 	I-D draft-ietf-tls-tls13-05: "The Transport Layer Security (TLS) Protocol Version 1.3".

[9] 	IETF RFC 5246: "The Transport Layer Security (TLS) Protocol Version 1.2".

[10] 	I-D draft-ietf-dice-profile-13: "A TLS/DTLS Profile for the Internet of Things".

[11]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[12]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[13]	3GPP TS 48.018: "General Packet Radio Service (GPRS); Base Station System (BSS) - Serving GPRS Support Node (SGSN); BSS GPRS protocol (BSSGP)".

[14]	3GPP TS 44.064: "Mobile Station - Serving GPRS Support Node (MS-SGSN); Logical Link Control (LLC) Layer Specification".

[15]	3GPP TS 44.065: "Mobile Station (MS) - Serving GPRS Support Node (SGSN); Subnetwork Dependent Convergence Protocol (SNDCP)".

[16]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[17]	Void.

[18]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[19]	3GPP TS 33.187: "Security aspects of Machine-Type Communications (MTC) and other mobile data applications communications enhancements".

[20]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[21]	3GPP TS 33.223: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA) Push function".

[22]	3GPP TS 22.368: "Service requirements for Machine-Type Communications (MTC); Stage 1".

[23]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification".

[24]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

[25]	3GPP TR 45.820: "Cellular system support for ultra-low complexity and low throughput Internet of Things (CIoT)".

[26]	NIST SP 800-38D.

[27]	3GPP TS 33.102: "3G security; Security architecture".

[28]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[29]	3GPP TS 29.060: "General Packet Radio Service (GPRS); GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface".

[30]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[31]	3GPP TS 23.002: "Network architecture".

[32]	IETF RFC 6347.: "Datagram Transport Layer Security" [33]	IETF RFC 4492. "Elliptic Curve Cryptography (ECC) Cipher Suites for Transport Layer Security (TLS) ".

[34]	3GPP TS 24.109: "Bootstrapping interface (Ub) and network application function interface (Ua); Protocol details".

[35]	3GPP TS 33.222: "Generic Authentication Architecture (GAA); Access to network application functions using Hypertext Transfer Protocol over Transport Layer Security (HTTPS)".

[36]	IETF RFC 4303: "IP Encapsulation Security Payload (ESP)".

[37]	3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security".

[38]	3GPP TS 55.241: " Specification of the GIA4 integrity algorithm for GPRS; GIA4 specification".

[39]	3GPP TS 55.251: " Specification of the GEA5 encryption and GIA5 integrity algorithms for GPRS; GEA5 and GIA5 specification".

[40]	3GPP TS 35.205: "3G Security; Specification of the MILENAGE Algorithm Set:  An example algorithm set for the 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*; Document 1: General".

[41]	3GPP TS 35.215: "Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2; Document 1: UEA2 and UIA2 specifications".

[42]	3GPP TS 35.401: ""

[43]	3GPP TS 35.221: " Specification of the 3GPP Confidentiality and Integrity Algorithms EEA3 & EIA3; Document 1: EEA3 and EIA3 specifications".

[44]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2)"

[45]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)"

[46]	3GPP TS 23.682:" Architecture enhancements to facilitate communications with packet data networks and applications"

[47]	3GPP TS 43.020: "Security related network functions"

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

E2m: End to middle

Gb architecture: a system architecture described in TS 23.060 [11] that uses the protocols defined in e.g. 3GPP TSs 24.008 [12], 48.018 [13], 44.064 [14] and 44.065 [15].

Note 1: 	It implies the use of a USIM application.

S1 architecture: a system architecture described in TS 23.401 [16] that uses the protocols defined in e.g. 3GPP TSs 24.301 [30] and 36.413 [18].

Note 2 : 	It implies the use of a USIM application.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

||	Concatenation

	Communication between

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AKA		Authentication and Key Agreement

AMF		Authentication Management Field

AuC		Authentication Centre

cIoT	Cellular Internet of Things

CK		Ciphering Key

CK'	replacement CK for ait interface security to withhold CK from the visited network

CoAP	Constrained Application Protocol

DH		Diffie-Hellman

DLTS	Datagram Transport Layer Security

E-UTRAN		Evolved Universal Terrestrial Radio Access Network

E2E		End to End

EESE	End to End Security Endpoint

EMKS	End to Middle Key Server

EMSE	E2M Security Endpoint

EPS		Evolved Packet System

GBA		Generic Bootstrapping Architecture

GERAN		GSM EDGE Radio Access Network

GGSN		Gateway GPRS Support Node

GTP		GPRS Tunnelling Protocol

HLR		Home Location Registry

HSS		Home Subscriber Server

IK		Integrity Key

IK'	replacement for IK for air interface security to withhold IK from the visited network

IoT	Internet of Things

KDF		Key Derivation Function

LLC		Logical Link Controller

MME		Mobility Management Entity

MTC	Machine Type Communications

P-GW		PDN Gateway

PDN		Packet Data Network

PLMN		Public Land Mobile Network

S-GW		Serving Gateway

SGSN		Serving GPRS Support Node

SQN		Sequence Number

TLS		Transport Layer Security

UE		User Equipment

UMTS		Universal Mobile Telecommunication System

UP		User Plane

USIM		Universal Subscriber Identity Module

# 4	Background and key objectives

## 4.0 	Motivation

End-to-end security protocols e.g. DTLS between the MS and an application server provides one way of securing the communication between MS and a cellular IoT application server, irrespective of the nature of the security over the radio access and within the cellular network domain (including both the home network and visited network).

One of the main drawbacks of supporting existing end-to-end security protocols for Cellular IoT devices is the amount of security related signalling – protocol overheads like DTLS handshakes - that needs to be exchanged between MS and the application server before any useful information can be sent (usually a small data packet). The signalling overhead will not only reduce the radio access capacity but, more importantly, increase the energy consumption by the M2M device. This may make the objective of having devices lasting for years with standard battery power unachievable.

From an application level perspective, the customer may not be satisfied with relying on the user plane security between MS and a visited SGSN (assuming this is implemented) and between the GGSN and the application server since there is a gap in user plane security between the visited SGSN and the GGSN (especially when NDS/IP is not implemented). It is to be noted that the communication between a visited SGSN and GGSN may run over thousands of Kilometres.

SGSN (Visited Network)

MS

GGSN (Home Network)

IoT Application server

Key:				User plane may be encrypted

No user plane encryption

Figure 4.0-1: User plane security gap between SGSN and GGSN

If user plane security could be extended to the GGSN, or to a nearby MTC server, this may remove the need for additional end-to-end security, which will in turn improve both the radio access capacity and the MS energy consumption. Otherwise, the signalling exchange over the radio access to establish application level security need to be optimised.

The above argument refers to the Gb architecture, but applies equally to the S1 architecture (Serving Gateway replacing SGSN, and PDN gateway replacing GGSN, as appropriate). The terms "Gb architecture" and "S1 architecture" are defined in the next clause.

It is recognised that lawful interception regulations may sometimes prohibit confidentiality from being applied between device and home network, particularly in a roaming scenario – see the paragraph beginning "When local or regional regulation allows …" in clause 5.1.3.

## 4.1	Architectural assumptions

The following should be taken into account:

- 	Security aspects need to be investigated to be compatible with both Gb and S1 architectures.

- 	"Gb architecture" implies a system architecture described in TS 23.060 [11] that uses the protocols defined in e.g. 3GPP TSs 24.008 [12], 48.018 [13], 44.064 [14] and 44.065 [15]. It implies the use of a USIM application.

- 	"S1 architecture" implies a system architecture described in TS 23.401 [16] that uses the protocols defined in e.g. 3GPP TSs 24.301 [30] and 36.413 [18]. It implies the use of a USIM application.

- 	Roaming is supported.

- 	The UE will roam in countries where encryption is allowed and where encryption is not used. SA1 adds that saying that there will be some UEs for which their HPLMN operator does not expect the UE to roam between countries allowing encryption and countries not allowing encryption.

- 	Inter-RAT mobility is not supported. SA2 has the same understanding. SA1 doesn't exclude the fact that some devices vendor may implement a multi-RAT UE which would perform PLMN reselection between those different RATs, but they do not precise whether security aspects should be addressed.

- 	GERAN believes that subscriptions used to access cellular IoT (CIoT) are not expected to be used for other 3GPP RATs and subscriptions used for other 3GPP RATs are not expected to be used to access CIoT. SA1 believes that in general 3GPP subscriptions can be used on any 3GPP access technology.

## 4.2	Clarification of "device to enterprise security" term

"Device to Enterprise" security refers to a broader scope which could be made from the following, non-exhaustive, combinations:

- 	security between the Enterprise and the MTC device;

- 	OR security between the Enterprise and the HPLMN, then security between the HPLMN and the MTC device;

- 	OR security between the Enterprise and the HPLMN, then security between the HPLMN and the VPLMN, then security between the VPLMN and the MTC device.

For those combinations described above, the security protection being applied to a communication would traverse intermediaries which are not able to perform security operations on the data being exchanged between those two endpoints. For instance the "security between the HPLMN and the MTC device" of user plane data may traverse a VPLMN and a GRX network which will not be able to perform security operation (e.g. MAC verification) on those exchanged user plane data between those two endpoints.

Figure 4.2-1: High level ecosystem

## 4.3	"Device to enterprise" user plane protection

In clause 7 "Security procedures for secure connection" of 3GPP TS 33.187 [19], GBA (see 3GPP TS 33.220 [20]) and GBAPush (see 3GPP TS 33.223 [21]) were selected as the preferred optional features for addressing the SA1 requirement in "Secure Connection" in clause 7.2.10 and the use case "End-to-end security for roaming MTC devices" in Annex A of 3GPP TS 22.368 [22].

Many current MTC users implement "device to enterprise" security. It is believed that these security mechanisms incur a significant data overhead by e.g. frequent security key renegotiation, and, that this data overhead could severely impact the useful battery life of such devices.

This document intends to perform a study of those "device to enterprise" security solutions and discover whether they are fit for purpose.

## 4.4	"Device to HPLMN" user plane protection

In that study, it is needed to "Investigate potential enhancements to 3GPP's security architecture(s) that enable the Home Operator to be able to offer well guaranteed security to enterprises e.g. to provide security between the UE and a node in the home operator domain.". End-to-middle security (e2m security) denotes security between an UE and a server that is not the communication endpoint. This server can reside in the HPLMN or in a third party network. If the latter the server needs to be authorised to obtain cryptographic keys from the HPLMN. Also it is needed to consider both Gb and S1 based architectures in this study.

In GPRS system, the user plane data is carried over the following nodes:

UE  BSS  SGSN  GGSN

In EPS system the user plane data is carried over the following nodes:

UE  eNB  S-GW P-GW

When a S4 SGSN is used, the user plane data is carried over the following nodes:

UE  BSS  SGSN  S-GW  P-GW

In GPRS system, the user plane data is carried in LLC frames between the UE and the SGSN. LLC frames are confidentiality protected between the UE and the SGSN (see 3GPP TS 44.064 [14]).

In EPS system, the user plane data is carried in PDCP PDUs (see 3GPP TS 36.323 [23]) between the UE and the eNB that may be confidentiality protected.

GTP-U (see 3GPP TS 29.281 [24]) packets carry the user plane data between:

- the SGSN and the GGSN,

- the SGSN and the S-GW if S4 interface is used,

- the eNB and the S-GW,

- the S-GW and the P-GW.

In the roaming case, the user plane data is carried between the SGSN and GGSN in a GPRS system (resp. S-GW and P-GW in an EPS system) over a GRX/IPX network (a.k.a inter-PLMN backbone network). The GRX/IPX network is intended to be a trusted network which interconnects several PLMN and service providers either with a direct communication link or through an third parties called GRX/IPX providers. Two PLMNs may communicate over a selected inter-PLMN backbone network that includes border gateway security functionalities (see clause "Packet Domain PLMN Backbone Networks" in clause 5.4.2 of 3GPP TS 23.060 [11]) however those security functionalities aren't uniformly used.

The user plane data is protected with key derived from GSM/UMTS AKA between the UE and the SGSN for GPRS systems.

The user plane data is protected with keys derived from EPS AKA between the UE and the eNB for EPS systems.

NDS/IP is used between the eNB and S-GW for EPS systems.

## 4.5	Battery usage challenges

According to the FS_IoT_LC Study Item in GP-140421, the types of the devices under study are required to last 10 years on 5Wh battery capacity only. Consequently, reducing the power consumption for security procedures should be taken into account.

A methodology for assessing the battery performance metrics of a particular RAT is described in clause 5.4 of TR 45.820 [25].

## 4.6	Cellular IoT traffic model

Cellular IoT traffic models are described in Annex E of TR 45.820 [25].

# 5	Key issues

## 5.1	Issue 1: N-PDU data tampering and eavesdropping

### 5.1.1	Issue description

Although the air interface may provide confidentiality protection (e.g. with GEA4), the following interfaces are not required to provide data confidentiality: Gp, Gn, S5, S8. This means that N-PDUs exchanged between the MTC device and the GGSN (resp. the P-GW) aren't consistently confidentiality protected.

Also the air interface, Gp, Gn, S5 and S8 do not provide integrity protection to plaintext or ciphered N-PDUs.

### 5.1.2	Threat description

In the cellular IoT case, battery constraints mean that higher layer protection (e.g. end to end TLS) is less likely to be present than for other sensitive data carried over cellular connections. The N-PDU packets are therefore likely to be subject to eavesdropping when carried over a GRX/IPX network, thus leaking sensitive data to an eavesdropper or to the visiting network.

In the cellular IoT case, battery constraints mean that higher layer protection (e.g. end to end TLS) is less likely to be present than for other sensitive data carried over cellular connections. The N-PDU packets are therefore likely to be tampered with at the air interface or over a GRX/IPX network, thus sending false/negative reports to the home network.

### 5.1.3	Security requirements

There should be a mechanism that provides integrity protection to the transmitted N-PDUs between the MTC device and a node within or beyond the home network.

When local or regional regulation allows, it should be possible to provide confidentiality protection to the transmitted N-PDUs that only the UE and a node within or beyond the home network should be able to encrypt/decrypt.

## 5.2	Issue 2: Efficient user data protection challenges

### 5.2.1	Issue description

N-PDUs may consist of the encapsulation of the following protocols: IP, UDP or TCP, TLS/DTLS, Application-level protocol such as HTTP or COAP. Other protocols could be used or could even be omitted from the N-DPU e.g. no IP header.

The maximum size of a MAR exception message is 200 bytes but let's assume that the N-PDU should not be greater than 200 bytes, then the following numbers are for consideration:

- 	An uncompressed IPv4 header may be 20 bytes, while an IPv6 header may be 40 bytes.

- 	A compressed IPv4/IPv6 header may be 4 bytes while an UDP header is 8 bytes long. The best case is that the application payload ends up with 188 bytes allocated.

- 	In a year time, a total of 73000 bytes of N-PDU may be exchanged if 200 bytes long MAR exception reports are exchanged on a daily basis.

- 	Assuming that a block cipher algorithm of 16 bytes block size is used, the best case is that the plaintext message is 192 bytes, then 176 bytes (192 minus 16), then 160 bytes (176 minus 16) of usable data.

Theseremainders might be used for something else such as a MAC. The table 5.2.1-1 below summarizes this potential distribution in case both confidentiality and integrity protection are provided.

Table 5.2.1-1

| Plaintext payload size (bytes) | MAC size (bytes) |
| --- | --- |
| 192 | 8 |
| 176 | 24 |
| 160 | 40 |

### 5.2.2	Threat description

### 5.2.3	Security requirements

The minimum key size for providing data confidentiality and/or integrity should be 128 bits.

If HMAC is used to protect an entire N-PDU, the computed or transmitted MAC value should not be less than 64 bits. The HMAC key should at least be refreshed every year.

If AES-GCM, which is an authenticated encryption algorithm, is used to protect an entire N-PDU, the length of MAC may be 64 bits. However, the integrity key should be refreshed after being used to protect plaintexts/ciphertexts of cumulated length of 215 bytes (meaning a key update two times a year).

NOTE 1: 	The values above are based on requirements and guidelines from Appendix C of NIST SP 800-38D [26].

If AES-CMAC is used to protect an entire N-PDU, the length of MAC may be 64 bits. However, the integrity key should be refreshed after being used to protect plaintexts/ciphertexts of cumulated length of 248 messages when the block size is 128 bits.

NOTE 2: 	The values above are based on requirements and guidelines from Appendix B of NIST SP 800-38B [26].

The values above are presented only as indications of how often (as a minimum) keys are likely to be updated. Key refresh will be done based on operator policy – there is no expectation of it being enforced by the 3GPP standard.

## 5.3	Issue 3: "VPLMN Specific Needs"

### 5.3.1	Issue description

Devices that roam in a visited network are subject to the regulations of the country of the visited network. A visited network might have certain requirements with respect to what (types of) algorithms are allowed. In particular, for end-to-end and end-to-middle security, lawful interception regulations in some countries might forbid the use of (some) encryption algorithms. Other countries however might allow this type of end-to-end and end-to-middle security and other countries again might forbid or allow specific CIoT devices to use end-to-end and end-to-middle security.

### 5.3.2	Threat description

### 5.3.3	Security requirements

Solutions dealing with end-to-end and end-to-middle security should have mechanisms to deal with the regulations depending on the class of CIoT UE device and the regulations applying to the visited network.

## 5.4	Issue 4: End-to-end security

### 5.4.1	Issue description

Requirements with respect to end-to-end security of large scale CIoT deployments could be driven by the requirements of large corporations using CIoT for their specific needs. In particular, multinationals, such as mining and oil companies that use the CIoT UEs for measurement devices will want to have:

-	End-to-end security, in particular with respect to the integrity of the measurement data;

-	Control over the key material, in particular with respect to how often keys should be refreshed.

### 5.4.2	Threat description

The threat in these cases is out of the scope of 3GPP, meaning that a violation of the integrity will cause a problem somewhere outside of the 3GPP or operators' domains. For example, the threat from a violation of the integrity of measurements could range from a wrong decision leading to a less than optimal processing in the plants to deadly mistakes based on misinformation.

### 5.4.3	Security requirements

The end-user should have control over the frequency of derivation of the key material.

The key-material exchange between the end-users premises and the network should be minimized.

Higher level key material should not leave the 3GPP or operators domain.

# 6	Candidate solutions

## 6.0	General

Clause 6 details processes that may eventually be in a normative document.  For clarity, shalls and shoulds are included in clause 6 however as this document is a TR all text in clause 6 is informative and not mandatory.

## 6.1	Solution #1: "UE to HPLMN" security solutions based on UMTS/EPS AKA enhancements.

### 6.1.1	Introduction

In clause 6.1, "e2m" refers to a UE to HPLMN security association.

The following defines enhanced authentication and key agreement procedures for mobile network with a Cellular Internet-of-Things RAT which extends the UMTS AKA (See 3GPP TS 33.102 [27], clause 6.3) and EPS AKA (3GPP TS 33.401 [28], clause 6.1) procedures:

- 	While remaining compatible with existing security procedures between the UE and the E-UTRAN defined in 3GPP TS 33.401 [28].

- 	While remaining compatible with existing security procedures between the UE and the GERAN when UMTS AKA is used as defined in 3GPP TS 33.102 [27].

- 	To derive additional keys (from IK, and from CK under specific circumstance) securing data in an end-to-end fashion between the UE and a network node within the Home PLMN.

"UE to HPLMN" protection of the user plane data is being provided from the UE to the home network. The new security functionalities the HPLMN needs to support can be implemented as a new network function called HPLMN Security Endpoint (HSE). The HSE may also be collocated with the GGSN/P-GW.

The new keys shared between UE and HSE are called e2mKEYSET. If e2m security uses separate encryption and integrity algorithms then this keyset will include a cipher key and an integrity key; if e2m security uses a combined authenticated encryption algorithm then this keyset will consist of a single key.

The UE (resp. "UE to HPLMN" security functionality) uses the relevant key to generate/verify a signature/MAC or cipher/decipher the user plane data.

### 6.1.2	Solution description

#### 6.1.2.1	"UE to HPLMN" security solution with HSE context establishment procedure

Key Establishment Procedure:

Figure 6.1.2.1-1: Procedure Diagram

Steps:

0. In addition to the generated authentication vectors as defined in UMTS AKA (resp. EPS AKA), the HSS/HLR:

- 	in the case of UMTS AKA:

- 	derives CK´/IK´ from a concatenation of CK and IK, see clause 6.1.2.4 (CK´/IK´ will be used for air interface security) and sets AMF bit to 1 to indicate to the USIM that KDF is required to be done on CK to obtain the usable air interface ciphering and integrity keys;

- 	derives e2mKEYSET from a concatenation of CK and IK, see clause 6.1.2.4;

- derives e2mKEYSET_ID from a concatenation of CK and IK, see clause 6.1.2.4.

- 	in the case of EPS AKA:

- 	from CK and IK, as well as deriving KASME in the usual way, also derives e2mKEYSET and e2mKEYSET_ID, see clause 6.1.2.4;

- 	sets an AMF bit to 1 to indicate to the UE whether e2m keys should be derived;

- 	builds the AV. For the UMTS AKA case, CK/IK is replaced with the newly generated CK´/IK´.

1a. Distribution of AV to the SGSN/MME

NOTE 1: 	It is recommended to distribute only one AV at a time as the frequency of AKA runs is very low in the MTC scenario.

1b. The HSS/HLR pushes to the HSE the following pair:

- 	e2mKEYSET

- 	e2mKEYSET_ID

- 	optionally the IMSI

2. SGSN sends the authenticate request message to the UE as defined in TS 33.102 [27] (resp. TS 33.401 [28]).

3. USIM runs UMTS/EPS AKA, and if the AMF bit indicating that additional key derivation is to be done is set to 1, performs the following additional key derivations:

-	in the case of UMTS AKA:

- 	CK´/IK´ (used for Air IF sec) from a concatenation of CK and IK, see clause 6.1.2.4;

-	 e2mKEYSET from a concatenation of CK and IK, see clause 6.1.2.4;

- 	e2mKEYSET_ID from a concatenation of CK and IK, see clause 6.1.2.4.

- 	in the case of EPS AKA:

- 	e2mKEYSET and e2mKEYSET_ID from CK and IK, see clause 6.1.2.4.

4. The UE sends the authenticate response message to the SGSN/MME as defined in TS 33.102 [27] (resp. TS 33.401 [28]).

5a. If no PDP context exist, the UE sends the Activate PDP context request message to the SGSN/MME with a PCO IE which should contain the e2mKEYSET_ID associated with the e2m keys to be used for the protection of the N-PDUs. The PCO IE will be transparently transmitted to the GGSN/P-GW as specified in 3GPP TS 29.060 [29].

5b. If an Update PDP Context request message is instead sent by the UE to the SGSN/MME, a PCO IE shall also be included in the message with the e2mKEYSET_ID associated with the e2m keys to be used for the protection of the N-PDUs. The PCO IE will be transparently transmitted to the GGSN/P-GW as specified in 3GPP TS 29.060 [29].

6. The GGSN/P-GW forwards the e2mKEYSET_ID along with the IMSI indicated in the Activate/Update PDP context message and the TEID as a triplet to the HSE.

6a. The HSE matches the information in the triplet with the recently pushed e2mKEYSET_ID and e2mKEYSET.

Note 2: 	If the HSE does not have a record of the received e2mKEYSET_ID within its memory, then:

- 	The HSE sends an error message back to the GGSN/P-GW, including the IMSI.

- 	On receipt of this error message, the GGSN/P-GW sends an error message to the UE in a PCO IE as specified in 3GPP TS 29.060 [29].

- 	On receipt of this error message, the UE sends a message to the SGSN/MME requesting a new authentication exchange using newly generated authentication vectors.

NOTE 3:	 This will be a newly standardised feature. Newly defined parameterisations of the Tracking Area Update and/or Routing Area Update messages (with new cause codes, and minimal additional information) may be used for this. Specially, a new Update Type can be defined to indicate a request for reauthentication, and for this Update Type the RAU/TAU message should be made as short as possible by making as many fields optional as possible (including, potentially, some that are mandatory today). Alternatively, new message types may be defined in 3GPP TS 24.008 [12] and 3GPP TS 24.301 [30].

- 	The new authentication exchange causes the whole process defined here to restart, in particular creating new e2m keys to be pushed to the HSE.

7. The HSE is ready to provide confidentiality and/or integrity protection to the N-PDUs.

The HSE will need a way to flush out e2m key pairs that are never used, rather than retaining them indefinitely. There seems no need to standardise a mechanism for this, but a time-based approach is recommended, deleting keys that remain unused for a specified period after they are generated. The process defined above includes a way to recover if an e2m key pair is deleted that would eventually have been used.

#### 6.1.2.2	"UE to HPLMN" security solution with HLR push procedure – Alternative

Figure 6.1.2.2-1: Procedure Diagram

Procedure:

Steps:

0. In addition to the generated authentication vectors as defined in UMTS AKA (resp. EPS AKA), the HLR/HSS:

-	in the case of UMTS AKA:

-	derives CK´/IK´ from a concatenation of CK and IK, see clause 6.1.2.4 (CK´/IK´ will be used for air interface security) and sets AMF bit to 1 to indicate to the USIM that KDF is required to be done on CK to obtain the usable air interface ciphering and integrity keys.

-	derives e2mKEYSET from a concatenation of CK and IK, see clause 6.1.2.4.

-	in the case of EPS AKA:

-	from CK and IK, as well as deriving KASME in the usual way, also derives e2mKEYSET, see clause 6.1.2.4.

-	sets an AMF bit to 1 to indicate to the UE whether e2m keys should be derived.

-	While building the AV and for the UMTS AKA case only, replaces CK/IK with the newly generated CK´/IK´.

1. Distribution of AV to the SGSN/MME

NOTE 1:	 It is recommended to distribute only one AV at a time as the frequency of AKA runs is very low in the MTC scenario.

2. The SGSN/MME sends the authenticate request message to the UE as defined in TS 33.102 [27] (resp. TS 33.401 [28]).

3. The USIM runs UMTS/EPS AKA, and if the AMF bit indicating that additional key derivation is to be done is set to 1, performs the following additional key derivations:

-	in the case of UMTS AKA:

-	CK´/IK´ (used for Air IF sec) from a concatenation of CK and IK, see clause 6.1.2.4

-	e2mKEYSET from a concatenation of CK and IK, see clause 6.1.2.4

-	in the case of EPS AKA:

-	e2mKEYSET from CK and IK, see clause 6.1.2.4

4. The UE sends the authenticate response message to the SGSN/MME as defined in TS 33.102 [27] (resp. TS 33.401 [28]).

5. Upon successful UMTS AKA (resp. EPS AKA) authentication result and successful PDP context activation as defined in 23.002 [31] (Resp. PDN Connectivity Request, see 3GPP TS 23.401 [16]), the SGSN/MME sends a notification message to the HLR/HSS with the IMSI, the SQN value (or RAND) and the GGSN/P-GW address as parameters.

6. The HLR/HSS pushes e2mKEYSET to the HSE.

7. UE initiates establishment of PDP Context at the GGSN/PGW. The need for the E2M security is indicated.

8. The GGSN/P-GW forwards the e2mKEYSET_ID along with the IMSI indicated in the Activate/Update PDP context message and the TEID as a triplet to the HSE. The HSE matches the information in the triplet with the recently received e2mKEYSET_ID and e2mKEYSET from the HSS.

Note 2: 	If the HLR/HSS does not have a record of the received e2mKEYSET_ID within its memory, then:

-	The HLR/HSS sends an error message back to the GGSN/P-GW, including the IMSI.

- 	On receipt of this error message, the GGSN/P-GW sends an error message to the UE.

- 	On receipt of this error message, the UE sends a message to the SGSN/MME requesting a new authentication exchange using newly generated authentication vectors for example using an adaptation of the Tracking Area Update request as explained in Note 3 of section 6.1.2.1.

- 	The new authentication exchange causes the whole process defined here to restart, in particular creating a new e2m KEYSET to be pushed to the HLR/HSS.

#### 6.1.2.3	"UE to HPLMN" security solution with HSE pull procedure

Key Establishment Procedure:

Figure 6.1.2.3-1: Process flow diagram

Steps:

0. In addition to the generated authentication vectors as defined in UMTS AKA (resp. EPS AKA), the HLR/HSS:

-	in the case of UMTS AKA:

-	derives CK´/IK´ from a concatenation of CK and IK, see clause 6.1.2.4 (CK´/IK´ will be used for air interface security) and sets AMF bit to 1 to indicate to the USIM that KDF is required to be done on CK to obtain the usable air interface ciphering and integrity keys;

-	derives e2mKEYSET from a concatenation of CK and IK, see clause 6.1.2.4.

-	in the case of EPS AKA:

-	from CK and IK, as well as deriving KASME in the usual way, also derives e2mKEYSET, see clause 6.1.2.4;

-	sets an AMF bit to 1 to indicate to the SGSN/MME/UE whether E2E keys should be derived;

-	While building the AV and for the UMTS AKA case only, replaces CK/IK with the newly generated CK´/IK´.

NOTE 1: The HSS keeps CK and IK in memory for a short period of time, e.g. a couple of minutes so that the HSE can pull the key set.

1. Distribution of AV to the SGSN/MME

NOTE 2:	It is recommended to distribute only one AV at a time as the frequency of AKA runs is very low in the MTC scenario.

2. The SGSN/MME sends the authenticate request message to the UE as defined in TS 33.102 [27] (resp. TS 33.401 [28]).

3. The USIM runs UMTS/EPS AKA, and if the AMF bit indicating that additional key derivation is to be done is set to 1, performs the following additional key derivations:

-	in the case of UMTS AKA:

-	CK´/IK´ (used for Air IF sec) from a concatenation of CK and IK, see clause 6.1.2.4;

-	e2mKEYSET from a concatenation of CK and IK, see clause 6.1.2.4.

-	in the case of EPS AKA:

-	e2mKEYSET from CK and IK, see clause 6.1.2.4.

4. The UE sends the authenticate response message to the SGSN/MME as defined in TS 33.102 [27] (resp. TS 33.401 [28]).

5a. If no PDP context exist, the UE sends the Activate PDP context request message to the SGSN/MME with a PCO IE which should contain the SQN value (or RAND) from the AV and associated IMSI to the GGSN. The PCO IE will be transparently transmitted to the GGSN/P-GW as specified in 3GPP TS 29.060 [29].

5b. If an Update PDP Context request message is instead sent by the UE to the SGSN/MME, a PCO IE shall also be included in the message. The PCO IE should contain the SQN value (or RAND) and associated IMSI. The PCO IE within the Update PDP Context request message will be transparently transmitted to the GGSN/P-GW as specified in 3GPP TS 29.060 [29].

6. The GGSN/P-GW forwards the SQN value (or RAND) along with the IMSI indicated in the Activate/Update PDP context message and the TEID as a triplet to the HSE.

7 & 8. The HSE fetches e2mKEYSET from the HSS/HLR by providing the SQN value (or RAND) and associated IMSI

Editor's note: it is FFS whether and how E2E CK/IK key could be retained or recreated by the HLR/HSS in order to be pulled by the HSE.

9. The HSE is ready to provide confidentiality and/or integrity protection to the N-PDUs.

#### 6.1.2.4	Key derivation rules

The following is one possible way of implementing the key derivation rules. Other possible implementations options may be defined.

The key derivation function should be supported by the UICC and the HLR/HSS and is written as KDF (Key, S) below.

The key derivation algorithm KDF should be HMAC SHA 256 (as defined in 3GPP TS 33.220 [20]).

For the UMTS AKA case, the following key derivation applies:

- CK´||IK´ = KDF (CK||IK, key type 1).

NOTE 1:	CK´ goes from bit 0 to bit 127, IK´ goes from bit 128 to bit 255. Key type 1 is an ASCII string pointing to the purpose of the key use, e.g. "VPLMN_CIoT".

- e2mKEYSET = KDF (CK||IK, key type 2).

NOTE 2: 	If e2mKEYSET includes more than 256 bits of key material, then multiple instances of KDF are used with different key type strings (key type 2A, key type 2B, etc).

- If the solution requires an e2mKEYSET_ID then e2mKEYSET_ID = KDF (CK||IK, key type 3).

For the EPS AKA case, the derivation of e2mKEYSET and e2mKEYSET_ID (if required) is the same as in the UMTS AKA case. For the visited network, though, KASME is derived as usual.

NOTE 3: 	If an AKA algorithm such as TUAK were used, that can generate 256-bit CK and IK, then the key derivation formulae above could be different, with keys for the visited network security generated purely from CK, and keys for the e2m security derived purely from IK. 256-bit CK and IK are currently outside the scope of 3GPP standards, however.

#### 6.1.2.5	Solution variant: End to Middle Key Server

Solution #2 uses an End to Middle Key Server (EMKS) (see clauses 6.2.2.1, 6.2.2.2), which sits logically between the HLR/HSS and what is called the EMSE (the equivalent of the HSE in this solution). This can be added as a variant to any of the solution #1 options described above. With this variant, there is no difference to how keys are derived for the visited network, but the e2m key is done differently, in two steps:

1.	Whenever an Authentication Information Request from the SGSN or the MME arrives at the HLR/HSS, then the HLR/HSS checks the subscription profile for the need to derive an e2m_int_key. If needed according to the subscription profile, the HLR/HSS derives e2m_int_key from CK, IK and a string pointing to the purpose of the key use, namely e2m security for CIoT, where the derivation of e2m_int_key has the form.

e2m_int_key = KDF (CK||IK, string)

and the string could be set to e.g. "e2m_ CIoT".

2.	The EMKS derives e2m_key from e2m_int_key, using the key derivation rule.

e2mKEYSET = KDF (e2m_int_key, key type 2)

and if necessary also

e2mKEYSET_ID = KDF (e2m_int_key, key type 3)

where key type 2 and key type 3 are as described in clause 6.1.2.4.

In all cases, keys are delivered from HLR/HSS to EMKS; either push or pull can be used from EMKS to HSE, according to the particular solution option.

#### 6.1.2.6	Solution variant: key derivation on the ME (EPS AKA only)

In all of the solution descriptions above, new key derivation operations are required on the USIM. (From a concatenation of CK and IK, the USIM derives one or more of CK´/IK´, e2mKEYSET, e2mKEYSET_ID, according to the particular solution variant. An AMF bit is used to indicate whether these additional key derivations are required.)

In the case of EPS AKA, new key derivation is only required to produce e2mKEYSET and (in one variant) e2mKEYSET_ID. Rather than doing this on the USIM, an alternative is for the new key derivation options to be performed by the ME. In this case the USIM does nothing new – it just generates CK and IK as usual. Key derivation operations are illustrated in Figure 6.1.2.6-1.

Figure 6.1.2.6-1: Key derivation

Use of the AMF bit is no longer necessary if this variant is adopted.

### 6.1.3	Solution evaluation

All of the variants in this clause create an e2m keyset, shared between UE and HSE, that can be used to provide confidentiality (where permitted) and integrity. This contributes towards satisfying key issue #1. However, this solution does not on its own specify what confidentiality and integrity mechanism (e.g. TLS, IPsec) makes use of the shared keyset.

Key issue 2 is satisfied well as long as keys are relatively long lived (both device-to-visited-network keys and e2m keys), so that authentication challenges and responses on the air interface are infrequent.

The solution requires new functionality in the HSS. It also requires new functionality in the USIM, except in the solution variant "key derivation on the ME (EPS AKA only)" in clause 6.1.2.C.

It seems necessary to keep the visited network and e2m security associations synchronised: if keys are updated for one, then they need to be updated for the other too.

Variant 6.1.2.2 requires the HLR/HSS to retain an e2m key pair until a notification message is received from the SGSN/MME. This type of stateful behaviour is not standard in today's HLRs. Adopting the End to Middle Key Server solution variant (clause 6.1.2.5) avoids this problem.

## 6.2	Solution #2: "End-to-middle security based on AKA"

### 6.2.1	Introduction

The present solution builds on solution#1. It is presented as a delta description to solution#1, i.e. the description of the present solution contains only those elements that differ from solution#1.

Solution#1 is explicitly meant for "mobile network with a Cellular Internet-of-Things RAT". The present solution can be used for CIoT as well (which is why it is included in the present document). It should be noted, though, that there is no technical reason why the present solution should be restricted to the CIoT use case; it can provide e2m security whenever AKA is run in a mobile network.

While solution#1terminates the additional user plane protection in a HPLMN Security Endpoint (HSE) the present solution is applicable to any endpoint, called "E2M Security Endpoint" (EMSE), that is authorised to obtain e2m keys from the E2M Key Server (EMKS). The EMKS derives an e2m key from an intermediate key (e2m_int_key) the EMKS obtains from the HSS.

An EMSE may reside in an operator's home network, but also in a trusted third party domain (e.g. a CIoT application provider platform). The EMKS always resides in an operator's home network.

There may be multiple instances of an EMSE, with which a UE engages simultaneously or sequentially. There is typically only one instance of an EMKS per HSS.

As usual in 3GPP standardisation, EMSE and EMKS denote functional entities that could be implemented stand-alone or combined with HSS or GGSN or P-GW. EMSE and EMKS could also be combined. The advantage of introducing the EMKS is that the HSS could continue to operate in a stateless fashion.

### 6.2.2	Solution description

#### 6.2.2.1	End-to-middle security solution based on AKA

Transparency for core network nodes: 
SGSNs, GGSNs, MMEs, S-GWs, P-GWs may work with the present solution without any CIoT-specific enhancements. This is not to say that such enhancements would not be useful for certain use cases, but it means that there is no mandatory pre-condition on the UMTS or EPS core network elements. This transparency of the solution for the core network is important for deployments as many operators may be reluctant to upgrade their core networks for the purpose of supporting CIoT. (To be sure, the HSS and HLR would need CIoT-specific enhancements.)

Transparency for the USIM: 
The possibility to re-use USIMs without CIoT-specific additions is important as particular CIoT use cases may not warrant the modification of USIMs for their purposes. The present solution can re-use existing USIMs. CIoT specific operations can be done in the ME.

In contrast to the present solution, solution#1 has the following text: "USIM … performs the following additional key derivations: - CK´/IK´…" and again "The key derivation function should be supported by the UICC…". These key derivations on the USIM are CIoT-specific. (This does not apply to the solution variant "key derivation on the ME (EPS AKA only)" in clause 6.1.2.6.)

Push vs. pull procedures

The present solution can work with any of the push or pull procedure variants from solution#1 that are used to transport the e2m key from the EMKS to the EMSE. For transporting the intermediate key e2m_int_key from the HSS to the EMKS, only a push procedure is used.

#### 6.2.2.2	Key derivation rules

As for solution#1, the present solution uses a new key pair CK´, IK´for UMTS PS or GPRS access security between UE and SGSN. For EPS, KASME can be re-used as defined today.

The present solution differs from solution#1 in the derivation of the additional user plane key. In solution#1, this key is called "e2mKEYSET". It is used between UE and HSE. In the present solution, this key is called "e2m_key". It is used between UE and EMSE.

The e2m key is derived in two steps:

(1)	Whenever an Authentication Information Request from the SGSN or the MME arrives at the HSS, then the HSS checks the subscription profile for the need to derive an e2m_int_key. The HSS then generates authentication vectors and sends them back to the SGSN or MME. If needed according to the subscription profile, the HSS derives e2m_int_key from CK, IK using a specific FC value, where the derivation of e2m_int_key has the form

e2m_int_key = KDF (CK||IK,)

. The HSS sends e2m_int_key to the EMKS.

(2)	The EMKS derives e2m_key from e2m_int_key. As there may be multiple instances of EMSE it becomes necessary to include an identifier of the EMSE in the key derivation in order to achieve key separation between EMSEs. The following is the key derivation rule:

e2m_key = KDF (e2m_int_key, EMSE_Id, string)

NOTE: A specific FC value is allocated for the purpose of the derivations of the keys in step 1) and step 2).

#### 6.2.2.3	Usage of e2m security

Solution#1 (except for the solution variant "key derivation on the ME (EPS AKA only)" in clause 6.1.2.C) mentions the use of a, yet to be specified, bit in the Authentication Management Field (AMF) to indicate to the UE whether the UE has to derive the keys CK´ and IK´ for access security. The information on whether this specific AMF-bit is to be set or not needs to be fed into the Authentication Centre. How this information is stored and fed into the AuC does not need to be standardised from an interoperability point of view. However, it may be beneficial to do so for HSS or HLR vendors as standardisation would reduce the number of implementation variants requested by customers.

As the information on whether this specific AMF-bit is to be set is subscriber-specific it should be stored in the subscriber records. However, it is not part of the subscriber profile that is sent to the serving node.

NOTE: 	The use of an AMF bit, as described above, is still open. If used it would imply that there are two kinds of AKA runs: (1) AMF bit set: access security keys for use between UE and SGSN and e2m keys for use between UE and EMSE are derived; (2) AMF bit not set: only access keys are derived. The third case, namely an AKA run that establishes ONLY e2m keys, should not arise as it would necessitate significant changes to the security as defined today between UE and SGSN: the SGSN would have to support running an AKA without deriving access keys, and, in particular, the SGSN would have to interpret the AMF bit. Cf., however, solution#3 where a different approach is taken to decouple AKA runs for access security from AKA runs for e2m security.

### 6.2.3	Solution evaluation

Solution 2 generalises solution 1. Solution 2 enables using the simplified key agreement mechanism for IoT purposes not only for IoT servers (EMSEs) residing in the operator's home network, but also for EMSEs residing in third party networks, if so desired. Furthermore, solution 2 leaves the possibility open to provide an end-to-middle key that was agreed according to the procedures in this clause to any application layer security protocol based on shared keys. Finally, solution 2 is not limited to protocols and architectures that require GGSN support for the distribution and use of the end-to-middle key.

## 6.3	Solution #3: "Independent VPLMN and e2m security associations"

### 6.3.1	Introduction

Solutions #1 and #2 use a single AKA run, with additional subsequent key derivations, to derive both e2m keys and keys for the usual UE-to-VPLMN security. This achieves efficiency, but comes with some challenges in terms of maintaining the synchronisation between the e2m and UE-to-VPLMN security associations.

The present solution, by contrast, uses separate, independent AKA runs for the e2m and UE-to-VPLMN key derivations. This simplifies some issues, at the cost of more AKA runs (with the consequent battery drain implications).

This solution can be used with a HPLMN Security Endpoint (HSE), as described in solution #1, or with a more general e2m Security Endpoint (EMSE) as described in solution #2. The following text refers to the HSE, but an EMSE could be used instead.

### 6.3.2	Solution description

#### 6.3.2.1	Independent VPLMN and e2m security associations

The AKA operations, key derivations and overall security flow for the "traditional" UE-to-VPLMN security are unchanged in this solution.

A new domain is defined for e2m security– neither Circuit Switched (CS) nor the usual Packet Switched (PS) but Home Packet Switched (HPS). The USIM maintains a separate HPS key set, with associated CKSN and KSI (in the same way that USIM already maintains separate CS and PS keysets).

Whenever the GGSN/PGW gets a Create Session Request from the S-GW it starts an AKA run. The P-GW is informed about the fact that the requesting UE is a CIoT UE through either:

-	The use of a PCRF. The PCRF informs the P-GW during the IP CAN session setup that this particular UE is a CIoT UE. The P-GW obtains the IMSI either through the PCRF IP CAN Session Setup message or through the Create Session Request message that it got from the S-GW. In either way, the IMSI is forwarded to the HSE to obtain the AV.

-	By means of the subscription profile stored in the HSS. The S-GW or the P-GW could obtain the subscription profile. If it is possible for the S-GW to obtain the subscription profile, it could inform the P-GW that this particular UE is a CIoT UE. In case the S-GW does not obtain the subscription profile the P-GW could request the subscription profile itself and find out that this UE is a CIoT UE. The IMSI is obtained through the Create Session Request and forwarded to the HSE.

Subsequent AKA runs piggy back on the same mechanism. Whenever a new session between the P-GW and CIoT UE is established, the HSE is triggered, requests a fresh AV and performs an AKA run. The sessions expire according to a policy set in the subscription profile. The term session refers to the data bearer between P-GW and CIoT. This implies that the UE keeps the CIoT security context in memory whenever the UE goes to sleep / idle mode.

The HSE requests an AKA vector from the HLR/HSS. Note: if the HLR/HSS uses the array scheme for SQN management (see TS 33.102 [27] clauses C.1.2 and also clause C.3.4) then it may use a set of IND values exclusively for the HPS domain. In the EPS AKA case, where the AV will contain KASME, the PLMN ID used as input for the derivation of KASME should refer to the HPLMN, but should be different from the PLMN_ID that would be used for that network in regular (non-e2m) KASME derivation.

The HSE derives e2mKEYSET from CK/IK (UMTS AKA case) or from KASME (EPS AKA case) using the key derivation KDF defined in 3GPP TS 33.220 [20]:

-	e2mKEYSET = KDF (CK||IK, key type).

NOTE: 	key type is an ASCII string specific to this key derivation operation, e.g. "e2m_CIoT". If e2mKEYSET includes more than 256 bits of key material, then multiple instances of KDF are used with different key type strings (key type A, key type B, etc.).

The HSE formulates an authentication challenge message, containing all the same AV fields as a standard authentication challenge, which it delivers to the GGSN/P-GW. The GGSN/P-GW sends this challenge message to the UE, via the PCO channel.

The USIM runs UMTS/EPS AKA as normal. The USIM also derives e2mKEYSET, following the same derivation rule as the HSE.

The UE sends the authentication response to the GGSN/P-GW via the PCO channel. The GGSN/P-GW passes this response to the HSE, which validates it against the expected response. Assuming that the authentication is successful, e2mKEYSET is now shared between the HSE and the UE, ready to be used to provide confidentiality and/or integrity protection to the N-PDUs.

### 6.3.3	Solution evaluation

This solution creates an e2m keyset, shared between UE and HSE, that can be used to provide confidentiality (where permitted) and integrity. This contributes towards satisfying key issue #1. However, this solution does not on its own specify what confidentiality and integrity mechanism (e.g. TLS, IPsec) makes use of the shared keyset.

Key issue 2 is satisfied well as long as keys are relatively long lived (both device-to-visited-network keys and e2m keys), so that authentication challenges and responses are infrequent (both the usual challenges from the VPLMN and the e2m challenges delivered over the PCO channel). The solution is not optimised to the same extent as solutions #1 or #2, because provisioning the two sets of shared keys requires two authentication challenges and responses, not one. However, there is a gain in simplicity, because there is no need to keep the two security associations in sync – either can be updated at any time with no impact on the other.

This solution has no impact on the HLR/HSS, unlike solutions #1 and #2.

## 6.4	Solution #4: "Security policies"

### 6.4.1	Introduction

This clause includes some security policy requirements and/or preferences that can be sent from the home network to the visited network, and applied in the visited network. These may be used irrespective of which key sharing solution is preferred from previous clauses. They may also be applied in a broader context – they are not specific to battery constrained use cases or to IoT use cases.

### 6.4.2	Solution description

#### 6.4.2.1	Authentication and key usage policy

The battery lifetime of CIoT devices could benefit from a reduced authentication frequency and, hence, a prolonged lifetime of keys. The visited network may not know about the need for a CIoT-specific authentication and key usage policy. It needs to be told by the home network.

On the other hand, the home network may know more about the security requirements applying to individual devices, and may know that there should be a certain minimum frequency at which visited network keys are changed. In solutions #1 and #2, where visited network and e2m keys are in effect derived simultaneously, this could also be a mechanism for the home network to ensure / request that e2m keys are changed with a certain minimum frequency.

It is proposed to include a new field in the GPRS, 3G or 4G subscriber profiles that determines the authentication and key usage policy required for this subscriber. Including this information in the subscriber profile has two benefits:

-	The specifications of the interfaces between serving node and HLR or HSS need not be modified as subscriber profiles can be carried over these interfaces today.

-	The solution can remain transparent for core network nodes as an SGSN or an MME that does not understand the new field in the subscriber profile simply ignores it.

#### 6.4.2.2	Algorithm policy

It is proposed to include a new field in the GPRS, 3G or 4G subscriber profiles that specifies which cryptographic algorithms are allowed to be used for radio interface security for this subscriber (including both AS and NAS in the 4G case).

Including this information in the subscriber profile has the same benefits as for the previous point. This may be particularly beneficial for IoT devices that have very long lifetimes, where there is a higher risk of weaknesses appearing in an algorithm during that lifetime.

Conversely, if the visited network is able to indicate to the home network what security features / algorithms the visited network will use, the home network may use this in determining its own e2m security policy.

#### 6.4.2.3	VPLMN Specific Algorithm policies

Different Visited PLMNs will have different requirements with respect to whether end-to-end or end-to-middle encryption is allowed from a Lawful Interception perspective. These LI requirements may depend on the type of subscription and the type of device used. To cater for these requirements, it is proposed to include a new field into the GPRS, 3G or 4G subscriber profile that specifies to which class of devices the CIoT UE belongs. What algorithms are allowed might also be subject to regulations or Lawful Interception.

Editor's Note: The expertise of SA3-LI is required to determine whether LI requirements in the VPLMN may indeed depend on the type of subscription and the type of device used, and whether information obtained from the HPLMN would be useful for the VPLMN in fulfilling its LI requirements. The usefulness of the proposed field in the subscriber profile depends on the answers from SA3-LI. It is ffs how the SGSN could learn about the type of device used as the subscriber profile contains only subscription-related information.

Additionally, roaming contracts should specify for which classes of CIoT UEs which encryption algorithms are allowed. During an AKA run, these policies will have to be enforced by the home network.

### 6.4.3	Solution evaluation

The authentication and key usage policy can help with key issue #2, ensuring that authentication challenges and responses are not too frequent.

The algorithm policy does not address the specific security requirements identified in clause 5.1.3, but can help to ensure that the visited network security is as good as possible, reducing some of the tampering and eavesdropping risks identified in clause 5.1.

The VPLMN Specific Algorithm Policies addresses key issue 5.3 if applied to the solutions that provide end-to-end and end-to-middle security.

## 6.5	Solution #5: "End-to-end security solution"

### 6.5.1	Introduction

The present solution builds on solutions #3 by reusing components and mechanisms already introduced in that solution. Solutions #3 was initially meant to be used for end-to-middle, but parts can be used with only minor modifications for end-to-end security. This clause describes how an end-to-end solution is constructed based on the components from solution #3.

### 6.5.2	Solution Description

#### 6.5.2.1	Specific e2e security association

This clause of the solution is based on solution #3 where it is proposed to have two separate key runs, one for the UE-to-VLPMN and one for the CIoT purpose. In this solution, it is proposed to extend this feature to have an additional set of keys for UE-to-Enterprise security.

In solution #3 an e2mKEYSET is derived and used in an HSE / EMSE to terminate the secure connection. In this solution, a new security end point node in the network of the enterprise is defined. The new node inside the enterprise network will be called EESE (End-to-End Security Endpoint). An EESE is basically an EMSE that is located in the enterprise network or even integrated with the CIoT application platform. The EESE may have interfaces that the EMSE does not have.

As described in solution #3, the UE and HSS derive an e2mKEYSET in a separate AKA run for a newly created HPS domain. In this solution, it is proposed to add another domain, the EnPS (Enterprise Packed Switched) domain and derive a key specifically for this domain. The HSS can distinguish between the two domains based on a new field in the subscriber profile that indicates whether for a CIoT UE the EnPS domain is applicable. It is proposed to derive the key as follows:

e2e_int_key = KDF(CK||IK, string),

where the string can be set to "CIoT End-to-end". The HSS forwards the e2e_int_key to the EMKS inside the operator's network and the EESE can obtain the e2e_int_key from the EMKS. Contrary to solution #3, the e2e_int_key will be used to derive further e2eKEYSETs which are the keys used for securing the data transfer between EESE and UE.

#### 6.5.2.2	Derivation of e2eKEYSET

The e2eKEYSET is derived according to:

e2eKEYSET = KDF(e2e_int_key, RAND),

where a RAND is introduced to ensure that derivation of the e2eKEYSET is unpredictable.

It is proposed that the EESE is the source of the RAND and to let the EESE transport the RAND to the UE. Alternatively, in case the EESE might not have a reliable random source available, the operator could provide the random to the EESE via the EMKS. Furthermore, the EESE derives the e2eKEYSET based on the RAND that is optionally provided by the EMKS.

#### 6.5.2.3	Triggering the key derivation

In order to ensure that both the UE and the EESE have the same keys and do the key derivation at the same time, it is necessary to trigger the key derivation. In this solution, the key derivation trigger is exchanged over the existing connection as a control message. What happens is as follows:

1. 	Upon deriving a e2eKEYSET a timer te2e is set in the EESE.

2.	Whenever the timer runs out, the EESE sets a flag invalidating the keys.

3.	Upon next contact from the UE, the EESE replies with an error message indicating the reason (key has expired). This step is skipped whenever the EESE triggers new contact.

4.	The EESE generates a fresh RAND (or triggers the EMKS that a new RAND is needed), derives the new e2eKEYSET and forwards the RAND to the UE, which derives the new e2eKEYSET.

5.	The UE or EESE transmit the message.

#### 6.5.2.4	Setting the timer

The value of the time te2e may be specified by the enterprise. The EESE may also support a local command for the enterprise to expire e2eKEYSET immediately – this is outside the scope of the standard.

#### 6.5.2.5	Interfaces of the EESE

The EESE may have a number of interfaces:

-	An interface between the EESE and the EMKS for the exchange of the e2e_int_key.

-	An interface to receive a RAND from e.g. the EMKS.

-	An interface that allows the CIoT operator platform to let the enterprise expire the keys immediately, which is out of scope of the standard.

### 6.5.3	Solution Evaluation

This solution fulfils the requirement of an end-to-end security mechanism between the Enterprise and the UE, thereby fulfilling the requirement 5.4. And because it builds upon solutions #3, it also fulfils the requirement #1 by deriving a key that can be used for encrypting data between the UE and the EESE and it can fulfils requirement #2 by picking the right KDF.

## 6.6	Solution #6: Bearer protection

### 6.6.1	Introduction

The present solution builds on the previous AKA key derivation solutions, such as solution #1 and #2, clauses 6.1 and 6.2 respectively. It is presented as a delta, i.e. the description of the present solution contains only elements in addition to those solutions. The solution proposes IPsec ESP for user plane protection utilizing the AKA key derivation solutions documented in those clauses.

### 6.6.2	Solution description

In order to protect the user plane data between the UE and the HSE it is required to implement IPsec ESP according to RFC 4303 [36] as profiled by TS 33.210 [37], with confidentiality, integrity and replay protection. The e2m key pairs shared between the UE and the HSE are derived as described in solutions #1 and #2 of the document. The keys are used for encryption and integrity protection by the e2m IPsec ESP Security Association (SA).  There are two options for lifecycle management of the SA, such as parameter negotiations and rekeying.  One option is to preconfigure necessary parameters and handle SA lifecycle via CIoT specific methods.  Another option is to use IKEv2[refr x].  See discussions on these two options below.

NOTE 1: 	The choice of crypto suites is not restricted further from the ones in TS 33.210[37], but due to computational efficiency, the use of AEAD algorithms combining authentication and encryption is preferred.

NOTE 2:	 The solution does not mandate the use of tunnel versus transport-mode ESP.

NOTE 3:	While header compression is expected to further improve efficiency of the solution, this contribution neither mandates it nor puts forward the use of specific compression algorithms.

Pre-configured ESP SA parameters and SA management

ESP SA parameters may be pre-configured and derived for simple IPsec tunnels without run-time overhead.  The e2m key pairs derived by the UE and HSS are directly used for the crypto processing of the SA.  There are obvious drawbacks to pre-configured SAs, such as:  there is no way to negotiate integrity checking and encryption algorithms, Traffic Selectors, NAT-T and to communicate the SPI.   These parameters have to be either pre-configured or derived.  Furthermore, there is no possibility of anti-replay protection, Dead Peer Detection (DPD), DiffServ Code Point (DSCP) mapping, re-keying, and Perfect Forwarding Secrecy (PFS).

NOTE 4:  The method of deriving per-SA parameters that cannot be preconfigured for each SA will be need to be described in the TS.

NOTE 5: ESP SA rekey may be triggered via the signaling message mechanism described in 6.5.2.3.

IKEv2 negotiated ESP SA parameters and SA management

Using the rich feature set of IKEv2 to negotiate the SA parameters fixes the above shortcomings.  All the SA parameters can be discovered and negotiated in two message exchanges.  IKEv2 may be also utilized for SA rekey and teardown.  Supporting IKEv2 with PSK is already required by TS 33.210 [37].  The e2m key pairs derived by the UE and HSS are used as input to the standard SA key generation process of IKEv2 [44].

### 6.6.3	Solution evaluation

The current solution satisfies key issue #1 and its requirements, by providing IPsec-based integrity and confidentiality protection against tampering and eavesdropping.

By pointing to the standard list of crypto suites in TS 33.210[37], the solution allows for the use of crypto algorithms that conform with regional and visited network requirements, thus satisfies the requirement of key issue 3.

## 6.7	Solution #X: "End-to-end" for solutions 1 and 2

### 6.7.1	Introduction

Each of the solutions #1 and #2 can be used for end-to-end encryption by placing the HSE or EMSE in the CIoT application domain (as is mentioned as well in solution #2). Even though such a solution may satisfy some clients needs, none of the solutions will satisfy key issue #4. This clause, is a solution that works on top of solution #1 and #2.

### 6.7.2	Solution #1 and #2 in End-to-End case

The solution #1 and #2 can be adapted to do both end-to-end and fulfil key issue #4 by deriving another key from e2mKEYSET (sol. #1) and e2m_key (sol. #2), namely the e2eKEYSET according to clause 6.5.2.2. In this case, the HSE (sol. #1) or EMSE (sol. #2) can be the source of the RAND and transport the RAND to the UE, but the HSE/EMSE can also obtain the RAND from an external or trusted source such as the EMKS or HSS.

The UE could know that it should derive an additional e2eKEYSET by receiving a signalling message that is replay, confidentiality and integrity protected form the HSE/EMSE that it requires e2e instead of e2m that includes a RAND. The HSE knows that it should apply e2e from a configuration setting that tells it to derive e2e keys.

Additionally, keys can be refreshed independently by using the mechanism described in clause 6.5.2.3. For this purpose the same signalling message that instructs the UE to derive a new key can be reused.

In order to satisfy key issue #4, the HSE/EMSE may be equipped with an interface that the CIoT application provider can use to initiate a key derivation. However, such an interface may be out of scope of the standardisation.

### 6.7.3	Solution Evaluation

This solution shows how using solutions #1 and #2 it is possible to also fulfil requirement #4. Compared to solution #5, this solution is slightly less flexible since the end-to-end keys are always refreshed whenever a new AKA run is necessary.

## 6.8	Solution #8: Complete end to middle solution

### 6.8.1	Introduction

This solution is a complete end to middle solution that includes the following features that minimise the security overhead whilst delivering all of the key objectives and addressing all of the key issues:

-	Optimisation of the transmitted data to reduce battery drain – The signalling and user plane headers between the UE and the eNodeB have been optimised so as to significantly reduce both the amount of header data added to every message and to reduce the processing required to generate a new message.

-	Optimisation of the key agreement - The number of authentication messages to setup and synchronise keys has been reduced significantly compared to a standard E2E security solution.  The ability for keys to survive power cycles and location / network changes, acts to reduce the security overhead per message and therefore the battery drain.  The ability to reuse the 3G authentication mechanisms significantly reduces the signalling required for key agreement.

-	Re-use of existing signalling – The use of the protocol configuration options element that it is passed transparently from the UE to the HSE via the existing LTE architecture for the initial signalling setup allows the overhead for this signalling to be minimised and allows the protocols specified here to be transparent to the visited network that does not know about the BEST service.  For a network that is aware of the BEST service, this also allows the visited network to send BEST service restriction requests to the home network to meet LI requirements.

-	Use of tunnelled bearer to reduce message overheads – This solution uses newly specified SGi PtP tunnelling based on UDP/IP bearer to reduce the transmitted data size.  With the SGi PtP tunnelling based on UDP/IP bearer the IP/UDP headers are created in the PGW and not by the UE saving a significant number of transmitted bytes.

The solution specifically addresses:

Service discovery.

Enabling and disabling the service.

Negotiation of parameters.

Key agreement and re-keying.

Starting a session.

Suspending and resuming a session.

Ending a session.

Consequences of network changes and errors.

The solution proposed includes the setup of the E2M solution at the request of the UE and at the request of the external data network.

### 6.8.2	Solution description

#### 6.8.2.1	Proposed Architecture

Figure 6.5.2.5.1 Proposed Architecture for Solution #8

The two new network elements proposed are:

-	Home Security Environment (HSE) – this is the endpoint for the secure communications in the home network.  It is expected that both the user plane data and the signalling data relevant to the BEST service will terminate here.  In line with the LTE architecture, the HSE may be split into a signalling part and a user data part.  The HSE has interfaces to the PDN gateway (UDP/IP and Diameter over SGi), the enterprise server (out of scope for 3GPP) and either the HSS (S6x) or the EMKS (new 2).

-	End to Middle Key Server (EMKS) – This is an optional key server element that manages the key communication with the HSS (for quintets)  and stores keys to reduce loading on the HSE and HSS.  Note: This is the same element described in solution #2.  The EMKS has interfaces to the HSS (S6y) and the HSE (new 2).

The four new network interfaces proposed are:

-	 S6x – An interface between the HSE and the HSS to allow the HSE to obtain subscription data and authentication vectors.

-	New 1 – An interface between the HSE and the Enterprise Server that allows the Enterprise Server to manage aspects of the BEST service and allows the transportation of user data to and from the Enterprise Server.

-	New 2 – An interface between the HSE and the EMSK that allows the HSE to retrieve authentication vectors for a user's subscription.

-	S6y – An interface between the EMSK and the HSS that allows the EMSK to retrieve a block of authentication vectors for later use by the HSE.

The SGi interface between the PDN Gateway and the HSE has 2 connections per subscriber.  A Diameter connection for "service discovery and negotiation" and a UDP/IP connection for the session signalling and user plane data.

#### 6.8.2.2	Service Discovery and Negotiation

This clause defines the BEST service discovery and initial BEST service definition.  This is run when the UE wants to use the BEST service and there is no BEST session active or as part of the power-on sequence.

In this solution, the BEST service only needs to be supported by the UE, the MME and the HSE in the home network to operate.  The visited network may have a requirement to influence the use of the BEST service to comply with local laws such as Lawful Interception.

Figure 6.8.2.2.1 shows the message flow for service discovery for LTE

Note: 	non relevant messages have been omitted for clarity.

The message flow for GSM and 3G is similar.

UE

Figure 6.8.2.2.1: Flow diagram showing the BEST service discovery message flow for LTE

The ability of the UE to support the BEST service shall be indicated in a BEST container within the protocol configuration options of a PDN_CONNECTIVITY_REQUEST.

The following BEST service aspects shall be indicated in the protocol configuration options as detailed in figure 8.6.2.2.2 (based on the protocol block format for the protocol configuration options IE in 3GPP TS 24.008 [12]):

| Bit 8 | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Upper byte of PCO protocol ID = UE BEST |  |  |  |  |  |  |  |
| Lower byte of PCO protocol ID = UE BEST |  |  |  |  |  |  |  |
| Length of Best protocol ID contents |  |  |  |  |  |  |  |
| BEST release supported by the UE |  |  |  | BEST counter schemes supported by the UE - BEST optimised counter scheme | Reserved for future use (set to 000) |  |  |
| BEST protocols supported - type 01 signalling plane EMSDP message | Reserved for future use (set to 000) |  |  | BEST protocols supported - type 01 user plane EMSDP message | Reserved for future use (set to 000) |  |  |
| Support for user data push to UE | BEST encryption algorithm GEA4 supported | BEST encryption algorithm GEA5 supported | BEST encryption algorithm UEA1 supported | BEST encryption algorithm UEA2 supported | BEST encryption algorithm EEA0 supported | BEST encryption algorithm 128-EEA1 supported | BEST encryption algorithm 128-EEA2 supported |
| BEST encryption algorithm 128-EEA3 supported | BEST integrity algorithm GIA4 supported | BEST integrity algorithm GIA5 supported | BEST integrity algorithm UIA1 supported | BEST integrity algorithm UIA2 supported | BEST integrity algorithm 128-EIA1 supported | BEST integrity algorithm 128-EIA2 supported | BEST integrity algorithm 128-EIA3 supported |

Figure 6.8.2.2.2: Definition of the UE BEST protocol ID element

-	BEST release supported by the UE – an indicator the release of the BEST solution that the UE has been designed to.  If no release is indicated this means that the BEST service is not supported.  This shall be a 4 bit field where "0000" = Rel.14 and "0001" to "1111" are RFU,

-	BEST protocols supported – 1 octet where each bit is used to indicate a BEST signalling protocol that is supported from the following list: type 01 signalling plane EMSDP message and type 01 user plane EMSDP message.  All other values are reserved for future use.

-	BEST encryption algorithms supported by the UE – a flag for each supported algorithm.  A further optimisation for this would be to assume that all algorithms indicated in the UE network capability information element are supported for the BEST service (currently GEA4, GEA5, UEA1, UEA2, EEA0, 128-EEA1, 128-EEA2 and 128-EEA3).

-	BEST integrity algorithms supported by the UE – a flag for each supported algorithm.  A further optimisation for this would be to assume that all algorithms indicated in the UE network capability information element are supported for the BEST service (currently GIA4, GIA5, UIA1, UIA2, 128-EIA1, 128-EIA2 and 128-EIA3).

-	BEST counter schemes supported by the UE – a flag for each supported scheme.  Currently only one scheme is being proposed, the optimised counter scheme, but other optimisations may be proposed in the future.

-	Support for user data push to UE – a flag to indicate whether the UE supports user data pushed to the UE without a request for user data from the UE.

-	Power capabilities of the UE.  This allows the home network and visited network to optimise their key agreement and signalling layers to take into account the power available.

In order to support regulatory requirements in certain countries (e.g. LI or where encryption is prohibited) then an MME that supports the BEST service may inspect the PCO provided by the UE in the PDN_CONNECTIVITY_REQUEST.  The MME may add a VISITED NETWORK BEST INDICATION container to the PCO with the following information:

-	BEST encryption Restricted – A flag that indicates that encryption is not allowed through this visited network.

The HSE defines how the BEST service shall be setup in a BEST container within the protocol configuration options of a ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST.

The following BEST service aspects shall be indicated in the protocol configuration options as detailed in figure 8.6.2.2.3 (based on the protocol block format for the protocol configuration options IE in 3GPP TS 24.008 [12]):

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Upper byte of PCO protocol ID = HSE BEST |  |  |  |  |  |  |  |
| Lower byte of PCO protocol ID = HSE BEST |  |  |  |  |  |  |  |
| Length of Best protocol ID contents |  |  |  |  |  |  |  |
| BEST Service Activated | BEST encryption algorithm GEA4 to be used | BEST encryption algorithm GEA5 to be used | BEST encryption algorithm UEA1 to be used | BEST encryption algorithm UEA2 to be used | BEST encryption algorithm EEA0 to be used | BEST encryption algorithm 128-EEA1 to be used | BEST encryption algorithm 128-EEA2 to be used |
| BEST signalling plane protocol identifier |  |  |  |  |  |  |  |
| BEST user plane protocol identifier |  |  |  |  |  |  |  |
| BEST encryption algorithm 128-EEA3 to be used | BEST integrity algorithm GIA4 to be used | BEST integrity algorithm GIA5 to be used | BEST integrity algorithm UIA1 to be used | BEST integrity algorithm UIA2 to be used | BEST integrity algorithm 128-EIA1 to be used | BEST integrity algorithm 128-EIA2 to be used | BEST integrity algorithm 128-EIA3 to be used |
| Reserved for future use (set to 000) |  |  | Local BEST configuration management allowed | EMSDP session ID length |  |  |  |
| Reserved for future use (set to 00) | New session required | EMSDP MAC length |  | Size of EMSDP Data Length |  |  |  |

Figure 6.8.2.2.3: Definition of the HSE BEST protocol ID element

-	BEST Service Activated – a bit flag that when set instructs the UE to use the BEST service and when clear instructs the UE not to use the BEST service,

-	BEST signalling plane protocol identifier – 1 octet that is used to determine the BEST signalling protocol to be used from the following list (only one shall be indicated): 01 = type 01 signalling plane EMSDP message. All other values are reserved for future use.

-	BEST user plane protocol identifier – 1 octet that is used to determine the BEST signalling protocol to be used from the following list (only one shall be indicated): 01 = type 01 user plane EMSDP message. All other values are reserved for future use.

-	BEST encryption algorithm to be used – 1 octet that is used to define which of the following algorithms to use for encryption: GEA0, GEA4, GEA5, UEA0, UEA1, UEA2, EEA0, 128-EEA1, 128-EEA2 and 128-EEA3. Only one algorithm from this list shall be indicated.  If the Visited network indicated that BEST encryption is restricted, then the HSE shall indicate EEA0.

-	BEST integrity algorithm to be used – 1 octet that is used to define which one of the following algorithms to use for integrity:GIA4, GIA5, UIA1, UIA2, 128-EIA1, 128-EIA2 and 128-EIA3). Only one algorithm from this list shall be indicated.

-	BEST counter scheme to be used – a flag for each supported scheme.  4 bits that are used to define which one of the following counter schemes to use: "0001" = the optimised EMSDP counter scheme. All other values are reserved for future use. Only one counter scheme from this list is allowed to be indicated.

-	Local BEST configuration management allowed – a flag to indicate that the software connected to the UE is allowed to manage the BEST service.

-	EMSDP session ID length – 4 bits that indicates how many octets in the EMSDP data packet the EMSDP session ID will be on.

-	New Session Required – 1 bit that indicates if a new session is required.  If this bit is set to 0 then the details agreed for the last session can be used and a new session is not required to be setup.

-	EMSDP MAC length – 2 bits that indicates how many octets in the EMSDP data packet the integrity checksum (MAC) will be on, as follows: "00"=4 octets, "01"=8 octets, "10"=12 octets and "11"= 16 octets.  This value shall not be set to a size that is greater than MAC size produced by the chosen algorithm.

-	Size of EMSDP Data Length – 4 bits that indicate how many octets are used for the EMSDP Data Length. "0000" is reserved for future use.

Any remaining bits are reserved for future use and are set to "0..0".

In order to ensure that the visited networks encryption restriction is honoured by the HSE, an MME that has indicated BEST encryption is restricted, should inspect the BEST information element in the PCO within the start session response from the PGW and if it indicates an encryption algorithm that is not EEA0, the MME should reject the start session response.  It is the HSE's responsibility to send a corrected start session response.

#### 6.8.2.3	Ability to enable and disable the BEST service

The HSE may allow the enterprise to manage the BEST service.  The service shall allow enable / disable of the BEST service down to an individual UE level. It is anticipated that the interface for the management of the BEST service between the enterprise and the HSE will be out of scope for 3GPP.  However it is expected that such an interface would have the following abilities:

-	a mechanism that allows the service to be removed for a specific subscriber at the network end (probably re-using  a service provisioning service).

-	the ability for the enterprise to enable / disable the E2M security service for one or many of their endpoints through communication with the operators CIoT interface.  (This may be because other security mechanisms are being used).

-	The ability for the home network to disable the BEST service due to non-payment or security concerns.

If the BEST service parameters in the HSE change as a result of the management of the BEST service for a specific UE and the BEST service is not currently active with that UE then the changes shall take effect the next BEST session.

If the BEST service parameters in the HSE change as a result of the management of the BEST service for a specific UE and the BEST service is currently active with that UE then the HSE shall end the current BEST session and where appropriate start a new BEST session using the new BEST service parameters.

The UE may allow the application connected to it to manage the BEST service if the Local BEST configuration management flag within the protocol configuration options of a ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST indicates this is allowed.

If the BEST service parameters in the UE change as a result of the management of the BEST service and the BEST service is not currently active then the changes shall take effect the next BEST session.

If the BEST service parameters in the UE change as a result of the management of the BEST service and the BEST service is currently active in that UE then the UE shall end the current BEST session and where appropriate start a new BEST session using the new BEST service parameters.

The UE may choose not to accept data that does not meet the defined security requirements.

When the BEST service is disabled it is anticipated that the HSE and UE shall act as though the BEST service is not supported.

#### 6.8.2.4	End to Middle Security User Plane and Signalling Plane

##### 6.8.2.4.1	Data transport

This solution uses the SGi PtP tunnelling based on UDP/IP bearer specified in 3GPP TS 23.401[16] to transport both user plane and signalling plane data between the UE and the HSE.  This bearer has been chosen as it significantly reduces the  size of the data header used over the air interface (and hence reduces power consumption) whilst still allowing IP/UDP to be used between the PGW and the HSE.

##### 6.8.2.4.2	End to Middle Secured Data Protocol (EMSDP)

A new data protocol between the UE and the HSE is detailed in this solution, called "End to Middle Secured Data Protocol" (EMSDP),  which has been optimised for efficient, secure transfer of both user plane and signalling plane data.

Figure 6.8.2.4.2.1 shows the data stack for the EMSDP transfers (based on 3GPP TS 23.401 [16]).

Figure 6.8.2.4.2.1: data stack for the EMSDP transfers

The UE selects the non-IP PDN data type as defined in clause 4.3.17.8 of 3GPP TS 23.401[16].  The PDN GW adds the IP and UDP layers for the communication with the HSE.

So that an MME that does not support the BEST service can restrict the use of the BEST service, as part of the normative work, a fixed APN structure will be investigated that allows filtering of the BEST service based on the APN requested by the UE.

##### 6.8.2.4.3	EMSDP general structure

This clause details a type 01 signalling plane EMSDP message and a type 01 user plane EMSDP message.

Figure 6.8.2.4.3.1 shows the general structure of the EMSDP type 01 message:

| Signalling Plane type 01 message: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| UP / SP Flag | Key ID | SP COUNTER | Session ID | EMSDP Command | Cmd Options | MAC |
| 1 bit | 4 bits | Note 4 | Note 1 Note 2 | Note 1 Note 2 Note 3 | Note 2 Note 3 | Note 1 Note 3 |
| User Plane type 01 message: |  |  |  |  |  |  |
| UP / SP Flag | Key ID | UP COUNTER | Session ID | Data Length | Data | MAC |
| 1 bit | 4 bits | Note 4 | Note 1 Note 2 | Note 1 Note 2 Note 3 | Note 2 Note 3 | Note 1 Note 3 |

Note 1: The length of these fields is indicated in the BEST service discovery from the HSE.

Note 2: These fields are included in the integrity protection calculation.

Note 3: These fields are encrypted when encryption is used.  When encrypted these fields are replaced by the encrypted output.

Note 4: The length of this field is as defined in the BEST Counter Scheme being used.

Figure 6.8.2.4.3.1: data stack for the EMSDP transfers

UP / SP Flag: 	This is a 1 bit field that when set means that the data packet is a User Plane message and when clear means that the message is a signalling plane message.

Key ID:	This is a 4 bit field that indicates the key being used for encryption and Integrity protection.

SP COUNTER:	This is a counter used for signalling plain data packets that is incremented every data packet.  It is used to protect data packets against replay and its length is set based on the counter scheme indicated in the BEST service discovery (minimum length is 3 bits).  There are separate counter values for UE to HSE and HSE to UE.

UP COUNTER:	This is a counter used for user plain data packets that is incremented every data packet.  It is used to protect data packets against replay and its length is set based on the counter scheme indicated in the BEST service discovery (minimum length is 3 bits). There are separate counter values for UE to HSE and HSE to UE.

Session ID:	This indicates the identifier for the current session.  The value is assigned by the HSE.  For signalling messages sent outside of a session, the Session ID shall be all 0's.  Its length is the value set in the BEST service discovery.

EMSDP Command:	This is a 1 byte field that contains the signalling command.  The defined commands are detailed below.

Cmd Options:	This is a TLV container that contains TLV elements that detail the options for the EMSDP command. The defined options TLVs are detailed below.

MAC:	This contains the truncated integrity result for this data packet calculated using the agreed integrity algorithm. Its length is set in the BEST service discovery.

Data length:	This holds the length of the following data in this data packet. Its length is set in the BEST service discovery.

Data:	This is the data being transferred.

##### 6.8.2.4.3	EMSDP Counter Schemes

6.8.2.4.3.1	Optimised EMSDP counter scheme

The optimised EMSDP counter scheme has a 3 bit counter length followed by the counter value.  This allows the counter length to be changed on a per message basis.  The 3 bits are used to indicate the number of octets the counter is on, the value "000" is reserved for future use.  So for instance:

Counter value "1" is represented as "001 00000001"

Counter value "257" is represented as "010 00000001 00000001"

The optimised EMSDP counter scheme is defined as scheme 01.

##### 6.8.2.4.4	EMSDP Integrity protection

The integrity protection algorithm to be used and the length of the MAC is indicated in the BEST service discovery.

Integrity protection is mandatory for all signalling plane and user plan data packets except for the following signalling plane commands when no valid keyset is agreed between the UE and HSE:

-	a Start EMSDP session request originating from the UE or the HSE

-	a Start EMSDP session response originating from the HSE

For these commands the MAC shall be set to "0…0".

For all other signalling plane and user plane data packets the MAC shall be computed as follows:

First the following fields are calculated (where needed for the chosen algorithm):

-	INPUT-I	set to the Data Packet Counter Value expanded and right padded with 0's to a fixed size of 4 bytes.

-	COUNT-C	set to the Data Packet Counter Value expanded and right padded with 0's to a fixed size of 4 bytes

-	M (GSM)	set to the length of Message in bytes. It is coded on 2 bytes.

-	LENGTH 	set to the length of Message in bytes. It is coded on 2 bytes.

-	MESSAGE	the fields marked for integrity protection in figure 6.8.2.4.3.1: "data stack for the EMSDP transfers" concatenated in the order they appear in the data packet.

-	M (LTE)	the fields marked for integrity protection in figure 6.8.2.4.3.1: "data stack for the EMSDP transfers" concatenated in the order they appear in the data packet.

-	DIRECTION	The DIRECTION bit shall be "0" for UE to HSE data packets and set to "1" for HSE to UE data packets.

-	BEARER	For signalling data packets this shall be set to "00000" and for user plane data packets this shall be set to "10101"

-	FRAMETYPE	For signalling data packets this shall be set to "00" and for user plane data packets this shall be set to "AA".

-	KI128	This is the agreed integrity key value truncated to the lowest 128 bits.

-	IK	This is the agreed integrity key value truncated to the lowest 128 bits.

If GIA4 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 55.241 [38] shall be used to generate the MAC value.  If the MAC length indicated in the BEST service discovery is less than the length of the MAC produced by the GIA4 function, then the MAC shall be truncated to the correct size from the right.

If GIA5 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 55.251 [39] shall be used to generate the MAC value. If the MAC length indicated in the BEST service discovery is less than the length of the MAC produced by the GIA5 function, then the MAC shall be truncated to the correct size from the right.

If UIA1 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.205 [40] shall be used to generate the MAC-I value.  The MAC shall be set to the MAC-I truncated to the correct size from the right.

If UIA2 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.215 [41] shall be used to generate the MAC-I value.  The MAC shall be set to the MAC-I truncated to the correct size from the right.

If 128-EIA1 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.401 [42] shall be used to generate the MAC-I value.  The MAC shall be set to the MAC-I truncated to the correct size from the right.

If 128-EIA2 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.401 [42] shall be used to generate the MAC-I value.  The MAC shall be set to the MAC-I truncated to the correct size from the right.

If 128-EIA3 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.221 [43] shall be used to generate the MAC-I value.  The MAC shall be set to the calculated MAC-I value truncated to the correct size from the right.

##### 6.8.2.4.5	EMSDP Encryption

The encryption protection algorithm to be used is indicated in the BEST service discovery.  If EEA0 is indicated then the data packet shall not be encrypted.

Encryption is mandatory for all signalling plane and user plan data packets when an encryption algorithm is indicated in the BEST service discovery, except for the following signalling plane commands when no valid keyset is agreed between the UE and HSE:

-	a Start EMSDP session request originating from the UE or the HSE

-	a Start EMSDP session response originating from the HSE

For all other data packets the following encryption shall be applied the fields indicated in figure 6.8.2.4.3.1: "data stack for the EMSDP transfers" to be encrypted.

First the following are computed (where relevant for the algorithm being used):

-	INPUT-I	set to the Data Packet Counter Value expanded and right padded with 0's to a fixed size of 4 bytes.

-	COUNT-C	set to the Data Packet Counter Value expanded and right padded with 0's to a fixed size of 4 bytes

-	M (GSM)	set to the length of Message in bytes. It is coded on 2 bytes.

-	LENGTH 	set to the length of Message in bytes. It is coded on 2 bytes.

-	MESSAGE	the fields marked for encryption protection in figure 6.8.2.4.3.1: "data stack for the EMSDP transfers" concatenated in the order they appear in the data packet.

-	M (LTE)	the fields marked for encryption protection in figure 6.8.2.4.3.1: "data stack for the EMSDP transfers" concatenated in the order they appear in the data packet.

-	DIRECTION	The DIRECTION bit shall be "0" for UE to HSE data packets and set to "1" for HSE to UE data packets.

-	BEARER	For signalling data packets this shall be set to "00000" and for user plane data packets this shall be set to "10101"

-	FRAMETYPE	For signalling data packets this shall be set to "00" and for user plane data packets this shall be set to "AA".

-	KI128	This is the agreed encryption key value truncated to the lowest 128 bits.

-	CK	This is the agreed encryption key value truncated to the lowest 128 bits.

If GEA4 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 55.241 [38] shall be used to generate the OUTPUT value. The OUTPUT value replaces the fields that are encrypted in the data packet.

If GEA5 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 55.251 [39] shall be used to generate the OUTPUT value. The OUTPUT value replaces the fields that are encrypted in the data packet.

If UEA1 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.205 [40] shall be used to generate the OBS value.  The OBS value replaces the fields that are encrypted in the data packet.

If UEA2 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.215 [41] shall be used to generate the OBS value.  The OBS value replaces the fields that are encrypted in the data packet.

If 128-EEA1 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.401 [42] shall be used to generate the OBS value.  The OBS value replaces the fields that are encrypted in the data packet.

If 128-EEA2 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.401 [42] shall be used to generate the OBS value.  The OBS value replaces the fields that are encrypted in the data packet.

If 128-EEA3 is indicated in the HSE BEST protocol ID element, then the algorithm specified in 3GPP TS 35.221 [43] shall be used to generate the OBS value.  The OBS value replaces the fields that are encrypted in the data packet.

##### 6.8.2.4.6	EMSDP Commands

6.8.2.4.6.0	Overview

The following EMSDP commands are proposed Table 6.8.2.4.6.1.1:

Table 6.8.2.4.6.1.1: EMSDP commands

| Code (Hex) | Command |
| --- | --- |
| 10 | Start EMSDP session request |
| 11 | Start new EMSDP session |
| 30 | Terminate EMSDP session request |
| 31 | Terminate EMSDP session response |
| 40 | Manage EMSDP Keys request |
| 41 | Manage EMSDP Keys response |

6.8.2.4.6.1		Start EMSDP session request

The Start EMSDP session request command is used by the UE to trigger a new BEST session from the HSE.

The cmd options for the Start EMSDP session request command are as follows:

Enterprise Setup Information Element TLV: This is a TLV element that contains information from the UE that is used by the HSE to setup the HSE to enterprise connection.

6.8.2.4.6.2		Start new EMSDP session

The Start new EMSDP session response command is used by the HSE to setup a new BEST session.  This message can be sent in response to a start EMSDP session request command from the UE or as an initial message from the HSE.

The Start new EMSDP session response command has the following cmd options:

HSE Identity TLV: this contains a 4 octet numeric identifier for the HSE.  This should be unique to the HSE being used within the home network.

Key agreement TLV: this contains the RAND IE and AUTN IE specified in 3GPP TS 24.008 [12] as detailed in table 6.8.2.4.6.2.1:

Table 6.8.2.4.6.2.1: Key Agreement TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| Key Agreement TLV | 1 byte | M | TBD |
| Length | 1 byte | M | 0 or 35 |
| RAND IE | 17 bytes | C | See 10.5.3.1 in 3GPP TS 24.008 [12] |
| AUTN IE | 18 bytes | C | See 10.5.3.1.1 in 3GPP TS 24.008 [12] |

If the Length of the Key agreement is set to 0 then this means use current keyset.

6.8.2.4.6.3		Terminate EMSDP session request

The Terminate EMSDP session request EMSDP command is used by either the UE or the HSE to terminate an existing EMSDP session.

This command has no cmd options.

6.8.2.4.6.4	Manage EMSDP Keys request

The Manage EMSDP Keys request EMSDP command is used by the HSE to negotiate new key material.

This command has no cmd options.

6.8.2.4.6.5		Manage new EMSDP Keys

The Manage EMSDP Keys response command is used by the HSE to agree new keys.

The Manage EMSDP Keys response command has the following cmd options:

Key agreement TLV: this contains the RAND IE and AUTN IE specified in 3GPP TS 24.008 [12] as detailed in table 6.8.2.4.6.5.1:

Table 6.8.2.4.6.5.1: Key Agreement TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| Key Agreement TLV | 1 byte | M | TBD |
| Length | 1 byte | M | 35 |
| RAND IE | 17 bytes | C | See 10.5.3.1 in 3GPP TS 24.008 [12] |
| AUTN IE | 18 bytes | C | See 10.5.3.1.1 in 3GPP TS 24.008 [12] |

#### 6.8.2.7	Key Agreement and Refreshing

##### 6.8.2.7.1	Overview

There are several key agreement solutions presented in this document that would be capable of agreeing keys between the UE and the HSE.  For this solution it is proposed to use the key agreement similar to the mechanism proposed in solution #3 as it enables complete confidentially between the home network and the UE and it remains unaffected by visited network changes (such as location area changes).

##### 6.8.2.7.2	Key setup messaging between HSE and UE

New keys are agreed either at the start of a BEST session or as required due to key aging or counter thresholds being met.  Key agreement is based on the 3GPP AKA mechanism detailed in TS33.102 [27] and the AKA challenge is transported between the HSE and the UE in any of the following EMSDP commands:

-  Start new EMSDP session EMSDP command from the HSE as part of a new session being setup by the HSE.

-	Manage new EMSDP Keys EMSDP command from the HSE as a result of key aging or counter thresholds being met.

Once in a session, a UE can request new keys by sending a Manage EMSDP Keys request command to the HSE.

Figure 6.8.2.7.2.1 shows the key agreement process

Without EMKS

With EMKS

Figure 6.8.2.7.2.1 BEST key agreement process

The Key agreement steps are:

1.	Start EMSDP session request command or Manage EMSDP Keys request command over the Air interface / core network interfaces. – If the UE wants to initialise the BEST session, it sends a "Start EMSDP session request command" through the SGI PtP tunnel (This is described in detail in clause 6.8.2.8). If the UE is already in a session and wants to renegotiate the keys it sends a or Manage EMSDP Keys request command secured with the current keys to the PDN gateway through the SGI PtP tunnel.

2.	Start EMSDP session request command or Manage EMSDP Keys request command over the SGi interface.  –The "Start EMSDP session request command" or Manage EMSDP Keys request command is forwarded to the HSE using the SGi interface.

3.	Keys required? - the HSE checks to see if there are valid keys with valid counter values available in the HSE for that UE then the following is checked:

-	If the "Start EMSDP session request command" has the correct counter value, is correctly encrypted and is correctly integrity protected then the HSE sends a "Start new EMSDP session command" – use current keyset, to the UE correctly encrypted, integrity protected and with a correct counter to the UE (step 6).

-	If the "Start EMSDP session request command" is not integrity protected or has any of the security parameters wrong (against the information the HSE has) and there are new authentication vectors available in the HSE for that UE, then the HSE goes to step 6 and uses one of the available unused authentication vectors.

-	If the "Start EMSDP session request command" is not integrity protected or has any of the security parameters wrong (against the information the HSE has) and there are no authentication vectors available in the HSE for that UE and the HSE has a direct connection to the HSS, then the HSE goes to step 4.

-	If the "Start EMSDP session request command" is not integrity protected or has any of the security parameters wrong (against the information the HSE has) and there are no authentication vectors available in the HSE for that UE and the HSE has an indirect connection to the HSS via a EMKS, then the HSE goes to step a.

4. Authentication-Information-Request over S6x interface – The HSE uses the S6x interface to the HSS to request one or more authentication vectors using the UE IMSI.  These vectors will be specific to the BEST service.  The IMSI of the UE is the IMSI passed in the accounting request (Start) for that UE when the SGi interface was setup.

5. Authentication-Information-Answer over S6x interface – The HSS uses the S6x interface to the HSE to return the requested authentication vectors.  These vectors are specific to the BEST service.

a. Authentication-Information-Request over the new2 interface – Where an EMKS is used, the HSE uses the new2 interface to the EMKS to request an authentication vector using the UE IMSI.  These vectors will be specific to the BEST service.  The IMSI of the UE is the IMSI passed in the accounting request (Start) for that UE when the SGi interface was setup.

b. Authentication-Information-Request over the S6y interface – The EMKS uses the S6y interface to the HSS to request one or more authentication vectors using the UE IMSI.  These vectors will be specific to the BEST service.

c. Authentication-Information-Answer over the S6y interface – The HSS uses the S6y interface to the EMKS to return the requested authentication vectors.  These vectors may be stored on the EMKS for later use.

d. Authentication-Information-Answer over the new2 interface – The EMKS uses the new2 interface to the HSE to return the requested authentication vector.

6. Start new EMSDP session command over the SGi interface -  The HSE sends a "Start new EMSDP session" command to the PDN Gateway using the SGi interface as follows:

-	If the new keys need to be agreed for the BEST service, then the HSE includes the Key Agreement TLV in the "Start new EMSDP session" command.  The Key Agreement TLV has the RAND and AUTN of the authentication vector used in the RAND IE and AUTN IE.

7. Start new EMSDP session command over the Air interface / core network interface -  The PDN Gateway sends a "Start new EMSDP session" command to the UE using the core network and air interfaces.

8. Calculate UE Keys - On successful reception of the "Start new EMSDP session" command, the UE checks the AKA challenge as detailed in 3GPP TS 33.xxx [x] and if it is valid, calculates the AKA response CK and IK.  Unlike the normal AKA challenge/response, the AKA response in not sent to the HSE as the HSE can determine that this was successful from the MAC value presented in any following message.  The UE calculates the KE2Menc key using the BEST key derivation mechanism, CK, IK, a HSE id and the key id with the Algorithm id set to "BEST encryption Key". The UE calculates the KE2Mint key using the BEST key derivation mechanism, CK, IK, a HSE id and the key id with the Algorithm id set to "BEST integrity Key".  The UE may choose to calculate one or a set of KE2Menc and KE2Mint key values depending on its optimal power strategy.  Each key id refers to a pair of KE2Menc and KE2Mint key values.

9. Calculate HSE Keys -  The HSE calculates the KE2Menc key using the BEST key derivation mechanism, CK, IK, a HSE id and the key id with the Algorithm id set to "BEST encryption Key". The HSE calculates the KE2Mint key using the BEST key derivation mechanism, CK, IK, a HSE id and the key id with the Algorithm id set to "BEST integrity Key".

The pair of KE2Menc and KE2Mint key values for the indicated key id are then used for all further EMSDP messages until either a new key ID is indicated or until the keys are re-negotiated.

##### 6.8.2.7.3	BEST key derivation mechanism

All key derivations (including input parameter encoding) for BEST shall be performed using the key derivation function (KDF) specified in TS 33.220 [20]. This clause specifies how to construct the input string, S, to the KDF (which is input together with the relevant key).

When deriving a KE2Menc or KE2Mint from CK, IK, a HSE id, key id and algorithm id, the following parameters shall be used to form the input S to the KDF.

-	FC = [TBD],

-	P0 = HSE id,

-	L0 = length of HSE id (i.e. 0x00 0x03),

-	P1 = SQN  AK

-	L1 = length of SQN  AK (i.e. 0x00 0x06)

-	P2 = algorithm id

-	L2 = length of algorithm id (i.e. 0x00 0x01)

The exclusive or of the Sequence Number (SQN) and the Anonymity Key (AK) is sent to the UE as a part of the Authentication Token (AUTN), see TS 33.102. If AK is not used, AK shall be treated in accordance with TS 33.102 [27], i.e. as 000…0.

The algorithm id shall be "BEST encryption key" for BEST encryption algorithms and "BEST integrity Key" for BEST integrity protection algorithms. The values 0x03 to 0xf0 are reserved for future use, and the values 0xf1 to 0xff are reserved for private use.

Table 6.8.2.7.3.1: Algorithm type distinguishers

| Algorithm distinguisher | Value |
| --- | --- |
| BEST encryption key (KE2Menc) | 0x01 |
| BEST integrity Key (KE2Mint) | 0x02 |

The input key shall be equal to the concatenation CK || IK of CK and IK.

#### 6.8.2.8	Starting a BEST service session

##### 6.8.2.8.1	UE initiated BEST session

2. Start EMSDP session request command over the SGi interface (UE initialised session).

Figure 6.8.2.8.2.1 UE initiated BEST session startup flow diagram

Once the data session is live (following the BEST service discovery procedure), if there is no current session running and typically when the UE has some data to transfer, the UE checks the key status and the BEST container received from the BEST service discovery procedure and does the following:

- If the BEST service is not activated in the BEST container then the session shall not be started.

- If the UE determines that the key needs to be re-agreed then the UE sends a "Start EMSDP session request" command through the SGI PtP tunnel (This is described in detail in clause 6.8.2.8) with the key id set to "00", Session id set to "00..00", no encryption and the MAC set to "00..00".

- If the UE determines that the key does not need to be re-agreed then the UE sends a "Start EMSDP session request" command with the indicated key and algorithms.  The HSE may trigger a key re-agreement if it chooses to do so.

On reception of the "Start EMSDP session request" command from the UE via the PDN gateway, the HSE checks the security of the message, the UE's subscription, the BEST service status for the location of the UE and the validity of the BEST service keys and carries out one of the following:

- If the HSE determines that the key does not need to be re-agreed then the HSE sends a "Start new EMSDP session" command with the indicated key, encrypted and integrity protected as indicated in the service discovery.  This message has one HSE Identity TLV containing the identity of the HSE and a Key agreement TLV with its length set to 00 and no AUTN IE or RAND IE.

- If the new keys need to be agreed for the BEST service, then the HSE includes one HSE Identity TLV containing the identity of the HSE and the Key Agreement TLV in the "Start new EMSDP session" command.  The Key Agreement TLV has the RAND and AUTN of the authentication vector to be used, in the RAND IE and AUTN IE.

On reception of the "Start new EMSDP session" command in the UE, the BEST session is considered setup and user plane messages are allowed to be sent.

##### 6.8.2.8.2	HSE initiated BEST session

Once the data session is live (following the BEST service discovery procedure), if there is no current session running and typically when the HSE has some data to transfer, the HSE checks the UE's subscription, the BEST service status for the location of the UE and the validity of the BEST service keys and carries out one of the following:

-	If the HSE determines that the key does not need to be re-agreed then the HSE sends a "Start new EMSDP session" command with the indicated key, encrypted and integrity protected as indicated in the service discovery.  This message has one HSE Identity TLV containing the identity of the HSE and a Key agreement TLV with its length set to 00 and no AUTN IE or RAND IE.

-	If the new keys need to be agreed for the BEST service, then the HSE includes one HSE Identity TLV containing the identity of the HSE and the Key Agreement TLV in the "Start new EMSDP session" command.  The Key Agreement TLV has the RAND and AUTN of the authentication vector to be used, in the RAND IE and AUTN IE.

On reception of the "Start new EMSDP session" command in the UE, the BEST session is considered setup and user plane messages are allowed to be sent.

#### 6.8.2.9	Resuming a BEST session following a power cycle at the UE or a re-attach

If the new session required is not set and the other BEST parameters in the HSE BEST IE are identical to the previous agreed HSE BEST IE, the UE may continue a previously started session by using the security details of the last agreed session.

#### 6.8.2.10	BEST service session operation

Once a BEST session is setup, boith signalling messages and user plan messages can be sent and received between the UE and the HSE secured by the agreed security mechanisms via the PDN gateway.

#### 6.8.2.11	Ending a BEST service session

The HSE or the UE can end a BEST session by sending a Terminate EMSDP request command.  This may be due to a request to do so by the enterprise or the user or due to an error condition.

### 6.8.3	Solution Evaluation

This solution completely describes a BEST service.

Key issue #1 is fully addressed by the described solution as it describes a secure message service the between the UE and an element in the home network called the HSE. The keys are agreed securely by re-using the strong LTE authentication mechanism.  The user data is protected by an integrity mechanism and optionally protected by encryption.  The user data messages are protected with a counter to prevent reply attacks.

Key issue #2  is fully met by this solution.

Key Issue #3 is fully met by the solution by the ability for the visited network to indicate restrictions to the HSE which in turn takes this information into account.

Key Issue #4 is fully met as this solution specifies integrity protection of all user data and allows the enterprise to control the security used either at the UE or at the HSE.

## 6.9	Solution #9: Complete end to end solution

### 6.9.1	Introduction

This solution is a complete end to end solution that includes the following features that minimise the security overhead whilst delivering all of the key objectives and addressing all of the key issues:

-	Optimisation of the transmitted data to reduce battery drain – The signalling and user plane headers between the UE and the eNodeB have been optimised so as to significantly reduce both the amount of header data added to every message and to reduce the processing required to generate a new message.

-	Optimisation of the key agreement - The number of authentication messages to setup and synchronise keys has been reduced significantly compared to an over the top E2E security solution.  The ability for keys to survive power cycles and location / network changes, acts to reduce the security overhead per message and therefore the battery drain.  The ability to reuse the 3G authentication mechanisms significantly reduces the signalling required for key agreement.

-	Re-use of existing signalling – The use of the protocol configuration options element that it is passed transparently from the UE to the EnSE via the existing LTE architecture for the initial signalling setup allows the overhead for this signalling to be minimised and allows the protocols specified here to be transparent to the visited network that does not know about the BEST service.  For a network that is aware of the BEST service, this also allows the visited network to send BEST service restriction requests to the home network to meet LI requirements.

-	Use of tunnelled bearer to reduce message overheads – This solution uses newly specified SGi PtP tunnelling based on UDP/IP bearer to reduce the transmitted data size.  With the SGi PtP tunnelling based on UDP/IP bearer the IP/UDP headers are created in the PGW and not by the UE saving a significant number of transmitted bytes.

-	Encryption of data from the UE all the way to the Enterprise – This solution describes the derivation of end-to-end keys between the UE and the Enterprise allowing for end-to-end encryption and integrity protection from the UE to the Enterprise while allowing the operator to assert control over the application of this encryption.

The solution specifically addresses:

-	Service discovery.

-	Enabling and disabling the service.

-	Negotiation of parameters.

-	Key agreement and re-keying.

-	Starting a session.

-	Suspending and resuming a session.

-	Ending a session.

-	Consequences of network changes and errors.

The solution proposed includes the setup of the E2E solution at the request of the UE and at the request of the external data network. The solution builds on the solution #8, the complete end to middle solution.

In comparison to solution #8, this solution adds:

-	An additional key derivation to derive a key shared between the Enterprise and the UE is proposed. This key derivation has two steps, namely, first derive an intermediate key at the HSE that is shared with the Enterprise and then in the EnSE deriving an E2E keyset using the intermediate key and an newly introduced Enterprise key.

-	An Enterprise key shared between the UE and the Enterprise and used in the derivation of the E2E keyset.

-	An Enterprise container that is signalled between the Enterprise and the UE to contain relevant information for the key derivation.

-	An endpoint, called the Enterprise Security Endpoint inside the Enterprises premises where the E2E session terminates.

-	Full control over the E2E session by running it through the HSE in the home network and keeping a session open between the HSE and the UE for full control.

-	Signalling messages between the HSE and the UE to control the E2E session between the UE and the EnSE.

### 6.9.2	Solution description

#### 6.9.2.1	Proposed Architecture

Figure 6.9.2.1.1 Proposed Architecture for Solution #9

The two new network elements proposed are:

-	Home Security Environment (HSE) – this is the endpoint for the secure communications in the home network.  It is expected that some user plane data and most of the signalling data relevant to the BEST service will terminate here.  In line with the LTE architecture, the HSE may be split into a signalling part and a user data part.  The HSE has interfaces to the PDN gateway (UDP/IP and Diameter over SGi), the enterprise server (out of scope for 3GPP), the EnSE (new 3) and either the HSS (S6x) or the EMKS (new 2).

-	Enterprese Security Endpoint (EnSE) - This is the endpoint for the secure communications in the domain of the enterprise. The user plane relevant for the BEST service will terminate here and some signalling traffic will terminate here. The EnSE has a new interface (new 3) that is used to pass key material from the HSE to the EnSE, to forward user plane data from the UE and to exchange relevant signalling between the HSE and the EnSE, such as the applicability of encryption or integrity algorithms. The EnSE maybe integrated with the Enterprise Server.

-	End to Middle Key Server (EMKS) – This is an optional key server element that manages the key communication with the HSS (for quintets)  and stores keys to reduce loading on the HSE and HSS.  Note: This is the same element described in solution #2.  The EMKS has interfaces to the HSS (S6y) and the HSE (new 2).

The four new network interfaces proposed are:

-	 S6x – An interface between the HSE and the HSS to allow the HSE to obtain subscription data and authentication vectors.

-	New 2 – An interface between the HSE and the EMSK that allows the HSE to retrieve authentication vectors for a user's subscription.

-	New 3 – An interface between the HSE and the EnSE that is used for carrying user plane traffic between the EnSE and the HSE and for signalling between the HSE and the EnSE.

-	New 4 – An interface between the EnSE and the Enterprise Server that allows the Enterprise Server to manage aspects of the BEST service and allows the transportation of user data to and from the Enterprise Server.

-	S6y – An interface between the EMSK and the HSS that allows the EMSK to retrieve a block of authentication vectors for later use by the HSE.

The SGi interface between the PDN Gateway and the HSE has 2 connections per subscriber.  A Diameter connection for "service discovery and negotiation" and a UDP/IP connection for the session signalling and user plane data.

Compared to solution #8, the complete End to Middle solution, the EnSE and the new-4 interface are added.

#### 6.9.2.2	Service Discovery and Negotiation

Clause 6.8.2.2 describes how this works for this solution.

#### 6.9.2.3	Ability to Enable and Disable the BEST service

Like solution #8, the HSE allows the enterprise to manage the BEST service. However, the interface may be provided through the EnSE and tunnelled to the HSE. In addition to solution #8, the EnSE allows the enterprise to turn E2E off without turning BEST off entirely, falling back to end-to-middle instead.

#### 6.9.2.4	End to Middle Security User Plane and Signalling Plane

##### 6.9.2.4.1	Data transport

Just like solution #8, this solution uses the SGi PtP tunnelling based on UDP/IP bearer specified in 3GPP TS 23.401[x] to transport both user plane and signalling plane data between the UE and the HSE.  This bearer has been chosen as it significantly reduces the  size of the data header used over the air interface (and hence reduces power consumption) whilst still allowing IP/UDP to be used between the PGW and the HSE.

##### 6.9.2.4.2	End to Middle Secured Data Protocol (EMSDP)

This solution uses the same EMSDP protocol as solution #8. A new data protocol between the UE and the HSE is detailed in this solution, called "End to Middle Secured Data Protocol" (EMSDP),  which has been optimised for efficient, secure transfer of both user plane and signalling plane data.

Figure 6.9.2.4.2.1 shows the data stack for the EMSDP transfers (based on 3GPP TS 23.401 [16]) for data transfer between the HSE and the UE.

Figure 6.9.2.4.2.1: data stack for the EMSDP transfers

The UE selects the non-IP PDN data type as defined in clause 4.3.17.8 of 3GPP TS 23.401[16].  The PDN GW adds the IP and UDP layers for the communication with the HSE. The HSE routes traffic between the UE and the EnSE based on the session ID in the EMDSP headers over the New-4 interface.

So that an MME that does not support the BEST service can restrict the use of the BEST service, as part of the normative work, a fixed APN structure will be investigated that allows filtering of the BEST service based on the APN requested by the UE.

The dashed box in HSE indicates that the control session between the UE and HSE terminates in the HSE and that the E2E session between the UE and the EnSE is forwarded by the HSE. Forwarding is left unspecified at this point in time and may include encapsulation in another protocol, such as XML over TCP/IP.

##### 6.9.2.4.3	EMSDP general structure

This solution uses the EMSDP general structure specified in 6.8.2.4.3 without modifications.

NOTE: With respect to the session ID, it is noted that in the end to end case, there is one session the HSE and the UE and one for data transfer between the EnSE and UE. The session between the HSE and the UE is the session that the HSE uses to control the session between the EnSE and the UE.

##### 6.9.2.4.3A	EMSDP Counter Schemes

There are no changes to the EMSDP Counter Schemes as specified in 6.8.2.4.3 .

##### 6.9.2.4.4	EMSDP Integrity protection

There are no changes to the EMSDP Integrity protection as specified in 6.8.2.4.4

##### 6.9.2.4.5	EMSDP Encryption

There are no changes to the EMSDP Encryption as specified in 6.8.2.4.5

##### 6.9.2.4.6	EMSDP Commands

6.9.2.4.6.0	Overview

There are no changes to the commands specified in 6.8.2.4.6.1

The following EMSDP commands for controlling the E2E session are proposed Table 6.9.2.4.6.1.1. These commands are exchanged between the UE and the HSE on the session, if it exists.

Table 6.9.2.4.6.1.1: EMSDP commands

| Code (Hex) | Command |
| --- | --- |
| 12 | Start EMSDP session request with E2E keys |
| 13 | Start EMSDP session response with E2E keys |
| 12 | Start EMSDP session request for E2E only |
| 13 | Start EMSDP session response for E2E only |
| 32 | Terminate EMSDP session request for E2E |
| 33 | Terminate EMSDP session response for E2E |
| 42 | Manage EMSDP Keys request for E2E keys |
| 43 | Manage EMSDP Keys request for E2E keys |
| 44 | Manage EMSDP Keys request for E2E keys only |

6.9.2.4.6.1	Start EMSDP session request with E2E keys

The Start EMSDP session request command is used by the UE to trigger a new BEST session from the HSE and at the same time start a session between the UE and the EnSE

The cmd options for the Start EMSDP session request command are as follows:

Enterprise Setup Information Element TLV: This is a TLV element that contains information from the UE that is used by the HSE to setup the HSE to enterprise connection.

Use E2E keys: Indicates the ability of the UE to setup a session with E2E encryption.

6.9.2.4.6.2		Start new EMSDP session response with E2E keys

The Start new EMSDP session response command is used by the HSE to setup a new BEST session.  This message can be sent in response to a start EMSDP session request command from the UE or as an initial message from the HSE.

The Start new EMSDP session response command has the following cmd options:

HSE Identity TLV: this contains a 4 octet numeric identifier for the HSE.  This should be unique to the HSE being used within the home network.

Key agreement TLV: this contains the RAND IE and AUTN IE specified in 3GPP TS 24.008 [12] as detailed in table 6.9.2.4.6.2.1:

Table 6.9.2.4.6.2.1: Key Agreement TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| Key Agreement TLV | 1 byte | M | TBD |
| Length | 1 byte | M | 0 or 35 |
| RAND IE | 17 bytes | C | See 10.5.3.1 in 3GPP TS 24.008 [12] |
| AUTN IE | 18 bytes | C | See 10.5.3.1.1 in 3GPP TS 24.008 [12] |

If the Length of the Key agreement is set to 0 then this means use current keyset.

E2E options: Indicates whether E2E is allowed

E2E session ID: Indicates the session ID to be used for the session between the UE and the EnSE

E2E derivation information: a container that contains the provided information for the key E2E derivation for the UE as provided by EnSE. The container may contain an identifier for the a private key to be used and optionally other values for key derivation.

Table 6.9.2.4.6.2.2: Key Agreement TLV for E2E

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| Key Agreement TLV | 1 byte | M | TBD |
| Length | 1 byte | M | Maximum length TBD |
| Container contents | Length | C | Provided by the EnSE |

6.9.2.4.6.3		Start EMSDP session request with E2E only

The Start EMSDP session request command is used by the UE to trigger a new BEST E2E session from the HSE to the same endpoint as the standing session. So, it serves as a request to turn the existing session between an HSE and the UE into a control session and obtain a new session for data transfer to the endpoint.

There are no cmd options for this command.

6.9.2.4.6.4		Start new EMSDP session response E2E only

The Start new EMSDP session response command is used by the HSE to add a new BEST E2E session to the existing session between the HSE and UE. This message can be sent in response to a start EMSDP session request for E2E only command from the UE or as an initial message from the HSE.

The Start new EMSDP session response for E2E only command has the following cmd options:

E2E options: Indicates whether E2E is allowed

E2E session ID: Indicates the new session ID to be used for the session between the UE and the EnSE

E2E derivation information: a container that contains the provided information for the key E2E derivation for the UE as provided by EnSE. The container may contain an identifier for the a private key to be used and optionally other values for key derivation.

Table 6.9.2.4.6.2.2: Key Agreement TLV for E2E

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| Key Agreement TLV | 1 byte | M | TBD |
| Length | 1 byte | M | Maximum length TBD |
| Container contents | Length | C | Provided by the EnSE |

6.9.2.4.6.5		Terminate EMSDP session request for E2E

The Terminate EMSDP session request for E2E EMSDP command is used by either the UE or the HSE to terminate an existing EMSDP session with E2E keys. This message is exchanged between the UE and the HSE on control session to terminate the E2E session between the UE and the EnSE. It has the following options:

Session ID to be terminated: The session ID of the E2E session to be terminated.

NOTE: This message is omitted whenever the control session between the UE and the HSE is terminated. Upon termination of the control session, the HSE and UE shall terminate the E2E session as well.

6.9.2.4.6.6	Manage EMSDP Keys request for E2E Keys

The Manage EMSDP Keys request for E2E keys EMSDP command is used by the UE to negotiate new key material.

Command options:

E2E_keys_only: indicates whether the UE wants to refresh all the keys or the E2E keys only.

6.9.2.4.6.7		Manage new EMSDP Keys for E2E Keys

The Manage EMSDP Keys response command for E2E keys is used by the HSE to agree new keys.

The Manage EMSDP Keys response command for E2E keys has the following cmd options:

Key agreement TLV: this contains the RAND IE and AUTN IE specified in 3GPP TS 24.008 [12] as detailed in table 6.8.2.4.6.5.1:

Table 6.8.2.4.6.5.1: Key Agreement TLV

| Name | Size | M / C / O | Value |
| --- | --- | --- | --- |
| Key Agreement TLV | 1 byte | M | TBD |
| Length | 1 byte | M | 35 |
| RAND IE | 17 bytes | C | See 10.5.3.1 in 3GPP TS 24.008 [12] |
| AUTN IE | 18 bytes | C | See 10.5.3.1.1 in 3GPP TS 24.008 [12] |

E2E derivation information: a container that contains the provided information for the key E2E derivation for the UE as provided by EnSE as specified in Table 6.9.2.4.6.2.2.

6.9.2.4.6.8		Manage new EMSDP Keys for E2E Keys only

The Manage EMSDP Keys response command is used by the HSE to agree new keys between EnSE and the UE only. The intermediate key material and the E2M keys remain unchanged. The command options are as follows:

E2E derivation information: a container that contains the provided information for the key E2E derivation for the UE as provided by EnSE as specified in Table 6.9.2.4.6.2.2.

#### 6.9.2.7	Key Agreement and Refreshing

##### 6.9.2.7.1	Overview

There are several key agreement solutions presented in this document that would be capable of agreeing keys between the UE and the enterprise.  For this solution it is proposed to use the key agreement similar to the mechanism proposed in solution #4 as it enables complete confidentially between the enterprise and the UE and it remains unaffected by visited network changes (such as location area changes).

##### 6.9.2.7.2	Key setup messaging between HSE and UE

New keys are agreed either at the start of a BEST session or as required due to key aging or counter thresholds being met.  Key agreement is based on the 3GPP AKA mechanism detailed in TS33.102 [27] and the AKA challenge is transported between the HSE and the UE in any of the following EMSDP commands:

-  Start new EMSDP session EMSDP command from the HSE as part of a new session being setup by the HSE.

-	Manage new EMSDP Keys EMSDP command from the HSE as a result of key aging or counter thresholds being met.

Once in a session, a UE can request new keys by sending a Manage EMSDP Keys for E2E request or Manage EMSDP Keys for E2E only request command to the HSE.

Figure 6.9.2.7.2.1 shows the key agreement process

Without EMKS

With EMKS

Figure 6.9.2.7.2.1 BEST key E2E agreement process

The Key agreement steps are:

1.	Start EMSDP session request with E2E command or Manage EMSDP Keys request for E2E (only) command over the Air interface / core network interfaces. – If the UE wants to initialise the BEST E2E session, it sends a "Start EMSDP session request command with E2E" through the SGI PtP tunnel (This is described in detail in clause 6.9.2.8). If the UE is already in a session and wants to renegotiate the keys it sends a or Manage EMSDP Keys request for E2E command secured with the current keys to the PDN gateway through the SGI PtP tunnel.

2.	Start EMSDP session request command with E2E or Manage EMSDP Keys request for E2E (only) command over the SGi interface.  –The "Start EMSDP session request with E2E command" or Manage EMSDP Keys for E2E (only) request command is forwarded to the HSE using the SGi interface.

3.	Keys required? - the HSE checks to see if there are valid keys with valid counter values available in the HSE for that UE then the following is checked:

-	If the "Start EMSDP session request with E2E command" has the correct counter value, is correctly encrypted and is correctly integrity protected then the HSE sends a "Start new EMSDP session with E2E command" – use current keyset, to the UE correctly encrypted, integrity protected and with a correct counter to the UE (step 6).

-	If the "Start EMSDP session request with E2E command" is not integrity protected or has any of the security parameters wrong (against the information the HSE has) and there are new authentication vectors available in the HSE for that UE, then the HSE goes to step 6 and uses one of the available unused authentication vectors.

-	If the "Start EMSDP session request command with E2E" is not integrity protected or has any of the security parameters wrong (against the information the HSE has) and there are no authentication vectors available in the HSE for that UE and the HSE has a direct connection to the HSS, then the HSE goes to step 4.

-	If the "Start EMSDP session request with E2E command" is not integrity protected or has any of the security parameters wrong (against the information the HSE has) and there are no authentication vectors available in the HSE for that UE and the HSE has an indirect connection to the HSS via a EMKS, then the HSE goes to step a.

-	If the command is the 'Manage EMSDP Keys request for E2E only’, then the HSE skips steps 4, 5, 6 and directly proceeds to step 7.

4. Authentication-Information-Request over S6x interface – The HSE uses the S6x interface to the HSS to request one or more authentication vectors using the UE IMSI.  These vectors will be specific to the BEST service.  The IMSI of the UE is the IMSI passed in the accounting request (Start) for that UE when the SGi interface was setup.

5. Authentication-Information-Answer over S6x interface – The HSS uses the S6x interface to the HSE to return the requested authentication vectors.  These vectors are specific to the BEST service.

a. Authentication-Information-Request over the new2 interface – Where an EMKS is used, the HSE uses the new2 interface to the EMKS to request an authentication vector using the UE IMSI.  These vectors will be specific to the BEST service.  The IMSI of the UE is the IMSI passed in the accounting request (Start) for that UE when the SGi interface was setup.

b. Authentication-Information-Request over the S6y interface – The EMKS uses the S6y interface to the HSS to request one or more authentication vectors using the UE IMSI.  These vectors will be specific to the BEST service.

c. Authentication-Information-Answer over the S6y interface – The HSS uses the S6y interface to the EMKS to return the requested authentication vectors.  These vectors may be stored on the EMKS for later use.

d. Authentication-Information-Answer over the new2 interface – The EMKS uses the new2 interface to the HSE to return the requested authentication vector.

6. Calculate HSE Keys -  The HSE calculates the KE2Menc key using the BEST key derivation mechanism, CK, IK, a HSE id and the key id with the Algorithm id set to "BEST encryption Key". The HSE calculates the KE2Mint key using the BEST key derivation mechanism, CK, IK, a HSE id and the key id with the Algorithm id set to "BEST integrity Key". The HSE calculates the (KInt_E2E)  key using the BEST key derivation mechanism, CK, IK, a HSE id and the key id with the Algorithm id set to " BEST end2end Intermediate Key".

7. The HSE sends a ‘UE E2E Session Request’ to the EnSE over the new 3 interface – The HSE uses the new 3 interface to the EnSE to inform the EnSE of the new E2E session request and forwards the intermediate E2E key (KInt_E2E)  to the EnSE.

8. The EnSE sends a ‘E2E Session Accept’ to the HSE with an Enterprise  service container – The EnSE replies with the Enterprise service container to be send to the UE.

9. Start new EMSDP session command over the SGi interface -  The HSE sends a "Start new EMSDP session with E2E" command to the PDN Gateway using the SGi interface as follows:

-	If the new keys need to be agreed for the BEST service, then the HSE includes the Key Agreement TLV and the provided Enterprise container in the "Start new EMSDP session" command.  The Key Agreement TLV has the RAND and AUTN of the authentication vector used in the RAND IE and AUTN IE. The Enterprise container is passed on transparently.

- A different session ID for the E2E session, i.e. for the user data that should not be decrypted by the HSE but passed on unchanged to the EnSE. This session ID is to be used between the UE and the EnSE.

10. Start new EMSDP session command over the Air interface / core network interface -  The PDN Gateway sends a "Start new EMSDP session with E2E" command to the UE using the core network and air interfaces.

11. Calculate UE Keys - On successful reception of the "Start new EMSDP session with E2E" command, the UE checks the AKA challenge as detailed in 3GPP TS 33.102 [27] and if it is valid, calculates the AKA response CK and IK.  Unlike the normal AKA challenge/response, the AKA response in not sent to the HSE as the HSE can determine that this was successful from the MAC value presented in any following message.  The UE calculates the End to Middle keys as follows:

-	KE2Menc key using the BEST key derivation mechanism, CK, IK, a HSE id and the key id with the Algorithm id set to "BEST encryption Key". The UE calculates the KE2Mint key using the BEST key derivation mechanism, CK, IK, a HSE id and the key id with the Algorithm id set to "BEST integrity Key".  The UE may choose to calculate one or a set of KE2Menc and KE2Mint key values depending on its optimal power strategy.  Each key id refers to a pair of KE2Menc and KE2Mint key values. The UE calculates the (KInt_E2E)  key using the BEST key derivation mechanism, CK, IK, a HSE id and the key id with the Algorithm id set to " BEST end2end Intermediate Key".

The UE calculates the E2E keys as follows:

The UE calculates the KE2Eenc key using the BEST key E2E derivation mechanism using the Enterprise key, optional additional information from the Enterprise container, the BEST end-to-end Intermediate Key (KInt_E2E) and the Algorithm ID set to "BEST e2e encryption key" and the UE calculates the KE2Eenc key using the BEST key E2E derivation mechanism using the Enterprise key, optional additional information from the Enterprise container,  the BEST end2end Intermediate Key (KInt_E2E) and the Algorithm ID set to "BEST e2e integrity key (KE2Eenc)".

NOTE: Upon successful " Manage EMSDP Keys request for E2E only", the UE only calculates the E2E keys.

12. Calculate EnSE Keys - The EnSE calculates the KE2Eenc key using the BEST key E2E derivation mechanism using the Enterpise key and optional additional information, the BEST end-to-end Intermediate Key (KInt_E2E) and the Algorithm ID set to "BEST e2e encryption key". The EnSE calculates the KE2Eenc key using the BEST key E2E derivation mechanism using the Enterprise key and optional additional information from the Enterprise container,  the BEST end2end Intermediate Key (KInt_E2E) and the Algorithm ID set to "BEST e2e integrity key (KE2Eenc)".

The pair of KE2Menc and KE2Mint key values for the indicated key id are then used for all further EMSDP messages between the HSE and the UE until either a new key ID is indicated or until the keys are re-negotiated. The pair of KE2Eenc and KE2Eint key values for the indicated key id are then used for all further EMSDP messages between the EnSE and the UE until either a new key ID is indicated or until the keys are re-negotiated.

##### 6.9.2.7.3	BEST key derivation mechanism

6.9.2.7.3.1	BEST End to middle and intermediate key derivation

All key derivations (including input parameter encoding) for BEST shall be performed using the key derivation function (KDF) specified in TS 33.220 [20]. This clause specifies how to construct the input string, S, to the KDF (which is input together with the relevant key).

When deriving a KE2Menc or KE2Mint from CK, IK, a HSE id, key id and algorithm id, the following parameters shall be used to form the input S to the KDF.

-	FC = [TBD],

-	P0 = HSE id,

-	L0 = length of HSE id (i.e. 0x00 0x03),

-	P1 = SQN  AK

-	L1 = length of SQN  AK (i.e. 0x00 0x06)

-	P2 = algorithm id

-	L2 = length of algorithm id (i.e. 0x00 0x01)

The exclusive or of the Sequence Number (SQN) and the Anonymity Key (AK) is sent to the UE as a part of the Authentication Token (AUTN), see TS 33.102. If AK is not used, AK shall be treated in accordance with TS 33.102 [27], i.e. as 000…0.

The algorithm id shall be "BEST encryption key" for BEST encryption algorithms and "BEST integrity Key" for BEST integrity protection algorithms. The values 0x04 to 0xf0 are reserved for future use, and the values 0xf1 to 0xff are reserved for private use.

Table 6.9.2.7.3.1: Algorithm type distinguishers

| Algorithm distinguisher | Value |
| --- | --- |
| BEST encryption key (KE2Menc) | 0x01 |
| BEST integrity Key (KE2Mint) | 0x02 |
| BEST end2end Intermediate Key (KInt_E2E) | 0x03 |

The input key shall be equal to the concatenation CK || IK of CK and IK.

6.9.2.7.3.2	BEST End to end key derivation

All key derivations (including input parameter encoding) for BEST shall be performed using the key derivation function (KDF) specified in TS 33.220 [20]. This clause specifies how to construct the input string, S, to the KDF (which is input together with the relevant key).

When deriving a KE2Eenc or KE2Eint from KInt_E2E, the following is the following parameters shall be used to form the input S to the KDF. The RAND in this derivation could be provided by inside the Enterprise container.

-	FC = [TBD],

-	P0 = EnSE id,

-	L0 = length of EnSE id (i.e. 0x00 0x03),

-	P1 = algorithm id

-	L1 = length of algorithm id (i.e. 0x00 0x01)

-	P2 = RAND

-	L2 = length of rand (0 if no rand found in the Enterprise container)

The algorithm id shall be "BEST e2e encryption key" for BEST encryption algorithms and "BEST e2e integrity Key" for BEST integrity protection algorithms. The values 0x03 to 0xf0 are reserved for future use, and the values 0xf1 to 0xff are reserved for private use.

Table 6.9.2.7.3.2: Algorithm type distinguishers

| Algorithm distinguisher | Value |
| --- | --- |
| BEST e2e encryption key (KE2Eenc) | 0x01 |
| BEST e2e integrity Key (KE2Eint) | 0x02 |

The input key shall be equal to the concatenation KInt_E2E || Enterprise_Key of KInt_E2E  and Enterprise_Key. When the Enterprise container specifies no Enterprise key, the Enterprse_Key shall be set to zero (0..0).

Note: The Enterprise_Key provisioning is outside the scope of 3GPP.

#### 6.9.2.8	Starting a BEST service session

##### 6.9.2.8.1	UE initiated BEST session

2. Start EMSDP session request with E2E command over the SGi interface (UE initialised session).

Figure 6.9.2.8.2.1 UE initiated BEST session startup flow diagram

Once the data session is live (following the BEST service discovery procedure), if there is no current session running and typically when the UE has some data to transfer, the UE checks the key status and the BEST container received from the BEST service discovery procedure and does the following:

- If the BEST service is not activated in the BEST container then the session shall not be started.

- If the UE determines that the key needs to be re-agreed then the UE sends a "Start EMSDP session request with E2E" command through the SGI PtP tunnel (This is described in detail in clause 6.8.2.8) with the key id set to "00", Session id set to "00..00", no encryption and the MAC set to "00..00".

- If the UE determines that the key does not need to be re-agreed then the UE sends a "Start EMSDP session request with E2E" command with the indicated key and algorithms.  The HSE may trigger a key re-agreement if it chooses to do so.

On reception of the "Start EMSDP session request with E2E" command from the UE via the PDN gateway, the HSE checks the security of the message, the UE's subscription, the BEST service status for the location of the UE and the validity of the BEST service keys and carries out one of the following:

- If the HSE determines that the key does not need to be re-agreed then the HSE sends a "Start new EMSDP session with E2E" command with the indicated key, encrypted and integrity protected as indicated in the service discovery.  This message has one HSE Identity TLV containing the identity of the HSE and a Key agreement TLV with its length set to 00 and no AUTN IE or RAND IE. This message may contain an empty enterprise container indicating that the stored E2E keys can be reused.

- If the new keys need to be agreed for the BEST service, then the HSE includes one HSE Identity TLV containing the identity of the HSE and the Key Agreement TLV in the "Start new EMSDP session with E2E" command.  The Key Agreement TLV has the RAND and AUTN of the authentication vector to be used, in the RAND IE and AUTN IE. Also, it includes whether E2E is allowed, includes the EnSE ID and provides the Enterprise container if E2E is allowed.

On reception of the "Start new EMSDP session with E2E" command in the UE, the BEST session is considered setup and user plane messages are allowed to be sent.

If no E2E session is allowed, the HSE can decide to reply with a "Start new EMDSP session" in stead telling the UE that it should commence a session without E2E. In this case, the UE falls back to the End to Middle scenario.

##### 6.9.2.8.2	HSE initiated BEST session

Figure 6.9.2.8.2.1: Flow diagram of HSE initiated BEST session

Once the data session is live (following the BEST service discovery procedure), if there is no current session running and typically when the HSE has some data to transfer, the HSE checks the UE's subscription, the BEST service status for the location of the UE and the validity of the BEST service keys and carries out one of the following:

-	If the HSE determines that the key does not need to be re-agreed then the HSE sends a "Start new EMSDP session with E2E" command with the indicated key, encrypted and integrity protected as indicated in the service discovery.  This message has one HSE Identity TLV containing the identity of the HSE and a Key agreement TLV with its length set to 00 and no AUTN IE or RAND IE. This message may contain an empty enterprise container indicating that the stored E2E keys can be reused.

-	If the new keys need to be agreed for the BEST service, then the HSE includes one HSE Identity TLV containing the identity of the HSE and the Key Agreement TLV in the "Start new EMSDP session with E2E" command.  The Key Agreement TLV has the RAND and AUTN of the authentication vector to be used, in the RAND IE and AUTN IE. Also, it includes whether E2E is allowed, includes the EnSE ID and provides the Enterprise container if E2E is allowed.

On reception of the "Start new EMSDP session with E2E" command in the UE, the BEST session is considered setup and user plane messages are allowed to be sent.

If no E2E session is allowed, the HSE can decide to send a "Start new EMDSP session" in stead telling the UE that it should commence a session without E2E. In this case, the UE falls back to the End to Middle scenario.

#### 6.9.2.9	Resuming a BEST session following a power cycle at the UE or a re-attach

If the new session required is not set and the other BEST parameters in the HSE BEST IE are identical to the previous agreed HSE BEST IE, the UE may continue a previously started session by using the security details of the last agreed session. If the control session is resumed, it is implicitly assumed that the E2E associated with that control session is resumed as well.

#### 6.9.2.10	BEST service session operation

Once a BEST session is setup, boith signalling messages and user plan messages can be sent and received between the UE and the HSE / EnSE secured by the agreed security mechanisms via the PDN gateway.

#### 6.9.2.11	Ending a BEST service session

The HSE or the UE can end a BEST session by sending a Terminate EMSDP request command.  This may be due to a request to do so by the enterprise or the user or due to an error condition.

### 6.9.3	Solution Evaluation

This solution completely describes a BEST service.

Key issue #1 is fully addressed by the described solution as it describes a secure message service the between the UE and an element in the Enteprise’s domain, the EnSE. The keys are agreed securely by re-using the strong LTE authentication mechanism.  The user data is protected by an integrity mechanism and optionally protected by encryption.  The user data messages are protected with a counter to prevent reply attacks.

Key issue #2  is fully met by this solution.

Key Issue #3 is fully met by the solution by the ability for the visited network to indicate restrictions to the HSE which in turn takes this information into account and also controls the E2E session.

Key Issue #4 is fully met as this solution specifies integrity protection of all user data and allows the enterprise to control the security used either at the UE or at the HSE through the EnSE.

## 6.10	Solution #10: "AKA-based session key generation for application protocols"

### 6.10.1	Introduction

The main purpose of the present solution is providing AKA-based session key generation as an independent module usable by any security protocol at the application layer.

Solution 10 leaves security context negotiation (apart from the keys) and protection of transferred data to the application layer protocol.

As an example, it is shown how the present solution will work with pre-shared key DTLS.

The present solution builds on solution #2. The present solution extends solution #2 by providing an example of a standardized interface between the EMSE and the EMKS  that enables any application, in the home network or in the trusted third party domain, to obtain e2m keys from the EMKS. Furthermore, the requirements on the application layer security protocols that need to be fulfilled so that the present solution can be used are stated. Furthermore, compared to solution 2, the text expands on the transport of the UE's application layer identities, the usefulness of key identifiers and two aspects of key refresh.

The present solution can be used for CIoT (which is why it is included in the present document). It should be noted, though, that there is no technical reason why the present solution should be restricted to the CIoT use case; it can provide e2m security whenever AKA is run in a mobile network.

This solution introduces E2M Security Endpoint (EMSE), an entity that is authorized to obtain e2m keys from the E2M Key Server (EMKS). An EMSE may reside in an operator's home network, but also in a trusted third party domain (e.g. a CIoT application provider platform). The EMKS always resides in an operator's home network.

There may be multiple instances of an EMSE, with which a UE engages simultaneously or sequentially. There is typically only one instance of an EMKS per HSS.

As usual in 3GPP standardisation, EMSE and EMKS denote functional entities that could be implemented stand-alone or combined with HSS or GGSN or P-GW. EMSE and EMKS could also be combined. The advantage of introducing the EMKS is that the HSS could continue to operate in a stateless fashion.

The following solution description first highlights the features of the present solution, then describes the interface between EMSE and EMKS, and finally show how the solution can be applied using the example of pre-shared key DTLS as the application layer security protocol between UE and EMSE.

### 6.10.2	Solution description

#### 6.10.2.1	Features

The present solution follows the same functional architecture as in solution #2.

Requirements on the application layer security protocol:

The present solution can be used as a key establishment mechanim for an application layer security protocol if the protocol fulfills the following requirements:

1) The protocol needs to support use with pre-shared keys.

2) The protocol needs to provide a means to transport the user identity (including a key identifier) to the EMSE.

NOTE: This identity transport mechanism necessarily is specific to the particular security protocol.

3) The protocol needs to be able to generate fresh session keys for consecutive instances of the protocol even when the pre-shared key remains the same (cf. also clause on key refresh below).

NOTE: All three requirements are satisfied for (D)TLS and IKE.

Considerations on the user identity in the application layer security protocol:

The UE could use the IMSI. But the UE’s application layer identity used in the security protocol between UE and EMSE does not have be identical to the IMSI. This depends on offline agreements between UE, EMSE, and the mobile network operator.

If the UE’s application layer identity is different from the IMSI, then two alternatives are available:

1)	EMSE translates the UE’s application layer identity to an IMSI and sends IMSI to EMKS.

2)	EMSE sends the UE’s application layer identity to the EMKS, and the EMKS translates the UE’s application identity to an IMSI.

For case 1, there may be privacy implications if MNO does not want to share the IMSI with the third party.

For case 2, it may be possible that the third party does not want to share the UE’s application layer identity with the MNO. However, case 2 seems less critical as, for any solution for BEST, the third party depends on the MNO for deriving pre-shared key for its communicaton protocol between the UE and EMSE, which means that the third party has to put a substantial amount of trust in the MNO anyhow. Case 2 therefore is the preferred solution.

Key synchronization between the UE and the EMSE:

Key synchronization issues could occur due to race condition or authentication failures. For example,

a) When HSS pushes key to EMKS the HSS does not know whether network access authentication will be successful. If not successful, the UE does not have the latest key pushed to the EMKS. This could happen repeatedly. So, when a UE establishes an application layer connection with the EMSE the UE could have a key that is older than the latest one available at the EMKS.

b) Assume an MME requests re-authentication (which is possible even while the UE is in connected state). The HSS then sends authentication vectors back to the MME and, at the same time, pushes e2m_int_key to the EMKS. At (roughly) the same time the UE establishes an application layer connection with the EMSE, but has not seen the latest Authentication challenge from the MME yet (race condition). Again, the UE could have a key that is older than the latest one available at the EMKS.

The present solution avoids key synchronization problem through the following set of features:

a) It is proposed that the EMKS does not immediately delete a e2m_int_key in storage when it receives a new e2m_int_key from the HSS for a given UE. The EMKS keeps a fixed (small) number of instances of e2m_int_key from the past. This is up to the policy of the EMKS.

b) Identify each instance of e2m_int_key with a unique key identifier. HSS pushes both the key and its identifier to EMKS during network access authentication. UE is independently able to generate the key identifier. Considerations on key identifier is discussed next.

c) UE provides the latest version of the key identifier to EMSE when it initiates setup of the application layer security prototocol. This is used by EMSE to query EMKS for the appropriate e2m_int_key.

d) The EMKS obtains the associated e2m_int_key from its store and responds to EMKS with the generated e2m_key.

Therefore, even if UE had an older version of e2m_int_key, by storing multiple instances of e2m_int_key in its store, EMKS can obtain the correct e2m_int_key and continue with the derivation of e2m_key.

Considerations on the key identifier:

The present solution proposes the use of a key identifier for the e2m_key. This key identifier is assumed to be sent along with the UE’s application layer identity from the UE to the EMSE, when application layer security is established, and further from the EMSE to the EMKS when the EMSE fetches a key from the EMKS.

The key identifier is useful to synchronize the use of e2m_key at UE and EMSE.

In order to support key synchronization, it is proposed that the EMKS does not immediately delete a e2m_int_key in storage when it receives a new e2m_int_key from the HSS for a given UE, but keeps a fixed (small) number of instances of e2m_int_key from the past. This is up to the policy of the EMKS.

The present solution envisages two alternatives for defining a key identifier:

1) Use a portion of RAND as contained in the authentication vector, e.g. the 32 least significant bits of RAND. With this approach, the HSS would have to push the relevant portion of RAND from the HSS to the EMKS, together with the e2m_int_key.

2) Use a short hash of the e2m_int_key. The hash would not serve any cryptographic purposes. It should just be long enough to achieve an acceptably small rate of accidental collisions. With this approach, the EMKS would have to compute hashes on all the e2m_int_key values it has stored for a particular UE when the EMKS receives a request from the EMSE to fetch a key.

Alternative 1 is the preferred approach.

Considerations on unique key per EMSE

The UE could connect to multiple application layer endpoints in the enterprise network at any given time. To achieve key separation of UE’s traffic between these endpoints, the present solution proposes to generate unique e2m_key for each endpoint by including an identifier for the endpoint in the key derivation.

In the present solution, each instance of EMSE is identified by a unique endpoint_id or EMSE_Id that is known to both the UE and EMSE.

When the UE initiates set up of a secure application layer connection with an instance of the EMSE, the EMSE requests a e2m_key from the EMKS. The EMKS includes EMSE_Id in the key deriviation to generate unique e2m_key for the connection. The EMSE could include EMSE_Id in every request for e2m_key or provide it to EMKS one-time during the initial registration.

The UE would correspondingly generate EMSE specific e2m_key by including EMSE_Id in its key derivation procedure.

Key derivation rules are discussed in a later clause.

Transparency for core network nodes:

The present solution will work transparently to core network. In other words, the present solution does not require any change to SGSNs, GGSNs, MMEs, S-GWs and P-GWs.

Transparency for the USIM:

As for solution#2, the present solution can re-use existing USIMs, with all CIoT-specific operations to be done in the ME.

Key derivation rules:

This solution reuses key derivation rules from solution #2. They are repeated here for the sake of readability of the present solution

As for solution#1, the present solution uses a new key pair CK´, IK´for UMTS PS or GPRS access security between UE and SGSN. This new key pair CK´, IK´ is generated in the HSS and included in the UMTS AKA authentication vector sent to the  SGSN. The SGSN will not notice the difference to legacy authentication vectors. For EPS, KASME can be re-used as defined today.

In the present solution, the key used between UE and EMSE is called "e2m_key".

The e2m key is derived in two steps:

(1)	Whenever an Authentication Information Request from the SGSN or the MME arrives at the HSS, the HSS checks the subscription profile for the need to derive an e2m_int_key. The HSS then generates authentication vectors and sends them back to the SGSN or MME. If there is a need to derive an e2m_int_key a new key pair CK´, IK´is included in the authentication vector for UMTS PS or GPRS. Furthermore, the HSS derives e2m_int_key from CK, IK and a string pointing to the purpose of the key use, namely e2m security for CIoT, where the derivation of e2m_int_key has the form

e2m_int_key = KDF (CK||IK, string)

and the string could be set to e.g. "e2m_ CIoT". The HSS the pushes the e2m_int_key to the EMKS.

NOTE: It is expected that the interface between HSS and EMKS will be based on DIAMETER, details will be defined in stage 3.

(2)	The EMKS derives e2m_key from e2m_int_key upon request from an EMSE. As there may be multiple instances of EMSE it becomes necessary to include an identifier of the EMSE in the key derivation in order to achieve key separation between EMSEs. The following is the key derivation rule:

e2m_key = KDF (e2m_int_key, EMSE_Id, string)

Editor's Note: it is ffs whether the additional input 'string' is needed in the derivation of e2m_key. It may be appropriate to allocate FC values for the purpose of the two key derivations in steps 1) and 2) according to TS 33.220 [20], B.2.2.

Key refresh:

There are two aspects to consider:

1) The application layer security protocol requires a fresh session key.

This is the case e.g. when the application layer session is torn down and re-established some time later. It could be that the e2m_key, which is used as pre-shared key, has not changed in the meantime. The present solution does not envisage a means for the application layer to trigger the generation of a new e2m_key. Therefore, it is stated further above as a requirement that application layer security protocol needs to be able to generate fresh session keys for consecutive instances of the protocol even when the pre-shared key remains the same.

2) A new e2m_int_key becomes available due to a re-authentication at the network access layer.

In general, it is not the case that an application layer security protocol can change the pre-shared key during an ongoing session. This means that a new pre-shared key derived from the new e2m_int_key can only be established when the session has been terminated, for reasons determined within the application, and a new session is set up. Then the EMSE will again contact the EMKS and obtain, in this way, a new e2m_key.

#### 6.10.2.2	Interface between EMKS and EMSE

##### 6.10.2.2.1	Introduction

Editor's Note: The material in this clause describes a candidate interface bewteen EMKS and EMSE. Other examples for this interface should be studied in a normative phase, e.g. the Tsp interface could be an object of study.

The interface between EMKS and EMSE is used by the EMSE for fetching UE-related key to be used in the application layer security protocol between UE and EMSE.

The interface should be standardized to achieve maximum interoperability.

In the following, we describe the example of a RESTful HTTP interface between EMKS and EMSE.

It is described as follows:

-	TCP provides communication service at the transport layer

-	TLS provides security to the communication

-	HTTP based transport of XML data

-	XML documents used to embed specific datastructures, such as keys etc.

Figure 6.10.2.2.1-1 illustrates the protocol stack of this reference point.

Figure: 6.10.2.2.1-1 RESTful HTTPS between EMKS and EMSE

EMKS and EMSE act as both HTTP client and server. Thus, there is a TCP connection for each direction. This permits bidirectional communication between EMKS and EMSE.

RESTful HTTP is used at the application delivery layer. The content type of the RESTful HTTP is XML.

The unsecured HTTP protocol shall be combined with TLS, as HTTPS, to provide confidentiality and integrity protection. Mutual authentication shall be enabled in TLS for authenticating and allowing only an authorized third party EMSE to access the EMKS. The profile for TLS implementation and usage shall follow the provisions given in 3GPP TS 33.310 [45], Annex E.

##### 6.10.2.2.2	Procedures over the RESTful HTTP reference point

Following procedures are supported over the RESTful HTTP reference point:

-	Initial registration by EMSE

-	Obtaining UE specific e2m_key from EMKS

-	Deregistration by EMSE

-	Subscription to Notification by EMKS

-	Notification by EMSE

Editor’s Note: Subscription by EMSE to a particular event, and Notification by EMKS is ffs

6.10.2.2.2.1	Initial registration by EMSE

The EMSE registers with EMKS by sending an HTTP POST message to the EMKS including its identifier (EMSE_Id). The EMKS establishes a session context for this EMSE and returns session id in HTTP 201 CREATED.

6.10.2.2.2.2	Obtaining UE specific e2m key from the EMKS

The EMSE obtains UE specific e2m key from the EMKS by sending an HTTP GET message including an identity of the UE and a key identifier received from the UE. This message is embedded in the following example information flow using DTLS-PSK.

6.10.2.2.2.3	Deregistration by EMSE

When the session needs to be terminated, EMSE may send an HTTP DELETE message including the session ID as the URL address to the EMKS.

#### 6.10.2.3	Example use of solution 10: information flow using pre-shared key DTLS

In this subclause we show how the present solution can be applied using the example of pre-shared key DTLS as the application layer security protocol between UE and EMSE.

Other candidate application layer security protocols include IKEv2 (with pre-shared keys) combined with ESP.

Prerequisite:

a)	HSS has processed the Authentication Information Request from the SGSN or MME, and pushed the intermediate key, e2m_int_key and the associated key identifier to the EMKS. 
Network access AKA run is completed and UE is in possession of the necessary inputs for deriving the e2m_int_key, together with the related key identifier.

b)	UE is ready to initiate setup of the chosen application layer security protocol with a particular EMSE. UE derives e2m_key using the EMSE_Id.

c)	EMSE has registered and established a session with the EMKS. The interface between EMSE and EMKS is integrity- and confidentiality-protected by TLS as shown in the protocol stack in Figure 6.10.2.2.1-1.

Figure: 6.10.2.3 DTLS-PSK session between UE and EMSE based on e2m_key

In the following sequence (D)TLS-PSK is used as an example:

1)	UE initiates establishment of a DTLS session by sending TLS message "ClientHello". This may, for example, contain COAP defined mandatory-to-implement cipher suite TLS_PSK_WITH_AES_128_CCM_8.

2)	EMSE responds to the UE with the "ServerHello" and "ServerHelloDone" messages.

3)	Using e2m_key (from prerequisite step b above) as the psk, UE generates TLS session keys.

4)	UE sends "ClientKeyExchange", "ChangeCipherSpec" and "Finished". UE’s application layer identity and key identifier are sent in the psk_identity value in the "ClientKeyExchange" message.

5)	EMSE initiates HTTP GET to obtain the latest UE specific e2m_key from EMKS. EMSE sends UE’s identity value and key identifier in this request message to EMKS.

6)	The EMKS selects the right e2m_int_key with the help of the key identifier and computes e2m_key using the EMSE_Id as input.

7)	The EMKS then returns e2m_key in the HTTP 201 response to EMSE.

8)	EMSE uses the received e2m_key as pre-shared secret as specified for DTLS.

9-11)	EMSE continues with rest of the DTLS session setup flow.

As a result, a DTLS security session is setup between the UE and EMSE.

### 6.10.3	Solution evaluation

Solution #10 builds upon Solution #2. Therefore all aspects of solution #2, as indicated in clause 6.2.3, apply to solution #10 as well. In particular, solution#10 is transparent to core network nodes, apart from the HSS; and does not require any changes to SGSN, GGSN, S-GW, or P-GW. In addition, solution #10 provides a standardized interface for the application layer security protocol in 3rd party networks, to fetch keys and use it as pre-shared secret. This enables any application layer protocol to use MNO generated AKA key to generate protocol-specific session keys between the UE and the application layer node (EMSE). Solution #10 provides an essential module in a modular approach to BEST.

## 6.11	Solution #11: A method for IoT service layer security bootstrapping solution

### 6.11.1	Introduction

This solution provides a mechanism for deriving session keys between the IoT device and the application server with the following advantages.

Note: This solution is not compatible with GPRS.

1)	Key Issue No. 1: It provides integrity protection and confidentiality keys to all communications between the IoT device and the application server which can be used by a transport protocol to secure communications. Confidentiality is invoked as per the legal regulatory of the serving network.

2)	Key Issue No. 2: Addresses the efficient user data protection challenges by allowing keys of at least 128 bits.

Address main performance objective of the study:

3)	Optimization of transmitted flows to reduce battery consumption by eliminating the need for another AKA procedure, i.e., GBA-AKA, and thus saves at least four message flows of the procedure, then reduces the power consumption at the IoT device.

4)	Use existing 3GPP architecture and network entities/nodes with minimum changes to existing interfaces to accomplish the objectives of this study.

### 6.11.2	Solution description

#### 6.11.2.1	Proposed architecture

Figure 6.11.2.1.1 Proposed Architecture for Solution #11

The key network element is:

-	Service Capability Exposure Function (SCEF): This is the key entity within the 3GPP architecture for service capability exposure that provides a means to securely expose the services and capabilities provided by 3GPP network interfaces. It is expected that signalling data relevant to the IoT application boostrapping service will terminate at this node. As defined in TS23.682, SCEF has interfaces to the MME/SGSN(T6a/T6b), the HSS(S6t), the application server. The HSE functionality could reside as needed with SCEF.

The functionality of relevant interfaces used by this solution are:

-	 T6a: An interface between the SCEF and the MME to allow the SCEF to obtain the UE application layer session key (Ks) based on the UE session Identifier (GUTI).

Editor’s Note: Whether using GUTI which doesn’t have one-to-one correspondence with KASME introduces any security threat is for FFS.

-	S6a: Utilize this existing interface between the HSS and MME to allow HSS to provide a per-UE application layer session key to MME (Ks); In case of HSS providing multiple AVs, HSS provide one session key per AV.

-	Ua: An interface between the UE and the application server that transfers user plane data

#### 6.11.2.2	Security boostrapping and key refreshing

##### 6.11.2.2.1 	Overview

This clause describes how to derive session keys between the UE and the application server utilizing existing 3GPP interfaces and the fundamental 3GPP SCEF functionality which is designed to expose 3GPP services capabilities to trusted third parties, e.g., the application servers. This solution provides an option which allows SCEF to pull UE application layer session keys from the MME serving the specific UE and then deliver the session key as requested to the application server.In this option, HSS derives a per-UE per-AV application layer master session key (Ks) and deliver the session key to the MME over interface S6a in User Authentication Info Respone message. This is a minimum impact on the HSS.

##### 6.11.2.2.2	Key agreement and boostrapping with HSS deriving master session key

New IoT application level master session key is agreed upon when the UE perform 3GPP-AKA procedure while accessing the network, i.e. at the phase when the UE and the network mutually authenticate each other. This proposal requires changes, as described in this document, at UE, MME and HSS while using 3GPP-AKA mechanism as detailed in 3GPP TS 33.102 [27]and 3GPP TS 33.401 [28].

Figure 6.11.2.2.2.1 shows the key agreement and boostrapping process.

Figure 6.11.2.2.2.1: 3GPP-AKA Key Agreement and IoT Application Security Bootstrapping Process

The key steps are as follows:

1)	MME starts access authentication request and requires the identity of the UE.

2)	The UE responses MME with IMSI to identify himself.

3)	The MME sends authentication data request to HSS to ask for materials to authenticate the UE mutually.

4)	HSS receives the request and searches the root key shared with UE using IMSI. HSS generates one or more AV, RAND, XRES, AUTH, CK, IK, KASME as in TS33.401. In addition, HSS will generate Ks based on CK, IK for the application service. The key Ks is generated as Ks=KDF(CK||IK, "End-to-End_IOT").

5)	HSS responds to MME with the Authentication Vector. Authentication Vector (AV) consists of AV=(RAND, AUTH, XRES, KASME) as defined in TS33.401. HSS adds the Application layer master session key (Ks) to map to one-to-ome with the AV.

6)	Note: If HSS sends multiple AVs to MME at a time, then HSS includes multiple  Ks.

7)	Note: It is recommended to distribute only one AV at a time as the frequency of AKA runs is very low in the IOT scenario.

8)	MME sends user authentication request to UE, this request consists of RAND and AUTH as defined in 3GPP TS 33.102 [27] / 3GPP TS 33.401 [28].

9)	UE generates keys and check AUTH to authenticate the network, and computes response message RES. All the procedures are defined as in 3GPP TS 33.102 [27] / 3GPP TS 33.401 [28].

10)	UE sends the authentication response message RES to MME as defined in 3GPP TS 33.102 [27] / 3GPP TS 33.401 [28].

11)	MME checks RES by comparing RES and XRES as defined in 3GPP TS 33.102 [27] / 3GPP TS 33.401 [28].

12)	MME sends Authentication Successful to the UE together with GUTI generated by MME. MME updates the UE context with the respective used AV and Ks.

13)	UE generates the master key Ks and then generates key Ks_AS for application service. Ks_AS is generated in the form Ks_AS = KDF(Ks, AS_ID), where AS_ID is the identity of the application server.

14)	UE sends Application Request to the application server over the Ua interface. This message contains the GUTI received from MME in step 10, and msg. The content of msg depends on specific protocol.

15)	Once receiving the Application Request, the application server sends Key Requset to SCEF with GUTI, AS_ID.

16)	SCEF receives GUTI, SCEF uses the GUTI and other 3GPP operator services, e.g., network DNS, to identify the serving MME.

17)	SCEF send a Mobile Data Request to the MME with the GUTI included.

Note: In case SCEF is not equipped to derive KS-AS, SCEF include the AS_ID in the Mobile Data Request to allow the MME to derive the Ks_AS.

18)	The MME search its database for the UE context of the received GUTI. MME responds to SCEF with Mobile Data Answer with the GUTI and the Ks included.

Note: If the MME receives the GUTI and the AS_ID, MME derives the KS-AS key as KDF (Ks, AS-ID) and return to SCEF in the Mobile Data Answer.

19)	The SCEF responds to the application server with the application key Ks_AS.

20)	The application server returns to the UE with Application Response.

6.11.2.2.2.1	Key Generation in Application Layer.

This proposal presents two possible solutions for the UE and Application to generate session keys. All the session key is based on  Ks_AS. In order to ensure the freshness of the session key, the UE will generate a random number as the input material of the session key.

Session Key Generation without Certificate: The UE generates a random number RAND_AS. In addition, the UE computes encryption key Ks_AS_enc and integrity key Ks_AS_int, respectively. The two keys are generated in the way Ks_AS_enc = KDF(Ks_AS, "enc") and Ks_AS_int = KDF(Ks_AS, "int"). The UE computes the session key as K_app = KDF(Ks_AS_enc, RAND_AS), then to protect the integrity of GUTI and RAND_AS, UE generates the message authentication code t = MACK_AS_int(GUTI, RAND_AS). The UE puts RAND_AS and t into msg in the Application Request. Once received Ks_AS, the application server generates Ks_AS_enc and Ks_AS_int and then verify the message authentication code t. If it is correct, then the application server computes the session key K_app.

Session Key Generation with Certificate: In this solution, the UE is configured with the certificate of the application server. This certificate contains the public key PK_AS of the application server. The UE generates a random number RAND_AS. In addition, the UE computes encryption key Ks_AS_enc and integrity key Ks_AS_int as the above solution. The session key K_app is also generated in the UE as K_app = KDF(Ks_AS_enc, RAND_AS). To protect GUTI and RAND_AS, the public key PK_AS is used to encrypt RAND_AS as C = EncPK_AS(RAND_AS) and the integrity is protected as t = MACKs_AS_int(GUTI, C). The UE puts C and t into msg in the Application Request (RAND_AS is not included in cleartext). Once received Ks_AS, the application server generates Ks_AS_enc and Ks_AS_int and then verify the message authentication code t. If it is correct, the application server uses the private key to decrypt C and get RAND_AS, then computes the session key K_app.

These two solutions ensure the freshness of the session key, this enables that two different sessions between the same UE and application server not to be the same. In addition, the certificate solution provides end-to-end security between the UE and the application server where the wireless operator has no access to the encrypted traffic.

##### 6.11.2.2.3	Key refreshing

In this proposal, the key refreshing could be triggle by both the UE and application server.

Initiated by UE: The procedure is described in Figure 6.11.2.2.3.1

Figure 6.11.2.2.3.1: Key Refreshing Initiated by the UE

The steps are as follows:

1)	UE initiates key refreshing by sending a Refresh Request to the application server. This message will contain the GUTI.

2)	The application server forwards the Refresh Request to the SCEF.

3)	A Bootstrapping Renegotiation Request is sent to MME by SCEF.

4)	MME then will re-authenticate the UE by initiating a 3GPP-AKA procedure.

Initiated by application server: The procedure is decribed in Figure 6.11.2.2.3.2

Figure 6.11.2.2.3.2: Key Refreshing Initiated by the Application Server

The steps are similar to the case that the UE intiating procedure except that the application server start the Refresh Request.

### 6.11.3	Solution evaluation

This solution creates an end-to-end key shared between UE, IoT Device, and the application server utilizing existing 3GPP architecture and interfaces, that can be used to provide confidentiality and integrity. This solution eliminates the need for GBA-AKA and optimizes the power usage at the IoT device. The solution addresses key agreement between UE and the Application Server.

## 6.12	Solution #12: A method for IoT service layer security bootstrapping solution

### 6.12.1	Introduction

This document proposes an IoT service layer security bootstrapping solution as described below. This solution provides a mechanism for deriving session keys between the IoT device and the application server with the following advantages.

Address all Key Issues:

1)	Key Issue No. 1: It provides integrity protection and confidentiality to the IoT device and the application server which can be used by a transport protocol to secure communications.. Confidentiality is invoked as per the legal regulatory of the serving network.

2)	Key Issue No. 2: Address the efficient user data protection challenges by allowing keys of at least 128 bits.

Address main performance objective of the study:

3)	Optimization of transmitted flows to reduce battery consumption by eliminating the need for another AKA procedure of GBA (see 3GPP TS 33.220[20]), i.e., GBA-AKA, and thus saves at least four message flows of the procedure, then reduces the power consumption at the IoT device.

4)	Use existing 3GPP architecture and network entities/nodes with minimum changes to existing interfaces to accomplish the objectives of this technical report.

### 6.12.2	Solution description

#### 6.12.2.1	Proposed architecture

Figure 6.12.2.1.1 Proposed Architecture for Solution #10

The key network element is:

-	Service Capability Exposure Function (SCEF): This is the key entity within the 3GPP architecture for service capability exposure that provides a means to securely expose the services and capabilities provided by 3GPP network interfaces. It expected that the some signalling data relevant to the boostrapping service will terminate at this node. As defined in 3GPP TS 23.682[46], SCEF has interfaces to the MME/SGSN(T6a/T6b), the HSS(S6t), the application server. The HSE functionality could reside as needed with SCEF.

The functionality of relevant interfaces used in this solution are:

-	T6a: An interface between the SCEF and the MME to allow the SCEF obtain the authentication result of the UE and return feedbacks to the MME.

-	S6t: An interface between the SCEF and the HSS that allows the SCEF to obtain the session key for application and subscription and UE related information.

-	S6a: Utilize this existing interface between the HSS and MME to allow HSS to provide an indication to MME that this UE supports current IoT security bootstapping and MME needs to push the network authentication result together with other subscriber information to SCEF.

-	Ua: An interface between the UE and the application server that transfers user plane data.

#### 6.12.2.2	Security boostrapping and key refreshing

##### 6.12.2.2.1	Overview

This clause describes how to derive session keys between the UE and the application server utilizing existing 3GPP interfaces and the fundamental 3GPP SCEF functionality which is designed to expose 3GPP services capabilities to trusted third parties, e.g., the application servers. This solution provides an option which allows HSS pushes the application layer key to SCEF serving the specific UE over the S6t interface.

##### 6.12.2.2.2	Key agreement and boostrapping

New IoT application level master session key is agreed upon when the UE perform 3GPP-AKA procedure while accessing the network, i.e. at the phase when the UE and the network mutually authenticate each other. This proposal requires changes, as described in this document, at UE, MME and HSS while using 3GPP-AKA mechanism as detailed in 3GPP TS 33.102 [27] and 3GPP TS 33.401 [28].

Figure 6.12.2.2.2.1 shows the key agreement and boostrapping process:

Figure 6.12.2.2.2.1: Key Agreement and Bootstrapping Process

The key steps are as follows:

1)	MME starts access authentication request and requires the identity of the UE.

2)	The UE responses MME with IMSI to identify himself.

3)	The MME sends authentication data request to HSS to ask for materials to authenticate the UE mutually.

4)	HSS receives the request and searches the root key shared with UE using IMSI. HSS generates one or more AV, RAND, XRES, AUTH, CK, IK, KASME as in TS33.401. In addition, HSS will generate Ks based on CK, IK for the application service. The key Ks is generated as Ks=KDF(CK||IK, "End-to-End_IOT").

5)	HSS responds to MME with the Authentication Vector. Authentication Vector (AV) consists of AV=(RAND, AUTH, XRES, KASME) as defined in 3GPP TS 33.401 [28]. HSS adds an indication to MME to indicate that MME needs to push the network authentication result and other subscribe information to SCEF.

5.1)	HSS pushes to SCEF the KeySet which is later used to be delivered to the application server. The KeySet contains the RAND from AV, IMSI and the key Ks.  This message is over S6t interface.

Note1 : If HSS sends multiple AVs to MME at a time, then HSS will pushes multiple RAND, Ks to SCEF (still with the same IMSI). SCEF will store all these information.

Note2: It is recommended to distribute only one AV at a time as the frequency of AKA runs is very low in the IOT scenario.

6)	MME sends user authentication request to UE, this request consists of RAND and AUTH as defined in 3GPP TS 33.102 [27] and 3GPP TS 33.401 [28].

7)	UE generates keys and check AUTH to authenticate the network, and computes response message RES. All the procedures are defined as in 3GPP TS 33.102 [27] and 3GPP TS 33.401 [28].

8)	UE sends the authentication response message RES to MME as defined in 3GPP TS 33.102 [27] and 3GPP TS 33.401 [28].

9)	MME checks RES by comparing RES and XRES as defined in 3GPP TS 33.102 [27] and 3GPP TS 33.401 [28].

10)	MME sends Authentication Successful to the UE together with GUTI generated by MME.

Editor’s Note: Whether using GUTI which doesn’t have one-to-one correspondence with KASME introduces any security threat is for FFS.

10.1 MME pushes the Notification message over the T6a interface to SCEF. The Notification message contains Authentication Result, RAND (from AV), IMSI and GUTI.

11)	UE generates the master key Ks and then generates key Ks_AS for application service. Ks_AS is generated in the form Ks_AS = KDF(Ks, AS_ID), where AS_ID is the identity of the application server.

12)	UE sends Application Request to the application server over the Ua interface. This message contains the GUTI received from MME in step 10, and msg. The content of msg depends on specific protocol.

13)	Once receiving the Application Request, the application server sends Key Requset to SCEF with GUTI, AS_ID.

14)	SCEF receives GUTI, searches the database and finds out the state of the  Authentication Result according to this GUTI, If the state is Successful, then SCEF will find the application key Ks with RAND and IMSI. Then, SCEF generate application key Ks_AS based on Ks and AS_ID.

15)	The SCEF responds to the application server with the application key Ks_AS.

16)	The application server returns to the UE with Application Response.

6.12.2.2.2.1	Key generation in application layer.

This proposal presents two options for the UE and Application to generate session keys depending on whether certificate is used or not. All the session keys is based on Ks_AS. In order to ensure the freshness of the session key, the UE will generate a random number as the input material of the session key.

Session Key Generation without Certificate: The UE generates a random number RAND_AS. In addition, the UE computes encryption key Ks_AS_enc and integrity key Ks_AS_int, respectively. The two keys are generated in the way Ks_AS_enc = KDF(Ks_AS, "enc") and Ks_AS_int = KDF(Ks_AS, "int"). The UE computes the session key as K_app = KDF(Ks_AS_enc, RAND_AS), then to protect the integrity of GUTI and RAND_AS, UE generates the message authentication code t = MACKs_AS_int(GUTI, RAND_AS). The UE puts RAND_AS and t into msg in the Application Request. Once received Ks_AS, the application server generates Ks_AS_enc and Ks_AS_int and then verify the message authentication code t. If it is correct, then the application server computes the session key K_app.

Session Key Generation with Certificate: In this solution, the UE is configured with the certificate of the application server. This certificate contains the public key PK_AS of the application server. The UE generates a random number RAND_AS. In addition, the UE computes encryption key Ks_AS_enc and integrity key Ks_AS_int as the above solution. The session key K_app is also generated in the UE as K_app = KDF(Ks_AS_enc, RAND_AS). To protect GUTI and RAND_AS, the public key PK_AS is used to encrypt RAND_AS as C = EncPK_AS(RAND_AS) and the integrity is protected as t = MACKs_AS_int(GUTI, C). The UE puts C and t into msg in the Application Request (RAND_AS is not included in cleartext). Once received Ks_AS, the application server generates Ks_AS_enc and Ks_AS_int and then verify the message authentication code t. If it is correct, the application server uses the private key to decrypt C and get RAND_AS, then computes the session key K_app.

These two options ensure the freshness of the session key, this enables that two different sessions between the same UE and application server not to be the same. In addition, the certificate solution provides end-to-end security between the UE and the application server where the wireless operator has no access to the encrypted traffic.

##### 6.12.2.2.3	Key refreshing

In this proposal, the key refreshing could be triggle by both the UE and application server.

Initiated by UE: The procedure is described in Figure 6.12.2.2.3.1

Figure 6.12.2.2.3.1: Key Refreshing Initiated by the UE

The steps are as follows:

1)	UE initiates key refreshing by sending a Refresh Request to the application server. This message will contain the GUTI.

2)	The application server forwards the Refresh Request to the SCEF.

3)	A Bootstrapping Renegotiation Request is sent to MME by SCEF.

4)	MME then will re-authenticate the UE by initiating a 3GPP-AKA procedure.

Initiated by application server: The procedure is decribed in Figure 6.12.2.2.3.2

Figure 6.12.2.2.3.2: Key Refreshing Initiated by the Application Server

The steps are similar to the case that the UE intiating procedure except that the application server start the Refresh Request.

### 6.12.3	Solution evaluation

This solution creates an end-to-end key shared between UE, IoT Device, and the application server utilizing existing 3GPP architecture and interfaces, that can be used to provide confidentiality and integrity. This solution eliminates the need for GBA-AKA and optimizes the power usage at the IoT device. The solution addresses key agreement between UE and the Application Server.

# 7	Conclusions

## 7.1	Issues identified

The present document identifies that:

-	Using current traditional end to end security methods such as IPsec and DLTS may have significant power and network traffic capacity overheads when used by battery efficient devices to transfer small data packets over a 3GPP system.

-	For low power MTC devices, end to middle security that reuses 3GPP security procedures and keys is likely to be significantly more efficient than end to end security. The current 3GPP system can only secure information between the UE and the SGSN of the visited network and does not extend to the home GGSN.

-	Any security mechanism specified by 3GPP for end to middle security needs to take into account local and home regulations on encryption of data and lawful intercept.

The report identifies the following key issues relating to battery efficient machine type communication:

-	N-PDU data tampering and eavesdropping – It recommends that an integrity mechanism be provided to prevent tampering of N-PDU data and a confidentiality mechanism (where allowed) to prevent eavesdropping on the data content.

-	Efficient user data protection – It recommends that the minimum key size for confidentiality should be 128 bits and where a HMAC is used this should not be less than 64bits.

-	Visited PLMN specific needs – It recommends that any proposed solution should have mechanisms to disable confidentiality protection where local laws in the visited country require this. Also mechanisms need to exist for Lawful intercept of data where applicable.

-	End to End security – The enterprise should have control over the frequency of derivation of the key material, the key material exchanges should be minimised and that higher level key material should not leave the 3GPP or operator domain.

## 7.2	Solution evaluation summary

The report proposes the following solutions:

-	Solution #1 "UE to HPLMN security based on UMTS/EPS AKA enhancements". This solution uses enhancements to the 3GPP AKA to generate key material used by the end to end or end to middle security mechanisms. The solution is intended for use over the cellular IoT RAT where the middle endpoint is a HSE. The solution proposes a mechanism that protects the key materials from being exposed to the visited network but does not cover the mechanisms that use these keys to deliver integrity protection and confidentiality protection. Variants of this solution are proposed that differ in how and when the key material is delivered to the non-UE end of the communication as follows:

-	"Variant A" where the HSS/HLR pushes key materials to the relevant HSE when an AKA generation occurs that includes end to middle key generation. In this solution the HSE needs to store the generated keys until they are used.

-	"Variant B" where the HSS/HLR pushes key materials to the relevant HSE when a PDP context activation occurs following a successful AKA exchange with the UE that includes end to middle key generation. In this solution the HSS/HLR needs to store the generated keys until they are used.

-	"Variant C" where the HSE pulls the key materials from the HSS/HLR when an PDP context activation occurs following a successful AKA exchange with the UE that includes end to middle key generation. In this solution the HSS/HLR needs to store the generated keys until they are used.

-	"Variant D" where the HSS/HLR pushes key materials to a new standardised key store entity called an End to Middle Key Server (EMKS) when an AKA generation occurs that includes end to middle key generation. The HSE pulls the key materials from the EMKS when a PDP context activation occurs following a successful AKA exchange with the UE that includes end to middle key generation. In this solution the EMKS needs to store the generated keys until they are used.

-	"Variant E" is a modification of variants a, b, c and d where the USIM only performs an unmodified AKA procedure and the creation of the local end to middle key values is calculated on the ME.

-	Solution #2 "End to middle security based on AKA with an EMSE". This solution defines a new middle endpoint called an E2M Security Endpoint (EMSE). This solution uses the EMKS defined in solution #1d to store the end to middle keysets and the processes in solution #1e where the UE end to middle keys are generated in the ME so that no modifications to the current USIM is required. The solution proposes a mechanism that protects the key materials from being exposed to the visited network but does not cover the mechanisms that use these keys to deliver integrity protection and confidentiality protection.

-	Solution #3 "Independent VPLMN and e2m security associations". This solution proposes a separate specific AKA run to generate the end to middle key material when they are needed so that there is no inter-relationship or related synchronisation issues with the AKA run for network access. The solution does not require modification of the HSS/HLR or the USIM but does not cover the mechanisms that use these keys to deliver integrity protection and confidentiality protection and uses more power/network resources to deliver as it requires 2 AKA runs. Although this solution requires two AKA procedures initially (Solution 1 and 2 only require one AKA procedure), further AKA runs only depend on the use and timing of the user plane data and not on any aspect of a 3GPP RAT.

-	Solution #4 "Security Policies". This solution describes the sharing of security policies between the HPLMN and Visited PLMN that relate to the frequency and type of security features supported so that the visited network can identify CIoT devices and adjust their security model appropriately. This solution does not address the agreement on key materials or the mechanisms that use these keys to deliver integrity protection and confidentiality protection.

-	Solution #5 "End to End security solution". This solution introduces a new element, the End to End Security Endpoint (EESE) that allows an authorised third party to fetch end to end keysets that have been agreed using one of the methods detailed in solutions 1, 2 or 3. It also presents a key aging and re-agreement mechanism using a timer. It does not however detail the security mechanisms that these keys are used with.

-	Solution #6 "Bearer protection". This solution introduces IPsec ESP for authentication, integrity and confidentiality protection of the user-plane between the UE and the HSE using the keys derived by the UE and HSS and distributed to the HSE according to solutions #1 or #2. For crypto algorithms the solution points the 33.210. The solution does not stipulate on tunnel vs. transport mode ESP and on compression methods. Furthermore the solution does not define the derivation or negotiation of the IPsec ESP Security Association parameters.

-	Solution #7 "End to End". The solution extends solutions 1 and 2 to End to End security. However this solution does not currently deal with mechanisms for LI and optional data cyphering.

-	Solution #8 "Complete end to middle solution".  This solution describes a full BEST service including service discovery, session management, secure operation and error management.  All of the key issues are addressed fully in this solution.-	Solution #9 "Complete end to emd solution".  This solution describes a full BEST service including service discovery, session management, secure operation and error management.  All of the key issues are addressed fully in this solution.

-	Solution #9 " Complete end to emd solution".  This solution describes a full BEST service including service discovery, session management, secure operation and error management.  All of the key issues are addressed fully in this solution.

- 	Solution #10 "AKA-based session key generation for application protocols": This solution provides a mechanism to use AKA-based session key generation as an independent module usable by any security protocol at the application layer. It leaves security context negotiation (apart from the keys) and protection of transferred data to the application layer protocol. The solution proposes a mechanism that protects the key materials from being exposed to the visited network but does not cover the mechanisms that use these keys to deliver integrity protection and confidentiality protection.

.-	Solution #11 "Service Layer Bootstrapping Solution".  This solution describes an IoT service layer security bootstrapping that allows key network element SCEF to pull application session keys from MME for the application layer. The solution addresses key agreement between UE and the Application Server.

.-	Solution #12 "Service Layer Bootstrapping Solution".  This solution describes an IoT service layer security bootstrapping that allows HSS to push application key to SCEF for the application layer. The solution addresses key agreement between UE and the Application Server.

The solutions are further summarised as follows:

Table 7.2-1: Summary overview of the proposed solutions

| Solution | Endpoints | Tampering and eavesdropping protection | Efficient device power and network use | Visited PLMN needs addressed | Confidentiality and integrity mechanism specified | Nodes potentially effected |
| --- | --- | --- | --- | --- | --- | --- |
| 1A | E2M | Yes | efficient | Yes | No | HSS/HLR, USIM, ME, SGSN, GGSN, MME, S-GW and P-GW |
| 1B | E2M | Yes | efficient | Yes | No | HSS/HLR, USIM, ME, SGSN, GGSN, MME, S-GW and P-GW |
| 1C | E2M | Yes | efficient | Yes | No | HSS/HLR, USIM, ME, SGSN, GGSN, MME, S-GW and P-GW |
| 1D | E2M | Yes | efficient | Yes | No | EMKS, HSS/HLR, USIM, ME, SGSN, GGSN, MME, S-GW and P-GW |
| 1E | E2M | Yes | efficient | Yes | No | EMKS, HSS/HLR, and ME |
| 2 | E2M | Yes | efficient | Yes | No | EMKS, HSS/HLR, USIM, ME, SGSN, GGSN, MME, S-GW and P-GW |
| 3 | E2M | Yes | less efficient | Yes | No | EMKS and ME |
| 4 | - | Yes | efficient | Yes | No | UE |
| 5 | E2E | Yes | efficient | No | No | EMKS, EESE and UE |
| 6 | E2M | Yes | efficient | No | No | HSS/HLR, USIM, ME, SGSN, GGSN, MME, S-GW and P-GW |
| 7 | E2E | Yes | efficient | No | No | HSS/HLR, USIM, ME, SGSN, GGSN, MME, S-GW and P-GW |
| 8 | E2M | Yes | efficient | Yes | Yes | UE, USIM, MME, HSS and HSE |
| 9 | E2E | Yes | Efficient | Yes | Yes | UE, USIM, MME, HSS, HSE and EnSE |
| 10 | E2E | Depends on the application layer security protocol | Depends on the application layer security protocol | No | No (Set up by the application layer protocol) | USIM, HSS/HLR, ME, EMKS |
| 11 | E2E | Depends on the application layer security protocol | Depends on the application layer security protocol | No | No | UE, MME, HSS |
| 12 | E2E | Depends on the application layer security protocol | Depends on the application layer security protocol | No | No | UE, MME, HSS |

## 7.3	Recommendation for normative work

This document recommends that solutions 8 (end to middle security) and 9 (end to end security) are the basis for normative specifications.  This should be carried out in a modular fashion to allow as much re-use of functionality as possible and so that the solution can be easily extended to new algorithms, new security transport protocols and new bearers.  Specifically, a module for the establishment of pre-shared keys based on AKA needs to be defined in the normative work that could be used by a pre-shared key protocol (e.g., pskDTLS, IKE).  Furthermore, the normative solution should aim not to require changes of following core network nodes MME, P-GW, S-GW, SGSN, and GGSN, except when required to meet LI requirements (e.g. in MME). It needs to be determined as part of the normative work as to whether this can be achieved by solution 8 and 9 or whether additional solutions need to be considered (e.g. solution #10).  As this technical report is focussed on Machine Type Communications, the normative specifications should be defined for both EPS and eGPRS.  These solutions could be specified for UMTS, but it is not currently a focus for Machine Type Communications.

Annex A: 
AKA procedures assessment in very low data throughput environment

# A.1	Introduction

In order to assess whether a security protocol is suitable for use in a situation wherein the battery capacity is limited and wherein the data rate is very low, it is necessary to provide measurable information to evaluate the potential impacts on the MTC device's battery life and the quality of service.

The following presents the description of UMTS AKA, DTLS or GBA procedures that are likely to be used in M2M or in a Cellular IoT context (as in defined by GERAN in 3GPP TR 45.820 [25]).

According to 3GPP TR 45.820 [25], the minimum data throughput over one channel is 160 bps for both uplink and downlink. This throughput will be taken as the base to calculate the time to transmit.

A.2 is the breakout example of the UMTS AKA procedure.

A.3 is the breakout example of DTLS 1.2  handshake procedure with ECDHE-ECDSA ciphersuite.

A.4 is the overhead description of a DTLS record header for ciphered data

A.5 is the breakout example of a TLS handshake session resumption procedure.

A.6 is the breakout of GBA with the shared-based mutual authentication procedure between UE and NAF.

# A.2	UMTS AKA

The following tables describe the messages during a successful UMTS AKA procedure which are exchanged between a UE and a SGSN. The authentication message request sent from the SGSN to the UE is defined in 3GPP TS 24.008 [12] and can be described as below:

Table A.2-1: Authentication and Ciphering Request

| Payload Name | M/O/C | Parameter name | min size (byte) | max size (byte) | min size (bits) | max size (bits) | Case (bits) | Transmit time (s) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Authentication and Ciphering Request |  |  |  |  |  |  |  |  |  |
|  | M | protocol discriminator |  |  | 4 | 4 | 4 |  |  |
|  | M | Skip indicator |  |  | 4 | 4 | 4 |  |  |
|  | M | message type | 1 | 1 | 8 | 8 | 8 |  |  |
|  | M | Ciphering algorithm |  |  | 4 | 4 | 4 |  |  |
|  | M | IMEISV request |  |  | 4 | 4 | 4 |  |  |
|  | M | Force to standby |  |  | 4 | 4 | 4 |  |  |
|  | M | A&C reference number |  |  | 4 | 4 | 4 |  |  |
|  | O | Authentication parameter RAND | 17 | 17 | 136 | 136 | 136 |  |  |
|  | C | Cipher key sequence number | 1 | 1 | 8 | 8 | 8 |  | only included if RAND is present |
|  | O | Authentication parameter AUTN | 18 | 18 | 144 | 144 | 144 |  |  |
| Final total |  |  |  |  | 320 | 320 | 320 | 2 |  |
| Ciphering Key Sequence Number IE |  |  |  |  |  |  |  |  |  |
|  |  | IEI |  |  |  | 4 |  |  |  |
|  |  | spare |  |  |  | 1 |  |  |  |
|  |  | key sequence |  |  |  | 3 |  |  |  |
| Total |  |  |  |  |  | 8 |  |  |  |
| Authentication Parameter IE |  |  |  |  |  |  |  |  |  |
|  |  | RAND IEI |  | 1 |  | 8 |  |  |  |
|  |  | RAND value |  | 16 |  | 128 |  |  |  |
| Total |  |  |  |  |  | 136 |  |  |  |
| Authentication Parameter AUTN IE |  |  |  |  |  |  |  |  |  |
|  |  | AUTN IEI |  | 1 |  | 8 |  |  |  |
|  |  | Length of AUTN |  | 1 |  | 8 |  |  |  |
|  |  | AUTN |  | 16 |  | 128 |  |  |  |
| Total |  |  |  |  |  | 144 |  |  |  |
| AUTN |  |  |  |  |  |  |  |  |  |
|  |  | SQN xor AK |  |  |  | 48 |  |  |  |
|  |  | AMF |  |  |  | 16 |  |  |  |
|  |  | MAC |  |  |  | 64 |  |  |  |
| Total |  |  |  |  |  | 128 |  |  |  |

The authentication message response sent from the UE to the SGSN is defined in 3GPP TS 24.008 [12] and can be described as below:

Table A.2-2: Authentication and Ciphering Response

| Payload Name | M/O/C | Parameter name | min size (byte) | max size (byte) | min size (bits) | max size (bits) | Case (bits) | Transmit time (s) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Authentication and Ciphering Response |  |  |  |  |  |  |  |  |  |
|  | M | protocol discriminator |  |  | 4 | 4 | 4 |  |  |
|  | M | Skip indicator |  |  | 4 | 4 | 4 |  |  |
|  | M | message type | 1 | 1 | 8 | 8 | 8 |  |  |
|  | M | A&C reference number |  |  | 4 | 4 | 4 |  |  |
|  | M | spare |  |  | 4 | 4 | 4 |  |  |
|  | O | Authentication Response parameter | 5 | 5 | 40 | 40 | 40 |  |  |
|  | O | IMEISV | 11 | 11 | 88 | 88 | 0 |  |  |
|  | O | Authentication Response parameter (extension) | 3 | 14 | 24 | 112 | 0 |  |  |
| Final total |  |  |  |  | 176 | 264 | 64 | 0.4 |  |
| Authentication Response parameter IE |  |  |  |  |  |  |  |  |  |
|  |  | IEI |  | 1 |  | 8 | 8 |  |  |
|  |  | RES (most significant bits) |  | 4 |  | 32 | 32 |  |  |
| Total |  |  |  |  |  | 40 | 40 |  |  |
| Authentication Response parameter (extension) IE |  |  |  |  |  |  |  |  |  |
|  |  | IEI |  | 1 |  | 8 |  |  |  |
|  |  | Length |  | 1 |  | 8 |  |  |  |
|  |  | RES (all but 4 most significant octets) | 0 | 12 |  | 96 |  |  |  |
| Total |  |  |  |  |  | 112 |  |  |  |

# A.3	DTLS handshake for ECDHE-ECDSA configuration

## A.3.1	DTLS handshake procedure measurement

The following describes the DTLS Record and Handshake messages exchanged for the ECDHE-ECDSA configuration sent between the UE and a Server is defined in IETF RFC 6347 [33] and IETF RFC 4492 [34].

Table A.3.1-1: High level DTLS Handshake procedure measurement

| DTLS handshake for ECDHE-ECDSA |  |  |  |  |
| --- | --- | --- | --- | --- |
| Direction | Step | Message Name | Size (bits) | transmit time (s) |
| C -> S | 1 | ClientHello | 608 |  |
| subtotal |  |  | 608 |  |
| subtotal with compressed IP/UDP headers |  |  | 704 | 4.4 |
| subtotal with IP/UDP headers |  |  | 928 | 5.8 |
| S -> C | 2 | ServerHello | 728 |  |
| S -> C | 2 | Server Certificate | 800 |  |
| S -> C | 2 | ServerKeyExchange | 1128 |  |
| S -> C | 2 | ServerHelloDone | 72 |  |
| subtotal |  |  | 2728 |  |
| subtotal with compressed IP/UDP headers |  |  | 2824 | 17.65 |
| subtotal with IP/UDP headers |  |  | 3048 | 19.05 |
| C -> S | 3 | ClientKeyExchange | 592 |  |
| C -> S | 3 | ChangeCipherSpec | 48 |  |
| C -> S | 3 | Finished | 168 |  |
| subtotal |  |  | 808 |  |
| subtotal with compressed IP/UDP headers |  |  | 904 | 5.65 |
| subtotal with IP/UDP headers |  |  | 1,128 | 7.05 |
| S -> C | 4 | ChangeCipherSpec | 48 |  |
| S -> C | 4 | Finished | 168 |  |
| subtotal |  |  | 216 |  |
| subtotal with compressed IP/UDP headers |  |  | 312 | 1.95 |
| subtotal with IP/UDP headers |  |  | 536 | 3.35 |
| TOTAL with compressed IP/UDP headers |  |  | 4,744 | 29.65 |
| TOTAL with IP/UDP headers |  |  | 5,640 | 35.25 |

## A.3.2	TLS Record and Handshake message measurement

The TLS Record message is defined as follow:

Table A.3.2-1: Record message header measurement

| Payload Name | M/O/C | Parameter name | min size (byte) | max size (byte) | min size (bits) | max size (bits) | ECDHE ECDSA Case (bits) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RECORD MESSAGE |  |  |  |  |  |  |  |  |
|  |  | Content Type | 1 | 1 | 8 | 8 | 8 |  |
|  |  | Protocol Version | 2 | 2 | 16 | 16 | 16 |  |
|  |  | Length | 2 | 2 | 16 | 16 | 16 |  |
|  |  | Fragment | 0 | 16384 | 0 | 131072 | message dependent | The Handshake messages will be contained within a Record Message |
| header total |  |  |  |  |  |  | 40 |  |
| final total |  |  | 5 | 16389 | 40 | 131112 | message dependent |  |

The TLS record encapsulates Handshake messages (as well as Change Cipher Spec messages) which are defined as follows:

Table A.3.2-2: Handshake message header measurement

| Payload Name | M/O/C | Parameter name | min size (byte) | max size (byte) | min size (bits) | max size (bits) | ECDHE ECDSA Case (bits) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HANDSHAKE MESSAGE |  |  |  |  |  |  |  |  |
|  |  | Handshake type | 1 | 1 | 8 | 8 | 8 |  |
|  |  | Length | 3 | 3 | 24 | 24 | 24 |  |
|  |  | body | 0 | 4095 |  |  | message dependent | The message content itself e.g. the ClientHello |
| header total |  |  |  |  |  |  | 32 |  |
| final total |  |  |  |  |  |  | message dependent |  |

The TLS Handshake messages are further described as follows:

Table A.3.2-3: ClientHello message measurement

| Payload Name | Parameter name | Size (bits) | Comments |
| --- | --- | --- | --- |
| ClientHello ECDHE ECDSA case (Direction: Client -> Server) |  |  |  |
|  | Protocol Version | 16 |  |
|  | Random | 256 |  |
|  | Session ID | 8 | first time (only length byte) |
|  | Cipher Suites | 48 | One cipher suite e.g. TLS_ECDHE_ECDSA_WITH_AES_128_CCM_8, 2 length bytes, renegotiation ciphersuite |
|  | Compression methods | 16 | Length byte + NULL |
|  | extensions | 192 | 2 byte total ext length field + 2 byte Sig Alg ext ID + 2 byte ext len + 2 byte list len + Sig Alg (2) + 2 byte Curve ext ID + 2 byte ext len + 2 byte list len + 2 byte curve ID + 2 byte Point Format ext ID + 2 byte ext len + 1 byte list len + 1 byte format. |
| final total |  | 536 |  |
| final total with record and handshake headers |  | 608 |  |

Table A.3.2-4: ServerHello message measurement

| Payload Name | Parameter name | Size (bits) | Comments |
| --- | --- | --- | --- |
| ServerHello ECDHE ECDSA case (Direction: Server -> Client) |  |  |  |
|  | Protocol Version | 16 |  |
|  | Random | 256 | Independently generated from the ClientHello.Random |
|  | Session ID | 256 | The assigned Session ID |
|  | Cipher Suite | 16 | One cipher suite e.g. TLS_ECDHE_ECDSA_WITH_AES_128_CCM_8 |
|  | Compression methods | 8 | null |
|  | extensions | 104 | e.g. brainpoolP256r1(26); 2 byte total ext length field + 2 byte Renego ext ID + 2 byte ext len + 1 byte + 2 byte Point Format ext ID + 2 byte ext len + 1 byte list len + 1 byte format |
| final total |  | 656 |  |
| final total with record and handshake headers |  | 728 |  |

Table A.3.2-5: Certificate message measurement

| Payload Name | Parameter name | Size (bits) | Comments |
| --- | --- | --- | --- |
| Certificate ECDHE ECDSA case (Direction: Server -> Client) |  |  |  |
| final total | ASN.1Cert certificate list | 728 | example Raw Public Certificate ECDSA P-256: 91 bytes |
| final total with record and handshake headers |  | 800 |  |

Table A.3.2-6: ServerKeyExchange message measurement

| Payload Name | Parameter name | Size (bits) | Comments |
| --- | --- | --- | --- |
| ServerKeyExchange ECDHE ECDSA case (Direction: Server -> Client) |  |  |  |
|  | ServerECDHParams | 544 |  |
|  | Signature | 512 |  |
| final total |  | 1056 |  |
| final total with record and handshake headers |  | 1128 |  |

Table A.3.2-7: ServerKeyExchange message measurement

| Payload Name | Parameter name | Size (bits) | Comments |
| --- | --- | --- | --- |
| ServerHelloDone ECDHE ECDSA case (Direction: Server -> Client) |  |  |  |
| total |  | 0 |  |
| final total with record and handshake headers |  | 72 |  |

Table A.3.2-8: ClientKeyExchange message measurement

| Payload Name | Parameter name | Size (bits) | Comments |
| --- | --- | --- | --- |
| ClientKeyExchange ECDHE ECDSA case (Direction: Client -> Server) |  |  |  |
|  | exchange keys | 520 | The client selects an ephemeral ECDH public key corresponding to the parameters it received from the server according to the ECKAS-DH1 scheme from IEEE 1363 |
| final total |  | 520 |  |
| final total with record and handshake headers |  | 592 |  |

Table A.3.2-9: ChangeCipherSpec message measurement

| Payload Name | Parameter name | Size (bits) | Comments |
| --- | --- | --- | --- |
| ChangeCipherSpec ECDHE ECDSA case (Direction: Client -> Server) |  |  |  |
|  | Change cipher spec | 8 | To help avoid pipeline stalls, ChangeCipherSpec is an independent TLS protocol content type, and is not actually a TLS handshake message. |
| final total |  | 8 |  |
| final total with record header |  | 48 |  |

NOTE: ChangeCipherSpec messages aren't handshake messages and are directly encapsulated in a record message.

Table A.3.2-10: Finished message measurement

| Payload Name | Parameter name | Size (bits) | Comments |
| --- | --- | --- | --- |
| Finished ECDHE ECDSA case (Direction: Client -> Server) |  |  |  |
|  | verify data | 96 |  |
| final total |  | 96 |  |
| final total with record and handshake headers |  | 168 |  |

Table A.3.2-11: ChangeCipherSpec message measurement

| Payload Name | Parameter name | Size (bits) | Comments |
| --- | --- | --- | --- |
| ChangeCipherSpec ECDHE ECDSA case (Direction: Server -> Client) |  |  |  |
|  | Change cipher spec | 8 | To help avoid pipeline stalls, ChangeCipherSpec is an independent TLS protocol content type, and is not actually a TLS handshake message. |
| final total |  | 8 |  |
| final total with record header |  | 48 |  |

Note: ChangeCipherSpec messages aren't handshake messages and are directly encapsulated in a record message.

Table A.3.2-12: Finished message measurement

| Payload Name | Parameter name | Size (bits) | Comments |
| --- | --- | --- | --- |
| Finished ECDHE ECDSA case (Direction: Server -> Client) |  |  |  |
|  | verify data | 96 |  |
| final total |  | 96 |  |
| final total with record and handshake headers |  | 168 |  |

# A.4	DTLS record header overhead description for ciphered data

## A.4.1	DTLS record header measurement

The following describes the overhead of DTLS record header per ciphered data:

Table A.4.1-1: DTLS Record Layer per-packet overhead measurement

| Parameter name | min size (bits) | max size (bits) | ECDHE ECDSA Case (bits) | Comments |
| --- | --- | --- | --- | --- |
| DTLS Record Layer Per-Packet Overhead (see also Appendix B of IETF dice profile 10) |  |  |  |  |
| DTLS record layer header | 104 | 104 | 104 |  |
| nonce | 64 | 64 | 64 | RFC 6655 [RFC6655] allows the nonce_explicit to be a sequence number or something else. This document makes this use more restrictive for use with DTLS: the 64-bit none_explicit needs to be the 16-bit epoch concatenated with the 48-bit seq_num. The sequence number component of the nonce_explicit field at the AES-CCM layer is an exact copy of the sequence number in the record layer header field. This leads to a duplication of 8-bytes per record. To avoid this 8-byte duplication RFC 7400 [RFC7400] provides help with the use of the generic header compression technique for IPv6 over Low-Power Wireless Personal Area Networks (6LoWPANs). Note that this header compression technique is not available when DTLS is exchanged over transports that do not use IPv6 or 6LoWPAN, such as the SMS transport described in Appendix A. |
| ICV | 64 | 64 | 64 |  |
| total |  |  | 232 |  |
| DTLS record layer header |  |  |  |  |
| content type | 8 | 8 |  |  |
| version | 16 | 16 |  |  |
| epoch | 16 | 16 |  |  |
| sequence number | 48 | 48 |  |  |
| length | 16 | 16 |  |  |

# A.5	TLS handshake session resumption

## A.5.1	TLS handshake session resumption procedure measurement

The following describes the TLS Record and Handshake messages exchanged for the ECDHE-ECDSA configuration for session resumption between the UE and a Server.

Table A.5.1-1: High level DTLS Handshake session resumption procedure measurement

| DTLS handshake for ECDHE-ECDSA |  |  |  |  |
| --- | --- | --- | --- | --- |
| Direction | Step | Message Name | Size (bits) | transmit time (s) |
| C -> S | 1 | ClientHello | 864 |  |
| subtotal |  |  | 864 |  |
| subtotal with compressed IP/UDP headers |  |  | 960 | 6 |
| subtotal with IP/UDP headers |  |  | 1184 | 7.4 |
| S -> C | 2 | ServerHello | 736 |  |
| subtotal |  |  | 736 |  |
| subtotal with compressed IP/UDP headers |  |  | 832 | 5.2 |
| subtotal with IP/UDP headers |  |  | 1056 | 6.6 |
| C -> S | 3 | ChangeCipherSpec | 48 |  |
| C -> S | 3 | Finished | 168 |  |
| subtotal |  |  | 216 |  |
| subtotal with compressed IP/UDP headers |  |  | 312 | 1.95 |
| subtotal with IP/UDP headers |  |  | 536 | 3.35 |
| S -> C | 4 | ChangeCipherSpec | 48 |  |
| S -> C | 4 | Finished | 168 |  |
| subtotal |  |  | 216 |  |
| subtotal with compressed IP/UDP headers |  |  | 312 | 1.95 |
| subtotal with IP/UDP headers |  |  | 536 | 3.35 |
| TOTAL with compressed IP/UDP headers |  |  | 2,416 | 15.1 |
| TOTAL with IP/UDP headers |  |  | 3,312 | 20.7 |

## A.5.2	TLS Record and Handshake message measurement

The TLS Record message is defined as follow:

Table A.5.2-1: Record message header measurement

| Payload Name | M/O/C | Parameter name | min size (byte) | max size (byte) | min size (bits) | max size (bits) | ECDHE ECDSA Case (bits) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RECORD MESSAGE |  |  |  |  |  |  |  |  |
|  |  | Content Type | 1 | 1 | 8 | 8 | 8 |  |
|  |  | Protocol Version | 2 | 2 | 16 | 16 | 16 |  |
|  |  | Length | 2 | 2 | 16 | 16 | 16 |  |
|  |  | Fragment | 0 | 16384 | 0 | 131072 | message dependent | The Handshake messages will be contained within a Record Message |
| header total |  |  |  |  |  |  | 40 |  |
| final total |  |  | 5 | 16389 | 40 | 131112 | message dependent |  |

The TLS record encapsulates Handshake messages (as well as Change Cipher Spec messages) which are defined as follows:

Table A.5.2-2: Handshake message header measurement

| Payload Name | M/O/C | Parameter name | min size (byte) | max size (byte) | min size (bits) | max size (bits) | ECDHE ECDSA Case (bits) | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HANDSHAKE MESSAGE |  |  |  |  |  |  |  |  |
|  |  | Handshake type | 1 | 1 | 8 | 8 | 8 |  |
|  |  | Length | 3 | 3 | 24 | 24 | 24 |  |
|  |  | body | 0 | 4095 |  |  | message dependent | The message content itself e.g. the ClientHello |
| header total |  |  |  |  |  |  | 32 |  |
| final total |  |  |  |  |  |  | message dependent |  |

The TLS Handshake messages are further described below:

Table A.5.2-3: ClientHello message measurement

| Payload Name | Parameter name | Size (bits) | Comments |
| --- | --- | --- | --- |
| ClientHello ECDHE ECDSA case (Direction: Client -> Server) |  |  |  |
|  | Protocol Version | 16 |  |
|  | Random | 256 |  |
|  | Session ID | 264 | 32 byte session + 1 length byte |
|  | Cipher Suites | 48 | One cipher suite e.g. TLS_ECDHE_ECDSA_WITH_AES_128_CCM_8, 2 length bytes, renegotiation ciphersuite |
|  | Compression methods | 16 | Length byte + NULL |
|  | extensions | 192 | 2 byte total ext length field + 2 byte Sig Alg ext ID + 2 byte ext len + 2 byte list len + Sig Alg (2) + 2 byte Curve ext ID + 2 byte ext len + 2 byte list len + 2 byte curve ID + 2 byte Point Format ext ID + 2 byte ext len + 1 byte list len + 1 byte format. |
| final total |  | 792 |  |
| final total with record and handshake headers |  | 864 |  |

Table A.5.2-4: ServerHello message measurement

| Payload Name | Parameter name | Size (bits) | Comments |
| --- | --- | --- | --- |
| ServerHello ECDHE ECDSA case (Direction: Server -> Client) |  |  |  |
|  | Protocol Version | 16 |  |
|  | Random | 256 | Independently generated from the ClientHello.Random |
|  | Session ID | 264 | The assigned Session ID + length byte |
|  | Cipher Suite | 16 | One cipher suite e.g. TLS_ECDHE_ECDSA_WITH_AES_128_CCM_8 |
|  | Compression methods | 8 | null |
|  | extensions | 104 | 2 byte total ext length field + 2 byte Renego ext ID + 2 byte ext len + 1 byte + 2 byte Point Format ext ID + 2 byte ext len + 1 byte list len + 1 byte format |
| final total |  | 664 |  |
| final total with record and handshake headers |  | 736 |  |

Table A.5.2-5: ChangeCipherSpec message measurement

| Payload Name | Parameter name | Size (bits) | Comments |
| --- | --- | --- | --- |
| ChangeCipherSpec ECDHE ECDSA case (Direction: Client -> Server) |  |  |  |
|  | Change cipher spec | 8 | To help avoid pipeline stalls, ChangeCipherSpec is an independent TLS protocol content type, and is not actually a TLS handshake message. |
| final total |  | 8 |  |
| final total with record header |  | 48 |  |

Note: 	ChangeCipherSpec messages aren't handshake message and are directly encapsulated in a record message.

Table A.5.2-6: Finished message measurement

| Payload Name | Parameter name | Size (bits) | Comments |
| --- | --- | --- | --- |
| Finished ECDHE ECDSA case (Direction: Client -> Server) |  |  |  |
|  | verify data | 96 |  |
| final total |  | 96 |  |
| final total with record and handshake headers |  | 168 |  |

Table A.5.2-7: ChangeCipherSpec message measurement

| Payload Name | Parameter name | Size (bits) | Comments |
| --- | --- | --- | --- |
| ChangeCipherSpec ECDHE ECDSA case (Direction: Server -> Client) |  |  |  |
|  | Change cipher spec | 8 | To help avoid pipeline stalls, ChangeCipherSpec is an independent TLS protocol content type, and is not actually a TLS handshake message. |
| final total |  | 8 |  |
| final total with record header |  | 48 |  |

Note: 	ChangeCipherSpec messages aren't handshake message and are directly encapsulated in a record message.

Table A.5.2-8: Finished message measurement

| Payload Name | Parameter name | Size (bits) | Comments |
| --- | --- | --- | --- |
| Finished ECDHE ECDSA case (Direction: Server -> Client) |  |  |  |
|  | verify data | 96 |  |
| final total |  | 96 |  |
| final total with record and handshake headers |  | 168 |  |

# A.6	GBA bootstrapping procedure

## A.6.1	Bootstrapping procedure description measurement

The following is an example signalling flow demonstrating a successful bootstrapping procedure found in 3GPP TS 24.109 [34].

Table A.6.1-1: Successful Bootstrapping Procedure Description

| Step | Example message | Character count |
| --- | --- | --- |
| Initial GET request (UE to BSF) | GET / HTTP/1.1 Host: bsf.home1.net:80 User-Agent: Bootstrapping Client Agent; Release-6; 3gpp-gba-tmpi Date: Thu, 08 Jan 2004 10:13:17 GMT Accept: */* Authorization: Digest username="user1_private@home1.net", realm="bsf.home1.net", nonce="", uri="/", response="" | 302 UTF-8 Chars 302 bytes |
| Unauthorized response (BSF to UE) | HTTP/1.1 401 Unauthorized Server: Bootstrapping Server; Release-6 Date: Thu, 08 Jan 2004 10:13:17 GMT WWW-Authenticate: Digest realm="bsf.home1.net", nonce= base64(A34Cm+Fva37UYWpGNB34JP), algorithm=AKAv1-MD5, qop="auth-int", opaque="5ccc069c403ebaf9f0171e9517f30e41" | 299 UTF-8 Chars 299 bytes |
| GET request (UE to BSF) | GET / HTTP/1.1 Host: bsf.home1.net:80 User-Agent: Bootstrapping Client Agent; Release-6; 3gpp-gba-tmpi Date: Thu, 08 Jan 2004 10:13:18 GMT Accept: */* Authorization: Digest username="user1_private@home1.net", realm="bsf.home1.net", nonce="base64(A34Cm+Fva37UYWpGNB34JP)", uri="/", qop=auth-int, nc=00000001, cnonce="6629fae49393a05397450978507c4ef1", response="6629fae49393a05397450978507c4ef1", opaque="5ccc069c403ebaf9f0171e9517f30e41", algorithm=AKAv1-MD5 | 530 UTF-8 Chars 530 bytes |
| 200 OK response (BSF to UE) | HTTP/1.1 200 OK Server: Bootstrapping Server; Release-6; 3gpp-gba-tmpi Authentication-Info: qop=auth-int, rspauth="6629fae49394a05397450978507c4ef1", cnonce="6629fae49393a05397450978507c4ef1", nc=00000001, opaque="5ccc069c403ebaf9f0171e9517f30e41", nextnonce="base64(A34Cm+Fva37UYWpGNB34JP)" Date: Expires: Thu, 08 Jan 2004 10:23:17 GMT Content-Type: application/vnd.3gpp.bsf+xml Content-Length: 255 <?xml version="1.0" encoding="UTF-8"?> <BootstrappingInfo xmlns="uri:3gpp-gba"> <btid>user@bsf.operator.com</btid> <lifetime>2004-05-28T13:20:00Z</lifetime> <Extension> <currenttime>2004-05-27T13:20:00Z</currenttime> </Extension> </BootstrappingInfo> | 705 UTF-8 Chars 705 bytes |
| total |  | 1836 UTF-8 Chars 1836 bytes 14688 bits |

The compressed IP and TCP header overhead per message is 24 bytes, therefore the total amount of data that is transmitted over the air is 15456 bits.

## A.6.2	PSK-TLS procedure measurement in GBA case.

The following describes the PSK-TLS procedures compliant with the shared key-based mutual authentication between UE and NAF procedure defined in clause 5.4 of 3GPP TS 33.222 [35].

Table A.6.2-1: Shared key-based mutual authentication between UE and NAF

| TLS handshake PSK-TLS for GBA |  |  |  |  |
| --- | --- | --- | --- | --- |
| Direction | Step | Message Name | Size (bits) | transmit time (s) |
| C -> S | 1 | ClientHello | 520 |  |
| subtotal |  |  | 520 |  |
| subtotal with compressed IP/TCP headers |  |  | 712 | 4.45 |
| S -> C | 2 | ServerHello | 640 |  |
| S -> C | 2 | ServerKeyExchange | 216 |  |
| S -> C | 2 | ServerHelloDone | 72 |  |
| subtotal |  |  | 928 |  |
| subtotal with compressed IP/TCP headers |  |  | 1120 | 7 |
| C -> S | 3 | ClientKeyExchange | 392 |  |
| C -> S | 3 | ChangeCipherSpec | 48 |  |
| C -> S | 3 | Finished | 168 |  |
| subtotal |  |  | 608 |  |
| subtotal with compressed IP/TCP headers |  |  | 800 | 5 |
| S -> C | 4 | ChangeCipherSpec | 48 |  |
| S -> C | 4 | Finished | 168 |  |
| subtotal |  |  | 216 |  |
| subtotal with compressed IP/TCP headers |  |  | 408 | 2.55 |
| TOTAL with compressed IP/TCP headers |  |  | 3,040 | 19 |

The TLS Handshake messages are further described as follows:

Table A.6.2-2: ClientHello message measurement

| Payload Name | Parameter name | Size (bits) | Comments |
| --- | --- | --- | --- |
| ClientHello PSK-TLS case (Direction: Client -> Server) |  |  |  |
|  | Protocol Version | 16 |  |
|  | Random | 256 |  |
|  | Session ID | 0 | First time |
|  | Cipher Suites | 32 | Two cipher suite e.g. TLS_PSK_WITH_AES_128_CBC_SHA256, one PSK-based cipher suite. |
|  | Compression methods | 8 | NULL |
|  | extensions | 136 |  |
| final total |  | 448 |  |
| final total with record and handshake headers |  | 520 |  |

Table A.6.2-3: ServerHello message measurement

| Payload Name | Parameter name | Size (bits) | Comments |
| --- | --- | --- | --- |
| ServerHello PSK-TLS case (Direction: Server -> Client) |  |  |  |
|  | Protocol Version | 16 |  |
|  | Random | 256 | Independently generated from the ClientHello.Random |
|  | Session ID | 256 | The assigned Session ID |
|  | Cipher Suite | 32 | Two cipher suite e.g. TLS_PSK_WITH_AES_128_CBC_SHA256, one PSK-based cipher suite. |
|  | Compression methods | 8 | null |
|  | extensions | 0 |  |
| final total |  | 568 |  |
| final total with record and handshake headers |  | 640 |  |

Table A.6.2-4: ChangeCipherSpec message measurement

| Payload Name | Parameter name | Size (bits) | Comments |
| --- | --- | --- | --- |
| ChangeCipherSpec PSK-TLS case (Direction: Server -> Client) |  |  |  |
|  | psk identity hint | 144 | 3GPP-bootstrapping |
| final total |  | 144 |  |
| final total with record header and handshake headers |  | 216 |  |

Table A.6.2-5: ServerHelloDone message measurement

| Payload Name | Parameter name | Size (bits) | Comments |
| --- | --- | --- | --- |
| ServerHelloDone PSK-TLS case (Direction: Server -> Client) |  |  |  |
| total |  | 0 |  |
| final total with record and handshake headers |  | 72 |  |

Table A.6.2-6: ClientKeyExchange message measurement

| Payload Name | Parameter name | Size (bits) | Comments |
| --- | --- | --- | --- |
| ClientKeyExchange PSK-TLS case (Direction: Client -> Server) |  |  |  |
|  | psk identity | 320 | 3GPP-bootstrapping;user@bsf.operator.com |
| final total |  | 320 |  |
| final total with record header and handshake headers |  | 392 |  |

Table A.6.2-7: ChangeCipherSpec message measurement

| Payload Name | Parameter name | Size (bits) | Comments |
| --- | --- | --- | --- |
| ChangeCipherSpec PSK-TLS case (Direction: Client -> Server) |  |  |  |
|  | Change cipher spec | 8 | To help avoid pipeline stalls, ChangeCipherSpec is an independent TLS protocol content type, and is not actually a TLS handshake message. |
| final total |  | 8 |  |
| final total with record header |  | 48 |  |

Table A.6.2-8: Finished message measurement

| Payload Name | Parameter name | Size (bits) | Comments |
| --- | --- | --- | --- |
| Finished PSK-TLS case (Direction: Client -> Server) |  |  |  |
|  | verify data | 96 |  |
| final total |  | 96 |  |
| final total with record and handshake headers |  | 168 |  |

Table A.6.2-9: ChangeCipherSpec message measurement

| Payload Name | Parameter name | Size (bits) | Comments |
| --- | --- | --- | --- |
| ChangeCipherSpec PSK-TLS case (Direction: Server -> Client) |  |  |  |
|  | Change cipher spec | 8 | To help avoid pipeline stalls, ChangeCipherSpec is an independent TLS protocol content type, and is not actually a TLS handshake message. |
| final total |  | 8 |  |
| final total with record header |  | 48 |  |

Table A.6.2-10: Finished message measurement

| Payload Name | Parameter name | Size (bits) | Comments |
| --- | --- | --- | --- |
| Finished PSK-TLS case (Direction: Server -> Client) |  |  |  |
|  | verify data | 96 |  |
| final total |  | 96 |  |
| final total with record and handshake headers |  | 168 |  |

Annex B: 
Review of security standardization efforts in other SDOs

# B.0	Introduction

This annex collects information on security standardization efforts in other standards organizations (SDOs) that are relevant for this study. This body of work includes security procedures that are optimized for constrained environments.

# B.1	(D)TLS optimization efforts in IETF

## B.1.1	Background

Editor's note: This clause presents the current status of evolving work, and would need to be updated after substantial progress in IETF.

CoAP is the most promising IoT protocol that also GERAN is expected to be used with Cellular IoT. CoAP was developed for constrained environments, and for this reason, it may seem strange that IETF choose DTLS as the primary security mechanism for CoAP. It is widely known that TLS is heavy in terms of negotiation roundtrips and amount of data sent during the negotiation. However, it seems that IETF wanted to re-use security mechanism that has already been proven, and widely deployed. By making this decision, IETF seemed to make also a commitment to solve the efficiency problems of DTLS to better fit the needs of CoAP. IETF community has been very active on finding solutions to the latency and overhead issues of DTLS. The commitment of solving the problems can be seen e.g. in the TLS working group charter that describes some of the main design goals for (D)TLS v1.3:

"Develop modes to reduce handshake latency, which primarily support HTTP-based applications, aiming for one roundtrip for a full handshake and one or zero roundtrip for repeated handshakes." [4]

The CoAP security story is based heavily on the use of DTLS with raw public keys. One deployment scenario assumes that IoT UEs would have a pre-installed asymmetric public key, and that a (truncated) hash of the public key would be used as the IoT UE identifier. The identifiers could be collected by using a barcode on the outside of the device, and stored in the server side. The identifiers would then be used to create access control lists about the IoT UEs that may use DTLS. Using strong cryptography for DTLS connection negotiation makes it possible to elongate the lifetime of the connection.

The other important assumption is that the IoT UE may not need to create DTLS sessions with several entities but only few, ideally only one. From this point of view, the situation is very different from a more typical TLS deployment scenario where TLS is supposed to be used with any server. This makes some of the DTLS protocol features look a little odd, e.g. why send the certificates (or even the information about the supported security algorithms) in every DTLS handshake if the end-points remains the same. There is obviously room for optimization in order to make the DTLS more suitable for CoAP.

## B.1.2	Existing and evolving TLS optimizations

The main technique for making DTLS more suitable for CoAP is to leave the DTLS session open for much longer period of time than it is left open in other application contexts. This mode of operation is described in CoAP protocol, and the basic idea seems to be that if the DTLS handshake is done using strong credentials, it needs not to be closed that soon. This saves energy and resources from IoT UE.

"DTLS connections in RawPublicKey and Certificate mode are set up using mutual authentication so they can remain up and be reused for future message exchanges in either direction. Devices can close a DTLS connection when they need to recover resources, but in general they should keep the connection up for as long as possible. Closing the DTLS connection after every CoAP message exchange is very inefficient." [5]

DTLS connection created using shared secrets is not mentioned, however, this does not mean that DTLS handshake created using 3GPP credentials stored in UICC would not meet the security requirements for leaving the DTLS connection open for a longer period of time.

Other reasons why DTLS connection can be left open more easily is that the proxy/server connected to UE is dedicated to server a restricted set of UEs. The amount of state maintained in proxy/server can be pre-calculated, and the risk of DoS attacks is not that severe. From the UE point of view, the amount of data that is sent over the DTLS connection at a time is very small. This means that there is less data available for a potential attacker to figure out the keys.

The dice WG (DTLS In Constrained Environments) is currently discussing about more optimized DTLS handshake procedures, and DTLS profiles for CoAP. For example, the draft-ietf-dice-profile [10] gives recommendations for chip manufacturers and software vendors on how to implement (D)TLS in CoAP devices. draft-ietf-dice-profile is a very useful document because it collects references to various RFCs and I-Ds together, and does not introduce any changes to (D)TLS but rather guides for better interoperability. It is based on (D)TLS 1.2, and strongly recommends resuming a (D)TLS sessions instead of running full handshake. However, draft-ietf-dice-profile is not a piece of work describing any real TLS optimizations as such. draft-ietf-dice-profile is currently in IESG review for publication.

## B.1.3	Making the full handshake lighter

There are at least two strategies for making the TLS handshake lighter. First one is trying to reduce the size of the certificates sent during the handshake, and the second one modifying the handshake itself in order to make it more compact.

On the certificate side, the raw public key certificates specified in RFC 7250 use ASN.1Cert format that is lighter than the full X.509 certificates. For the constrained environments, even more lightweight certificate format has been proposed [6]. There is also work going on where the certificates are cached in the TLS client removing the need of sending it every time a new TLS connection is negotiated [7]. This would be a useful feature in CoAP where the communication end-point seldom changes.

TLS 1.3 that is currently under development in IETF is trying to reduce the number of messages sent in the full TLS handshake [8]. The current version includes a 1,5 roundtrips handshake (instead of 2 roundtrips of TLS 1.2). The 1,5 roundtrips mode was only possible when resuming existing TLS 1.2 sessions. This more optimized TLS handshake is very likely to be included in the next TLS standard (Figure B.1.3-1).

ClientHello

ClientKeyShare            -------->

ServerHello

ServerKeyShare

<--------                Finished

Finished                  -------->

Figure B.1.3-1: Full TLS 1.3 handshake (optional/situational messages omitted)

## B.1.4	Resuming existing connection

A full TLS 1.2 handshake requires 2-roundtrips (four messages) before the handshake is completed, and the application can start sending application data [9]. This means not only an increase in latency but also more security related data to be sent. It is also possible to resume earlier TLS sessions but also in this case the handshake requires 1,5-roundtrips (three messages). The amount of bits is reduced radically if compared to full handshake. It is estimated that the saving would be roughly 60%.

The TLS working group is currently discussing on more radical modes of resuming the connections in TLS 1.3 namely the "1-roundtrip" and "0-roundtrip" modes. There seems to be a strong commitment within the working group to include the 0-roundtrip handshake for TLS session resumption partly because DTLS is an integral part of security of constrained environments. Interestingly, it is estimated that the 1-roundtrip mode would not save the number of bits if compared to TLS 1.2 resumption. It would still be roughly 60% less bits if compared to full handshake. But the 1-roundtrip mode is still interesting because it reduces the latency. Note that the 0-roundtrip may not mean that all of the TLS handshake messages would be omitted. It means that the client can start sending application data directly, and the server may still reply with some TLS resume messages (Figure B.1.4-1).

Client                                                Server

ClientHello

ClientKeyShare

{Certificate*}

{CertificateVerify*}

{Finished}

[Application Data]            -------->

ServerHello

ServerKeyShare

<--------            {Finished}

[Application Data]            <------->    [Application Data]

Figure B.1.4-1: Overview of 0-RTT Flow

The 0-roundtrip functionality may be limited to certain use cases only, and could be used only if both the client and the server agree to use it. There is going to be new key derivation procedure for the TLS master shared secret when the session is resumed. One of the current problems is related to the TLS anti-replay feature that is based on the idea that each side provides a random value that is mixed into the keying material. The anti-reply can be achieved for the 0-roundtrip client but not to the server. It is currently discussed how this can be solved, e.g. if the server should simply drop the 0-roundtrip data when the server thinks the 0-roundtrip is not possible. The server needs to tell the client that this was done for the client application to retransmit. However, the 0-roundtrip is seen as a feature for limited cases where the application domains can safely keep state, and can profile the use of TLS.

Annex C: Proposed normative changes

# C.1	Introduction

For the normative work, the key best functions need to be designed in a modular fashion so that they can be re-used for each solution and for other solutions in the future.

As cellular internet of things can be delivered over GPRS or LTE, it is expected that the normative specifications shall be available both in GPRS and LTE.  These specifications may be available in UMTS.

The normative work could be specified either as Change requests to existing specifications or as a new technical specification.  This technical report recommends that the BEST feature be delivered as changes to existing technical specifications.

The following specifications could be modified to include the BEST specifications:

-	3GPP TS 33.401 [28]: This covers LTE use.

-	3GPP TS 43.020 [47]: This covers use in GPRS.

This TR recommends that the changes be put into 3GPP TS 33.401[28] and references to the changes in 3GPP TS 33.401 [28] be put into 3GPP TS 43.020 [47].  This means only one version of the BEST normative specifications exists which is easier to maintain.

# C.2	Proposed changes to 3GPP TS 33.401 [28]

## C.2.1	Overview of changes

The BEST changes can be achieved through the following changes:

-	Clause 3: Definitions and abbreviations

-	New clause  17 "Security Procedures for Extended User Plane protection (or simply "BEST")" with all of BEST.  This should consist of both the modular building blocks and the way of using these blocks to deliver end to middle security and end to end security.

-	New Annex A.19: Key derivations for BEST

# C.3	Proposed changes to 3GPP TS 43.020 [47]

The BEST changes can be achieved through the following changes:

-	Clause 3: Definitions and abbreviations

-	New Annex "Security Procedures for Extended User Plane protection (or simply "BEST")" with references to all of  BEST in TS 33.401 [28].

Annex D: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2016-09 | SA#73 | SP-160575 |  |  |  | Presented for approval | 2.0.0 |
| 2016-09 | SA#73 |  |  |  |  | Upgrade to change control version | 14.0.0 |
| 2017-03 | SA#75 | SP-170103 | 0004 | 1 | F | BEST: Fixing error in Figure 6.10.2.3 of Solution #10 | 14.1.0 |
| 2017-06 | SA#76 | SP-170430 | 0006 | - | F | BEST: Clarifying key synchronization aspect in Solution #10 | 14.2.0 |
| 2017-06 | SA#76 | SP-170430 | 0007 | - | F | BEST: Clarification on generating unique pre-shared key per Enterprise server (EMSE) | 14.2.0 |