# 3GPP TSG SA WG3 (Security) Meeting #87	S3-171624

# Revision of S3-171255

# 15 – 19 May, 2017, Ljubljana, Slovenia

# Source:	Huawei, Hisilicon

# Title:	Questions related to Key Issue #8.3: Security on UEs’ access to slices

# Document for:	Discussion and Approval

Agenda Item:	8.3.8 Network Slicing Security

# 1	Decision/action requested

This contribution adds questions and interim agreements for Issue #8.3: Security on UEs’ access to slices

# 2	References

[1] 3GPP TR 33.899v1.1.0 (2017-03) Study on the security aspects of the next generation system

[2] 3GPP TS 22.261v2.0.0 (2017-03) Service requirements for the 5G system; Stage 1

# 3	Rationale

This contribution proposes a list of questions that may lead to interim agreements, addressing key issue #8.3: Security on UEs’ access to slices, for TR33.899.

# 4	Detailed proposal

It is proposed to approve below pCR. Since all texts are new, revision mark is not used.

**********************Begin of changes********************************

### E.8.3	Questions and Interim Agreements for Security on UEs’ access to slices

#### E.8.3.1   UE accesss to slices

##### E.8.3.1.1 Description of Questions

As specified in 3GPP TR 22.261,

- The 5G system shall allow the operator to authorize a 3rd party to create, modify and delete network slices, subject to an agreement between the 3rd party and the network operator.

- The 5G system shall be able to support identification of subscriptions independently of identification of equipment.

- For a private network using 5G technology, the 5G system shall support network access using identities, credentials, and authentication methods provided and managed by a 3rd party and supported by 3GPP.

Thus services offered by the network slice could either be managed by the MNO or a 3rd party service provider.

For different application scenarios, the access to the slices may be different. Hopefully, the answers to the following questions are helpful for reaching agreement on this key issue.

Question 1: Is there any mutual authentication performed before UE can access a slice?

##### E.8.3.1.2 Interim Agreement

1. Yes.

- For UE access to slices, UE shall be authenticated through Primary Authentication

- NOTE1: Secondary authentication is not related to Slice authentication: slices are inside the 3GPP network, but secondary authentication is between the UE and the external data network and is transparent to the 3GPP network

*********************************End of changes*******************************