3GPP TSG-SA WG3 Meeting #87 	S3-171345

# Ljubljana, Slovenia, 15-19 May 2017

Source:	Juniper Networks

Title:	Note on EMSDP Payload Format in BEST-MTC TS33.zyz

Document for:	Approval

Agenda Item:	7.6.15

Work Item / Release:	BEST-MTC / Rel 15

Abstract of the contribution:  Adding the note in section 6.2.2 of TS33.xyz to put the EMSDP data payload format out of scope for this release.  Deleting the corresponding EN in section 6.2.1.

---------------------------------------------------------------------------------------------------------------------

Discussion:

In the current TS, the format and content of the data payload carried in the EMSDP UP is undefined.  We have several options including the following four.

The payload data type could be

a)  indicated in the EMSDP CP message Command Option TLV during BEST session setup,

b)  indicated in the EMSDP UP message as part of each message,

c)  configured by policy, or

d) considered out of scope in this release.

This contribution proposes the last options and adds the note “The content and processing of the BEST UP EMSDP data payload is out of scope in this release.”

---------------------------------------------------------------------------------------------------------------------

************************ Start of Changes ***************************

## 6.2	EMSDP Protocol Description

### 6.2.1	Data Stack

Figures 6.2.1-1, 6.2.1-2 and 6.2.1-3 shows examples with the LTE network of the data stacks for the EMSDP transfers (based on 3GPP TS 23.401 [4]) between the HSE and the UE.

EMSDP

PDCP

RLC

MAC

L1

UE

RLC

MAC

E-UTRAN

UDP/IP

L2

L1

L1

PDCP

GTP-U

Relay

LTE - Uu

UDP/IP

L2

Serving GW

UDP/IP

L2

L1

L1

GTP-U

GTP-U

Relay

S1-U

UDP/IP

L2

IP

L2

L1

L1

S5

PDN GW

Transformation

GTP-U

UDP

EMSDP

UDP

IP

L2

L1

HSE

SGi

New-1

Figure 6.2.1-1: Non-IP data stack for the EMSDP transfers over UP

NAS

PDCP

RLC

MAC

L1

UE

E-UTRAN

Relay

LTE - Uu

Serving GW

S1-MME

PDN GW

HSE

SGi

RLC

L1

MAC

PDCP

SCTP/IP

L2

L1

S1AP

EMSDP

IP

L2

L1

EMSDP

Relay

SCTP/IP

L1

L2

NAS

S1AP

MME

Relay

IP

L1

L2

UDP

GTP-U

GTP-U

IP

L2

L1

UDP

UDP

Relay

IP

L1

L2

IP

L2

L1

UDP

UDP

GTP-U

IP

L2

L1

GTP-U

UDP

S11

S5

Figure 6.2.1-2: Non-IP data stack for the EMSDP transfers over NAS

If the UE selects the non-IP PDN data type as defined in clause 4.3.17.8 of  3GPP TS 23.401[2], the  PDN GW encapsulates the non-IP EMSDP payload in PtP (IP and UDP as detailed in clause 4.3.17.8.3.3.2 of 3GPP TS 23.401[4]) for the communication with the HSE.  In case of the EMSDP is in IP PDN, the PtP encapsulation is unnecessary, and the UE’s IP packet is directly forwarded to the HSE.  This is shown on Figure 6.2.1-2.

IP/UDP

PDCP

RLC

MAC

L1

UE

RLC

MAC

E-UTRAN

UDP/IP

L2

L1

L1

PDCP

GTP-U

Relay

LTE - Uu

UDP/IP

L2

Serving GW

UDP/IP

L2

L1

L1

GTP-U

GTP-U

Relay

S1-U

UDP/IP

L2

L2

L1

L1

S5

PDN GW

EMSDP

GTP-U

L2

L1

HSE

SGi

New-1

EMSDP

IP/UDP

Figure 6.2.1-3: IP data stack for the EMSDP transfers

The dashed box indicates that for UE-to-EAS BEST sessions, the HSE may forward the EMSDP user plane unmodified to the EAS. The signalling plane always terminates in the HSE.

Note: 	It is up to the operator to decide the protocol between the HSE and the EAS. For example, an operator may agree to use the optional Annex B protocol. In that case, the HSE will have to encapsulate the user data in an agreed message format.

Editors Notes: 	How does the UE know the IP address of the HSE.

### 6.2.2	EMSDP general structure

This clause details a type 01 control plane EMSDP message and a type 01 user plane EMSDP message.

Figure 6.2.2-1 shows the general structure of the EMSDP type 01 message:

| Control Plane type 01 message: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| UP / SP Flag | RFU | Key ID | CP COUNTER | Session ID | EMSDP Command | Cmd Options | MAC |
| 1 bit | 1 bit | 3 bits | Note 4 | Note 1 Note 2 | Note 1 Note 2 Note 3 | Note 2 Note 3 | Note 1 Note 3 |
| User Plane type 01 message: |  |  |  |  |  |  |  |  |
| UP / SP Flag | RFU | Key ID | UP COUNTER | Session ID | Data Length | Data | MAC |
| 1 bit | 1 bit | 3 bits | Note 4 | Note 1 Note 2 | Note 1 Note 2 Note 3 | Note 2 Note 3 | Note 1 Note 3 |

Note 1: The length of these fields is indicated in the BEST service discovery from the HSE.

Note 2: These fields are included in the integrity protection calculation.

Note 3: These fields are encrypted when encryption is used.  When encrypted these fields are replaced by the encrypted output.

Note 4: The length of this field is as defined in the BEST Counter Scheme being used.

Figure 6.2.2-1: data stack for the EMSDP transfers

UP / CP Flag: 	This is a 1 bit field that when set to ‘1’ means that the data packet is a User Plane message and when set to ‘0’ means that the message is a control plane message.

RFU:	This is a 1 bit field that is reserved for future use. Set to ‘0’.

Key ID:	This is a 3 bit field that indicates the key being used for encryption and Integrity protection.  If no keys have been agreed between the HSE and the UE then this shall be ‘000’.

CP COUNTER:	This is a counter, used for control plane data messages, that is incremented every control plane message.  It is used to protect control plane data messages against replay attacks and its length is set based on the counter scheme indicated in the BEST service discovery (minimum length is 3 bits).  There are separate counter values for UE to HSE and HSE to UE.

UP COUNTER:	This is a counter, used for user plain data messages, that is incremented every user plain data message.  It is used to protect user plain data message against replay and its length is set based on the counter scheme indicated in the BEST service discovery (minimum length is 3 bits). There are separate counter values for UE to HSE and HSE to UE.

Session ID:	This indicates the identifier for the current session.  The value is assigned by the HSE.    Its length is the value set in the BEST service discovery. For signalling messages sent outside of a BEST session, the Session ID length shall be 1 octet and the Session ID value shall be all 0’s.

EMSDP Command:	This is a 1 byte field that contains the signalling command.  The defined commands are detailed below.

Cmd Options:	This is a TLV container that contains TLV elements that detail the options for the EMSDP command. The defined options TLVs are detailed below.

MAC:	This contains the truncated integrity result for this data packet calculated using the agreed integrity algorithm. Its length is set in the BEST service discovery.  For an EMSDP session request command the MAC shall not be present.

Data length:	This holds the length of the following data in this data packet. Its length is set in the BEST service discovery.  This is not present if the data length is set to 0 in the BEST Service Discovery.

Data:	This is the data being transferred.

Note: The content and processing of the BEST UP EMSDP data payload is out of scope in this release.

****************** End of Changes ***********************