3GPP TSG-SA WG3 Meeting #87 	S3-171348

# 15-19 May 2017, Ljubljana, Slovenia

Source:	China Mobile

Title:	Key Management Procedure between the HSE and EAS

Document for:	Approval

Agenda Item:	7.6.15

Work Item / Release:	BEST_MTC_Sec / Rel 15

Abstract of the contribution:  Proposing the key management procedure between the HSE and the EAS.

---------------------------------------------------------------------------------------------------------------------

Introduction:

The UE-to-EAS keys (KE2Eenc , KE2Eint) and the EAS specific pre-shared key (KInt_EAS_PSK) are derivative from KInt_E2E and KInt_psk . KInt_E2E and KInt_psk  are negotiated  keys between UE and the HSE. Hence, if the EAS or the HSE wants to re-negotiate UE-to-EAS keys, the HSE shall trigger an EMSDP Manage Keys procedure.

pCR

************************ Start of Changes ***************************

4.4.4 Key Management

At any time during the BEST session, either the HSE or the EAS may trigger a re-negotiation of the keys being used for the BEST service.

When the HSE triggers a key re-negotiation:

- The HSE and UE execute key management procedures by EMSDP Manage Keys Request and Response exchange.

- The HSE and EAS execute key management procedures by EAS Manage Keys Request and Response exchange.

When the EAS triggers a key re-negotiation:

- The EAS initiates key management between HSE and EAS by sending the EAS Keys Request message.

- The HSE and UE execute key management procedures by EMSDP Manage Keys Request and Response exchange.

- After successful derivation of  intermediate keys,  the HSE forwards intermediate keys to EAS by EAS Keys Response message.

****************** End of Changes ***********************