# 3GPP TSG SA WG3 (Security) Meeting #87	S3-171553

# 15 – 19 May, 2017, Ljubljana, Slovenia	update of S3-170305

# Source:	Nokia

# Title:	Solution variant to #7.3

# Document for:	Approval, Discussion

Agenda Item:	8.3.7 (Subscription privacy)

# 1	Decision/action requested

Solution variant to solution #7.3 – resubmission from SA3#86Bis

# 2	References

[1]       3GPP TR 33.899 "Study on security aspects of the Next Generation System".

# 3	Rationale

# 4	Detailed proposal

**** Begin Change ****

#### 5.7.4.3.a	Solution #7.3.a: HN asymmetric key (solution variant to #7.3)

Solution #7.3 describes the usage of public key encrypted IMSIs in two cases: for the initial attach and when UE is requested to provide its real identity. Otherwise pseudonyms established between the UE and the home network are used. Solution #7.3.a is a variant of #7.3 as it demands that the only ways to provide the identifier to the network are:

- public key encryption of the long-term identifier

- Temporary identifier assigned by the serving network.

##### 5.7.4.3.a.2	Solution details

The 5G UE and the home PLMN UDM share a long-term identifier (SUPI) and a long-term key K.

A home PLMN has a public key, which it has made available to the UE. Since this is a relationship between the operator and its subscribers, this can be a raw public key, i.e. no need for certificates. The home PLMN can pre-provision the public key in the UE or use OTA mechanism to distribute it.

Every time the 5G UE is required to send its long-term identifier, the 5G UE encrypts the parts of the long-term identifier not needed for routing purpose with the public key of the home PLMN and sends the encrypted identifier to the serving or home PLMN. Long-term identifiers are needed for initial attach and if the serving network does not have the temporary identity available. Other reasons for sending long-term identifiers could be possible.

##### 5.7.4.3.a.3	Evaluation

This solution provides 5G UE privacy at the 5G RAN.

There is no need of a Global PKI. Impact to HSS is limited to decryption of the received identity. Home PLMN needs to have means to update public keys in its subscribers' devices.

Editor's. Note: the sending of SUPI within NAS Security Mode Complete is ffs.

**** End Change ****