# 3GPP TSG SA WG3 (Security) Meeting #88	S3-171948

# 7 – 11 August, 2017, Dali, China

# Source:	Nokia

# Title:	pCR  to TR: Agreement - Avoiding SUPI Paging

# Document for:	Approval

Agenda Item:	8.3.7 / TR 33.899 – Subscriber privacy

# 1	Decision/action requested

Approve the pCR below

# 2	References

[1] 3GPP TR 33.899 V1.1.0

# 3	Rationale

Using the permanent subscription identifier SUPI as the UE id while sending paging messages and the subsequent call origination by the UE is been reported as a privacy threat to the user. Using the SUPI Paging active/passive listeners, fake eNBs etc can track down people with reasonable accuracy to a specific geographic area. This has serious security implications to a user.  It is very desirable in 5G systems if we can avoid IMSI paging and protect the subscriber privacy.

Within the CN, many network nodes may be using the permanent identifier IMSI for legacy reasons, such as billing etc. This usage doesn’t become a privacy threat to the user nor a threat for the internal nodes. CN nodes can continue using the IMSI and the SUPI as long as they are not exposed to public.

The following potential requirements have been formulated in a new key issue #7.X in [1]:

Next generation networks shall avoid the permanent subscription identifier IMSI / SUPI while sending paging alerts to UEs while they are in idle mode.

Next generation networks shall use only temporary network identifier of a UE while sending paging alerts to UEs while they are in idle mode.

The temporary network identifier such as the S-TMSI should be assigned or reassigned periodically in such a way to avoid mapping with any other identity and tracking the user is difficult.

*** START OF CHANGES ***

### E.7.X 	Questions and interim agreements for key issue #7.X

Editor’s Note: how to handle IMSI paging and privacy in interworking scenarios is ffs.

#### E.7.X.1 	Privacy requirements for SUPI paging

##### E.7.X.1.1 	Description of Question

Key issue #7.X lists requirements on avoiding the use of SUPI over the air interface.

Question: Shall a 5G system allow the sending of SUPI in paging messages?

##### E.7.X.1.2 	Interim Agreement

It is agreed in phase 1, paging the UE with SUPI in clear-text over-the-air shall be avoided, while sending paging alerts to UE while they are in idle mode.

*** END OF CHANGES ***