# 3GPP TSG SA WG3 (Security) Meeting #90-bis	S3-180901

# 26 Feb- 2 March, 2018, San Diego (US)	revision of S3-180530

# Source:	China Mobile

# Title:	pCR to resolving authentication Editor’s Notes in section 9.1.3.4

# Document for:	Approval

Agenda Item:	4.1.13.4

1	Decision/action requested

To accept pCR to resolve Editor’s Note in section 9.1.3.4

2	References

[1]	3GPP TS 33.501 Security Architecture and Procedures for 5G System (Release 15)

[2]	3GPP TS 23.501 System Architecture for the 5G System; Stage 2 (Release 15)

[3]	IETF draft-ietf-tls-tls13-23 The Transport Layer Security (TLS) Protocol Version 1.3 https://datatracker.ietf.org/doc/draft-ietf-tls-tls13/

3	Rationale

In TS 33.501[1] section 9.1.3.4, it specifies authentication and authorization for service based architecture.

In section 9.1.3.4.2, an editor’s note is : It is FFS how to perform authentication in case a non-transparent proxy is in between the NF and NRF. In such case the NRF will only see the client-certificate of the non-transparent proxy and not the actual NF.

Non-transparent proxy means that NF will communicate with NRF in other PLMN so a non-transparent proxy is needed to hide some information.

To make editor’s note clear. It should be revised as follow:

Editor’s Note: It is FFS how to perform authentication between NRFs in case a non-transparent proxy (SEPP) is in between them.

.

.

4	Detailed proposal

============================BEGIN OF 1ST CHANGE===========================

9.1.3.4	Authorization and authentication aspects

9.1.3.4.1	General

Editor's Note: this subclause is to cover how an HTTP-based service request can be authorized.

9.1.3.4.2	Authorization and authentication between network functions and the NRF

NRF and NF shall authenticate each other during discovery and registration. If the PLMN uses protection at the transport layer, the protection at the transport layer shall be used for mutual authentication of the NRF and NF.

Procedures for transport layer protection of service base interfaces are specified in clause 9.1.3.2 of the present specification.

If the PLMN does not use protection at the transport layer, mutual authentication of NRF and NF may be implicit by NDS or physical security.

After successful authentication between NRF and NF, the NRF shall decide whether the NF is authorized to perform discovery and registration.

Editor’s Note: It is FFS how to perform authentication between NRFs in case a non-transparent proxy (SEPP) is in between them.

For non-roaming scenario, the NRF authorizes the Nnrf_NFDiscovery_Request based on the profile of the expected NF/NF service and the type of the NF service consumer, the NRF determines whether the NF service consumer is allowed to discover the expected NF instance(s). As described in clause 4.17.4 of TS23.502 [8].

For roaming scenario, the NRF of the NF Provider shall authorize the Nnrf_NFDiscovery_Request based on the profile of the expected NF/NF Service, the type of the NF service consumer and the serving network ID.

9.1.3.4.3	Authorization of NF service access

Editor’s Note: This content addresses the authorization of NF service access.

============================END OF 1ST CHANGE===========================