3GPP TSG-SA3 Meeting #91	S3-181165

# Belgrade, Serbia, 16th Apr 2018 - 20th Apr 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 0031 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolution of Editor's Note on authentication vectors |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN N.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SEC |  |  |  |  |  | Date: |  |  | 2018-04-04 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Resolution of Editor’s Note in |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes new definitions, new abbreviations and textual changes to clear up the usage of the word ‘authentication vector’. It also deletes the Editor’s Note that clarifies this. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not approving this CR leaves an Editor’s Note in that indicates that confusion may occur. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 6.1.1.4.3, 6.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

5G security context: The state that is established locally at the UE and a serving network domain and represented by the "5G security context data" stored at the UE and a serving network.

NOTE 1:	The "5G security context data" consists of the 5G NAS security context, and the 5G AS security context for 3GPP access and/or the 5G AS security context for non-3GPP access.

NOTE 2:	A 5G security context has type "mapped", "full native" or "partial native". Its state can either be "current" or "non-current". A context can be of one type only and be in one state at a time. The state of a particular context type can change over time. A partial native context can be transformed into a full native. No other type transformations are possible.

5G AS security context for 3GPP access: the cryptographic keys at AS level with their identifiers, the Next Hop parameter (NH), the Next Hop Chaining Counter parameter (NCC) used for next hop access key derivation, the identifiers of the selected AS level cryptographic algorithms, and the counters used for replay protection.

NOTE 3:	NH and NCC need to be stored also at the AMF during connected mode.

5G AS security context for non-3GPP access: The key KN3IWF, the cryptographic keys, cryptographic algorithms and tunnel security association parameters used at IPsec layer for the protection of IPsec SA.

5G Authentication Vector: a vector consisting of RAND, AUTN, XRES*, and KAUSF. This vector is received by the AUSF from the UDM/ARPF in the Nudm_Authentication_Get Response.

5G Home Environment Authentication Vector: a vector consisting of RAND, AUTN, HXRES*, and KSEAF. This vector is received by the SEAF from the AUSF in the Nausf_Authentication_Authenticate Response.

5G NAS security context: context consisting of KAMF with the associated key set identifier, the UE security capabilities, and the uplink and downlink NAS COUNT values.

NOTE 4:	The distinction between native and mapped 5G security contexts also applies to 5G NAS security contexts. The 5G NAS security context is called "full" if it additionally contains the integrity and encryption keys and the identifiers of the selected NAS integrity and encryption algorithms.

activation of security context: the process of taking a security context into use.

authentication vector: a vector consisting of CK, IK, RAND, AUTN, and XRES.

authentication data: 5G Authentication Vector or transformed authentication vector.

backward security: The property that for an entity with knowledge of Kn, it is computationally infeasible to compute any previous Kn-m (m>0) from which Kn is derived.

NOTE 5:	In the context of KgNB key derivation, backward security refers to the property that, for a gNB with knowledge of a KgNB, shared with a UE, it is computationally infeasible to compute any previous KgNB that has been used between the same UE and a previous gNB.

CM-CONNECTED state: This is as defined in TS 23.501 [2]. The term CM-CONNECTED state corresponds to the term 5GMM-CONNECTED mode used in TS 24.501 [35].

CM-IDLE state: As defined in TS 23.501 [2]. The term CM-IDLE state corresponds to the term 5GMM-IDLE mode used in TS 24.501 [35].

current 5G security context: The security context which has been activated most recently. Note that a current 5G security context originating from either a mapped or native 5G security context may exist simultaneously with a native non-current 5G security context.

forward security: The property that for an entity with knowledge of Km that is used between that entity and a second entity, it is computationally infeasible to predict any future Km+n (n>0) used between a third entity and the second entity.

NOTE 6:	In the context of KgNB key derivation, forward security refers to the property that, for a gNB with knowledge of a KgNB, shared with a UE, it is computationally infeasible to predict any future KgNB that will be used between the same UE and another gNB. More specifically, n hop forward security refers to the property that a gNB is unable to compute keys that will be used between a UE and another gNB to which the UE is connected after n or more handovers (n=1 or more).

full native 5G security context: A native 5G security context for which the 5G NAS security context is full according to the above definition. A full native 5G security context is either in state "current" or state "non-current".

mapped 5G security context: An 5G security context, whose KAMF was derived from EPS keys during interworking and which is identified by mapped ngKSI.

native 5G security context: An 5G security context, whose KAMF was created by a run of primary authentication and which is identified by native ngKSI.

non-current 5G security context: A native 5G security context that is not the current one.

NOTE 7:	A non-current 5G security context may be stored along with a current 5G security context in the UE and the AMF. A non-current 5G security context does not contain 5G AS security context. A non-current 5G security context is either of type "full native" or of type "partial native".

partial native 5G security context: A partial native 5G security context consists of KAMF with the associated key set identifier, the UE security capabilities, and the uplink and downlink NAS COUNT values, which are initially set to zero before the first NAS SMC procedure for this security context.

NOTE 8:	A partial native 5G security context is created by primary authentication, for which no corresponding successful NAS SMC has been run. A partial native context is always in state "non-current".

RM-DEREGISTERED state: This is as defined in TS 23.501 [2]. The term RM-DEREGISTERED state corresponds to the term 5GMM-DEREGISTERED mode used in TS 24.501 [35].

RM-REGISTERED state: As defined in TS 23.501 [2]. The term RM-REGISTERED state corresponds to the term 5GMM-REGISTERED mode used in TS 24.501 [35].

subscription identifier de-concealing function: This service is offered by the network function UDM in the home network of the subscriber. The Subscription Identifier De-concealing Function (SIDF) is responsible for de-concealing the SUPI from the SUCI.

subscription concealed identifier: The SUbscription Concealed Identifier (SUCI) is a one-time use subscription identifier, which contains the concealed subscription identifier, e.g. MSIN.

security anchor function: The network function that serves as the anchor for security in 5G.

subscription credential(s): set of values in the USIM and the ARPF, consisting of at least the long-term key(s) and the subscription identifier SUPI, used to uniquely identify a subscription and to mutually authenticate the UE and 5G core network.

transformed authentication vector: an authentication vector where CK and IK have been replaced with CK' and IK'.

UE security capabilities: The set of identifiers corresponding to the ciphering and integrity algorithms implemented in the UE.

NOTE 9:	This includes capabilities for NG-RAN and 5G NAS, and includes capabilities for EPS, UTRAN and GERAN if these access types are supported by the UE.

UE 5G security capability: The UE security capabilities for 5G AS and NAS.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

5G-AN	5G Access Network

5G-RAN	5G Radio Access Network

5G AV	5G Authentication Vector

5G HE AV	5G Home Environment Authentication Vector

AES	Advanced Encryption Standard

AKA	Authentication and Key Agreement

AMF	Access and Mobility Management Function

AMF	Authentication Management Field

NOTE:	If necessary, the full word is spelled out to disambiguate the abbreviation.

ARPF	Authentication credential Repository and Processing Function

AUSF	Authentication Server Function

AUTN	AUthentication TokeN

AV	Authentication Vector

AV'	transformed Authentication Vector

CP	Control Plane

CTR	Counter (mode)

CU	Central Unit

DN	Data Network

DNN	Data Network Name

DU	Distributed Unit

EAP	Extensible Authentication Protocol

EMSK	Extended Master Session Key

EPS	Evolved Packet System

gNB	NR Node B

GUTI	Globally Unique Temporary UE Identity

HRES	Hash RESponse

HXRES	Hash eXpected RESponse

IKE	Internet Key Exchange

KSI	Key Set Identifier

LI	Lawful Intercept

MSK	Master Session Key

N3IWF	Non-3GPP access InterWorking Function

NAI	Network Access Identifier

NAS	Non Access Stratum

NDS	Network Domain Security

NEA	Encryption Algorithm for 5G

NF	Network Function

NG	Next Generation

ng-eNB	Next Generation Evolved Node-B

ngKSI	Key Set Identifier in 5G

NIA	Integrity Algorithm for 5G

NR	New Radio

NSSAI	Network Slice Selection Assistance Information

PDN	Packet Data Network

PEI	Permanent Equipment Identifier

QoS	Quality of Service

RES	RESponse

SEAF	SEcurity Anchor Function

SEG	Security Gateway

SIDF	Subscription Identifier De-concealing Function

SMC	Security Mode Command

SMF	Session Management Function

SN Id	Serving Network Identifier

SUCI	Subscription Concealed Identifier

SUPI	Subscription Permanent Identifier

TLS	Transport Layer Security

UE	User Equipment

UEA	UMTS Encryption Algorithm

UDM	Unified Data Management

UIA	UMTS Integrity Algorithm

ULR	Update Location Request

UP	User Plane

UPF	User Plane Function

USIM	Universal Subscriber Identity Module

XRES	eXpected RESponse

# *** End of First Change ***

# *** Second Change ***

##### 6.1.1.4.3	Construction of the serving network name by the SEAF

The SEAF shall construct the serving network name as follows:

1.	It shall set the service code to "5G".

2.	It shall set the network identifier to the SN Id of the serving network to which the authentication data is sent by the AUSF.

3.	Concatenate service code and the SN Id with the separation character ":".

NOTE:	AUSF gets the serving network name from the SEAF. Before using the serving network name, AUSF checks that the SEAF is authorized to use it, as specified in clause 6.1.2.

# *** End of Second Change ***

# *** Third Change ***

### 6.1.3	Authentication procedures

#### 6.1.3.1	Authentication procedure for EAP-AKA'

EAP-AKA' is specified in RFC 5448 [12]. The 3GPP 5G profile for EAP-AKA' is specified in the normative Annex F.

EAP-AKA' is applied within the 5G authentication framework whenever the UDM/ARPF has received an Nudm_UEAuthentication_Get Requestmessage from the AUSF, and has chosen EAP-AKA' as the authentication method, cf. sub-clause 6.1.2 of the present document.

The authentication procedure for EAP-AKA' works as follows, cf. also Figure 6.1.3.1-1:

0.	The AUSF shall send a Nudm_UEAuthentication_Get Request message to the UDM/ARPF, if SUCI was included in the Nudm_UEAuthentication_Get Request, UDM/SIDF shall deconceal the SUPI and then choose EAP-AKA’ as authentication method as described in clause 6.1.2.

1.	The UDM/ARPF shall first generate an AV with Authentication Management Field (AMF) separation bit = 1 as defined in TS 33.102 [9]. The UDM/ARPF shall then compute CK' and IK' as per the normative Annex A and replace CK and IK by CK' and IK'.

2.	The UDM/ARPF shall subsequently send this transformed authentication vector AV' (RAND, AUTN, XRES, CK', IK') to the AUSF from which it received the Nudm_UEAuthentication_Get Request together with an indication that the AV' is to be used for EAP-AKA' using a Nudm_UEAuthentication_Get message.

NOTE:	The exchange of a Nudm_UEAuthentication_Get Request message and an Nudm_UEAuthentication_Get Responsemessage between the AUSF and the UDM/ARPF described in the preceding paragraph is the same as for trusted access using EAP-AKA' described in TS 33.402 [11], sub-clause 6.2, step 10, except for the input parameter to the key derivation, which is the value of <network name>. The "network name" is a concept from RFC 5448 [12]; it is carried in the AT_KDF_INPUT attribute in EAP-AKA'. The value of <network name> parameter is not defined in RFC 5448 [12], but rather in 3GPP specifications. For EPS, it is defined as "network access identity" in TS 24.302 [13], and for 5G, it is defined as "serving network name" in sub-clause 6.1.1.4 of the present document.

Editor's Note: The number of the stage 3 specification TS 24.yyy [xx] is FFS.

Figure 6.1.3.1-1: Authentication procedure for EAP-AKA'

In case SUCI was included in the Nudm_UEAuthentication_Get Request, UDM will include the SUPI in the Nudm_UEAuthentication_Get Response.

The AUSF and the UE shall then proceed as described in RFC 5448 [12] until the AUSF is ready to send the EAP-Success.

3.	The AUSF shall send the EAP-Request/AKA'-Challenge message to the SEAF in a Nausf_UEAuthentication_Authenticate Response message.

4.	The SEAF shall transparently forward the EAP-Request/AKA'-Challenge message to the UE in a NAS message Auth-Req message. This message shall also include the ngKSI that will be used by the UE and AMF to identify the partial native security context that is created if the authentication is successful. The ME shall forward the RAND and AUTN received in EAP-Request/AKA'-Challenge message to the USIM.

Editor's Note:	The SEAF needs to understand that the authentication method used is an EAP method. How the SEAF learns about the type of authentication method based on the Nausf_UEAuthentication_Authenticate Response message is specified by CT4.

5.	At receipt of the RAND and AUTN, the USIM shall verify the freshness of the AV' by checking whether AUTN can be accepted as described in TS 33.102[4]. If so, the USIM computes a response RES. The USIM shall return RES, CK, IK to the ME. If the USIM computes a Kc (i.e. GPRS Kc) from CK and IK using conversion function c3 as described in TS 33.102 [4], and sends it to the ME, then the ME shall ignore such GPRS Kc and not store the GPRS Kc on USIM or in ME.The ME shall derive CK' and IK' according to TS 33.402 [11], sub-clause 6.2, step 15.

If the verification of the AUTN fails on the USIM, then the USIM and ME shall proceed as described in sub-clause 6.1.3.2.1.

6.	The UE shall send the EAP-Response/AKA'-Challenge message to the SEAF in a NAS message Auth-Resp message.

7.	The SEAF shall transparently forwards the EAP-Response/AKA'-Challenge message to the AUSF in Nausf_UEAuthentication_Authenticate Request message.

8.	The AUSF shall verify the message, and if the AUSF has successfully verified this message it shall continue as follows, otherwise it shall return an error.

9.	The AUSF and the UE may exchange EAP-Request/AKA'-Notification and EAP-Response /AKA'-Notification messages via the SEAF. The SEAF shall transparently forward these messages.

Editor's note: the condition needs to be clarified.

10.	The AUSF uses the first 256 bits of EMSK as the KAUSF and then calculates KSEAF from KAUSF as described in clause A.6. The AUSF shall send an EAP Success message to the SEAF inside Nausf_UEAuthentication_Authenticate Response, which shall forward it transparently to the UE. Nausf_UEAuthentication_Authenticate Response message contains the KSEAF. If the AUSF received a SUCI from the SEAF when the authentication was initiated (see sub-clause 6.1.2 of the present document), then the AUSF shall also include the SUPI in the Nausf_UEAuthentication_Authenticate Response message.

Editor's Note: For LI, the AUSF sending SUPI to SEAF is necessary but not sufficient. Further steps such as acquiring SUPI from the UE are required.

11.	The SEAF shall send the EAP Success message to the UE in the N1 message.

The key received in the Nausf_UEAuthentication_Authenticate Response message shall become the anchor key, KSEAF in the sense of the key hierarchy in sub-clause 6.2 of the present document. The SEAF shall then derive the KAMF from the KSEAF and the SUPI according to Annex A.7 and send it to the AMF On receiving the EAP-Success message, the UE calculates KAUSF and KSEAF in the same way as the AUSF and shall derive the KAMF from the KSEAF and the SUPI according to Annex A.7..

The further steps taken by the AUSF upon receiving a successfully verified EAP-Response/AKA'-Challenge message are described in sub-clause 6.1.4 of the present document.

If the EAP-Response/AKA'-Challenge message is not successfully verified, the subsequent AUSF behaviour is determined according to the home network's policy.

If the AUSF and SEAF determines that the authentication was successful, then the SEAF provides the ngKSI and the KAMF to the AMF.

Editor's Note: Error cases are typically handled in stage 3 specifications and should be elaborated jointly by SA3 and CT4.

#### 6.1.3.2	Authentication procedure for 5G AKA

##### 6.1.3.2.0	5G AKA

5G AKA enhances EPS AKA [10] by providing the home network with proof of successful authentication of the UE from the visited network. The proof is sent by the visited network in an Authentication Confirmation message.

5G AKA is applied within the 5G authentication framework whenever the UDM/ARPF has received a Nudm_UEAuthentication_Get Request message from the AUSF, and chosen 5G AKA as the authentication method, cf. sub-clause 6.1.2 of the present document.

NOTE 1:	5G AKA does not support requesting multiple 5G AVs, neither the SEAF pre-fetching 5G AVs from the home network for future use.

Figure 6.1.3.2-1: Authentication procedure for 5G AKA

The authentication procedure for 5G AKA works as follows, cf. also Figure 6.1.3.2-1:

0.	The AUSF shall sentd a Nudm_UEAuthentication_Get Request message to the UDM/ARPF, if SUCI was included in the Nudm_UEAuthentication_Get Request, UDM/SIDF shall deconceal the SUPI and then chose 5G-AKA as authentication method for the SUPI as described in Clause 6.1.2.

1.	For each Nudm_Authenticate_Get Request,  the UDM/ARPF shall create a 5G HE AV. The UDM/ARPF does this by generating an AV with the Authentication Management Field (AMF) separation bit set to "1"as defined in TS 33.102 [9], The UDM/ARPF shall then derive KAUSF as per Annex A.2, and XRES* as per Annex A.4. Finally, creating the UDM/ARPF shall create a 5G HE AV from RAND, AUTN, XRES*, and KAUSF.

2.	The UDM/ARPF shall then return the 5G HE AV to the AUSF together with an indication that the 5G HE AV is to be used for 5G-AKA in a Nudm_UEAuthentication_Get Response. In case SUCI was included in the Nudm_UEAuthentication_Get Request, UDM will include the SUPI in the Nudm_UEAuthentication_Get Response..

3.	The AUSF may store the XRES* temporarily until it expires. The AUSF may store the KAUSF.

4.	The AUSF shall then generate the 5G AV from the 5G HE AV received from the UDM/ARPF by computing the HXRES* from XRES* according to Normative Annex A.5 5 and KSEAF from KAUSF according to Annex A.6, and replacing the XRES* with the HXRES* and KAUSF with KSEAF in the 5G HE AV.

Editor's Note: It is FFS whether HXRES* should be computed in the ARPF.

5.	The AUSF shall then return the 5G AV (RAND, AUTN, HXRES*, KSEAF) to the SEAF in a Nausf_UEAuthentication_Authenticate Response.

6.	The SEAF shall send RAND, AUTN to the UE in a NAS message Auth-Req. This message shall also include the ngKSI that will be used by the UE and AMF to identify the KAMF and the partial native security context that is created if the authentication is successful. The ME shall forward the RAND and AUTN received in NAS message Auth-Req to the USIM.

7.	At receipt of the RAND and AUTN, the USIM shall verify the freshness of the 5G AV by checking whether AUTN can be accepted as described in TS 33.102[4]. If so, the USIM computes a response RES. The USIM shall return RES, CK, IK to the ME. If the USIM computes a Kc (i.e. GPRS Kc) from CK and IK using conversion function c3 as described in TS 33.102 [4], and sends it to the ME, then the ME shall ignore such GPRS Kc and not store the GPRS Kc on USIM or in ME. The ME then shall compute RES* from RES according to Annex A.4. The UE shall calculate KSEAF from KAUSF according to clause A.6.

8.	An ME accessing 5G shall check during authentication that the "separation bit" in the AMF field of AUTN is set to 1. The "separation bit" is bit 0 of the AMF field of AUTN.

NOTE:	This separation bit in the AMF field of AUTN can not be used anymore for operator specific purposes as described by TS 33.102 [9], Annex F.

The UE shall return RES* to the SEAF in a NAS message Auth-Resp.

9.	The SEAF shall then compute HRES* from RES* according to Annex A, and the SEAF shall compare HRES* and HXRES*. If they coincide, the SEAF shall consider the authentication successful. If not, the SEAF proceed as described in sub-clause 6.1.3.2.2.

10.	The SEAF shall send RES*, as received from the UE, in a Nausf_UEAuthentication_Authenticate Request message (containing the SUPI or SUCI and the serving network name) to the AUSF.

11.	When the AUSF receives the Nausf_UEAuthentication_Authenticate Request message including a RES* it may verify whether the AV has expired. If the AV has expired the AUSF may consider the confirmation unsuccessful. AUSF shall compare the received RES* with the stored XRES*. If the RES* and XRES* are equal, the AUSF shall consider the confirmation message as successfully verified.

12.	The AUSF shall indicate to the SEAF in the Nausf_UEAuthentication_Authenticate Response whether the confirmation was successful or not. In case the AUSF received a SUCI from the SEAF when the authentication was initiated (see subclause 6.1.2 of the present document), and if the confirmation was successful, then the AUSF shall also include the SUPI in 5G-ACA.

If the authentication was successful, the key KSEAF received in 5G AV shall become the anchor key in the sense of the key hierarchy in sub-clause 6.2 of the present document. Then the SEAF shall derive the KAMF from the KSEAF and the SUPI according to Annex A.7, and shall provide the ngKSI and the KAMF to the AMF.

If a SUCI was used for this authentication, then the SEAF shall only provide ngKSI and KAMF to the AMF after it receives the 5G-ACA message containing SUPI; no communication services will be provided to the UE until the SUPI is known to the serving network.

The further steps taken by the AUSF upon receiving a successfully verified confirmation message are described in sub-clause 6.1.4 of the present document.

# *** End of Third Change ***