3GPP TSG-SA3 Meeting #91	S3-181460

# Belgrade, Serbia, 16th Apr 2018 - 20th Apr 2018 		merger of S3-181356, 
S3-181166, S3-181291, S3-181165, S3-181439

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 0160 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Authentication Framework |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, KPN N.V., Nokia, NTT Docomo |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SEC |  |  |  |  |  | Date: |  |  | 2018-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Cleaning up the specification by deleting a Editor’s Note on the option of leaving the KAUSF in the AUSF. Furthermore, editorial changes are performed and text is aligned with defintions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Deletion of Editor’s Note and editorial modification of text. Clarifying that there are no use cases presently for KAUSF. Explaining that leaving key at AUSF is operator-specific and out of scope of this spec. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary editor’s note is kept, which may lead to confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.1, 6.1.1.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

## 6.1	Primary authentication and key agreement

### 6.1.1	Authentication framework

#### 6.1.1.1	General

The purpose of the primary authentication and key agreement procedures is to enable mutual authentication between the UE and the network and provide keying material that can be used between the UE and the serving network in subsequent security procedures. The keying material generated by the primary authentication and key agreement procedure results in an anchor key called the KSEAF provided by the AUSF of the home network to the SEAF of the serving network.

Keys for more than one security context can be derived from the KSEAF without the need of a new authentication run. A concrete example of this is that an authentication run over a 3GPP access network can also provide keys to establish security between the UE and a N3IWF used in untrusted non-3GPP access.

The anchor key KSEAF  is derived from an intermediate key called the KAUSF. The KAUSF may be securely stored in the AUSF based on the home operator's policy on using such key.

NOTE 1:	This feature is an optimization that might be useful, for example, when a UE registers to different serving networks for 3GPP-defined access and untrusted non-3GPP access (this is possible according to TS 23.501 [2]). The details of this feature are operator-specific and not in scope of this specification.

NOTE 2:	A subsequent authentication based on the KAUSF stored in the AUSF gives somewhat weaker guarantees than an authentication directly involving the ARPF and the USIM. It is rather comparable to fast re-authentication in EAP-AKA'.

NOTE 3:	There are no use cases defined for the usage of a stored KAUSF.

UE and serving network shall support EAP-AKA' and 5G AKA authentication methods.

The USIM shall reside on a UICC. The UICC may be removable or non-removable.

Editor's Note: ETSI SCP is working on a new secure element called SSP. The SSP can be included as another solution where the USIM can reside on, if the SSP is defined in the Release 15 timeframe and if it complies with the security requirements defined in the present document.

NOTE 3:	For non-3GPP access networks USIM applies in case of terminal with 3GPP access capabilities.

If the terminal supports 3GPP access capabilities, the credentials used with EAP-AKA' and 5G AKA for non-3GPP access networks shall reside on the UICC.

NOTE 4:	EAP-AKA' and 5G AKA are the only authentication methods that are supported in UE and serving network, hence only they are described in sub-clause 6.1.3 of the present document. For a private network using the 5G system as specified in [7] an example of how additional authentication methods can be used with the EAP framework is given in the informative Annex B.

#### 6.1.1.2	EAP framework

The EAP framework is specified in RFC 3748 [27]. It defines the following roles: peer, pass-through authenticator and back-end authentication server. The back-end authentication server acts as the EAP server, which terminates the EAP authentication method with the peer. In the 5G system, when EAP-AKA' is used, the EAP framework is supported in the following way:

-	The UE takes the role of the peer.

-	The SEAF takes the role of pass-through authenticator.

-	The AUSF takes the role of the backend authentication server.

#### 6.1.1.3	Granularity of anchor key binding to serving network

The primary authentication and key agreement procedures shall bind the KSEAF to the serving network. The binding to the serving network prevents one serving network from claiming to be a different serving network, and thus provides implicit serving network authentication to the UE.

This implicit serving network authentication shall be provided to the UE irrespective of the access network technology, so it applies to both 3GPP and non-3GPP access networks.

Furthermore, the anchor key provided to the serving network shall also be specific to the authentication having taken place between the UE and a 5G core network, i.e. the KSEAF shall be cryptographically separate from the key KASME delivered from the home network to the serving network in earlier mobile network generations.

The anchor key binding shall be achieved by including a parameter called "serving network name" into the chain of key derivations that leads from the long-term subscriber key to the anchor key.

The value of serving network name is defined in sub-clause 6.1.1.4 of the present document.

The chain of key derivations that leads from the long-term subscriber key to the anchor key is specified in sub-clause 6.1.3 of the present document for each (class) of authentication methods. The key derivation rules are specified in Annex A.

NOTE:	No parameter like 'access network type' is used for anchor key binding as 5G core procedures are supposed to be access network agnostic.

#### 6.1.1.4	Construction of the serving network name

##### 6.1.1.4.1	Serving network name

The serving network name is used in the derivation of the anchor key. It serves a dual purpose, namely:

-	It binds the anchor key to the serving network by including the SN Id.

-	It makes sure that the anchor key is specific for authentication between a 5G core network and a UE by including a service code set to "5G".

In 5G AKA, the serving network name has a similar purpose of binding the RES* and XRES* to the serving network.

The serving network name is the concatenation of a service code and the SN Id such that the service code prepends the SN Id with a separation character ":".

NOTE:	No parameter like 'access network type' is used for serving network name as it relates to a 5G core procedure that is access network agnostic.

##### 6.1.1.4.2	Construction of the serving network name by the UE

The UE shall construct the serving network name as follows:

1.	It shall set the service code to "5G".

2.	It shall set the network identifier to the SN Id of the network that it is authenticating to.

3.	Concatenate the service code and the SN Id with the separation character ":".

##### 6.1.1.4.3	Construction of the serving network name by the SEAF

The SEAF shall construct the serving network name as follows:

1.	It shall set the service code to "5G".

2.	It shall set the network identifier to the SN Id of the serving network to which the authentication data is sent by the AUSF.

3.	Concatenate service code and the SN Id with the separation character ":".

NOTE:	AUSF gets the serving network name from the SEAF. Before using the serving network name, AUSF checks that the SEAF is authorized to use it, as specified in clause 6.1.2.

# *** End of First Change ***