# 3GPP TSG SA WG3 (Security) Meeting #96-Adhoc	S3-193361

# Chongqing (China), 14-18 October 2019	revision of S3-192936

# Source:	Qualcomm Incorporated

# Title: 	Shared key based MIB/SIBs integrity information provided by gNB

# Document for:	Approval

Agenda Item:	5.5

# 1	Decision/action requested

Approve the solution

# 2	References

[1]	3GPP TR 33.809: “Study on 5G security enhancements against false base stations”

# 3	Rationale

Solution #14 describes a shared key based MIB/SIB protection scheme, where UE sends a hash of MIB/SIB that it has read to the gNB so that gNB can determine whether the UE has read the correct MIB/SIBs. Alternatively, the gNB can provide the hash values of MIB/SIBs in the first protected RRC message during the AS security establishment (e.g., AS security mode command) so that UE can determine the correctness of the MIB/SIBs that it has read. This enables UE to verify the SIB(s) that it may read after the security setup procedure without extra signalling in addition to the ones that UE has already read.

# 4	Detailed proposal

It is proposed that SA3 approve the below pCR for inclusion in the TR 33.809 [1].

***** START OF FIRST CHANGES *****

## 6.x Solution #x: Shared key based MIB/SIBs integrity information provided by gNB

### 6.x.1	Introduction

This solution addresses the security requirement of Key Issue #2: Security protection of system information.

The basic idea is to have gNB provide the hash of the MIB/SIBs to the UE after it has established an AS security context. For the initial access, the AS security mode command is the first message that is integrity protected by the gNB, hence is used to carry the hash values of the MIB/SIBs.

UE determines whether it has read the correct MIB/SIBs by comparing the gNB provided hash values with the locally computed ones. If hash mismatch is detected, the UE requests the concerned MIB/SIBs.

### 6.x.2 	Solution details

Figure 6.x.2-1. System information protection

The procedure is as follows.

1. UE sends an initial NAS message to the network.

2. (optional) UE performs a primary authentication with the network.

3. (optional) AMF sends a NAS Security Mode Command to the UE optionally with indication of support for system information protection. If the UE receives the indication of support for system information protection, the UE stores it as part of the security context.

NOTE 1: If the indication of support for system information protection is indicated in the NAS SMC, system information protection is applied to the entire system (i.e., all gNBs in the PLMN).

4. (optional) UE sends a NAS Security Mode Complete to the AMF.

5. gNB sends an AS Security Mode Command to the UE with a list of hash values of MIB/SIBs.

NOTE 2: Hash of each SIB is provided separately so that UE can only validate the SIBs that it has read.

6. UE compares the received hash values of the MIB/SIBs with the locally computed ones and determines whether the MIB/SIBs that it has read are correct.

NOTE 2: UE may determine to change the cell if a MitM false base station is highly suspected based on the received MIB/SIBs from the gNB.

7. UE sends an AS Security Mode Complete optionally including a list of MIB/SIB(s) whose hash mismatch is identified in step 6.

8. (optional) gNB sends RRC message to UE that includes the MIB/SIBs if UE provided a list of MIB/SIB(s) in step 7.

The rest of the procedure is same as in TS 23.502 [13].

### 6.x.3 Evaluation

This solution fulfils the potential security requirement of the KI #2 when the UE is in the RRC-Connected state.

This solution does not filful the potential security requirement of the KI #2 when the UE is in the RRC-Idle or the RRC-Inactive state.

This solution does not require any additional key provisioning and setup procedure. This solution only requires transporting the hashes of MIB/SIBs or MIB/SIBs using secure RRC signalling.

This solution allows UE to verify not only the MIB/SIBs that it has already read but also those it would read later time.

This solution requires RAN node to provide the hashes of all MIB/SIBs to UE including those that UE is not interested in.

Since the MIB/SIBs mostly contain the radio configuration information that is used for the UE to make a connection to the network and stay in the connected state, enabling to detect the modification of MIB/SIBs during the connection establishment would significantly reduce the impact of SI modification by false base station.

***** END OF FIRST CHANGES *****