# 3GPP TSG SA WG3 (Security) Meeting #96	S3-192680

# 26-30 August, Wroclaw (Poland)	revision of S3-19XXXX

# Source:	China Mobile

# Title:	Key issue to mitigate the SUPI guessing attacks

# Document for:	Approval

Agenda Item:	8.16

1	Decision/action requested

The pCR identifies a new key issue regarding the SUPI guessing attacks for TR 33.846, and is kindly asked to be approved by SA3.

2	Reference

[1]		S.R. Hussain, M. Echeverria, O. Chowdhury, N. Li, E. Bertino. " Privacy Attacks to the 4G and 5G Cellular Paging Protocols Using Side Channel Information,”  Network and Distributed System Security (NDSS) Symposium 2019, February 24–27, San Diego, CA

3	Rationale

5G system applies the ECIES scheme to concealing SUPI so that an attacker is unable to obtain the SUPI.  However, as shown in [1], an adversary could launch a SUPI guessing attack to ascertain whether a guessed SUPI is valid or not in the network. The reason for this is that the network responds to a Registration Request message in a different way considering whether the SUPI is valid or not.  As a result, the adversary could figure out the whole SUPI database to a dedicated network by repeating the SUPI guessing attack many times. Furthermore, the adversary could detemine whether a geninue SUPI (the guessed SUPI is verified) belongs to a dedicated victum or not. This is achieved by forwarding the Authentication Request message to the victim and observing the message type of  victim’s response.  To protect the  permanent subscribtion identifier, there is need to mitigate the SUPI guessing attack.

4	Detailed proposal

*************** Start of Change ****************

5.3	Key issues on availability aspects of SUCI usage

5.3.X	Key Issue #3.X: < Key issue to mitigate the SUPI guessing attacks >

5.3.X.1	Key issue details

It is plausible that the attacker knows the network’s public key provisioned in the attacker-controlled SIM card. The attacker could launch a SUPI guessing attack as follows. It generates a guess SUPI and converts it into SUCI by using network’s public key. Then it sends a fabricated Registration Request message contining the SUCI. Up on receipt of a Registration Request message, the network decrypts SUCI and obtains the SUPI. If SUPI is valid, the network sends an Authenticiation Requst message; otherwise, the network issues a Registration Reject message.  Thus the attacker can ascertain that the guess SUPI is valid if it receiving Authenticiation Requst message.  After a great number of SUPI guessing attacks, the adversary could figure out the whole database of SUPIs for a dedicated network.

The attacker further could determine whether the valid SUPI belongs to a dedicated victim or not. It forwards the received Authenticiation Requst message to the victim. If the victim replies with the Authentication Failure message, then the valid SUPI does not match the victim. If the victim responds with Authentication Request message, then the victim with the valid SUPI is found.

5.3.X.2	Security threats

The attacker could retrieve the whole SUPI database for a dedicated network by using the SUPI guessing attacks. It can further determine whether the valid SUPI belongs to a dedicated victim or not.

5.3.X.3	Potential security requirements

5G system shall provide the mechanisms to mitigate the SUPI guessing attacks.

*************** End of Change ****************