3GPP TSG-SA WG3 Meeting #96			S3-192782

Wroclaw (Poland), 26-30 August 2019

Title:	DRAFT LS to RAN2 on Protection of RRC Reject Message

Response to:

Release:	Rel-16

Work Item:	FS-FBS

Source:	Huawei to be SA3

To:	RAN2

Cc:

Contact Person:

Name:

Tel. Number:

E-mail Address:	x dot x at huawei dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA3 has concluded a solution to protect RRC Reject message as depicted in solution 16 in TR 33.809. The solution details are:

On the UE side, when the UE receives an RRC Reject with a Wait Time:

- When the Wait Time is less than a threshold, the UE shall directly use the value of Wait Time to deny the access.

- When the Wait Time is greater than the threshold, the UE shall check whether there is a RejectMAC-I:

- If not, the UE shall ignore the RRC Reject message.

- If yes, the UE shall verify RejectMAC-I

- If success, the UE shall directly use the value of Wait Time to deny the access.

- If fail, the UE shall ignore the RRC Reject message.

- RejecMAC-I is calculated with source C-RNTI, source PCI, target Cell-ID, resume cause, target C-RNTI.

On the RAN side, when the RAN wants to reject the UE’s access:

- When the RAN has no AS security context, the RAN shall not set the value of Wait Time over the threshold.

- When the RAN has AS security context, if the RAN wants to set the value of Wait Time over the threshold, the RAN shall calculate a RejectMAC-I, and includes it in RRC Reject message.

- RejecMAC-I is calculated with source C-RNTI, source PCI, target Cell-ID, resume cause, target C-RNTI.

How to determine the threshold value:

SA3 knows that in R15, the max value of the waitTime is defined as 16s for eMBB UEs. In order to support backward compatibility, we suggest to standardize the fixed threshold at both R16 eMBB UE and RAN as 16s. The waitTime can be extended longer for R16 eMBB UEs.

For R16 CIoT/URLLC UEs, since it is a new UE, the threshold and value of waitTime can be defined by RAN2.

2. Actions:

To RAN2 group:

ACTION: SA3 kindly ask RAN2 to specify the proposed solution above, and provide feedback for defined threshold for different kind of UEs.

3. Date of Next TSG-SA WG3 Meetings:

SA3#96AdHoc			14-18 October 2019					China (TBD)

SA3#97	18-22 November 2019					Reno, US