# 3GPP TSG SA WG3 (Security) Meeting #96	S3-192847

# Wroclaw (Poland), 26-30 August 2019	revision of S3-19xabc

# Source:	China Mobile

# Title:	Adding Generic assets and threats of GVNP for type 3 into clause 5.2.3.4

# Document for:	Approval

Agenda Item:	8.11

1	Decision/action requested

It is proposed to add Generic assets and threats of GVNP for type 3 into clause 5.2.3.4.

2	References

[1] 3GPP TR 33.916: “Security Assurance Methodology (SCAS) for 3GPP network products”

[2] 3GPP TR 33.926: “Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes”

[3] 3GPP TR 33.848: “Study on Security Impacts of Virtualisation”

[4] ETSI GS NFV 002: “Network Functions Virtualisation (NFV); Architectural Framework”

3	Rationale

This contribution analyzes the generic assets and threats of GVNP for type 3 and adds related texts into clause 5.2.3.4.

4	Detailed proposal

****************** Start of the change ******************

5.2.3	Security Problem Definition (SPD) for 3GPP virtualised network products class

5.2.3.4 	Generic assets and threats for GVNP of type 3

5.2.3.4.1 Generic assets for GVNP of type 3

In addition to the critical assets for GVNP of type 2 described in clause 5.2.3.3.1, GVNP of type 3 also has the following critical assets:

-    Interface between hardware and Virtualised Infrastructure Manager (VIM), for Hardware resource configuration and state information (e.g. events) exchange

Moreover, for inerface between virtualisation layer and hardware, compared to GVNP of type 2, it is only considered when virtualisation layer is decoupled from hardware.

5.2.3.4.2 Generic threats for GVNP of type 3

5.2.3.4.2.1 Introduction

Compared to GVNP of type 2, GVNP of type 3 has hardware besides virtualisation layer. So the generic threats of GVNP for type 2 in clause 5.2.3.3.2.1 can be basically applied to GVNP for type 3. The following subclauses will describe the critical threats for GVNP of type 3.

5.2.3.4.2.2 Threats relating to 3GPP-defined interfaces

All texts from clause 5.2.3.3.2.2 also apply to GVNP of type 3.

5.2.3.4.2.3 Threats relating to ETSI-defined interfaces

In addition to threats described in clause 5.2.3.3.2.3, GVNP of type 3 also has following threats relating to ETSI-defined interfaces:

-     The threats on interface between hardware and Virtualised Infrastructure Manager (VIM): an attacker can tamper the hardware resource configuration to cause resource configuration errors or an attacker can intercept hardware state information to lead the information disclosure.

Moreover, the threats on interface between virtualisation layer and hardware only are considered when virtualisation layer is decoupled from hardware.

Editor’s note: This document will refer threats that are described in 3GPP TR 33.848 or/and ETSI specifications.

5.2.3.4.2.4 Spoofing identity

5.2.3.4.2.4.1 Default Accounts

The threat in clause 5.3.3.1 of TR 33.926 also applies to GVNP of type 3. In addition to using default account to access GVNP, an attacker can also utilize default account to access VNF of GVNP of type 3 through VNC (Virtual Network Console).

5.2.3.4.2.4.2 Weak Password Policies

The threat in clause 5.3.3.2 of TR 33.926 also applies to GVNP of type 3. In addition to using weak password to access GVNP, an attacker can also utilize weak password to access VNF of GVNP of type 3 through VNC (Virtual Network Console).

5.2.3.4.2.4.3 Password peek

The threat in clause 5.3.3.3 of TR 33.926 also applies to GVNP of type 3. In addition to using peeked password to access GVNP, an attacker can also utilize peeked password to access VNF of GVNP of type 3 through VNC (Virtual Network Console).

5.2.3.4.2.4.4 Direct Root Access

The threat in clause 5.3.3.4 of TR 33.926 also applies to GVNP of type 3.

5.2.3.4.2.4.5 IP Spoofing

All texts from TR 33.926, clause 5.3.3.5 also apply to GVNP of type 3. The objectives of unauthorized access include VNF in addition to computer.

5.2.3.4.2.4.6 Malware

The threat in clause 5.3.3.6 of TR 33.926 also applies to GVNP of type 3.

5.2.3.4.2.4.7 Eavesdropping

The threat in clause 5.3.3.7 of TR 33.926 also applies to GVNP of type 3.

5.2.3.4.2.5 Tampering

5.2.3.4.2.5.1 Software Tampering

The threat in clause 5.3.4.1 of TR 33.926 also applies to GVNP of type 3.

5.2.3.4.2.5.2 Ownership File Misuse

The threat in clause 5.3.4.2 of TR 33.926 also applies to GVNP of type 3.

5.2.3.4.2.5.3 Boot for GVNP of type 3

All texts in clause 5.2.3.3.2.5.3 also apply to GVNP of type 3.

5.2.3.4.2.5.4 Log Tampering

The threat in clause 5.3.4.4 of TR 33.926 also applies to GVNP of type3.

5.2.3.4.2.4.5 OAM traffic Tampering

The threat in clause 5.3.4.5 of TR 33.926 also applies to GVNP of type 3.

5.2.3.4.2.4.6 File Write Permissions Abuse

The threat in clause 5.3.4.6 of TR 33.926 also applies to GVNP of type 3.

5.2.3.4.2.4.7 User Session Tampering

The threat in clause 5.3.4.7 of TR 33.926 also applies to GVNP of type3.

5.2.3.4.2.6 Repudiation

5.2.3.4.2.6.1 Lack of User Activity Trace

The threat in clause 5.3.5.1 of TR 33.926 also applies to GVNP of type 3.

5.2.3.4.2.7 Information disclosure

The threat in all subclauses of clause 5.3.6 for TR 33.926 also applies to GVNP of type 3.

5.2.3.4.2.8 Denial of Service

All texts from clause 5.2.3.2.2.8 also apply to GVNP of type 3.

5.2.3.4.2.9 Elevation of privilege

The threat in all subclauses of clause 5.3.8 in TR 33.926 also applies to GVNP of type 3.

****************** End of the change ******************