3GPP TSG-SA WG3 Meeting #96	 		S3-192850

Wroclaw (PL) 26 -30 July 2019

Title:	LS on AUSF role in Secondary slice specific authentication

Release:	Rel-16

Work Item:	FS_eNS_SEC

Source:	SA3

To:	SA2

Cc:

Contact Person:

#### Name:	Henrik Normann

Tel. Number:

E-mail Address:	henrik dot normann at ericsson dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA3 would like to inform SA2 that SA3 is progressing on the TR on Study on Security Aspects of Enhanced Network Slicing, TR 33.813, and has concluded on the solutions basis for the Network Slice-Specific Authentication and Authorization procedure.

SA3’s conclusion in TR 33.813 concluded to base the normative work for Key Issue #1 (Authentication for access to specific Network Slices) on a merge of Solution#1, Solution#2 and Solution#4.

In summary, according to these solutions, the AMF is the initiator of the Network Slice-Specific Authentication and Authorization procedure, after primary authentication and UE registration in 5GC is completed. The AMF executes the secondary slice specific authentication with a AAA-S using EAP acting as an EAP authenticator. Optionally, the EAP authentication exchange may take place via a AAA-Proxy between the AMF and the AAA-S if the AAA-S is placed in an external domain to the 3GPP 5GC.

SA3 would like to indicate to SA2 that the solutions agreed by SA3 in TR 33.813 do not involve the AUSF during the Network Slice-Specific Authentication and Authorization procedure.

SA3 has noticed that the flow specified in TS 23.502 clause 4.2.9.2 does however involves the AUSF in all the interactions between the AMF and the AAA-S. SA3 believes that the AAA-Proxy functionality when needed may be provided by the AUSF, but its presence should not be mandated if AAA-S resides within the HPLMN of the 5GC operator. Otherwise, the involvement of the AUSF as defined in TS 23.502 does not seem to be sufficiently justified from SA3 point of view.

SA3 has not yet defined normative flows and description for the Network Slice-Specific Authentication and Authorization procedure in SA3 specifications and would like to request the opinion of SA2 of the reason why AUSF should be involved in the execution of the Network Slice-Specific Authentication and Authorization procedure.

2. Actions:

ACTION: SA3 kindly requests SA2 for the reason why the AUSF is involved in the Network Slice-Specific Authentication and Authorization procedure as defined in TS 23.502 clause 4.2.9.2.

SA3 additionally asks SA2 not to resolve the corresponding Editor’s Note present in TS 23.502 clause 4.2.9.1 until the need for possible alignment between SA2 and SA3 specifications is clarified.

3. Date of Next TSG-SA WG3 Meetings:

SA3#96Ad Hoc	                10 - 14 October 2019			Chongqing, China

SA3#97	                18 - 22 November 2019			Reno, USA