3GPP TSG-SA3 Meeting #96	S3-193021

# Wroclaw, Poland, 26th Aug 2019 - 30th Aug 2019							Revision of S3-192519

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.180 | CR | 0115 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications for Protected MCData |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Airbus DS SLC |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCSec |  |  |  |  |  | Date: |  |  | 2019-07-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current specification of Protected MCData cannot have a fully compliant implementation. The main issues are: - The main incoherency is that TS 33.180 clause 8.5.4.1 defines a new type of message (protected payload) which does not have any message type defined in TS 24.282. On the contrary, TS 24.282 uses two information elements (IE) “security parameters” and “security parameters and payload”; the format of these IE should be defined in TS 33.180, but it is not. - There is no reason why TS 24.282 contains messages with separate “security parameters” IE and “payload” IE, and on the other hand messages with “security parameters and payload” IE. - TS 33.180 clause 8.5.4.1 mentions the possibility that “MCData Protected Payload message is distributed as part of protected FD Signalling” but TS 24.282 clause 15.1.3 does not propose to include security parameters in a FD SIGNALLING message. - The value of the “message type” for the MCData Protected Payload to be used for the Key Properties in TS 33.180 clause E.6.1 has not been defined. The preferred correction is to follow TS 24.282 and to have TS 33.180 define the required Information Element. The other possible correction would have been to have TS 33.180 define a new type of message, and to change TS 24.282 to remove the “protected” bit in the message type and modify all references to security parameters for existing messages. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The “protected” bit in the message type is defined to be the indication that a MCData message is protected. It results in the following changes: - In TS 24.282 - Replace the “security parameters and payload” IE from clauses 15.1.4 and 15.1.12 with an optional 22-bytes TV element “security parameters”, referring to TS 33.180, and explaining that this optional IE is present if and only if the “protected” bit is set, and defining the parameters to encrypt all payloads. - Update clause 15.1.7 with the same statement on the “protected” bit. - Update clause 15.1.3 to add the possibility that a FD SIGNALLING message is protected. - In TS 33.180 - Modify Table 8.5.4.1-1 to describe the “security parameters” IE and add Table 8.5.4.1-2 to describe the “security parameters and payload” IE. - Explain which part of the Payload IE and of the Metadata IE are protected when there is a “security parameters” IE in the MCData message. Specify that the content of E.6.1 General Extension payload is a protected MCData message of type DATA PAYLOAD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The implementation of MCData Protected Messages cannot meet the specified requirements and is therefore incompatible with vendors. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 24.282 CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************ Start of change 1 *********************************

### 8.5.4	Payload protection

#### 8.5.4.1	Format of protected payloads

All protected payloads shall have “security parameters” in the format defined in table 8.5.4.1-1 or table 8.5.4.1-2:

# Table 8.5.4.1-1: Security Parameters IE

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Security Parameters IEI |  |  |  |  |  |  |  | octet 1 |
| Algorithm (see 8.5.4.2) |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 3 |
| IV |  |  |  |  |  |  |  | … |
|  |  |  |  |  |  |  |  | octet 18 |
|  |  |  |  |  |  |  |  | octet 19 |
| DPPK-ID |  |  |  |  |  |  |  | octet 20 |
|  |  |  |  |  |  |  |  | octet 21 |
|  |  |  |  |  |  |  |  | octet 22 |

# Table 8.5.4.1-2: Security Parameters and Payload IE

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Security Parameters and Payload IEI |  |  |  |  |  |  |  | octet 1 |
| Algorithm (see 8.5.4.2) |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 3 |
| IV |  |  |  |  |  |  |  | … |
|  |  |  |  |  |  |  |  | octet 18 |
|  |  |  |  |  |  |  |  | octet 19 |
| DPPK-ID |  |  |  |  |  |  |  | octet 20 |
|  |  |  |  |  |  |  |  | octet 21 |
|  |  |  |  |  |  |  |  | octet 22 |
| Length of Payload contents |  |  |  |  |  |  |  | octet 23 |
|  |  |  |  |  |  |  |  | octet 24 |
|  |  |  |  |  |  |  |  | octet 25 |
| Protected payload contents |  |  |  |  |  |  |  | … |
|  |  |  |  |  |  |  |  | octet n |

In a Protected MCData message contains Payload IE, then it shall contain a “Security Parameters” IE and the “Payload contents” (cf TS 24.282 clause 15.2.13) of all Payload IE is encrypted and integrity protected using the Security Parameters.

In a Protected MCData message contains Metadata IE, then it shall contain a “Security Parameters” IE and the “Metadata Contents” (cf. TS 24.282 clause 15.1.17) is encrypted and integrity protected using the Security Parameters.

The “Protected payload contents” of a Security Parameters and Payload IE is the “Payload contents” encrypted and integrity protected using the security parameters.

************************ End of change 1 *********************************

************************ Start of change 2 *********************************

## E.6.1	General

The parameters associated with the key shall be contained in the 'General extension payload' specified in IETF RFC 3830 [22] using the '3GPP key parameters ' Type value and contained within the signed envelope of the MIKEY-SAKKE I_MESSAGE specified in clause E.2. The format and cryptography of the payload are specified in this subclause.

The General Extensions Field Name '3GPP key parameters' type takes on the IANA assigned value of '7' [52].

The payload consist of a series of information elements. The standard format and encoding rules for the information elements follow that defined for the MCPTT Off-Network Protocol (MONP) as documented in Annex I of 3GPP TS 24.379 [10].

The four octets consisting of the header of the 'General extension payload' shall be formatted according to IETF RFC 3830 [22].

The contents of the 'General extension payload' shall be an MCData Protected DATA PAYLOAD type of message as defined in TS 24.282 clause 15.1.4 with a payload containing BINARY type of Payload content, consisting of the 'Key Parameters' payload defined in this clause. The DPPK-ID of the Security Parameters of the Protected Data Payload shall be the same as the CSB-ID of the encapsulating MIKEY payload (e.g. GMK, MuSiK, etc).

The 'Key Parameters' payload is a type 6 information element composing a 1 byte Key Parameters IEI, a 2 byte length of the Key Parameters payload contents, and the Key Parameters payload content itself. The Key Parameters payload content shall be of the format specified in Table E.6.1-1.

# Table E.6.1-1: Key Parameters Payload content

| Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Key Type | The type of key. Clause E.6.11. | M | V | 1 |
| Status | The current status of the key. Clause E.6.9. | M | V | 4 |
| Activation Time | Date and Time when the key may start to be used. Clause E.6.4. | M | V | 5 |
| Expiry Time | Date and Time when the key may no longer be used. Clause E.6.10. | M | V | 5 |
| Text | A human-readable name for the key Clause E.6.5. | M | LV-E | 2-x |
| MC Group Ids | The MC Group IDs associated with the key (if any) Clause E.6.3. | C | LV-E | 2-x |
| Reserved | Additional information associated with the key (if any) Clause E.6.6. | O | TLV-E | x |

NOTE:	The 'MC group IDs' IE is only present in the Key Parameters payload if the key type is 'GMK', 'MKFC' or 'MuSiK'.

The IEs in the Key Parameters Payload are described in the following subclauses.

************************ End of change 2 *********************************