# 3GPP TSG SA WG3 (Security) Meeting #96	S3-193183

# Wroclaw (Poland), 26-30 August 2019	revision of S3-192842

# Source:	China Mobile

# Title:	Adding the description of Generic Vitualized Network Product model of type 1

# Document for:	Approval

Agenda Item:	8.11

1	Decision/action requested

It is proposed to add the description of Generic Vitualized Network Product model of type 1.

2	References

[1] 3GPP TR 33.916: “Security Assurance Methodology (SCAS) for 3GPP network products”

[2] 3GPP TR33.926: “Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes

3	Rationale

This contribution describes the vitualized network product class description. It also describes generic virtualized network product model of type 1.

4	Detailed proposal

****************** Start of the change ******************

2	References

Editor’s Note: This clause will outline that the present document contains references.

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]  3GPP TR 33.916: "Security Assurance Methodology (SCAS) for 3GPP network products"

[2]  3GPP TR 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes"

[3]  3GPP TR 33.117: “Catalogue of general security assurance requirements”

[4]  3GPP TR 21.905:” Vocabulary for 3GPP Specifications”

[5] 3GPP TS 28.500: "Management concept, architecture and requirements for mobile networks that include virtualized network functions"

[6] ETSI GS NFV-SEC 001: "Network Functions Virtualisation (NFV); NFV Security; Problem Statement"

[7] GSMA Network Equipment Security Assurance Scheme – Vendor Development and Product Lifecycle Requirements and Accreditation Process

[x] ETSI GS NFV 002: " Network Functions Virtualisation (NFV); Architectural Framework"

****************** Next of the change ******************

5.2.x Generic vitualized network product model class description

5.2.x.1 Introduction

According to the definition of virtualized network product class, a virtualized network product class is the class of products that implement 3GPP defined network functionalities running on Network Function Virtualisation Infrastructure (NFVI). There are three types of the classes that are described in clause 4.0.1. The generic virtualized network product model classes are described in the following clauses.

5.2.x.2 Generic virtualized network product model of type 1

For the virtualized network product class type 1 (i.e. implementing 3GPP defined functionalities only), the following figure 5.2.x.2-1 depicts the components of a generic network product model at a high level.

Figure 5.2.x.2-1 GVNP model

Editor’s Note: How to involve containers into this model is FFS.

Editor’s Note: The figure needs to be updated.

The components in the figure 5.2.x.2-1are further described in the following subclauses.

5.2.x.2.1 Functions defined by 3GPP

For a generic virtualized network function, it will implement 3GPP-defined functions. Unlike a generic physical network product, the 3GPP-denfined functions can be deployed in multiple VMs and which feature is deployed in a VM of the GVNP is implemented by vendor.

To maintain generality and avoid overlap, the GVNP SCAS intends to explicitly address all GVNP functions that, if present in a GVNP network product, need to be evaluated and hence covered by requirements in the GVNP SCAS.

5.2.x.2.2 Other functions

A GVNP will also contain functionalities not or not fully covered in 3GPP specifications.

Examples include, but are not limited to, remote management functions.

5.2.x.2.3 Operating system (OS)

The present document assumes that the functions of GVNP are implemented on multiple VMs. Each VM which is running on common platform requires a guest operating system to run.

5.2.x.2.4 Interfaces

Compared to generic physical network product, GVNP has one type of logical interface, i.e.remote logical interfaces. These remote logical interfaces are interfaces which can be used to communicate with the GVNP from another network node and also include the remote access interfaces to the GNP for its maintenance through e.g. an Element Management System (EMS), a Virtualised Network Function Manager (VNFM).

# A GVNP hosts the following remote logical interfaces:

-	Service interfaces that are defined in pertinent 3GPP specifications

-	Service interfaces that are not defined by 3GPP

-	Remote OAM interface

-	EMS (Element Management System) interface

-	Interfaces that are defined by ETSI NFV specifications[x]:

- Interfaces related to virtualisation and virtualisation resource management

- Interfaces related to execution environment provision

Editor’s Note: How ETSI NFV interface and 3GPP interfaces are mapped is FFS.

****************** End of the change ******************