- 3GPP TSG SA WG3 (Security) Meeting #98e	S3-200086

- 02 - 06 March 2020, e-meeting	revision of S3-20xabc

- Source:	LG Electronics

- CTitle:	Common security procedures over V2X PC5

- Document for:	Approval

Agenda Item:	3.15

# 1	Decision/action requested

This contribution proposes to capture procedures that are commonly applied to all types of NR PC5 connection

# 2	References

[1]	3GPP TS 33.536 v0.2.0: "Security aspect of 3GPP support for advanced Vehicle-to-Everything (V2X) services".

[2]	3GPP TR 33.836 v0.5.0: "Study on Security Aspects of 3GPP support for Advanced V2X Services".

[3]	3GPP TS 23.287 v16.1.0: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services (Release 16) ".

# 3	Rationale

This contribution is prepared to capture the security procedure that can be applied to all types of NR PC5 connection from TR [2]. There is only one procedure identified so far.

# 4	Detailed proposal

It is proposed that SA3 agree the below pCR for inclusion in the TS [1].

**** START OF CHANGES ****

## 5.2	Common security

### 5.2.3	Procedures

5.2.3.1	Cross-RAT PC5 control authorization indication

During the Registration procedure for a UE, AMF shall manage cross-RAT PC5 control authorization by including the cross-RAT RC5 control authorization indication if applicable based on the subscription data received from UDM, in the NGAP message and sending it to NG-RAN. More details are specified in TS 23.287 [3] clause 6.5.

**** END OF CHANGES ****