3GPP TSG-SA3 Meeting #98e 	S3-200413

- e-meeting, 2 – 6 March 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | Draft CR | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ERP for TNAP mobility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  |  | Date: |  | 2020-02-17 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | So far ERP for TNAP mobility is not introduced but already part of TS 23.502. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of ERP for TNAP mobility |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7A.b.y, 7A.b.z, A.Ya (new), A.Yb (new), A.Yc (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** START OF CHANGES ****

- 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[4]	IETF RFC 4303: "IP Encapsulating Security Payload (ESP)".

[5]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[6]	IETF RFC 4301: "Security Architecture for the Internet Protocol".

[7]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[8]	3GPP TS 23.502: "Procedures for the 5G System".

[9]	3GPP TS 33.102: "3G security; Security architecture".

[10]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[11]	3GPP TS 33.402: "3GPP System Architecture Evolution (SAE); Security aspects of non-3GPP accesses".

[12]	IETF RFC 5448: " Improved Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA')".

Editor’s note:	This reference will be removed and references to it updated when the IETF updates the RFC and publishes a new RFC that supercedes this RFC.

[13]	3GPP TS 24.301: " Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[14]	3GPP TS 35.215: " Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2; Document 1: UEA2 and UIA2 specifications".

[15]	NIST: "Advanced Encryption Standard (AES) (FIPS PUB 197)".

[16]	NIST Special Publication 800-38A (2001): "Recommendation for Block Cipher Modes of Operation".

[17]	NIST Special Publication 800-38B (2001): "Recommendation for Block Cipher Modes of Operation: The CMAC Mode for Authentication".

[18]	3GPP TS 35.221: " Specification of the 3GPP Confidentiality and Integrity Algorithms EEA3 & EIA3; Document 1: EEA3 and EIA3 specifications".

[19]	3GPP TS 23.003: "Numbering, addressing and identification".

[20]	3GPP TS 22.101: "Service aspects; Service principles".

[21]	IETF RFC 4187: "Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA)".

[22]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[23]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[24]	3GPP TS 33.117: "Catalogue of general security assurance requirements".

[25]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2)"

[26]	IETF RFC 4303: "IP Encapsulating Security Payload (ESP)"

[27]	IETF RFC 3748: "Extensible Authentication Protocol (EAP)".

[28]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[29]	SECG SEC 1: Recommended Elliptic Curve Cryptography, Version 2.0, 2009. Available http://www.secg.org/sec1-v2.pdf

[30]	SECG SEC 2: Recommended Elliptic Curve Domain Parameters, Version 2.0, 2010. Available at http://www.secg.org/sec2-v2.pdf

[31]	3GPP TS 38.470: "NG-RAN; F1 General aspects and principles".

[32]	3GPP TS 38.472: "NG-RAN; F1 signalling transport".

[33] 	3GPP TS 38.474: "NG-RAN; F1 data transport".

[34]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)"

[35]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[36] 	3GPP TS 35.217: "Specification of the 3GPP Confidentiality and Integrity Algorithms UEA2 & UIA2; Document 3: Implementors' test data".

[37] 	3GPP TS 35.223: "Specification of the 3GPP Confidentiality and Integrity Algorithms EEA3 & EIA3; Document 3: Implementors' test data".

[38]	IETF RFC 5216: "The EAP-TLS Authentication Protocol".

[39]	IETF RFC 4346: "The Transport Layer Security (TLS) Protocol Version 1.1".

[40]	IETF RFC 5246: "The Transport Layer Security (TLS) Protocol Version 1.2".

[41]	3GPP TS 38.460: "NG-RAN; E1 general aspects and principles".

[42]	Void.

[43]	IETF RFC 6749: "OAuth2.0 Authorization Framework".

[44]	IETF RFC 7519: "JSON Web Token (JWT)".

[45]	IETF RFC 7515: "JSON Web Signature (JWS)".

[46]	IETF RFC 7748: "Elliptic Curves for Security".

[47]	IETF RFC 7540: " Hypertext Transfer Protocol Version 2 (HTTP/2)".

[48]	IETF RFC 5280: "Internet X.509 Public Key Infrastructure Certificate and Certificate Revocation List (CRL) Profile".

[49]	IETF RFC 6960: "X.509 Internet Public Key Infrastructure Online Certificate Status Protocol - OCSP".

[50]	IETF RFC 6066: "Transport Layer Security (TLS) Extensions: Extension Definitions".

[51]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[52]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[53]	3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

[54]	3GPP TS28.533: " Management and orchestration; Architecture framework".

[55]	3GPP TS28.531: "Management and orchestration of networks and network slicing; Provisioning".

[56]	IETF RFC 4279 "Pre-Shared Key Ciphersuites for Transport Layer Security (TLS)".

[57]	IETF RFC 7542: "The Network Access Identifier".

[58]	IETF RFC 6083: " Datagram Transport Layer Security (DTLS) for Stream Control Transmission Protocol (SCTP)".

[59]	IETF RFC 7516: "JSON Web Encryption (JWE)".

[60]	IETF RFC 8446: "The Transport Layer Security (TLS) Protocol Version 1.3".

[61]	IETF RFC 5705,"Keying Material Exporters for Transport Layer Security (TLS)".

[62]	IETF RFC 5869 "HMAC-based Extract-and-Expand Key Derivation Function (HKDF)".

[63]	NIST Special Publication 800-38D: "Recommendation for Block Cipher Modes of Operation: Galois Counter Mode (GCM) and GMAC".

[64]	IETF RFC 6902: "JavaScript Object Notation (JSON) Patch".

[65]	3GPP TS 31.115: "Secured packet structure for (Universal) Subscriber Identity Module (U)SIM Toolkit applications.

[66]	3GPP TS 31.111: "Universal Subscriber Identity Module (USIM), Application Toolkit (USAT)".

[67]	Internet draft draft-ietf-emu-rfc5448bis: "Improved Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA')".

[68]	3GPP TS 29.510: "5G System; Network function repository services".

[69]	3GPP TS 36.331: "Radio Resource Control (RRC); Protocol specification".

[xx]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)"

[xy]	IEEE Std 802.11-2016 (Revision of IEEE Std 802.11-2012) - IEEE Standard for Information technology—Telecommunications and information exchange between systems Local and metropolitan area networks—Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications.

[xz]	IETF RFC 2410 "The NULL Encryption Algorithm and Its Use With IPsec".

[ya]	IETF RFC 6696 "EAP Extensions for the EAP Re-authentication Protocol (ERP)"

[yb]	IETF RFC 5295 "Specification for the Derivation of Root Keys from an Extended Master Session Key (EMSK)"

**** Start of 2nd   CHANGE ****

### 7A.b.y Mobility handling for Trusted non-3GPP Access

Editor's note:	This clause will capture the security of mobility handling for a UE accessing to the 5GC via trusted non-3GPP access.

The ERP procedure can be only successfully executed if supported by UE, TNAP, TNGF, AMF and AUSF.

If the trusted non-3GPP access network supports ERP [ya], then the TNGF shall send the ERP Request together with the Registration Request in step 6b of Figure 6.Y.4-1 to the AMF, which is further forwarded to AUSF in step 8a.

The AUSF does not send a response to the ERP Request if it does not support ERP or if it selects 5G-AKA as authentication method. The AUSF only sends a response if it supports ERP and the selected authentication method is EAP-AKA’. In this case, the AUSF derives the EAP re-authentication root key rRK from the EMSK (as specified inRFC 6696 [ya]) and provides it to the AMF together with the SEAF key in step 8c. The rRK key is further send from the AMF to the TNGF together with the TNGF key in step 10a.

If the TNGF sends an ERP Request but does not receive an rRK in step 10a, then the ERP exchange cannot be performed and the UE will initiate a full re-authentication after moving to a new TNAP.

The TNGF will derive the rIK and rMSK from the rRK per TNAP according to RFC 6696 [ya] and the UE will derive the keys accordingly as well when it receives the EAP-Initiate/Re-auth Start message from the target TNAP.

The ERP procedure for Trusted non-3GPP Access is defined in clause 4.12a.9 of TS 23.502[8].

Editor’s Note: The ERP procedure in this specification should be checked against the RFC 6696.

**** Start of 3rd    CHANGE ****

### 7A.b.z Key hierarchy for Trusted non-3GPP Access

The key hierarchy described in clause 6.2.1 applies, with the following changes:

The key derived for non-3GPP access is called KTNGF in the context of trusted access.

The key KTNGF received from AMF is used for two different purposes; to setup IPSec SAs between the UE and the TNGF and to create WLAN keys between the UE and the TNAP.

To separate the keys for these purposes, the key hierarchy in Figure 7A.b.z-1 shall be used. The KTIPSec key is used to setup IPSec SAs and the KTNAP key is used to setup access security.

The keys KTIPSec and KTNAP are derived as described in Appendix A.X.

In case ERP is supported for EAP-AKA’, then the EAP re-authentication root key rRK is derived from the EMSK in the AUSF and provisioned to the TNGF via AMF. The subsequent re-authentication MSK (rMSK) keys and the integrity protection key for the ERP messages are derived in the ME and the TNGF from rRK. The rMSK act as the TNAP Key and it is used to setup access security.

Figure 7A.b.z-1 Key hierarchy for trusted non-3GPP access

**** Start of 4th     CHANGE ****

**** END OF CHANGES ****