Clean version of DTS/LI-00024 after TC LI#13 Stockholm

ETSI DTS 102 232-05 V0.5.0 (2006-09)

Technical Specification

Lawful Interception (LI);

Service-specific details for IP Multimedia Services

Reference

DTS/LI-00024

Keywords

IMS, lawful interception, IP, security

ETSI

650 Route des Lucioles

F-06921 Sophia Antipolis Cedex - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - NAF 742 C

Association à but non lucratif enregistrée à la

Sous-Préfecture de Grasse (06) N° 7803/88

Important notice

Individual copies of the present document can be downloaded from:
http://www.etsi.org

The present document may be made available in more than one electronic version or in print. In any case of existing or perceived difference in contents between such versions, the reference version is the Portable Document Format (PDF). In case of dispute, the reference shall be the printing on ETSI printers of the PDF version kept on a specific network drive within ETSI Secretariat.

Users of the present document should be aware that the document may be subject to revision or change of status. Information on the current status of this and other ETSI documents is available at http://portal.etsi.org/tb/status/status.asp

If you find errors in the present document, send your comment to:
editor@etsi.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© European Telecommunications Standards Institute 2006.

All rights reserved.

DECTTM, PLUGTESTSTM and UMTSTM are Trade Marks of ETSI registered for the benefit of its Members.
TIPHONTM and the TIPHON logo are Trade Marks currently being registered by ETSI for the benefit of its Members. 3GPPTM is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners.

Contents

Intellectual Property Rights	4

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions and abbreviations	7

3.1	Definitions	7

3.2	Abbreviations	7

4	General	9

4.1	Reference Model for Lawful Interception	9

4.2	Reference system model	10

4.2.1	Network layer Interception	10

4.2.2	Service layer Interception	11

4.3	General Requirements	11

5	Interception of IP Multimedia services	12

5.1	Identification of target of interception	12

5.1.1	SIP Target Identification	12

5.1.2	H.323 Target Identification	12

5.2	Interception of signalling	12

5.2.1	Provisioning of the SIP IRI IIF	12

5.2.2	Provisioning of the H.323 IRI IIF	12

5.3	Assigning a value to the Communication Identity Number	13

5.3.1	Assigning a CIN value to SIP related IRI	13

5.3.2	Assigning a CIN value to H.323 related IRI	13

5.4	Events and IRI record types	13

5.5	Interception of Content of Communication	14

6	Handover Interface	15

6.1	Intercepted Related Information	15

6.2	Correlation of IRI and CC	15

7	ASN.1 for IRI and CC	16

7.1	ASN.1 syntax tree for HI2 and HI3 headers	16

7.2	ASN.1 specification	17

Annex A (informative): Interception using H.248	19

A.1	Purpose of this annex	19

A.2	Notes on interception using H.248.	19

A.2.1	Target identification (see also clause 5.1)	19

A.2.2	Provisioning of the H.248 IRI IIF (see also clause 5.2)	20

A.3	Problems in H.248 interception	20

A.3.1	Missing information in H.248 signalling	20

A.3.2	Missing call content	21

Annex B (normative): Minimum set of functional attributes to be provided	22

B.1	General requirements	22

B.2	Result of interception	23

B.3	Location information	23

B.4	Time constraints	23

B.5	Technical handover interfaces and format requirements	24

Annex C (informative): Change Request history	25

History	26

# Intellectual Property Rights

IPRs essential or potentially essential to the present document may have been declared to ETSI. The information pertaining to these essential IPRs, if any, is publicly available for ETSI members and non-members, and can be found in ETSI SR 000 314: "Intellectual Property Rights (IPRs); Essential, or potentially Essential, IPRs notified to ETSI in respect of ETSI standards", which is available from the ETSI Secretariat. Latest updates are available on the ETSI Web server (http://webapp.etsi.org/IPR/home.asp).

Pursuant to the ETSI IPR Policy, no investigation, including IPR searches, has been carried out by ETSI. No guarantee can be given as to the existence of other IPRs not referenced in ETSI SR 000 314 (or the updates on the ETSI Web server) which are, or may be, or may become, essential to the present document.

# Foreword

This Technical Specification (TS) has been produced by ETSI Technical Committee Lawful Interception (LI).

# Introduction

The present document focuses on Lawful Interception of IP Multimedia Services. It is to be used in conjunction with TS 102 232-01 [2], in which the handling of the intercepted information is described.

# 1	Scope

The present document specifies interception of Internet Protocol (IP) Multimedia (MM) Services based on the Session Initiation Protocol (SIP) and Real time Transport Protocol (RTP) and IP MM services as described by the ITU-T Recommendations H.323 [6] and H.248 [7].

The present document is consistent with the definition of the Handover Interface, as described in TS 102 232-01 [2].

The present document does not override or supersede any specifications or requirements in 3GPP TS 33.108 [9] and TS 101 671 [1].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication and/or edition number or version number) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.

Referenced documents which are not found to be publicly available in the expected location might be found at http://docbox.etsi.org/Reference.

[1]	ETSI TS 101 671: “Lawful Interception (LI); Handover interface for the lawful interception of telecommunications traffic”.

NOTE:	Periodically TS 101 671 is published as ES 201 671. A reference to the latest version of the TS as above reflects the latest stable content from ETSI/TC LI.

[2]	ETSI TS 102 232-01: “Lawful Interception (LI); Handover Specification for IP Delivery”.

[3]	ITU-T Recommendation X.680: "Abstract Notation One (ASN.1): Specification of basic notation".

[4]	IETF RFC 3261 - SIP: Session Initiation Protocol (SIP), June 2002

[5]	IETF RFC 3550 - RTP: A Transport Protocol for Real-Time Applications, July 2003

[6]	ITU-T Recommendation H.323: “Packet-based multimedia communications systems”, July 2003

[7]	ITU-T Recommendation H.248.1: “Gateway control protocol”, Version 2, May 2002

[8]	ETSI TR 102 528: “Lawful Interception (LI); Interception Domain Architecture for IP Networks”.

[9]	ETSI TS 133 108: Universal Mobile Telecommunications System (UMTS);3G security;Handover interface for Lawful Interception (LI) (3GPP TS 33.108)

[10]	ETSI TS 101 331: “Lawful Interception (LI); Requirements of Law Enforcement Agencies”.

[11]	ATIS-1000678.2006, Lawfully Authorized Electronic Surveillance (LAES) for Voice over Packet Technologies in Wireline Telecommunications Networks, Version 2 (revision of ANS T1.678-2004), March 2006.

[12]	ITU-T Recommendation H.225.0: “Call signalling protocols and media stream packetization for packet-based multimedia communication systems”, July 2003

[13]	ITU-T Recommendation H.245: “Control protocol for multimedia communication”, July 2003

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions apply, in addition to those provided in TS 101 671 [1] and TS 102 232-01 [2]:

Multimedia: The use of computers to present text, graphics, video, animation, and sound in an integrated way.

Multimedia Service: A communication service that offers Multimedia communication to end-users.

IP Multimedia Service: A Multimedia service that utilizes the Internet Protocol (IP) for the transport of data.

TSAP identifier: The piece of information used to multiplex several transport connections of the same type on a single H.323 entity with all transport connections sharing the same Network Address, (e.g. the port number in a TCP/UDP/IP environment). TSAP identifiers may be (pre)assigned statically by some international authority or may be allocated dynamically during the setup of a call. Dynamically assigned TSAP identifiers are of transient nature, i.e. their values are only valid for the duration of a single call.

Termination: An entity in ITU-T Recommendation H.248 [7] that acts as a source or sink of media. Terminations may be physical, such as a given channel on a TDM line, or ephemeral, such as an IP endpoint.

Context: A logical collection of ITU-T Recommendation H.248 [7] terminations.

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

AF	Administration Function

ASN.1	Abstract Syntax Notation One

CC IIF	CC Interception Function

CC	Content of Communication

CCTF	Content of Communication Trigger Function

CCCI	Content of Communication Control Interface

CCTI	Content of Communication Trigger Interface

CID	Communication Identifier

CIN	Communication Identification Number

CSP	Communications Service Provider (covers all AP/NWO/SvP)

GW	Gateway

HI1	Handover Interface 1 (for Administrative Information)

HI2	Handover Interface 2 (for Intercept Related Information)

HI3	Handover Interface 3 (for Content of Communication)

IAP	Internet Access Provider

INI	Internal Network Interface

IP	Internet Protocol

IRI	Intercept Related Information

IIF	Internal Interception Function

IRI IIF	IRI Interception Function

LEA	Law Enforcement Agency

LEMF	Law Enforcement Monitoring Facility

LI	Lawful Interception

LIID	Lawful Interception Identifier

MF	Mediation Function

MG	Media Gateway

MGC	Media Gateway Controller

MM	Multi Media

PDU	Protocol Data Unit

RTP	Real time Transport Protocol

SDP	Session Description Protocol

SIP 	Session Initiation Protocol

SvP	Service Provider

TCP	Transmission Control Protocol

TSAP	Transport layer Service Access Point

UDP	User Datagram Protocol

URI	Uniform Resource Identifier

URL	Uniform Resource Locator

# 4	General

## 4.1	Reference Model for Lawful Interception

This document adopts the generic reference model for the interception domain from TR 102 528 [8], its internal intercept functions, IRI IIF, CCTF, and CC IIF, and the internal interfaces INI1, INI2, INI3, CCTI and CCCI as shown in figure 1.

HI1

HI2

HI3

INI2

INI3

HI

INI1a

CCCI

LEA DOMAIN

NWO/AP/SP DOMAIN

CCTI

CC Internal Intercept

Function

(CC IIF)

LEA

Administration

Function

Law

Enforcement

Monitoring

Facility

(LEMF)

INI1b

INI1c

Administration Function (AF)

CC Trigger Function

(CCTF)

IRI Internal Intercept

Function

(IRI IIF)

Mediation

Function

(MF)

Figure 1: Reference Model for Lawful Interception

The reference model depicts the following functions and interfaces:

- INI1a provisions Intercept Related Information Internal Interception Function (IRI IIF).

- INI1b may (statically) provision Content of Communications Trigger Function (CCTF).

- INI1c provisions the Mediation Function (MF).

- Intercept Related Information Internal Intercept Function (IRI IIF) generates IRI.

- Content of Communication Internal Intercept Function (CC IIF) generates CC.

- Content of Communication Trigger Function (CCTF) controls CC IIF.

- Content of Communication Control Interface (CCCI) provisions CC IIF.

- Content of Communication Trigger Interface (CCTI) may trigger CCTF for provisioning of the CC IIF.

- Content of Communication Control Interface (CCCI) may dynamically provision the CC IIF.

- Internal interface INI1 carries provisioning information from LIAF to the Internal Intercept Functions (IIF).

- Internal interface INI2 carries Intercept Related Information (IRI) from IRI IIF to the MF.

- Internal interface INI3 carries Content of Communication (CC) information from CC IIF to the MF.

For an in-depth explanation of the functions and interface, is referred to clause 4 of TR 102 528 [8].

## 4.2	Reference system model

The reference system model applied in this document, as depicted in figure 2, provides a simplified model of a technology independent, IP MM service platform, access by multiple different access networks. The access networks may provide different forms of network access, using different technologies; they all have in common that they provide IP connectivity among end-users and between end-users and the IP MM services provided by the IP MM service platform.

Figure 2: Reference System Model

Access from the access networks into the IP Core network of the IP MM service provider is assumed to be protected by some gateway device (e.g. a session border controller, border gateway controller or a firewall/router combination).

The IP MM Service platform contains a signalling server that provides session initiation functionality (e.g. a SIP call manager or an H.323 gatekeeper) among end-users and between end-users and communication services (e.g. unified messaging, audio or video conference servers).

## Network layer Interception

Network Layer interception requires a copy of all signalling information as well as call content exchanged in the platform to be available at a central point in the infrastructure.

Figure 3: Network layer Interception Model

This can be achieved by means of span-ports in the layer 2 switching backbone or by means of passive splitters (either copper or fibre) at strategic points in the SvP’s core network. Either way, due to the bandwidth of copied network traffic, some form of filtering will most likely be required (e.g. by means of a layer 3 switch).

## 4.2.2	Service layer Interception

Service Layer interception requires LI interfaces and functionality to be available in both the signalling server and the gateways.

Figure 4: Service layer Interception Model

Typically the IRI IIF in the signalling server is provisioned with the target ID. At detection of a session setup for a target, the IRI IIF will provide the IRI for the intercepted session and may provide session information to be used for ad-hoc provisioning of the gateway devices. In some implementations, the gateway devices are also provisioned with the target ID beforehand and are capable of detecting sessions independent from the signalling server.

## 4.3	General Requirements

The following requirements regarding the interception of signalling shall apply:

- Annex B provides the functional description of the minimal set of information that is to be provided to Law Enforcement for each intercepted communication.

- This standard supports the interception of communication services defined in the following IETF/ITU-T standards and recommendations:

Editor’s Note: Contributions are sought to identify this list. (FFS)

- Any deviation from the supported IETF/ITU-T specifications identified in item 2, e.g. vendor specific parameters, shall be agreed in advance between the CSP and LEA.

- This standard specifies the hand over of intercepted signalling containing all information required in TS 101 331 [10] by encapsulating that intercepted signalling.

- IRI that is not part of intercepted signalling shall also be delivered.  The format of such information on the handover interface shall be agreed in advance between the CSP and LEA.

- As a national option, mapping of the IRI information onto specific messages at the handover interface may be mandated, e.g., according to the ANS T1.678 [11] specification.

# 5	Interception of IP Multimedia services

## 5.1	Identification of target of interception

### 5.1.1	SIP Target Identification

The target identity is not a network layer or transport layer address. The target identity shall be an address type that uniquely identifies the target in the CSP’s network and by means of which sessions among users can be established, such as:

- TEL URI;

- SIP URI;

- E.164 Number.

### 5.1.2	H.323 Target Identification

The target identity is not a network layer or transport layer address. The target identity shall be an address type that uniquely identifies the target in the CSP’s network and by means of which sessions among users can be established, such as:

- H.323 URL;

- H.323 ID;

- E.164 Number.

## 5.2	Interception of signalling

### 5.2.1	Provisioning of the SIP IRI IIF

SIP messaging RFC 3261 [4] is reported as Intercept Related Information (IRI) for the interception of multi-media service. All SIP messages executed on behalf of a target subscriber are subject to interception at the IRI Internal Interception Function (IIF). Based upon network configuration, the Administration Function (AF) shall provision IRI IIF with SIP Uniform Resource Identifier (URI) or TEL Uniform Resource Locator (URL) target identifiers. These resulting intercepted SIP messages shall be sent to the Mediation Function (MF) over the INI2 interface for mediation prior to transmittal across the HI2 interface.

### 5.2.2	Provisioning of the H.323 IRI IIF

H.323 call signalling, call control and subscriber controlled input messages are reported as Intercepted Related Information (IRI) for the interception of multi-media services. H.323 call signalling and control messages refer to the basic call signalling (H.225.0), call control (H.245) and those messages required for the signalling of supplementary services (i.e.: H.450.x). Subscriber controlled input messages refer to those messages generated as a result of user procedures for the control of Supplementary Services (activation/deactivation/interrogation).

All H.323 call signalling, call control and subscriber controlled input messages that are transmitted on behalf of the target subscriber are subject to intercept at the IRI IIF. Based upon the network configuration, the AF shall provision IRI IIF with either a H.323 Unique Resource Locator (H.323-URL), or a H.323 Identity (H.323-ID), or a public E.164 telephone number.

If available events related to the Registration, Administration and Status (i.e., H.323 RAS) of the target subscriber’s terminal equipment are also subject to intercept at the IRI IIF.

## 5.3	Assigning a value to the Communication Identity Number

In order to produce useful IRI records from events, the IRI records of a communication session must be correlated with a single value for the Communication Identity Number (CIN) field.

### 5.3.1	Assigning a CIN value to SIP related IRI

All IRI events resulting from SIP messages in a single call signalling leg (i.e. with an identical value for the SIP field “Call-ID”) will be assigned the same value for the CIN. A call may consist of two or more call signalling legs (e.g. when communicating via a SIP proxy). The various related call signalling legs are matched using the O field in the SDP part of the SIP messages; call legs with an identical O field will be assigned the CIN value of the active session from which they are spawned.

This is depicted in figure 5, where all resulting IRI-Event are assigned the same CIN:

UA A

SIP Proxy

UA B

Msg 1 – Call-ID = X

Msg 2 – Call-ID = X – SDP O = ABC

Msg 3 – Call-ID = Y – SDP O = ABC

Msg 4 – Call-ID = Y

Msg 5 – Call-ID = Y – SDP O = XYZ

Msg 6 – Call-ID = Y

Msg 7 – Call-ID = X – SDP O = XYZ

Msg 8 – Call-ID = X

Figure 5: Relation between call signalling legs

### 5.3.2	Assigning a CIN value to H.323 related IRI

All IRI events resulting from messages within a single H.323 call will be assigned the same value for the CIN. Messages within a H.323 call can be identified as those messages containing, or associated with the same unique H.225.0 Call ID. The term H.323 Call is defined in ITU-T Recommendation H.323 [6]. The term Call ID also referred to as H.225.0 Call ID and is described in ITU-T Recommendations H.323 [6] and H.225.0 [12].

## 5.4	Events and IRI record types

The following requirements need to be met:

-	All multimedia signalling and control messages to or from a targeted subscriber, and all multimedia signalling and control messages executed on behalf of or related to a targeted subscriber are intercepted by the IRI IIF and sent to the MF over the INI2 interface as IRI records.

-	IRI records for all events related to a single communication session, which may consist of multiple media streams, that are being exchanged among the possibly more than two users, will be assigned the same CIN value.

- 	In addition, information on non-transmission related actions of a target constitute IRI is assigned CIN and is sent via HI2 (e.g. information on subscriber controlled input).

NOTE:	This includes events related to the target service handled internally by network functions (e.g., communications among GWs, Signalling Servers and Communication Services). The format of such information on the handover interface should be agreed in advance between the CSP and LEA.

For each event, a record is sent to the LEMF. A new value for the CIN field is assigned if an event is detected that is not related to an already existing communication session. This event is reported as an IRI-BEGIN record. Every subsequent record related to this communication session is reported as a IRI-CONTINUE record. At the detection of the event that denotes the end of a session an IRI-END record is sent.

In case of implementation issues, operational flaws or problems, events may need to be reported for a communication session for which the IRI-END record already has been sent. These events are reported as an IRI-REPORT with the proper value for the CIN.

The following table summarises the mapping between event type and record type sent to the LEMF.

Table 1: Mapping between IP MM Events and HI2 Records Type

| Event | IRI Record Type |
| --- | --- |
| At assignment of a new CIN value | BEGIN |
| All intermediate signalling, other than the last event | CONTINUE |
| The last event related to a communication session | END |
| Delayed IRI events related to an already ended session | REPORT |

Editor's note: The handling of single message exchange may require an IRI-REPORT (FFS).

Editor's note: The use of just IRI-REPORT to handover all IRI, was discussed, but no consensus was reached. This proposal will be discussed further off-line (FFS).

## 5.5	Interception of Content of Communication

The Communication Content (CC) provided is a copy of the multimedia stream, i.e. every RTP and RTCP packet [5] sent through the SvP’s network that is addressed to, sent from or related to the targeted subscriber as described in the intercepted call signalling and call control information. In addition to IRI messages, all RTP and RTCP packets identified to be associated with a specific intercepted call or session shall be assigned the same CIN.

The CC payload contains a copy of the multimedia stream.

NOTE 1:	The ASN.1 definition for CC is presented as the iPMultimediaPDU in clause 7.

NOTE 2:	The CC also contains the target media streams that are not transferred via RTP or RTCP (for example, those streams handled by a GW and not passing the core network). The format of such information on the handover interface should be agreed in advance between the CSP and LEA.

# 6	Handover Interface

## 6.1	Intercepted Related Information

The Communication Identity Number (CIN) is used to uniquely identify a communication session (as described in TS 102 232-01 [2] and TS 101 671 [1]). Applied to IP MM Services, a communication session refers to a single self contained transaction or a series of protocol data units that together form a self-contained communication such as a SIP or H.323 Session.

## 6.2	Correlation of IRI and CC

To assure correlation between the independently transmitted Content of Communication and the Intercepted Related In formation (IRI) of an intercepted call the following parameters are used:

- Lawful Interception Identifier (LIID);

- Communication Identifier (CID).

NOTE: 	The target identifier may not necessarily be found in both communication content and interception related information.

# 7	ASN.1 for IRI and CC

## 7.1	ASN.1 syntax tree for HI2 and HI3 headers

Figure 6 shows the object identifier tree from the point of view of packet-switched lawful interception.

Figure 6: Object identifier tree

## 7.2	ASN.1 specification

The ASN.1 (ITU-T Recommendation X.680 [3]) module that represents the information in the present document and meets all stated requirements is shown below:

-- ====================================

-- Description of the IP Multimedia PDU

-- ====================================

IPMultimediaPDU

{itu-t(0) identified-organization(5) etsi(0) securityDomain(2) lawfulIntercept(2) li-ps(5) iPMultimedia(5) version1(1)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

IMPORTS

-- from TS 101 671 [1]

IPAddress

FROM HI2Operations

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulIntercept(2) hi2(1) version10(10)};

-- Object Identifier Definition

iPMMIRIObjId RELATIVE-OID			::= {li-ps(5) iPMultimedia(5) version1(1) iRI(1)}

iPMMCCObjId RELATIVE-OID			::= {li-ps(5) iPMultimedia(5) version1(1) cC(2)}

-- both definitions relative to:

-- {itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulIntercept(2)}

-- =====================================

-- IP Multimedia Communications Contents

-- =====================================

IPMMCC		::= SEQUENCE

{

iPMMCCObjId 				[0] RELATIVE-OID,

rTPCCContents				[1] OCTET STRING

-- Copy of the multimedia stream, i.e. all related RTP and RTCP packets,

-- including the original IP and UDP headers

}

-- ========================================================

-- Intercept-related information for IP Multimedia sessions

-- ========================================================

IPMMIRI			::= SEQUENCE

{

iPMMIRIObjId 				[0] RELATIVE-OID,

iPMMIRIContents				[1] IPIRIContents,

...

}

IPIRIContents	::= CHOICE

{

originalIPMMMessage			[0] OCTET STRING,

-- Copy of the IP MM signalling packet including the original IP and UDP/TCP headers

sIPMessage					[1] SIPMessage,

-- Copy of the SIP content and the source and destination IP address

h323Message					[2] H323Message,

-- Copy of the H.323 content and the source and destination IP address

...

}

SIPMessage		::= SEQUENCE

{

iPSourceAddress 			[0] IPAddress,

iPDestinationAddress		[1] IPAddress,

sIPContent					[2] OCTET STRING,

...

}

H323Message		::= SEQUENCE

{

iPSourceAddress 			[0] IPAddress,

iPDestinationAddress		[1] IPAddress,

h323Content					[2] H323MessageContent,

...

}

H323MessageContent		::= CHOICE

{

H225CSMessageContent		[0] OCTET STRING,

H225RASMessageContent		[1] OCTET STRING,

H245MessageContent			[2] OCTET STRING,

GenericMessageContent		[3] OCTET STRING,

...

}

END -- end of IP Multimedia PDU

Annex A (informative):
Interception using H.248

# A.1	Purpose of this annex

There are a number of problems with extracting useful IRI from H.248 signalling. These problems are largely due to the fact that ITU-T Recommendation H.248 [7] is a gateway control protocol, not an end-to-end call initiation protocol like SIP or ITU-T Recommendation H.323 [6]. Often, useful information may be more easily extracted from other signalling links such as SIP, or from on-switch interception solutions. However, in certain circumstances it may be necessary to use the H.248 link. For example, for security or architecture reasons, it may be that the simpler options are not possible or acceptable. This annex provides some notes on how LI could be performed on a H.248 link (section A.2) and describes some of the problems and issues that arise (section A.3). It is not intended to be a complete or comprehensive description and solutions to the problems and issues are not presented.

# A.2	Notes on interception using H.248.

## A.2.1	Target identification (see also clause 5.1)

H.248 messages typically do not contain authoritative target identifiers, such as E.164 phone numbers. Instead, they will typically contain termination and context identifiers. Hence, a probe provisioned with a list of target identifiers, and sniffing an H.248 link, will not be able to discriminate between target and non-target H.248 messages.

A possible solution would be to provide a mapping between authoritative identifiers such as E.164 numbers, and H.248 identifiers such as termination IDs. The MGC would provide the IF or MF with a list of the termination IDs in each MG it controls, and the corresponding authoritative identifier for each termination. This list of correspondences, along with the target list held by the IF or MF, would allow the IF to target H.248 transactions based on termination ID.

Information such as the IP endpoint for the RTP stream will be sent to a different termination. However, it will be in the same context as the target termination. Hence, it will be necessary for the probe to extract the relevant Context ID from any messages it intercepts by Termination ID, and intercept subsequent messages by this Context ID.

Figure A.1: Mapping of Authoritative Identifiers to Termination and Context Identifiers

There is still a problem with this method. There is no guarantee that the first termination to be added to a context will be the target physical termination. For example, the MGC may choose to add an ephemeral IP termination first, followed by the target termination. The IF will only intercept signalling after the target termination is added. Information contained in the first Add message is missed. This message may contain the IP endpoints and codec for the media stream, and so missing it means that the call content cannot be intercepted

It is worth noting that typically, there is no advantage to adding the ephemeral termination first, and so ITU-T Recommendation H.248 [7] implementations tend to add the physical termination at the same time as, or before, the ephemeral one.

The recommendation is therefore that:

- The IF will intercept H.248 signalling using a physical termination ID as an identifier,

- The IF will subsequently intercept any H.248 signalling to the same context as the physical termination,

- The MGC provides a mapping of authoritative identifiers to physical termination IDs.

The problems with this approach are:

- Any signalling to a context sent before a target termination is added to that context may be missed. This may mean important information such as details of the call content media stream are missed.

Other methods of identifying target H.248 transactions (for example, examining some other signalling link) are also possible.

## A.2.2	Provisioning of the H.248 IRI IIF (see also clause 5.2)

For H.248, the IRI IIF is provisioned with a mapping between H.248 target identifiers and physical termination IDs. All H.248 signalling to and from a targeted termination is subject to interception. In addition, to retrieve information about the subsequent media path, any signalling sent to or from the same context as a targeted termination is also subject to interception.

# A.3	Problems in H.248 interception

## A.3.1	Missing information in H.248 signalling

In some circumstances, target H.248 messages may not contain all of the information an end user might expect. The following list shows what information may be missing.

- If a non-target calls a target, the non-target’s number may be unavailable. A CLI signal may be used to discover the caller’s number, but if there is no CLI package in place, or the user withholds their number, the calling number is unavailable.

- A failed attempt to call the target (because, for example, the target was busy) may not generate any target signalling, as the MGC will determine that the call cannot be made.

- If a target is called, the H.248 signalling gives no indication whether the call was made directly, or if the call was, for example, diverted or transferred to the target. If the call was diverted, there is no way of obtaining the original number (unless it is provided in some kind of package)

- Similarly, when a call is torn down, unless the target hung up, there is no indication of why the call ended.

- In some implementations, features such as call-holding or conference calling may be dealt with by a separate dedicated server (not the MGC). For example, a user may flash-hook, and be automatically redirected to a dedicated conferencing server, which would subsequently listen for the DTMF digits of the person the user wants to conference with. In this case, the digits the user dials are carried over the media stream to the conferencing server, rather than over H.248 signalling to the MGC. This number would only be found by analysing the call content afterwards.

## A.3.2	Missing call content

Local call turnaround creates a problem in capturing the call media. If the call is turned around locally at a Media Gateway, then the RTP media stream is never presented to the IP network. Therefore no interception function in the IP network will be able to intercept the call media.

Annex B (normative):
Minimum set of functional attributes to be provided

This annex provides the functional description of the minimal set of information that is to be provided to Law Enforcement for each intercepted communication.

The full law enforcement requirements are in TS 101 331 [10].

The present annex describes the requirements from a Law Enforcement Agency's (LEA's) point of view.

Not all requirements necessarily apply in one individual nation.

These requirements shall be used to derive specific network requirements and furthermore to standardize handover interfaces.

# B.1	General requirements

a)	The obligation of the CSP as to which telecommunications traffic shall be intercepted is subject to national laws.

b)	In accordance with the relevant lawful authorization a CSP shall ensure that:

b.1)	the entire content of communication associated with a target identity being intercepted can be intercepted during the entire period of the lawful authorization;

b.2)	any content of communication associated with a target identity being intercepted which is routed to technical storage facilities or is retrieved from such storage facilities can be intercepted during the entire period of the lawful authorization;

b.3)	the delivery of the intercept related information is reliable. If the intercept related information can not be delivered immediately to the relevant LEMF, then the intercept related information shall be buffered until they can be delivered;

b.4)	the delivery of the content of communication is reliable. If the content of communication can not be delivered immediately to the relevant LEMF, then the content of communication shall be buffered if this is required by national laws;

b.5)	the CSP shall not monitor or permanently record the results of interception.

c)	The ability to intercept telecommunications shall be provided relating to the interception subjects operating permanently within a telecommunications system (e.g. a subscriber or account).

d)	The ability to intercept telecommunications shall be provided relating to the interception subjects operating temporarily within a telecommunications system (e.g. a visiting mobile subscriber or a visiting subscriber using an access network to a home service).

e)	The results of interception relating to a target service shall be provided by the CSP in such a way that any telecommunications that do not fall within the scope of the lawful authorization shall be excluded by the CSP.

NOTE 1:	It is assumed that the intercepting system exercises best effort to exclude non-authorized interception patterns (e.g. transferred communication).

- All results of interception provided at the handover interface shall be given a unique identification relating to lawful authorization.

NOTE 2: 	Information used for the IRI is expected to be part of standard network signalling procedures. No additional signalling is expected for the IRI.

# B.2	Result of interception

The CSP shall, in relation to each target service:

a)	provide the content of communication;

b)	remove any service coding or encryption which has been applied to the content of communication (i.e. en clair) and the intercept related information at the instigation of the CSP;

c)	provide the LEA with any other decryption keys whose uses include encryption of the content of communication, where such keys are available for CSP;

d)	intercept related information shall be provided:

d.1)	when communication is attempted;

d.2)	when communication is established;

d.3)	when no successful communication is established;

d.4)	on change of status (e.g. in the access network);

d.5)	on change of service or service parameter;

d.6)	on change of location (this can be related or unrelated to the communication or at all times when the apparatus is switched on);

e)	intercept related information shall contain:

e.1)	the identities that have attempted telecommunications with the target identity, successful or not;

e.2)	identities used by or associated with the target identity;

e.3)	details of services used and their associated parameters;

e.4)	information relating to status;

e.5)	time stamps;

f)	the conditions mentioned above also apply to multi-party or multi-way telecommunication if and as long as the target identity participates.

# B.3	Location information

An LEA may request location information relating to locations, in a number of forms: the current geographic, physical or logical location of the target identity. This information is expected to be made available from normal network operation.

# B.4	Time constraints

a)	A CSP shall make the necessary arrangements to fulfil his obligation to enable the interception and delivery of the result of interception from the point in time when the telecommunication installation commences commercial service.

b)	The above requirement applies accordingly to the introduction of modifications to the telecommunication installation or to new operational features for existing telecommunications services to the extent of their impact on existing interception capabilities.

c)	When a lawful authorization is presented a CSP provider shall co-operate immediately.

d)	After a lawful authorization has been issued, provision of the results of interception of a target identity shall proceed on a real-time basis.

# B.5	Technical handover interfaces and format requirements

d)	The configuration of the handover interface shall ensure that the quality of service of the telecommunications traffic provided at the handover interface is not inferior to that offered to the target service for each particular call.

f)	Each interception target shall be uniquely associated with a single instance of the handover interface. This could be achieved by separate channels or the use of identifiers.

g)	The correlation between the content of communication and intercept related information shall be unique.

Annex C (informative):
Change Request history

| Status of the present document Service-specific details for IP Multimedia Services |  |  |
| --- | --- | --- |
| Date | Version | Remarks |
|  |  | First publication of the TS after approval by ETSI/TC LI#xx. Version 2.1.1 prepared by Johan Bakker (KPN) (rapporteur v2.1.1) |

| Document history |  |  |
| --- | --- | --- |
| V0.0.0 | June 2005 | Document layout |
| V0.0.1 | October 2005 | First draft |
| V0.0.2 | November 2005 | Incorporated proposals from TC LI 10 in Sorrento |
| V0.1.0 | December 2005 | Incorporated the work from TC LI RAP 11 in Amsterdam |
| V0.1.1 | April 2006 | Incorporated the work from TC LI 11 in St. Maarten |
| V0.2.0 | April 2006 | Incorporated the work from TC LI RAP 12 in London |
| V0.2.1 | April 2006 | Clean version v.0.2.0 with some additional cleaning up |
| V0.3.1 | May 2006 | Incorporated the work from TC LI 12 in Limasol |
| V0.4.0 | September 2006 | Clean version v.0.4.0 with some additional cleaning up |
| V0.4.1 | September 2006 | Incorporated the work from TC LI 13 in Stockholm |
| v0.5.0 | September 2006 | clean version as DTS 102 232-05 to SA3-LI |

# History

| Document history |  |  |
| --- | --- | --- |
| V2.1.1 |  |  |