3GPP TSG-SA3-LI Meeting #24 	Tdoc SA3LI07_023

New Orleans (USA), 6-8 February 2007

# Title: 	Concerning LS from SA2 on the Direct Tunnel

## Source: 				Nokia, Siemens

## Document for:	Information

# Introduction

An LS from SA2 in tdoc SA3LI07_003 outlines the new optional feature of establishing a direct user plane tunnel between GGSN and RNC. The LS however describes respective LI related implications, which clearly is out of SA2 scope. Moreover, these proposals are not based on the respective CR0570r6 to 23.060 and therefore are misplaced.

The CR actually reads:

Direct Tunnel shall not be used in following traffic cases:

1)	In roaming case

-	The SGSN needs to know whether the GGSN is in the same or different PLMN.

2)	SGSN has received CAMEL Subscription Information in the subscriber profile from the HLR.

-	If Direct Tunnel is established then volume reporting from SGSN is not possible as the SGSN no longer has visibility of the User Plane. Since a CAMEL server can invoke volume reporting at anytime during the life time of a PDP Context, the use of Direct Tunnel shall be prohibited for a subscriber whose profile contains CAMEL Subscription Information.

3)	GGSN does not support GTP protocol version 1.

Therefore, there is nothing about LI implication in the CR.

# 2. Proposal

When discussion the LS, it is proposed to ignore the proposals that are not based on the CR0570r6-23.060.