| CR-Form-v9.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.107 | CR | CRNum |  rev | - |  Current version: | 7.4.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | NSAPI (Network layer Service Access Point Identifier) optional in IRI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Nokia Siemens Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | SA 3 LI |  |  |  |  |  |  |  |  |  |
| Work item code:  |  |  |  |  |  |  |  | Date:  |  | 16/04/2007 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | Allow to send additional inforamtion which could be quite helpful for the combined use of the GGSN for PDG according 23.234 Annex F Figure F.1: PDG implementation re-using GGSN functionality |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Additional effort for the LI implemention in the GGSN |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Missing information |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 7.3.2, 7.4.3, 7.4.4, 7.4.5, 9.3.2, 9.4.3, 9.4.4, 9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  | x |  | Other core specifications  |  |  | 33.108 |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

***First Change***

7.3.2	Structure of the events

There are several different events in which the information is sent to the DF2 if this is required. Details are described in the following clause. The events for interception are configurable (if they are sent to DF2) in the 3G GSN or the HLR and can be suppressed in the DF2.

The following events are applicable to 3G SGSN:

-	Mobile Station Attach;

-	Mobile Station Detach;

-	PDP context activation;

-	Start of interception with mobile station attached (national option);

-	Start of intercept with PDP context active;

-	PDP context modification;

-	PDP context deactivation;

-	RA update;

-	SMS.

NOTE:	3G GGSN interception is a national option. Location information may not be available in this case.

The following events are applicable to the 3G GGSN:

-	PDP context activation;

-	PDP context modification;

-	PDP context deactivation;

-	Start of interception with PDP context active.

The following events are applicable to the HLR:

-	Serving System.

A set of elements as shown below can be associated with the events. The events trigger the transmission of the information from 3G GSN or HLR to DF2. Available IEs from this set of elements as shown below can be extended in the 3G GSN or HLR, if this is necessary as a national option. DF2 can extend available information if this is necessary as a national option e.g. a unique number for each surveillance warrant.

Table 2: Information Events for Packet Data Event Records

Observed MSISDN MSISDN of the target subscriber (monitored subscriber).

Observed IMSI IMSI of the target subscriber (monitored subscriber).

Observed IMEI IMEI of the target subscriber (monitored subscriber),it shall be checked for each activation over the radio interface.

Event type Description which type of event is delivered: MS attach, MS detach, PDP context activation, Start of intercept with PDP context active, PDP context deactivation, SMS, Serving System, Cell and/or RA update.

Event date Date of the event generation in the 3G GSN or the HLR.

Event time Time of the event generation in the 3G GSN or the HLR. Timestamp shall be generated relative to GSN or HLR internal clock.

PDP address The PDP address of the target subscriber. Note that this address might be dynamic.

Access Point Name The APN of the access point. (Typically the GGSN of the other party).

Location Information Location Information is the Service Area Identity (SAI), RAI and/or location area identity that is present at the GSN at the time of event record production.

Old Location Information Location Information of the subscriber before Routing Area Update

PDP Type The used PDP type.

Correlation Number The correlation number is used to correlate CC and IRI.

SMS The SMS content with header which is sent with the SMS-service. The header also includes the SMS-Centre address.

Network Element Identifier Unique identifier for the element reporting the ICE.

Failed attach reason Reason for failed attach of the target subscriber.

Failed context activation reason Reason for failed context activation of the target subscriber.

IAs The observed Interception Areas.

Initiator The initiator of the PDP context activation, deactivation or modification request either the network or the 3G MS.

SMS Initiator SMS indicator whether the SMS is MO or MT.

Deactivation / termination cause The termination cause of the PDP context.

QoS This field indicates the Quality of Service associated with the PDP Context procedure.

Serving System Address Information about the serving system (e.g. serving SGSN number or serving SGSN address).

NSAPI Network layer Service Access Point Identifier The NSAPI information element contains an NSAPI identifying a PDP Context in a mobility management context specified by the Tunnel Endpoint Identifier Control Plane.

***End first change***

***Second change***

### 7.4.3	Packet Data PDP context activation

When a PDP context activation is generated a PDP context activation-event is generated. These elements will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

PDP address of observed party

Event Type

Event Time

Event Date

Correlation number

Access Point Name

PDP Type

Network Element Identifier

Location Information

Failed context activation reason

IAs (if applicable)

Initiator (optional)

QoS (optional)

NSAPI (optional)

### Start of interception with PDP context active

This event will be generated if interception for a target is started and if the target has at least one PDP context active. If more then one PDP context are open, for each of them an event record is generated. These elements will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

PDP address of observed party

Event Type

Event Time

Event Date

Correlation number

Access Point Name

PDP Type

Network Element Identifier

Location Information

Old Location Information (optional)

IAs (if applicable)

QoS (optional)

Initiator (optional)

NSAPI (optional)

Presence of the optional Old Location Information field indicates that PDP context was already active, and being intercepted. However, the absence of this information does not imply that interception has not started in the old location SGSN for an active PDP context.

Start of interception with PDP context active shall be sent regardless of whether a Start of interception with mobile station attached has already been sent.

### Packet Data PDP context deactivation

At PDP context deactivation a PDP context deactivation-event is generated. These elements will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

PDP address of observed party

Event Type

Event Time

Event Date

Correlation number

Access point name

Network Element Identifier

Location Information

IAs (if applicable)

Deactivation cause

Initiator (optional)

NSAPI (optional)

***End second change***

***Begin third change***

### 9.3.2	3GPP WLAN Interworking LI Events and Event Information

The following events are applicable to AAA Server:

-	I-WLAN Access Initiation;

-	I-WLAN Access Termination;

-	I-WLAN Tunnel Establishment;

-	I-WLAN Tunnel Disconnect;

-	Start of Intercept with I-WLAN Communication Active;

The following events are applicable to the PDG:

-	I-WLAN Tunnel Establishment;

-	I-WLAN Tunnel Disconnect;

-	Start of Intercept with I-WLAN Communication Active.

A set of possible elements as shown below is used to generate the events. Information associated with the events are transmitted from the PDG or AAA server to DF2.

Note – some of these parameters apply to the PDG and some apply to the AAA server.  Parameters sent from the PDG or AAA server is dependent on what is available at the network element.

Table 3: Information Events for WLAN Interworking Event Records

| Element | PDG | AAA Server |
| --- | --- | --- |
| Observed MSISDN MSISDN of the target subscriber (monitored subscriber). | Available, see 3GPP TS 29.234 | Available, see 3GPP TS 29.234 |
| Observed NAI NAI of the target subscriber (monitored subscriber). | Not available | Available, see 3GPP TS 29.234 |
| Observed IMSI IMSI of the target subscriber (monitored subscriber). | Available, see 3GPP TS 24.234 | Available, see 3GPP TS 29.234 |
| Event type Description which type of event is delivered: I-WLAN Access Initiation, I-WLAN Access Termination, I-WLAN Tunnel Establishment, I-WLAN Tunnel Disconnect, Start of Intercept with I-WLAN Communication Active. | Available from ICE | Available from ICE |
| Event date Date of the event generation in the PDG or the AAA server. | Available from ICE | Available from ICE |
| Event time Time of the event generation in the PDG or the AAA server. Timestamp shall be generated relative to the PDG or AAA server internal clock. | Available from ICE | Available from ICE |
| WLAN UE Local IP address The WLAN UE Local IP addess of observed party. The WLAN UE Local IP address field specified in 3GPP TS 24.234 and IETF RFC 2409, represents the IPv4/IPv6 address of the WLAN UE in the WLAN AN. It is an address used to deliver the packet to a WLAN UE in a WLAN AN. Note that this address might be dynamic. | Available, see 3GPP TS 24.234 and IETF RFC 2409 | Not available |
| WLAN UE MAC address The WLAN MAC address of the target subscriber. Note that this address might be dynamic and the validity of the MAC Address is outside of the scope of 3GPP. | Not available | Available, see 3GPP TS 29.234 |
| WLAN UE Remote IP address The WLAN UE Remote IP addess of observed party. The WLAN UE Remote IP address field specified in 3GPP TS 24.234, represents the IPv4/IPv6 address of the WLAN UE in the network being accessed by the WLAN AN. It is an address used in the data packet encapsulated by the WLAN UE-initited tunnel and is the source address used by applications in the WLAN UE. Note that this address might be dynamic. | Available, see 3GPP TS 24.234 | Not available |
| WLAN Access Point Name The W-APN of the access point. | Available, see 3GPP TS 24.234 | Available, see 3GPP TS 29.234 |
| WLAN Operator Name The name of the WLAN operator name serving the target subscriber. | Not available | Available, see 3GPP TS 29.234 |
| WLAN Location Name The name of the location of the WLAN serving the target subscriber (e.g., string like "coffee shop" or "airport", etc.). | Not available | Available, see 3GPP TS 29.234 |
| WLAN Location Information Location Information regarding the WLAN as provided in RADIUS or DIAMETER signalling exchanged with the AAA server. | Not available | Available, see 3GPP TS 29.234 |
| Correlation Number The correlation number is used to correlate CC and IRI. The correlation number is also used to allow the correlation of IRI records. In case of the AAA server, the Correlation Number is only used to correlate IRI records. | Generated for LI by PDG | Generated for LI by AAA server |
| Network Element Identifier Unique identifier for the element reporting the ICE. | Generated for LI by PDG | Generated for LI by AAA server |
| Initiator The initiator of the request either the network or the WLAN UE. | Generated for LI by PDG | Generated for LI by AAA server |
| NAS IP/IPv6 address The IP or IPv6 address of the NAS in the WLAN. | Not available | Available, see 3GPP TS 29.234 |
| Visited PLMN ID Identity of the visited PLMN to which the user is terminating their WLAN tunnels or through which the user is establishing their WLAN tunnels. | Not available | Available, see 3GPP TS 29.234 |
| Session Alive Time The amount of time in seconds during which the target subscriber can be registered for WLAN access. | Not available | Available, see 3GPP TS 29.234 |
| Failed access reason Provides the reason for why a WLAN access attempt failed ("Authentication Failed"). | Not available | Available from ICE |
| Session termination reason Provides a reason for why a WLAN access session is terminated. | Not available | Available, see 3GPP TS 29.234 |
| Failed tunnel establishment reason Provides a reason for why a WLAN tunnel establishment failed ("Authentication failed" or "Authorization failed"). | Available from ICE | Available from ICE |
| NSAPI Network layer Service Access Point Identifier The NSAPI information element contains an NSAPI identifying a PDP Context in a mobility management context specified by the Tunnel Endpoint Identifier Control Plane. | Not available | Optional available according 23.234 Annex F; defined 29.060 7.7.17 |

***End third change***

***Begin fourth change***

### 9.4.3	I-WLAN Tunnel Establishment

For I-WLAN Tunnel Establishment, a I-WLAN tunnel establishment-event is generated. The elements, shown in Table 6 and Table 7, will be delivered to the DF2 if available, by the PDG or AAA server, respectively.

Table 6: I-WLAN Tunnel Establishment - PDG

Observed MSISDN

Observed IMSI

Observed NAI

Event Type

Event Time

Event Date

Correlation number

WLAN UE Local IP address

WLAN UE Remote IP address

WLAN Access Point Name

Network Element Identifier

Failed tunnel establishment reason

NSAPI (optional)

Table 7: I-WLAN Tunnel Establisment - AAA Server

Observed MSISDN

Observed IMSI

Observed NAI

Event Type

Event Time

Event Date

Correlation number

WLAN Access Point Name

Network Element Identifier

Visited PLMN ID

Failed tunnel establishment reason

### 9.4.4	I-WLAN Tunnel Disconnect

At I-WLAN Tunnel Disconnect, a I-WLAN tunnel disconnect event is generated. The elements, shown in Table 8 and Table 9, will be delivered to the DF2, if available, by the PDG or AAA server, respectively.

Table 8: I-WLAN Tunnel Disconnect - PDG

Observed MSISDN

Observed IMSI

Observed NAI

Event Type

Event Time

Event Date

Correlation number

WLAN UE Local IP Address

WLAN UE Remote IP address

WLAN Access Point Name

Network Element Identifier

Initiator (optional)

NSAPI (optional)

Table 9: I-WLAN Tunnel Disconnect - AAA Server

Observed MSISDN

Observed IMSI

Observed NAI

Event Type

Event Time

Event Date

Correlation number

Tunnel address of observed party

WLAN Access Point Name

Network Element Identifier

Initiator (optional)

### 9.4.5	Start of Intercept with I-WLAN Communication Active

This event will be generated if interception for a target is started and if the target has one or more active I-WLAN Access sessions or one or more I-WLAN Tunnels established. The elements, shown in Table 10 and Table 11, will be delivered to the DF2, if available, by the PDG or AAA server, respectively.

Table 10: Start of Intercept with I-WLAN Communication Active - PDG

Observed MSISDN

Observed IMSI

Observed NAI

Event Type

Event Time

Event Date

Correlation Number

WLAN UE Local IP Address

WLAN UE Remote IP address

WLAN Access Point Name

Network Element Identifier

NSAPI (optional)

Table 11: Start of Intercept with I-WLAN Communication Active - AAA Server

Observed MSISDN

Observed IMSI

Observed NAI

Event Type

Event Time

Event Date

Correlation Number

WLAN Access Point Name

Network Element Identifier

WLAN Operator Name

WLAN Location Name

WLAN Location Information

NAS IP/IPv6 address

Visited PLMN ID

***End fourth change***