| CR-Form-v9.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.108 | CR | CRNum |  rev | - |  Current version: | 8.0.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | WLAN IRI at AAA for re-authentication mandatory |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Nokia Siemens Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | SA 3 LI |  |  |  |  |  |  |  |  |  |
| Work item code:  |  |  |  |  |  |  |  | Date:  |  | 27/06/2007 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | Allow to send additional information and not only the first time |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Add a new trigger point for re-authentication at the AAA server |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | This info couldn’t be provided |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 8.5.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  | X |  | Other core specifications  |  |  | 33.107 |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

***Change***

#### 8.5.1.2	REPORT record information

The REPORT record is used to report non-communication related subscriber actions (events) and for reporting unsuccessful packet-mode communication attempts.

The REPORT record shall be triggered when:

-	the intercept subject's WLAN UE performs a (successful or unsuccessful) I-WLAN access initiation procedure (triggered by AAA server);

-	the intercept subject’s WLAN UE performs a (successful or unsuccessful) re-authentication at the AAA server

-	the intercept subject's WLAN UE performs a I-WLAN access termination detach procedure (triggered by AAA server);

-	the intercept subject's WLAN UE is unsuccessful at performing a I-WLAN tunnel establishment procedure (triggered by AAA server or PDG);

-	the interception of a subject’s communications is started and the WLAN UE has already successfully performed a I-WLAN access initiation procedure (triggered by AAA server), but there are no tunnels established.

***End***