3GPP TSG-SA3 LI Meeting#26				SA3LI07_085

Key West, USA, 31 Jul – 2 Aug 2007

Title:	Conference Call, Related Change Information

Source:	PIDS

Document for:	Information

Conference Call

In the Riga meeting the ongoing discussions around the requirements of the Conference Call has been forwarded. On one hand the Conference Call can be the LI target. On the other hand a Conference Call can be the tele service in which the target participates or has been initiated by the target. Although not all these situations are supported in all jurisdictions it is definitely more than a requirement in an individual country.

This informational document discusses the requirements on the Related Changes initiated by a user in a Conference Call and the 4G/NGN angle of these type of services.

The requirement to report also Related Changes as part of the LI is gives in TS 102 331 sections 4.3.d 4, 5 & 6. Assumption is that the Conference Call is part of the services offered by the targeted CSP.

The Related Change refer to changes made during the conference or outside the conference.

A major part of these Related Changes can concern features not standardized in 3G.

Examples of IRI associated with related changes are:

- Create/change time window for the conference call

- Create/change participant list that can subscribe to the conference call

- Identification of initiator

- Change of ownership

- Create conference ID

- Create conference access codes

- Create/change list of potential conferees

- Type of conference

In relation to many supplementary services a CSP can choose to use a fully 3G implementation, a partial 3G implementation or even a not 3G implementation. How to deal with the standardized and not 3G standardized features in relation to the delivery of LI?

For the fully 3G implementations the task for SA3LI is clear. For partial or not 3G implementations the obligation to fulfill LI is still on the CSP. Examples could be web-access, service desk access, separate conference platform. Choices can be made to have a combined delivery or to have separate delivery for the information strictly outside the 3G scope. In the later case correlation has to be provided between the two means of delivery for the same target.

SA3LI is asked to incorporate related changes for conference calls.

SA3LI is asked to discus the handling of CSP services not strictly part of 3G. The creation of “open” field in the 3G delivery could be considered and practical correlation could be discussed.

Associated documents: Td050, Td051, Td055, TS 24.147.