3GPP TSG-SA WG3-LI Meeting #37 	DocNumber SA3LI10_039

- New York, USA, 4th-6th May 2010

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.106 | CR | CRNum |  rev | - |  Current version: | 9.0.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Encryption Requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | BT Group, OTHERS TBC |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | SA3-LI |  |  |  |  |  |  |  |  |  |
| Work item code:  | LI-10 |  |  |  |  |  |  | Date:  |  | 26/04/2010 |
| Category:  | C |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change:  |  | The current version of 33.106 does not fully describe LI requirments concerned with the use of network applied encryption, especially in relation service layer encryption capabilities. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | The CR adds a new section 5.X with detailed requirements for LI relating to the use of operator applied encryption. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | 33.106 will continue to not clearly define LI requirements relating to network applied encryption. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 5.X (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

*** FIRST MODIFICATION ***

5.X LI Requirements for Encrypted Services

Section 5.1.2 provides a general description of requirements relating to network applied encryption. In addition to the general requirments, the following additonal LI requirements shall apply to network provided and/or network administered end to end or end to middle encryption, where this encryption prevents direct plain text capture of all required IRI and CC.

- When an encrypted service is provided by the PLMN, lawful interception shall take place as for normal PLMN use. In addition encrypted content shall be decrypted, or the decryption keys provided to the LEMF.

- Where a key server based solution is used, it is a national option for the key server to be directly available to the LEMF to provide keys and any required associated information (e.g.  roll over counters) for the decryption of communications.

- A surveillance mechanism shall be performed in such a manner as to avoid detectability by the Surveillance Subject or others to the maximum extent possible. In particular:

a.	There should be no significant difference in latency during call setup or during communications.

NOTE: Additional latency due to re-routing is not restricted by this requirement.

b.	Surveillance of a Subject shall not prevent the use of key exchange applications which provide a user key confirmation mechanism.

NOTE: Key confirmation mechanisms such as an authentication string to be exchanged verbally are commonly used to provide additional assurance of authentication.

c.	Should interception fail during a call (or during call setup), the call shall be unaffected.

- The PLMN operator shall not be required to perform additional processing within the communication channel of the Surveillance Subject over that required for interception in normal PLMN use.

- An encryption solution shall not prohibit:

a.	Retrospective decryption of retained encrypted communications content and associated key material.

b.	Commencement of Surveillance during an existing call.

NOTE: These requirements do not explicitly place a requirement on the operator to retain key material after the end of a communication (except if required by national law) but do require that the encryption mechanism shall not prevent retrospective decryption by design if key material is still available.

As for requirements in section 5.1.2, the requirements in this section are not considered to apply to any encryption which a Surveillance Subject  may use which is outside the control of the network operator (eg 3rd party end to end VOIP software).

In addition the requirements do not apply where encryption is provided by the network between any network nodes or UEs where this encryption does not affect the ability of the core network to perform interception (eg hop by hop IMS signalling security or End to Access Edge radio bearer encryption).

*** END OF MODIFICATIONS ***