SA WG3LI Meeting #62	S3i160280

18-21 July 2016, Sophia Antipolis, France	Revised from

Source:	Ericsson

Title:	P-CR proposing new Key Issue to LIV Study – Start of interception with active VoLTE call

Document for:	Approval

Agenda Item:	10-Continued Discussion and New Business

Work Item / Release:	Rel-14

Abstract of the contribution: The contribution proposes a new Key Issue #X, Start of interception with active VoLTE call.

**************************************************start of first change*******************************

## 5.X	Key issue #X – Start of interception with ongoing VoLTE call

### 5.X.1	Description

There is a requirement for the Operator to allow start of interception for an already established VoLTE call in case the warrant is activated while the target has an ongoing VoLTE call.

### 5.X.2	Architectural requirements

IMS information shall be stored in the LMISF for possible later use in case a user becomes a target while he has an ongoing session.

### 5.X.3	Stage 3 requirements

No stage 3 requirements

### 5.X.4	Solution approaches

The LMISF shall store all the IMS information retrieved at session setup, as the P-CSCF is required to do in case of LBO, hence the LMISF needs to get the GTP-U related to the VOLTE bearer for all subscribers irrespectively whether they are intercepted or not. In case LI is activated for a target over X1 interface, the LMISF shall check if there is any already established VoLTE call for the target; in such case, LI shall start for the ongoing VoLTE call.

************************************end of last change*****************************************