SA WG3LI Meeting #62	S3i160303

18th-21st July 2016, ETSI, Sophia Antipolis, France

Source:	SPRINT, NTT Docomo

Title:	P-CR on proposed clean-up of key issue #6

Document for:	Approval

Agenda Item:	10.1 S8HR, VoLTE & Roaming

Work Item / Release:	FS_LIV8/Rel- 14

***************************Start of first change***************************************************

## 5.6	Key issue #6 – Detecting targeted signalling unknown or unrecognized by the VPLMN

### 5.6.1	Description

There is the possibility of some SIP/IMS signalling and signalling elements transiting the VPLMN between the UE and HPLMN which can be any of the following:

- Proprietary extensions to IMS/SIP methods

- Proprietary information elements in standardized IMS/SIP methods

- Standardized IMS/SIP methods or headers supported by the HPLMN but not yet supported or recognized by the VPLMN

- Unrecognized or unknown SIP methods, syntax, semantics or methods usage.

The applicable standards may define the conditions are errors and prevent further processing.  The applicable may also define the conditions that are to be ignored if unknown or unrecognized and passed unmodified to the next IMS/SIP hop. Deliberate use of these later conditions in implementations is dependent on the following conditions;

- The UE is able to determine the roaming arrangement (LBO or S8HR) deployed in the VPLMN and act accordingly.

- VPLMNs pass any unknown or unrecognized IMS/SIP signalling elements which are not error conditions to the HPLMN unmodified.

The LI service may need to take these conditions into account by detecting unknown or unrecognized SIP signalling elements as well as potentially taking action when detected.

It is assumed that these conditions could be more likely encountered in S8HR arrangements than in LBO arrangements since for LBO there are active IMS network elements in the VPLMN participating in and processing the IMS signalling flows whereas for S8HR there are no IMS network elements in the VPLMN participating in or processing the IMS signalling flows.

Subclause 5.10 deals specifically with SIP information elements used for negotiating encryption.

### 5.6.2	Architectural requirements

Any solution needs to provide for VPLMN operator policies to:

*	Specify what constitutes unknown or unrecognized IMS/SIP protocol aspects from an LI service reporting perspective which otherwise would not otherwise cause a protocol processing exception.

*	Specify what actions to take when an unknown or unrecognized IMS/SIP protocol aspect is detected.  Possible actions include but not limited to: terminating the IMS bearers; reporting the SIP/IMS protocol aspect to the LEA as an unknown element; logging the element for further offline follow up.

### 5.6.3	Stage 3 requirements

No specific stage 3 requirement is expected.

### 5.6.4	Solution approaches

### 5.6.4.1	Solution #2 approach

The following summarizes the key steps of this approach:

- The LMISF provides the operator policy function for detecting unknown or unrecognized IMS/SIP protocol aspects not otherwise given IMS/SIP error treatment.

- The LMISF initiates the action specified by operator policy when an unknown or unrecognized IMS/SIP protocol aspect is detected and not otherwise given an IMS/SIP error treatment.  This can include but not limited to OAM&P element interaction for offline actions, reporting the elements to the LEA and/or initiate PCC controls to terminate any IMS sessions for the user.

***********************End of first change*************************************************************************