3GPP SA3LI#81-e-a	S3i210241

- eMeeting, 12-16 April 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0186 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Location: clarification on the payload direction field |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Nokia, Nokia Shanghai Bell) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI17 |  |  |  |  |  | Date: |  |  | 2021-04-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | All the xIRIs have a field named Payload Direction. The ETSI TS 103-221-2 provides different values that can be populated into the field. The values can be 1 (not known to the POI), 2 (sent to the target), 3 (received from the target), 4 (both sent to, received from, the target) or 5 (not applicable). However, for some xIRIs, it is obvious which values have to be used, and for some, it is not. This CR provides the clarity on the value to be used for location related xIRIs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For the LALSReport and MDFCellSiteReport, the value 5 are used. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Differing implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.2.4, 7.3.1.4, 7.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S3i210241 |  |  |  |  |  |  |  |  |

##### *** First change  ***

#### 7.3.1.4	Generation of xIRI over LI_X2

The IRI-POI provided by the LI-LCS client shall deliver the target location reports to respective MDF(s) as xIRI over the LI_X2 interface.

Table 7.3.1.4-1: LALSReport record

| Field name | Description | M/C/O |
| --- | --- | --- |
| sUPI | SUPI of the target, if used for the service (see NOTE). | C |
| pEI | PEI of the target, if used for the service (see NOTE). | C |
| gPSI | GPSI of the target, if used for the service (see NOTE). | C |
| location | Location of the target, if obtained successfully. Encoded as a positioningInfo parameter (location>positioningInfo). Both the positionInfo (location>positioningInfo>positionInfo) and the mLPPositionData (location>positioningInfo>rawMLPResponse>mLPPositionData) are present in the case of successful positioning. In the case of positioning failure only the mLPErrorCode (location>positioningInfo>rawMLPResponse>mLPErrorCode) is present. See Annex A. | C |
| NOTE: At least one of the SUPI, PEI or GPSI fields shall be present. |  |  |

The LI-LCS Client generating an xIRI containing an LALSReport record shall set the Payload Direction field in the PDU header to not applicable (Direction Value 5, see ETSI TS 103 221-2 [8] clause 5.2.6).

The LI_X2 header (as per clause 5.3.2) of the LALSReport record presented in Table 7.3.1.4-1 shall contain the correlation ID (if provided) from a respective LI_T2 ActivationTask message.

*** Next change  ***

#### 7.3.2.2	Delivery of cell site information over LI_HI2

The cell site information is encoded as the cellSiteInformation ASN.1 parameter and delivered either within the location field of an IRI message carrying the respective cell identity, or in a stand-alone IRI message containing the MDFCellSiteReport record.

The MDF2 shall use the IRI message containing the MDFCellSiteReport record to convey cell site information retrieved asynchronously with the sending of the IRI message that caused the retrieval. The MDFCellSiteReport record shall be delivered as an IRI REPORT (see ETSI TS 102 232-1 [9] clause 5.2.10) and allocated the same CIN, if any, as the IRI message that caused the retrieval.

When the cell site information is readily available at MDF2 or is retrieved synchronously (i.e., blocking the sending of the IRI message until the retrieval is complete), the cell site information shall be conveyed within the location field of the IRI message that caused the retrieval.

The cell site information for multiple cell identities can be delivered to the LEMF within an IRI message that carries the respective cell identities or within the IRI message containing the MDFCellSiteReport record (see Annex A).

The MDF2 generating the IRI message MDF2CellSiteRecord shall set the Payload Direction field in the PDU header to not applicable (Direction Value 5, see ETSI TS 103 221-2 [8] clause 5.2.6).

##### ** End of all changes **