3GPP SA3LI#81-e-a	S3i210273

- eMeeting, 12-16 April 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.127 | CR | 0122 | rev | 3 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IMS: Removing IBCF from the alternate option for non-local ID in the VPLMN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Nokia, Nokia Shanghai Bell) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI17 |  |  |  |  |  | Date: |  |  | 2021-04-13 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The IMS stage 2 specifies two deployment options: default and alternate to support S-CSCF based approach (default) or P-CSCF-based approach (alternate option). However, in the VPLMN there is no S-CSCF and hence, having an alternate option is not intended to be specified except for the emegency sessions since the E-CSCF is present even in the VPLMN. For the target non-local ID, the specification provides the IBCF approach as an alternate option in the VPLMN for the LBO approach. With LBO home-routed media path, there will be one IBCF in the VPLMN. With the optimal media path, there can be two IBCFs (or one IBCF and one MGCF) with the first IBCF not having an associated TrGW. Providing details on how and when the IBCF has to provide the LI functions and when not, can be complex while the option itself can be viewed as unnecessary. Therefore, it is suggested that the alternative approach be used only when the home-routed media is used. Also, allow the default and alternate options in the VPLM being mutually independent of the default and alternate options of HPLMN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The deployment options for non-local ID target ID are mutually independent in HPLMN and VPLMN. Furthermore, the alternate option in the VPLMN is used only for home-routed media. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Too many unresolved cases remain unclarified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.6.2, 7.4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S3i210273 |  |  |  |  |  |  |  |  |

##### *** First change  ***

#### 7.4.6.2	IMS Network Functions providing the IRI-POI

The IMS Network Functions that handle the target side of the session provide the IRI-POI functions except when the alternate option is used for the non-local ID target. When the alternate option is used for the non-local ID target, the IMS network function that handles the session-leg of the local served user connected directly to the non-local ID target.

Table 7.4.6.2-1 below identifies the IMS Network Functions in providing the IRI-POI functions in a non-roaming case for various session scenarios.

Table 7.4.6.2-1: IMS Network Functions providing the IRI-POI functions (non-roaming case)

| Session type/target type | Default | Alternative option |
| --- | --- | --- |
| Normal sessions | S-CSCF | P-CSCF |
| Emergency sessions | E-CSCF | P-CSCF (NOTE 1) |
| Redirected sessions: intra-PLMN | S-CSCF | P-CSCF |
| Redirected sessions: inter-PLMN (CS domain) | S-CSCF | MGCF |
| Redirected sessions: inter-PLMN (IMS domain) | S-CSCF | IBCF |
| Conference (NOTE 2) | Conf-AS/MRFC | - |
| PTC | PTC-Server | - |
| Non-local ID in CS domain (NOTE 3, NOTE 3A)) | MGCF | S-CSCF |
| Non-local ID in IMS domain (NOTE 3, NOTE 3A) | IBCF | S-CSCF |

NOTE 1:	For originated emergency sessions handled in the fixed networks, where S-CSCF is also part of an emergency session, the S-CSCF based IRI-POI as a deployment option may also be considered.

NOTE 2:	A conference ID can also be a target. Conf-AS stands for conference AS (see NOTE 2 in clause 7.4.4.1). When a normal session is extended to a conference session, the IMS signalling functions that provide the IRI-POI functions prior to the conference may continue to provide the IRI-POI functions in addition to the conference AS/MRFC.

NOTE 3:	Non-roaming means that the local served user is non-roaming.

NOTE 3A:	The default/alternate option used when the target is non-local ID is mutually independent of default/alternate option used when the target is local served user.

7.4.6.2-2 below identifies the IMS Network Functions in providing the IRI-POI functions in a roaming case for various session scenarios.

Table 7.4.6.2-2: IMS Network Functions providing the IRI-POI functions (roaming case)

| Session type/target type | Local Breakout (LBO) |  |  |  | Home Routed (HR) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | HPLMN |  | VPLMN |  | HPLMN |  | VPLMN |  |
|  | Default | Alternate Option | Default | Alternate Option | Default | Alternate Option | Default | Alternate Option |
| Normal sessions | S-CSCF | IBCF | P-CSCF | - | S-CSCF | P-CSCF | N9HR/S8HR | - |
| Emergency sessions | - | - | E-CSCF | P-CSCF | - | - | E-CSCF | P-CSCF |
| Redirected sessions | S-CSCF | See table 7.4.6.2-3 | - | - | S-CSCF | See table 7.4.6.2-3 | - | - |
| Conference (NOTE 2) | Conf-AS/MRFC | - | - | - | Conf-AS/MRFC | - | - | - |
| PTC | PTC-Server | - | - | - | PTC-Server | - | - | - |
| Non-local ID (E.164) in CS domain (NOTE 3A, NOTE 4, NOTE 4B) | MGCF | S-CSCF | P-CSCF | IBCF (NOTE 4C) | MGCF | S-CSCF (NOTE 3A) | N9HR/S8HR | - |
| Non-local ID in SIP/IMS domain (NOTE 3A, NOTE 4, NOTE 4B) | IBCF | S-CSCF) | P-CSCF | IBCF (NOTE 4C) | IBCF | S-CSCF (NOTE 3A) | N9HR/S8HR | - |

NOTE 4:	For roaming, this means the local served user is roaming. Also, see NOTE 3.

NOTE 4B:	The default/alternate options used in the HPLMN and default/alternate options used in the VPLMN are mutually independent.

NOTE 4C:	This alternate option may be used only in the VPLMN for IMS sessions with home-routed media.

The interception capabilities for normal sessions as defined in tables 7.4.6.2-1 (non-roaming) and 7.4.6.2-2 (roaming) shall be used for the cases where the Conf-AS and the PTSC-Server are not under the control of CSP serving the warrant.

Table 7.4.6.2-3: Extension of table 7.4.6.2-2

| Session type/target type |  | Local Breakout (LBO) | Home Routed (HR) |
| --- | --- | --- | --- |
| Redirected sessions: Intra-PLMN | Redirected-to party non-roaming | P-CSCF | P-CSCF |
|  | Redirected-to party is roaming | IBCF | P-CSCF |
| Redirected sessions Inter-PLMN | Redirected-to party in CS domain | MGCF | MGCF |
|  | Redirected-to party in IMS domain | IBCF | IBCF |

Table 7.4.6.2-3 shows the IMS Network Functions that provide the IRI-POI functions in the HPLMN for redirected sessions in a roaming case when the alternate option is used to provide the IRI-POI functions for the normal case.

NOTE 5:	For the redirected do not answer related sessions, the IMS Network Functions that provide the IRI-POI functions prior to the redirection are as illustrated in table 7.4.6.2-2 (normal case) and after the redirection are as illustrated in table 7.4.6.2-3.

*** Next change  ***

#### 7.4.6.3	IMS Network Functions providing the CC-TF and CC-POI functions

The IMS Network Functions that handle the target side (including the non-local ID target) of the session provide the CC-TF and CC-POI functions. For redirected scenarios, the IMS Network Functions that handle the redirected-to-user side of the session provide the CC-TF and CC-POI functions.

Table 7.4.6.3-1 provides the IMS Network Functions that provide the CC-TF functions when the CC-POI functions are provided by the IMS Media Functions as indicated (also see clause 7.4.4.1).

Table 7.4.6.3-1: Mapping between the IMS Network Functions providing the CC-TF and the CC-POI functions

| CC-POI | CC-TF |
| --- | --- |
| PGW (NOTE 1) | P-CSCF |
| PGW-U (NOTE 1) | P-CSCF |
| IMS-AGW | P-CSCF |
| MRFP | Conference AS/MRFC |
| PTC-Server | PTC-Server |
| TrGW | IBCF |
| IM-MGW | MGCF |

NOTE 1: 	This is defined in TS 33.107 [11] and outside the scope of the present document.

Table 7.4.6.3-2 below identifies the IMS Media Functions that provide the CC-POI functions in a non-roaming case for session scenarios (PGW and PGW-U based options are not shown in the table).

Table 7.4.6.3-2: IMS Media Functions providing the CC-POI functions (non-roaming case)

| Session type/target type | CC-POI |
| --- | --- |
| Normal sessions | IMS-AGW |
| Emergency sessions | IMS-AGW |
| Redirected sessions: intra-PLMN | IMS-AGW |
| Redirected sessions: inter-PLMN (CS domain) | IM-MGW |
| Redirected sessions: inter-PLMN (IMS-domain) | TrGW |
| Conference (NOTE 4) | MRFP |
| PTC | PTC- Server |
| Non-local ID in CS domain (NOTE 2) | IM-MGW |
| Non-local ID in IMS domain (NOTE 2) | TrGW |

NOTE 2:	Non-roaming means that the local served user is non-roaming.

Table 7.4.6.3-3 below identifies the IMS Media Functions that provide the CC-POI functions in a roaming case for various session scenarios (PGW and PGW-U based options are not shown in the table).

Table 7.4.6.3-3: IMS Media Functions providing the CC-POI functions (roaming case)

| Session type/target type |  | Local Breakout (LBO) |  |  | Home Routed (HR) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | HPLMN | VPLMN |  | HPLMN | VPLMN |
|  |  |  | Default | Alternate Option |  |  |
| Normal sessions |  | TrGW | IMS-AGW | - | IMS-AGW | N9HR/ S8HR |
| Emergency sessions |  | - | IMS-AGW | - | - | IMS-AGW |
| Redirected sessions: intra-PLMN | Redirected-to-party non-roaming | IMS-AGW | - | - | IMS-AGW | - |
|  | Redirected-to-party roaming | TrGW | - | - | IMS-AGW | - |
| Redirected sessions: inter-PLMN | Redirected-to-party in CS domain | IM-MGW | - | - | IM-MGW | - |
|  | Redirected-to-party in IMS domain | TrGW | - | - | TrGW | - |
| Conference (NOTE 4) |  | MRFP | - | - | MRFP | - |
| PTC |  | PTC-Server | - | - | PTC-Server | - |
| Non-local ID in CS domain (NOTE 3) |  | IM-MGW | IMS-AGW | TrGW (NOTE 5) | IM-MGW | N9HR/ S8HR |
| Non-local ID in IMS domain (NOTE 3) |  | TrGW | IMS-AGW | TrGW (NOTE 5) | TrGW | N9HR/S8HR |

NOTE 3:	Roaming means that the local served user is roaming.

NOTE 4:	When a normal session is extended to a conference session, the IMS-AGW that provides the CC-POI functions prior to the conference may continue to provide the CC-POI functions as an alternate, or in addition, to the MRFP. In that case, the P-CSCF provides the CC-TF functions for the CC-POI in the IMS-AGW.

NOTE 5:	This is applicable only for IMS sessions with home-routed media with a TrGW present in the VPLMN.

##### ** End of all changes **