3GPP SA3LI#83-e-b	S3i210829

- eMeeting, 1-5 November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.127 | CR | 0154 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IMS stage 2 clarification due to STIR/SHAKEN and RCD/eCNAM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Nokia, Nokia Shanghai Bell) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI17 |  |  |  |  |  | Date: |  |  | 2021-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The STIR/SHAKEN and RCD/eCNAM clauses have introduced the LI functions to Telephony AS and the IBCF. The IMS stage 2 clause perhaps needs to mention or give reference that too. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Telephony AS is listed among the rest of the NFs that provide IRI-POI and a reference is added to the STIR/SHAKEN and RCD/eCNAM clause. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing clarity. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.3.1, 7.4.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S3i210829 |  |  |  |  |  |  |  |  |

##### *** First change ***

#### 7.4.3.1	General

The IRI-POI detects the SIP messages that are related to a target subscriber and then generates and delivers the related xIRI to the MDF2 over LI_X2.

The following IMS Network Functions (i.e. IMS Signalling Functions) that handle SIP signalling for IMS sessions may provide the IRI-POI functions:

-	S-CSCF.

-	E-CSCF.

-	P-CSCF.

-	IBCF.

-	MGCF.

-	Conference AS/MRFC.

-	Telephony AS.

-	PTC server.

Clause 7.4.6 gives more information from network topology/session perspective how different IMS Network Functions are to be used in providing the IRI-POI functions. The Telephony AS is one of the IMS Network Functions that provides the IRI-POI for STIR/SHAKEN and RCD/eCNAM (see clause 7.14.2).

##### *** Next change ***

#### 7.4.6.2	IMS Network Functions providing the IRI-POI

The IMS Network Functions that handle the target side of the session provide the IRI-POI functions except when the alternate option is used for the non-local ID target. When the alternate option is used for the non-local ID target, the IMS network function that handles the session-leg of the local served user connected directly to the non-local ID target.

Table 7.4.6.2-1 below identifies the IMS Network Functions in providing the IRI-POI functions in a non-roaming case for various session scenarios.

Table 7.4.6.2-1: IMS Network Functions providing the IRI-POI functions (non-roaming case)

| Session type/target type | Default | Alternative option |
| --- | --- | --- |
| Normal sessions | S-CSCF | P-CSCF |
| SMS over IMS | S-CSCF | P-CSCF |
| Emergency sessions | E-CSCF | P-CSCF (NOTE 1) |
| SMS over IMS to emergency services | E-CSCF | P-CSCF (NOTE1) |
| Redirected sessions: intra-PLMN | S-CSCF | P-CSCF |
| Redirected sessions: inter-PLMN (CS domain) | S-CSCF | MGCF |
| Redirected sessions: inter-PLMN (IMS domain) | S-CSCF | IBCF |
| Conference (NOTE 2) | Conf-AS/MRFC | - |
| PTC | PTC-Server | - |
| Non-local ID in CS domain (NOTE 3, NOTE 3A) | MGCF | S-CSCF |
| Non-local ID in IMS domain (NOTE 3, NOTE 3A) | IBCF | S-CSCF |
| Non-local ID for SMS over IMS (NOTE 3) | S-CSCF | P-CSCF (NOTE 3A) |

NOTE 1:	For originated emergency sessions (or SMS over IMS to emergency services centre) handled in the fixed networks, where S-CSCF is also part of an emergency session, the S-CSCF based IRI-POI as a deployment option may also be considered.

NOTE 2:	A conference ID can also be a target. Conf-AS stands for conference AS (see NOTE 2 in clause 7.4.4.1). When a normal session is extended to a conference session, the IMS signalling functions that provide the IRI-POI functions prior to the conference may continue to provide the IRI-POI functions in addition to the conference AS/MRFC.

NOTE 3:	Non-roaming means that the local served user is non-roaming.

NOTE 3A:	The default/alternate option used when the target is non-local ID is mutually independent of default/alternate option used when the target is local served user.

7.4.6.2-2 below identifies the IMS Network Functions in providing the IRI-POI functions in a roaming case for various session scenarios.

Table 7.4.6.2-2: IMS Network Functions providing the IRI-POI functions (roaming case)

| Session type/target type | Local Breakout (LBO) |  |  |  | Home Routed (HR) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | HPLMN |  | VPLMN |  | HPLMN |  | VPLMN |  |
|  | Default | Alternate Option | Default | Alternate Option | Default | Alternate Option | Default | Alternate Option |
| Normal sessions | S-CSCF | IBCF | P-CSCF | - | S-CSCF | P-CSCF | N9HR/S8HR | - |
| SMS over IMS | S-CSCF | IBCF | P-CSCF | - | S-CSCF | P-CSCF | N9HR/S8HR | - |
| Emergency sessions/SMS over IMS | - | - | E-CSCF | P-CSCF | - | - | E-CSCF | P-CSCF |
| SMS over IMS to emergency services | - | - | E-CSCF | P-CSCF | - | - | E-CSCF | P-CSCF |
| Redirected sessions | S-CSCF | See table 7.4.6.2-3 | - | - | S-CSCF | See table 7.4.6.2-3 | - | - |
| Conference (NOTE 2) | Conf-AS/MRFC | - | - | - | Conf-AS/MRFC | - | - | - |
| PTC | PTC-Server | - | - | - | PTC-Server | - | - | - |
| Non-local ID (E.164) in CS domain (NOTE 3A, NOTE 4, NOTE 4A) | MGCF | S-CSCF | P-CSCF | IBCF (NOTE 4B) | MGCF | S-CSCF (NOTE 3A) | N9HR/S8HR | - |
| Non-local ID in SIP/IMS domain (NOTE 3A, NOTE 4, NOTE 4A) | IBCF | S-CSCF | P-CSCF | IBCF (NOTE 4B) | IBCF | S-CSCF (NOTE 3A) | N9HR/S8HR | - |
| Non-local ID for SMS over IMS (NOTE 4) | S-CSCF | IBCF | P-CSCF | - | S-CSCF | P-CSCF | N9HR/S8HR | - |

NOTE 4:	For roaming, this means the local served user is roaming. Also, see NOTE 3.

NOTE 4A:	The default/alternate options used in the HPLMN and default/alternate options used in the VPLMN are mutually independent.

NOTE 4B:	This alternate option may be used only in the VPLMN for IMS sessions with home-routed media.

The interception capabilities for normal sessions as defined in tables 7.4.6.2-1 (non-roaming) and 7.4.6.2-2 (roaming) shall be used for the cases where the Conf-AS and the PTSC-Server are not under the control of CSP serving the warrant.

Table 7.4.6.2-3: Extension of table 7.4.6.2-2

| Session type/target type |  | Local Breakout (LBO) | Home Routed (HR) |
| --- | --- | --- | --- |
| Redirected sessions: Intra-PLMN | Redirected-to party non-roaming | P-CSCF | P-CSCF |
|  | Redirected-to party is roaming | IBCF | P-CSCF |
| Redirected sessions Inter-PLMN | Redirected-to party in CS domain | MGCF | MGCF |
|  | Redirected-to party in IMS domain | IBCF | IBCF |

Table 7.4.6.2-3 shows the IMS Network Functions that provide the IRI-POI functions in the HPLMN for redirected sessions in a roaming case when the alternate option is used to provide the IRI-POI functions for the normal case.

NOTE 5:	For the redirected do not answer related sessions, the IMS Network Functions that provide the IRI-POI functions prior to the redirection are as illustrated in table 7.4.6.2-2 (normal case) and after the redirection are as illustrated in table 7.4.6.2-3.

The IMS Network Functions that provide the IRI-POI for STIR/SHAKEN and RCD/eCNAM are listed in clause 7.14.2.

##### *** End of all changes ***