3GPP SA3LI#88e-v	S3i230149

- Electronic, 21-23 February 2023

Title:	Reply LS on lawful interception EPS fallback for 5G inbound roamer

Response to:	LS from GSMA on lawful interception EPS fallback for 5G inbound roamer (S3i230134)

Release:	Release 18

Work Item:	n/a

Source:	SA3-LI

To:	GSMA NRG

Cc:	SA, SA3

Contact Person:

Name:	Nag Rao

Tel. Number:

E-mail Address:	Nagaraja.rao@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description

SA3-LI thank GSMA NRG for their LS requesting clarification on the lawful interception mechanism for 5G inbound roamers in case of EPS fallback with voice calls.

SA3-LI would clarify that the lawful interception shall apply for 5G inbound roamers when EPS fallback is applied for voice calls. The lawful interception of voice calls initiated from, or terminated to, 5G inbound roamers are described in TS 33.127 clause 7.4.7 and TS 33.128 clause 7.10. The LMISF as described in those clauses provides POI functions for voice calls involving the inbound roaming UEs independent of the access network (i.e. 5GS or EPS).

Even though the above clauses do not mention the word "EPS Fallback", the lawful interception for the particular use case described in the LS can be addressed through the concepts described in TS 33.127 clause 7.4.7.4.12 Target UE Mobility. The scope of LI functions provided within the SMF and UPF (required anyway to support the data interception in 5GS) shall also include the BBIFF capabilities described in the above clauses.

SA3-LI would like to point out that some BBIFF capabilities (e.g. providing LMISF with the identities involved in the IMS communication) shall be supported by the SMF/UPF even if 5G voice service is provided via the EPS fallback solution for inbound roamers.

A call-flow based illustration of the BBIFF relocation aspects are described in the TR 33.929 clause 5.8.6.

2. Actions:

To GSMA NRG:

ACTION: 	SA3-LI kindly ask GSMA NRG  to consider the above clarification for the use case described in the LS.

3. Date of Next SA3-LI WG Meetings:

SA3-LI Meeting #89		25-28 April 2023                                 Washington DC, US