3GPP SA3LI#91e-b	s3i230706

# 27-28 November 2023, electronic

Source:	Ericsson, Nokia, Nokia Shanghai Bell, OTD_US

Title:	Further comments on DL Indirect Data Forwarding

Document for:	Discussion

Agenda Item:	4

Work Item / Release:	Rel-18

Abstract of the contribution: Dependency between N4 and LI functions

In SA3LI#91 meeting (Sydney), tdoc s3i230606 “Additions to prevent redundant interception of packets from DL data forwarding tunnel” was discussed. The outcome of the meeting was that the document was not pursued, however content of CR is agreeable, but deferred pending confirmation by CT4. A LS was sent to CT4 in tdoc s3i230618, asking to update the TS 29.244 so that the Source Interface Type and the Destination Interface type are included in the PFCP session related requests sent to the UPF for DL data forwarding tunnels.

However, the proposed solution in tdoc s3i230606 creates a dependency on the N4 (which is a non-LI interface) parameters by the LI functions.

Such a dependency for the given use case could interfere with the LI functions.

It is then proposed to rediscuss the topic and assess whether the proposal is considered appropriate from that perspective.