BacSA3#95-LI	 S3i240622

Las Vegas; October 29-November 1, 2024

Source:	Nokia,

Title:	pCR to TR 33.929-8: MMS LI – MMS background with scope of LI – part I

Document for:	Agreement

Agenda Item:	9

Work Item / Release:	LI19

Abstract of the contribution: This pCR provides input text to TR 33.929-8 LI for MMS. Provides some MMS background information with LI scope – this is part I.

BACKGROUND

The TR 33.929-8 provides the implementation guidance for the LI aspects related to the MMS. This pCR gives some background on MMS with functional elements, flow-diagrams.

PROPOSAL

Incorporate the following to the TR 33.929-8.

***** New text ****

# 4	Illustration of LI for MMS

## 4.1	General

Multimedia Messaging Service (MMS) is a session-based messaging service handled by the Network Functions (NFs) referred to as MMS Relay and MMS Server together referred to as MMS Relay/Server in the present document.

An MMS client can interact with variety of other clients via the Multimedia (MM) messages. The details of MMS architecture are described in TS 23.140 [18].

In the subsequent clauses, the application within MMS client's device is referred to as:

-	MMS User Agent when the MM message originated from, or received at with, a phone number (e.g. MSISDN) as the identity.

- 	Email Client when the MM message originated from, or received at with, an Email Address as the identity.

NOTE 1:	The email message may be treated as an MM message in the illustrations of the present document.

The LI aspects of MMS are defined in TS 33.127 [3], TS 33.128 [4] and TR 33.928 [9]. The term MMS LI used in the present document refers to the LI functions provided by the MMS Relay/Server. The OMA specification [19] and TS 33.128 [4] refer to the MMS Relay/Server as MMS Proxy-Relay.

An overview of the MMS LI architecture is depicted in figure 4.1-1 below:

Figure 4.1-1: An overview of MMS LI architecture

With a warrant to intercept the MMS related communications of a target, the LEA submit an intercept request to the CSP (i.e. to the ADMF) over the LI_HI1 interface. The LIPF within the ADMF provisions the target identities to IRI-POI and the CC-POI present in the MMS Relay/Server. The target identity can be local to the MMS Relay/Server or can be a non-local ID.

The IRI-POI generates the xIRIs from the MM1/MM4 messages (see clause 4.2.3) and the send the same over the LI_X2 interface to the MDF2. The MDF2 forwards the same to the LEMF over the LI_HI2 interface as IRI messages.

The CC-POI generates the xCC from the MM1/MM4 messages that contain the media information and the send the same over the LI_X3 interface to the MDF3. The MDF3 forwards the same to the LEMF over the LI_HI3 interface as IRI messages.

The illustrations given in the present document focus on scope of target identifiers (see clause 4.3) and scope of MMS messages (see clause 4.4) for generating the xIRIs. No further illustration of xCC generation is presented.

## 4.2	MMS background with LI scope

### 4.2.1	Background

#### 4.2.1.1	MMS functional elements and the interfaces

The figure 4.2.1.1-1 below gives an overview of the MMS concepts and the interfaces based on the architecture defined in TS 23.140 [18]. To illustrate the concept, two MMS User Agents (A and B) are shown.

Figure 4.2.1.1-1: MMS concepts per TS 23.140

NOTE 1: MMS User Agent (A) and MMS User Agent (B) may also be served by the same MMS Relay/Server.

The MM messages to or from an MMS User Agent flow through the MMS Relay Server.

To send an MM message, the originating MMS User Agent submits the MM message to the MMS Relay/Server over the MM1 interface. The MM message may be stored at the MMBox.

The terminating MMS Relay/Server would store the MM message and notify the terminating MMS User Agent with the link for the stored MM message over MM1 interface. The terminating MMS User Agent would then (can also be automatic) retrieve the stored MM message. The MM message may also be stored/retrieved using an MMBox.

If two different MMS Relay/Servers serve the two involved MMS User Agents, then the MM messages are transferred between the two MMS Relay/Servers over the MM4 interface.

Once the message is retrieved (at the terminating end), a delivery report may be sent from the terminating MMS Relay/Server to the originating MMS User Agent via originating MMS Relay/Server.

A read report may be sent from the terminating MMS User Agent to the originating MMS User Agent via MMS Relay/Servers.

Alternatively, the terminating MMS User Agent may forward or delete the MM message stored at the MMS Relay/Server without retrieving. Also, the terminating MMS Relay/Server may cancel the retrieved MM message at the terminating MMS User Agent.

The MM message can also be stored, uploaded, viewed, retrieved through the MMBox.

Other functional entities and the interfaces may come into play when an MM message cannot be transferred over the MM4 interface. For example, the MM message may also be initiated by an MMS VAS application which would be handled in the same except that MMS application would submit the message via MM7 interface. Likewise, if the MM message happens to be an email message, then the originating MMS Relay/Server would forward that email message via the External Server (i.e. email server) over the MM3 interface. Likewise, a terminating MMS Relay/Server may receive an email message destined to an MMS User Agent via the External Server (i.e. Email Server) over the MM3 interface.

For LI, however, the focus is on the message exchanges between the MMS Relay/Server and the MMS User Agent over the MM1 interface and the message exchanges between the two MMS Relay/Servers over the MM4 interface. Therefore, all interfaces and functional entities shown in figure 4.2.1.1-1 except MM1, MM4, MMS User Agent, MMS Relay/Server are outside the scope of present document even though the MM3 has an indirect influence and hence, referenced along with the External Server (Email).

#### 4.2.1.2	Flow diagrams

##### 4.2.1.2.1	MMS User Agent A to MMS User Agent B

An abstract level flow diagram depicting the flow of an MM message between the two MMS User Agents served by two different MMS Relay/Servers is shown in figure 4.2.1.2.1-1 (based of TS 23.140 [18]).

4.2.1.2.1-1: MMS flow (A-to-B) at an abstract level (TS 23.140 [18] based)

Brief description of the flow:

The MMS User Agent (A) submits the MM message over the MM1 interface using the MM1_submit.REQ message. The originating MMS Relay/Server (A) forwards the MM message to the terminating MMS Relay/Server (B) over the MM4 interface using the MM4_forward.REQ message. An intermediate delivery report is sent to the MMS User Agent (A) using the MM1_delivery_report (intermediate) message.

The MMS Relay/Server (B) notifies the MMS User Agent (B) using the MM1_notification.REQ message. The MMS User Agent (B) retrieves the MM message using the MM1_retrieve.REQ message.

A delivery report is sent by the MMS Relay/Server (B) when it receives a confirmation of the retrieval (MM1_acknowledge.REQ) using the MM4_delivery_report.REQ (retrieved) message. The MMS Relay/Server (A) sends the MM1_delivery_report.REQ (retrieved) message to the MMS User Agent (A).

The MMS User Agent (B) sends a read-reply report using the MM1_read_reply_receipient.REQ to the MMS Relay/Server (B). The MMS Relay/Server (B) forwards the same to the MMS Relay/Server (A) using the MM4_read_reply_report.REQ message. The MMS Relay/Server (A) forwards the read-reply report to the MMS User Agent (A) using the MM1_read_reply_originator.REQ message.

Not elaborated or not shown:

Sending of delivery reports are conditional, in the sense, sent only when requested so by the MMS User Agent (A) and permitted (except the intermediate) by the MMS User Agent (B). Sending of the read-reply report is also conditional in the sense, sent by the MMS User Agent (B), if requested by the MMS User Agent (A).

The retrieval of the MM message at the terminating end can also be automatic, in the sense, without any user action. When it is not automatic, other alternate steps mentioned in clause 4.2.1.1 are possible (e.g. forward).

The MMBox, at either end of the flow (i.e. within the MMS Relay/Servers) may be used. The MMBox related steps are not shown in figure 4.2.1.2.1-1.

##### 4.2.1.2.2	MMS User Agent A to MMS User Agent B forwarded to MMS User Agent C

An abstract level flow diagram depicting the flow of an MM message between the three MMS User Agents served by three different MMS Relay/Servers is shown in figure 4.2.1.2.2-1.

4.2.1.2.2-1: MMS flow (A-to-B-to-C) at an abstract level (TS 23.140 [18] based)

Brief description of the flow:

The MMS User Agent (A) submits the MM message over the MM1 interface using the MM1_submit.REQ message. The originating MMS Relay/Server (A) forwards the MM message to the MMS Relay/Server (B) over the MM4 interface using the MM4_forward.REQ message. An intermediate delivery report is sent to the MMS User Agent (A) using the MM1_delivery_report (intermediate) message.

The MMS Relay/Server (B) notifies the MMS User Agent (B) using the MM1_notification.REQ message. The MMS User Agent (B) initiates the forwarding to MMS User Agent (C) using the MM1_forward.REQ message. The MMS Relay/Server (B) forwards the MM message to the MMS Relay/Server (C) over the MM4 interface using the MM4_forward.REQ message. An intermediate delivery report is sent to the MMS User Agent (B) using the MM1_delivery_report (intermediate) message. A forwarded delivery report is sent toward the MMS User Agent (A) by the MMS Relay/Server (B) using the MM4_delivery_report.REQ (forwarded). The MMS Relay Server (A) sends the MM1_delivery_report.REQ (forwarded) to the MMS User Agent (A).

The MMS Relay/Server (C) notifies the MMS User Agent (C) using the MM1_notification.REQ message. The MMS User Agent (C) retrieves the MM message using the MM1_retrieve.REQ message.

A delivery report is sent by the MMS Relay/Server (C) when it receives a confirmation of the retrieval (MM1_acknowledge.REQ) using the MM4_delivery_report.REQ (retrieved) message. The MMS Relay/Server (B) sends the MM1_delivery_report.REQ (retrieved) message to the MMS User Agent (B).

The MMS User Agent (C) sends a read-reply report using the MM1_read_reply_receipient.REQ to the MMS Relay/Server (C). The MMS Relay/Server (C) forwards the same to the MMS Relay/Server (B) using the MM4_read_reply_report.REQ message. The MMS Relay/Server (B) forwards the read-reply report to the MMS User Agent (B) using the MM1_read_reply_originator.REQ message.

Not elaborated or not shown:

Sending of delivery reports are conditional, in the sense, for A-to-B leg, sent only when requested so by the MMS User Agent (A) permitted (except the intermediate) by the MMS User Agent (B). Likewise, for B-to-C leg, sent only when requested so by the MMS User Agent (B) permitted (except the intermediate) by the MMS User Agent (C). Sending of the read-reply report (for B-to-C leg) is also conditional in the sense, sent by the MMS User Agent (C), if requested by the MMS User Agent (B).

The retrieval of the MM message at the terminating end (MMS User Agent (C) can also be automatic, in the sense, without any user action. When it is not automatic, other alternate steps mentioned in clause 4.2.1.1 are possible (e.g. further forward).

The MMBox, at all three ends of the flow (i.e. within the MMS Relay/Server), may be used. The MMBox related steps are not shown in figure 4.2.1.2.2-1.

***** End of the new text ****