SA3LI#95	 S3i240624

Las Vegas; October 29-November 1, 2024

Source:	Nokia,

Title:	pCR to TR 33.929-8: MMS LI: Scope of target identifiers – Part I (without redirection, multiple MMS Relay/Servers)

Document for:	Agreement

Agenda Item:	9

Work Item / Release:	LI19

Abstract of the contribution: This pCR provides input text to TR 33.929-8 LI for MMS. Scope of target identifiers without redirection are illustrated in a topology view format. Here, two MMS Relay/Servers are used.

BACKGROUND

The TR 33.929-8 provides the implementation guidance for the LI aspects related to the MMS. The target identities can be a phone number or an email address. Several permutations are possible when messages are exchanged between two clients represented by those identities.

PROPOSAL

Incorporate the following to the TR 33.929-8.

# 4	Illustration of LI for MMS

## 4.1	General

## 4.2	MMS background with LI scope

*** New Text ***

## 4.3	Scope of target identities

### 4.3.1	General

As described in clause 4.2 and as illustrated in figure 4.2-2, the scope of MMS LI depends on the source and destination identity of the MM message.

For the purpose of illustrations given in the present document, a phone number is referenced either as a "phone number" or as an "MSISDN" that may be any of the following:

- MSISDN.

- GPSI (MSISDN).

- E.164 number.

Furthermore, in the illustrations, two forms of email addresses are referenced:

- Generic Email Address.

- MSISDN based Email Address.

The generic Email Address is used by the Email Client. The MSISDN based Email Address is a specific case where the MSISDN (i.e. any of E.164 number, GPSI (MSISDN), or MSISDN) of the remote MMS User Agent is in the user-id part of that Email Address. The MSISDN based Email Address is used to address the remote MMS User Agent in an email format and allows an Email Client to send an MM message to an MMS User Agent and vice-versa. The domain name of the MSISDN based Email Address may belong to the CSP and may be helpful in finding the MMS Relay/Server that serves the MMS User Agent.

An IMSI or SUPI (IMSI) can be the targets for MMS LI in the following circumstances:

- During the initial authorization, the MMS/Relay Server is able to associate the target identities IMSI or SUPI (IMSI ) to an MSISDN. In this case, all MM messages using that MSISDN are intercepted.

- When the MMS Relay/Server as a part of discovering the destination MMS/Relay Server gets the IMSI or SUPI (IMSI) of the terminating MMS User Agent. In that case, the MM messages destined to the associated MSISDN are intercepted. In this case, such an IMSI or GPSI (IMSI) are treated as target non-local ID.

Editor's Note: This shaded text can be removed, if the CRs in S3i240614 to S3i240619 are approved.

For the purpose of illustrations, the NAI (SUPI or GPSI) are treated as a generic Email Address. The present document limits the discussions of NAI (SUPI or GPSI), IMSI, or SUPI (IMSI) based target identities to this clause.

The IMPU and IMPI based target identities are not considered in the present document.

The subsequent clauses focus only on phone number (E.164 number, MSISDN, GPSI (MSISDN)) and the Email Address (generic or MSISDN based) based target identities.

In the illustrations, the indicated addresses are at the SMTP layer for the messages transferred over the MM3 interface (between MMS Relay/Server and the External Server) and are at the application layer for the messages transferred over the MM1 (between MMS User Agent and MMS Relay/Server) and the MM4 (between the two MMS Relay/Servers) interfaces.

### 4.3.2	Topology views - without redirection

#### 4.3.2.1	General

In the illustrations shown in the following sub-clauses, the MMS User Agent (A) or Email Client (AA) sends an MM message to MMS User Agent (B) or to an Email Client (BB). The two MMS User Agents may be served by different MMS Relay Servers or by the same MMS Relay Server. Likewise, the two Email Clients may have the same or different Email domains.

#### 4.3.2.2	Multiple MMS Relay/Servers

##### 4.3.2.2.1	Overview

The figure 4.3.2.2.1-1 illustrates the scenarios of different identities that are applicable for MMS LI when there is no redirection and two MMS Relay/Servers are involved in the MMS flow.

Figure 4.3.2.2.1-1: Scope of target identities without redirection (multiple Relay/Servers)

As shown in figure 4.3.2.2.1-1, MM messages originated from an Email Client (i.e. generic Email Address) do not hit the MMS Relay/Server at the originating end. Likewise, MM messages destined to an Email Client (i.e. generic Email Address) do not hit the MMS Relay Server at the terminating end. Therefore, the MM messages from a generic Email Address to a generic Email Address are outside the scope of MMS LI.

However, MM messages do hit the MMS Relay Server when at least one end of the MM message is a phone number allowing the Email Address to be the identity of the remote user. Because of this, an Email Address can only be a target non-local ID.

The phone number may be in an email format within the MM message (i.e. MSISDN based Email Address). In this case, the local MMS Relay/Server (originating end or terminating end) adds or removes the domain name. The email domain name could identify the MMS Relay/Server.

The figure 4.3.2.2.1-1 shows five scenarios with permutations of different identities used at either end of the MM message flow. These five scenarios are further illustrated in the clause 4.3.2.2.2.

##### 4.3.2.2.2	Scenario descriptions

###### 4.3.2.2.2.1	General

Figure 4.3.2.2.1-1 illustrates the following five scenarios:

-	MM message originated from a phone number, destined to a phone number, terminated at a phone number. This is referenced as MSISDN to MSISDN – case 1.

-	MM message originated from a phone number, destined to an MSISDN based Email Address, terminated at a phone number. This is referenced as MSISDN to MSISDN – case 2.

-	MM message originated from a phone number, destined to a generic Email Address, terminated at an Email Client (generic Email Address). This is referenced as MSISDN to generic Email Address.

-	MM message originated from at an Email Client (generic Email Address), destined to an MSISDN based Email Address, terminated at a phone number. This is referenced as generic Email Address to MSISDN.

-	MM message originated from at an Email Client (generic Email Address), destined to a generic Email Address, terminated at an Email Client (generic Email Address). This is referenced as generic Email Address to generic Email Address.

In MSISDN to MSISDN (case 2), the termination end of an MSISDN based Email Address is a phone number (MSISDN, GPSI (MSISDN), E.164 number). From the originating end perspective, the destination is an MSISDN based Email Address. From the terminating end's perspective, the originating point is an MSISDN based Email Address.

###### 4.3.2.2.2.2	Scenario 1: MSISDN to MSISDN - case 1

MMS User Agent (A) uses the own phone number (MSISDN, GPSI (MSISDN), E.164 number) and addresses the destination (MMS User Agent (B)) also with a phone number (MSISDN, GPSI (MSISDN), E.164 number).

Figure 4.3.2.2.2.2-1: MM message from MSISDN (A) to MSISDN (B) – case 1

In this scenario, phone number A can be a target in the MMS Relay/Server (A) and can be a target non-local ID in the MMS Relay/Server (B). Likewise, phone number B can be a target in the MMS Relay/Server (B) and can be a target non-local ID in the MMS Relay/Server (A).

###### 4.3.2.2.2.3	Scenario 2: MSISDN to MSISDN – case 2

MMS User Agent (A) uses the own phone number (MSISDN, GPSI (MSISDN), E.164 number) and addresses the destination (MMS User Agent (B)) with an MSISDN based Email Address (shown as B@mmsc-b).

The MMS Relay/Server (A) adds the domain name (shown as @mmsc-a) to the A's identity (shown as A@mmsc-a) before forwarding the MM message to the External Server serving the domain mmsc-a.

The MMS Relay/Server (B) would remove the domain name (shown as @mmsc-b) from B@mmsc0b, at the terminating end from B@mmsc-b.

The phone number B is seen as an MSISDN based Email Address in the MMS Relay/Server (A) and the phone number A is seen as an MSISDN based Email Address in the MMS Relay Server (B).

Figure 4.3.2.2.2.3-1: MM message from MSISDN (A) to MSISDN (B) – case 2

In this scenario, the phone number A can be a target in the MMS Relay/Server (A) and A@mmsc-a can be a target non-local ID in the MMS Relay/Server (B). Likewise, phone number B can be a target in the MMS Relay/Server (B) and B@mmsc-b can be a target non-local ID in the MMS Relay/Server (A).

###### 4.3.2.2.2.4	Scenario 3: MSISDN to generic Email Address

MMS User Agent (A) using the own phone number (MSISDN, GPSI (MSISDN), E.164 number) sends an MM message to a generic Email Address (shown as BB@email). This MM message goes from the external server (email) to the Email Client BB.

The MMS Relay/Server (A) adds the domain name (shown as @mmsc-a) to the A's identity (shown as A@mmsc-a) before forwarding the MM message to the External Server serving the domain mmsc-a.

Figure 4.3.2.2.2.4-1: MM message from MSISDN (A) to generic Email Address (BB)

In this scenario, the phone number A can be a target in the MMS Relay/Server (A). The Email Address BB@email can be a target non-local ID in the MMS Relay/Server (A).

###### 4.3.2.2.2.5	Scenario 4: Generic Email Address to MSISDN

Email Client (shown as AA) sends an MM message to a MSISDN based Email Address (shown as B@mmsc-b).

The MMS Relay/Server (B) would remove the domain name (shown as @mmsc-b) at the terminating end.

Figure 4.3.2.2.2.5-1: MM message from generic Email Address (AA) to MSISDN (B)

In this scenario, the phone number B can be a target in the MMS Relay/Server (B). The Email Address AA@email can be a target non-local ID in the MMS Relay/Server (B).

###### 4.3.2.2.2.6	Scenario 5: Generic Email Address to generic Email Address

Email Client AA sends an email message to Email Client BB.

Figure 4.3.2.2.2.6-1: Email message from generic Email Address (AA) to generic Email Address (BB)

MMS LI does not apply to this scenario, since the email message from AA@email to BB@email does not hit the MMS/Relay Server.

***** End of the new text ****