SA3LI#95	 S3i240625

Las Vegas; October 29-November 1, 2024

Source:	Nokia,

Title:	pCR to TR 33.929-8: MMS LI: Scope of target identifiers – Part II (without redirection, single MMS Relay/Server)

Document for:	Agreement

Agenda Item:	9

Work Item / Release:	LI19

Abstract of the contribution: This pCR provides input text to TR 33.929-8 LI for MMS. Scope of target identifiers without redirected are illustrated in a topology view format. Here, one MMS Relay/Server is used.

BACKGROUND

The TR 33.929-8 provides the implementation guidance for the LI aspects related to the MMS. The target identities can be a phone number or an email address. Several permutations are possible when messages are exchanged between two clients represented by those identities.

In the illustrations shown here, both MMS User Agents are served by the same MMS Relay/Server. Interesting to note here is that target non-local ID comes into play here even if both MMS User Agents are served by the same MMS Relay/Server.

PROPOSAL

Incorporate the following to the TR 33.929-8.

# 4	Illustration of LI for MMS

## 4.1	General

## 4.2	MMS background with LI scope

## 4.3	Scope of target identities

### 4.3.1	General

### 4.3.2	Topology views - without redirection

#### 4.3.2.1	General

#### 4.3.2.2	Multiple MMS Relay/Servers

*** New Text ***

#### 4.3.2.3	One MMS Relay/Server

##### 4.3.2.3.1	Overview

The figure 4.3.2.3.1-1 illustrates the cases of different identities that are applicable for MMS LI when there is no redirection and only MMS Relay Server is involved in the MMS flow.

In this case, it is presumed that MMS User Agent (A) and MMS User Agent (B) are served by the same MMS Relay/Server.

Figure 4.3.2.3.1-1 Scope of target identities without redirection (one Relay/Server)

As shown in figure 4.3.2.3.1-1, MM messages originated from an Email Client (i.e. generic Email Address) do not hit the MMS Relay/Server at the originating end. Likewise, the MM messages destined to an Email Client (i.e. generic Email Address) do not hit the MMS Relay Server at the terminating end. Therefore, the MM messages from a generic Email Address to a generic Email Address are outside the scope of MMS LI. However, the MM messages do hit the MMS Relay Server when at least one end of the MM message is a phone number (MSISDN, GPSI (MSISDN), E.164 number).

As illustrated in clause 4.3.2.2, the phone number may be in an email format within the MM message (i.e. MSISDN based Email Address). In this case, the local MMS Relay/Server (originating end or terminating end) adds or removes the domain name. The email domain name could identify the MMS Relay/Server. Even though the MMS User Agent (A) and MMS User Agent (B) are served by the same MMS Relay Server, when an MSISDN based Email Address is used for either of them, they can only be a target non-local ID. When an MMS User Agent sends or receives an MM message from a generic Email Address, that generic Email Address can be a target not local ID.

The five scenarios illustrated in figure 4.3.2.3.1-1 are further described in clause 4.3.2.3.2.

##### 4.3.2.3.2	Scenario descriptions

###### 4.3.2.3.2.1	General

The same five scenarios described in clause 4.3.2.2.2 are applicable to the illustrations shown in figure 4.3.2.3.1-1 as well.

###### 4.3.2.3.2.2	Scenario 1: MSISDN to MSISDN - case 1

MMS User Agent (A) uses the own phone number (MSISDN, GPSI (MSISDN), E.164 number) and addresses the destination (MMS User Agent (B)) also with a phone number (MSISDN, GPSI (MSISDN), E.164 number).

Figure 4.3.2.3.2.2-1: MM message from MSISDN (A) to MSISDN (B) - case 1

In this scenario, phone number A and the phone number B can be the targets in the MMS Relay/Server.

###### 4.3.2.3.2.3	Scenario 2: MSISDN to MSISDN - case 2

MMS User Agent (A) uses the own phone number (MSISDN, GPSI (MSISDN), E.164 number) and addresses the destination (MMS User Agent (B)) with an MSISDN based Email Address (shown as B@mmsc).

The MMS Relay/Server (A) adds the domain name (shown as @mmsc) to the A's identity (shown as A@mmsc) before forwarding the MM message to the External Server serving the domain mmsc.

The MMS Relay/Server (B) would remove the domain name (shown as @mmsc) at the terminating end.

Figure 4.3.2.3.2.3-1: MM message from MSISDN (A) to MSISDN (B) – case 2

In this scenario, the phone number A and the phone number B can be the targets in the MMS Relay/Server and MSISDN base Email Address A@mmsc and the MSISDN based Email Address B@mmsc can be the target non-local IDs in the MMS Relay/Server.

Note that it is the same MMS Relay/Server at the two ends of the MM message path.

###### 4.3.2.3.2.4	Scenario 3: MSISDN to generic Email Address

MMS User Agent (A) using the own phone number (MSISDN, GPSI (MSISDN), E.164 number) sends an MM message to a generic Email Address (shown as BB@email). This MM message goes from the external server (email) to the Email Client BB.

The MMS Relay/Server (A) adds the domain name (shown as @mmsc) to the A's identity (shown as A@mmsc) before forwarding the MM message to the External Server serving the domain mmsc.

Figure 4.3.2.3.2.4-1: MM message from MSISDN (A) to generic Email Address

In this scenario, the phone number A can be a target in the MMS Relay/Server. The Email Address BB@email can be a target non-local ID in the MMS Relay/Server.

###### 4.3.2.3.2.5	Scenario 4: Generic Email Address to MSISDN

Email Client (shown as AA) sends an MM message to an MSISDN based Email Address (shown as B@mmsc).

The MMS Relay/Server removes the domain name (shown as @mmsc) at the terminating end.

Figure 4.3.2.3.2.5-1: MM message from generic email Address to MSISDN

In this scenario, the phone number B can be a target in the MMS Relay/Server. The Email Address AA@email can be a target non-local ID in the MMS Relay/Server.

###### 4.3.2.3.2.6	Scenario 5: Generic Email Address to generic Email Address

Email Client AA sends an email message to Email Client BB.

Figure 4.3.2.3.2.6-1: Email message from generic email address to generic email address

MMS LI does not apply to this scenario, since the email message from AA@email to BB@email does not hit the MMS/Relay Server. Note that this case same as the one shown in figure 4.3.2.2.6-1.

***** End of the new text ****