DeletSA3#95-LI	 S3i240639

Las Vegas; October 29-November 1, 2024

Source:	Nokia,

Title:	pCR to TR 33.929-8: MMS LI – scope of MMS messages – Part VII (redirection scenarios, target is local to the redirecting MMS Relay/Server)

Document for:	Agreement

Agenda Item:	9

Work Item / Release:	LI19

Abstract of the contribution: This pCR provides input text to TR 33.929-8 LI for MMS. Scope of MMS messages reported as xIRIs for the redirecting case. The target is local to the redirecting MMS Relay/Server. In other words, the redirecting MMS User Agent is the target. The origination can be from a phone number, MSISDN based Email Address or a generic Email Address. Likewise, the redirected-to destination can be a from a phone number, MSISDN based Email Address or a generic Email Address.

BACKGROUND

The TR 33.929-8 provides the implementation guidance for the LI aspects related to the MMS. The MMS User Agent upon receiving a notification of an incoming MM message can request the MMS Relay/Server to forward (i.e. redirect) the MM message to another destination, which can be a phone number, MSISDN based Email Address or a generic Email Address.

The origination of the initial MM message can be a phone number, MSISDN based Email Address or a generic Email Address.

From a redirecting MMS Relay/Server perspective, a generic Email Address and MSISDN based Email Address or treated the same.

PROPOSAL

Incorporate the following to the TR 33.929-8.

# 4	Illustration of LI for MMS

## 4.1	General

## 4.2	MMS background with LI scope

## 4.3	Scope of target identities

## 4.4	Scope of MMS messages reported

### 4.4.1	General

### 4.4.2	3GPP, OMA and MMS LI messages

### 4.4.3	Topology views

#### 4.4.3.1	General

#### 4.4.3.2	MMBox specific operations

#### 4.4.3.3	Originating MMS domain

#### 4.4.3.4	Terminating MMS domain

***** New text ****

#### 4.4.3.5	Redirecting MMS domain

##### 4.4.3.5.1	General

###### 4.4.3.5.1.1	Scope

The MMS User Agent that is notified of an incoming MM message may forward the message to different destination (Email Client or MMS User Agent). The forwarding happens instead of the retrieval. Note that the forwarding of the MM message applies only for the deferred retrieval case.

In the subsequent descriptions, originating end refers to the originator the initial MM message and the destination refers to the destination after the forwarding (i.e. redirected-to end). The MMS User Agent who does the act of forwarding is referred to as redirecting MMS User Agent.

As far as the MMS LI is concerned, two aspects have to be addressed:

- LI for the incoming leg MM messages.

- LI for forwarded leg MM messages.

The LI for the incoming leg of MM message is similar to the MMS LI in a terminating MMS domain and the LI for the forwarded leg (i.e. redirected leg) is similar to the MMS LI in an originating MMS domain, except the messages used on the MM1 interface.

The following three cases are illustrated in the subsequent clauses:

- Redirecting MMS User Agent is a target (local to the MMS Relay/Server).

- Originating MMS User Agent is a target non-local ID at the redirecting MMS Relay/Server.

- The redirected-to (i.e. destination) MMS User Agent is a target non-local ID at the redirecting MMS/Relay/Server.

In the case of target non-local ID, the illustrations show the use of three separate MMS Relay/Servers.

Two alternatives of forwarding:

The MMS User Agent may first retrieve the MM message and then forward the retrieved MM message using the MMS origination procedures (i.e. using the m-send-req message). In this case, the MMS origination procedures as described in clause 4.4.3.3 are used.

Yet as another alternative, through a prior setup, the MMS Relay/Server could automatically, forward the incoming MM message to an MMS User Agent to a pre-setup destination. In this a user-action, at the time, when an MM message is not required.

These are not described as an alternate methods of forwarding in TS 23.140 [18] or the OMA spec [19], but mentioned here, as some deployments may have such implementations.

###### 4.4.3.5.1.2	MMBox

When the MMBox is supported, the incoming MM message is automatically stored in the MMBox, if setup and configured so, by the redirecting MMS User Agent. The status of thus stored MM message would be "New". In that case, redirecting MMS User Agent would point to the MM message stored in the MMBox in the forwarding request. Once forwarded, the status of the MM message in the MMBox is changed to "forwarded".

If the MMBox is supported, but not configured to store the incoming MM message automatically, then the MMS User Agent may request the MMS Relay/Server to store the MM message in the MMBox, when a forwarding request is sent. The status of the thus stored MM message would be "forwarded".

If the MMBox is not supported or not setup by the MMS User Agent, then MMBox related steps shown in the subsequent clauses are not applicable, and in that case, the MMS Relay/Server (B) stores the MM message locally and a reference then would be to the internally stored MM message.

###### 4.4.3.5.1.3	Delivery reports

Delivery report to the originating MMS domain

The sending of delivery report to the originating MMS domain is conditional. The delivery report is sent only if the originating MMS User Agent has requested such a report and the redirecting MMS User Agent permits the sending of such a report.

The delivery report is not sent if the origination from an Email Address.

Delivery report from the redirected-to MMS domain

A delivery report may be received from a redirected-to MMS domain if the requests for such a report and the redirected-to MMS User Agent permits the sending of the same. When the redirection happens, an intermediate delivery report is sent to the redirecting MMS User Agent, if the same is requested by that redirecting MMS User Agent.

###### 4.4.3.5.1.4	Read reports

A read report is not sent to the originating MMS domain.

A read report may be received from the redirected-to MMS domain to the redirecting MMS domain, if the redirecting MMS User Agent requests for such a report and the redirected-to MMS User Agent sends such a report.

The read report is not received, if the MMS is forwarded to an Email Address.

##### 4.4.3.5.2	Target is local to MMS Relay Server

###### 4.4.3.5.2.1	Overview

The figure 4.4.3.5.2.1-1 gives an overview of the MMS LI messages applicable to the redirecting MMS domain when the target is local to the MMS Relay/Server. The redirecting MMS domain is nothing but the terminating MMS domain except that here the terminating MMS User Agent forwards the MM message instead of retrieval or deletion.

Figure 4.4.3.5.2.1-1: MMS LI messages – redirecting MMS domain (target is local)

As shown in figure 4.4.3.5.2.1-1, the MM message may have been originated from an MMS User Agent or an Email Client. The MM message may be forwarded to an MMS User Agent or to an Email Client.

The type of MMS origination (i.e. Email Client Vs MMS User Agent) or the destination end (Email Client Vs MMS User Agent), i.e. redirected to end, has no impact on the MMS LI when the redirecting MMS User Agent is a target and local to the MMS Relay/Server.

###### 4.4.3.5.2.2	MM messages to MMS LI messages

The figure 4.4.3.5.2.2-1 illustrates the case where the MMS User Agent (B), the target, is notified and subsequently forwards the MM message.

In this illustration, the origination and the destination (i.e. redirected-to) is an MMS User Agent (identified with a phone number) served by a different MMS Relay/Server (the other MMS Relay/Server is not shown in the figure).

Figure 4.4.3.5.2.2-1: MMS LI messages from MM messages (forward)

As illustrated, the xIRIs generated a mixture of the xIRIs generated for the terminating MMS domain and originating MMS domain cases.

When the MMS User Agent (B) forwards the message, the xIRI MMSForward is sent when the m-forward-conf is sent to the MMS User Agent (B). The information from the MM message retrieved from the MMBox is used for the xIRI MMSForward message.

As shown, there are two instances where the m-delivery-ind is sent to the MMS User Agent (B):

- When the MM message is forwarded to the destination (using MM4_forward.REQ).

- When a delivery report is received from the destination (using MM4_delivery-report.REQ) .

Since the receipt of delivery report from the destination (MM4_delivery-report-REQ) is conditional, the sending of associated m-delivery-ind is also conditional. Therefore, the generation of the xIRI MMSDeliveryReport is conditional.

Since the receipt of the read-reply report from the destination (MM4_read-reply-report.REQ) is conditional, the sending of m-read-orig-ind is also conditional. Therefore, the generation of the xIRI MMSReadReport is conditional.

###### 4.4.3.5.2.3	Destination is an email address

The figure 4.4.3.5.2.3-1 gives an overview of the MMS LI messages applicable to the Redirecting MMS domain when the target is local to the MMS Relay/Server and the destination (i.e. redirected-to) is an Email Client or an MMS User Agent with an MSISDN based email address.

Figure 4.4.3.5.2.3-1: MMS LI messages from MM messages – MMS Relay/Server (B)

When the MMS User Agent (B) forwards the message, the xIRI MMSForward is sent when the m-forward-conf is sent to the MMS User Agent (B). The information from the MM message retrieved from the MMBox is used for the xIRI MMSForward message.

As shown, m-delivery-ind is sent to the MMS User Agent when the MM message (in the email format) is forwarded to the destination (using MM3: SMTP Send).

As can be inferred, there are subsequent delivery reports or a read report from the destination end.

###### 4.4.3.5.2.4	Origination is an email address

The figure 4.4.3.5.2.4-1 gives an overview of the MMS LI messages applicable to the Redirecting MMS domain when the target is local to the MMS Relay/Server and the origination is an Email Client or an MMS User Agent with an MSISDN based email address.

Figure 4.4.3.5.2.4-1: MMS LI messages from MM messages – MMS Relay/Server (B)

When the MMS User Agent (B) forwards the message, the xIRI MMSForward is sent when the m-forward-conf is sent to the MMS User Agent (B). The information from the MM message retrieved from the MMBox is used for the xIRI MMSForward message.

As shown, there are two instances where the m-delivery-ind is sent to the MMS User Agent (B):

- When the MM message is forwarded to the destination (using MM4_forward.REQ).

- When a delivery report is received from the destination (using MM4_delivery-report.REQ) .

Since the receipt of delivery report from the destination (MM4_delivery-report-REQ) is conditional, the sending of associated m-delivery-ind is also conditional. Therefore, the generation of the xIRI MMSDeliveryReport is conditional.

Since the receipt of the read-reply report from the destination (MM4_read-reply-report.REQ) is conditional, the sending of m-read-orig-ind is also conditional. Therefore, the generation of the xIRI MMSReadReport is conditional.

###### 4.4.3.5.2.5	Origination and destination are email addresses

The figure 4.4.3.5.2.5-1 gives an overview of the MMS LI messages applicable to the Redirecting MMS domain when the target is local to the MMS Relay/Server and the origination and destination are Email Clients or an MMS User Agents with an MSISDN based email addresses.

Figure 4.4.3.5.2.5-1: MMS LI messages from MM messages – MMS Relay/Server (B)

When the MMS User Agent (B) forwards the message, the xIRI MMSForward is sent when the m-forward-conf is sent to the MMS User Agent (B). The information from the MM message retrieved from the MMBox is used for the xIRI MMSForward message.

As shown, m-delivery-ind is sent to the MMS User Agent when the MM message (in the email format) is forwarded to the destination (using MM3: SMTP Send).

As can be inferred, there are subsequent delivery reports or a read report from the destination end.

***** End of the new text ****