DeletSA3#95-LI	 S3i240644

Las Vegas; October 29-November 1, 2024

Source:	Nokia,

Title:	pCR to TR 33.929-8: MMS LI – scope of MMS messages – Part XII (single MMS Relay/Server case with redirection - redirected MMS User Agent deletes)

Document for:	Agreement

Agenda Item:	9

Work Item / Release:	LI19

Abstract of the contribution: This pCR provides input text to TR 33.929-8 LI for MMS. Scope of MMS messages reported as xIRIs when only one MMS Relay Server is used and redirection happens. In the illustrations the redirected-to MMS User Agent deletes the MM message without retrieving – three illustrations, A is the target, B is the target, or C is the target.

Since only one MMS Relay/Server is used, all users are presumed to be MMS User Agents identified with a phone number.

For the scenario where the MMS User Agent even though served by the same MMS Relay/Server but addressed using an MSISDN-based Email Address is not included here since in the process of MM messages going over the MM3 interface, it is treated as if in a different MMS Relay/Server.

BACKGROUND

The TR 33.929-8 provides the implementation guidance for the LI aspects related to the MMS.

In the illustrations shown in this pCRs, originating, redirecting, and redirected MMS User Agents are served by the same MMS Relay/Server. This pCR covers the case when the redirected-to MMS User Agent deletes the MM message without a prior retrieval.

The originating MMS User Agent or redirected-to MMS User Addressed using an MSISDN based Email Address are not illustrated here because from an MMS LI perspective, messages are transferred over MM3 interface.

The cases are the same as in multiple MMS Relay/Server scenario illustrated before.

PROPOSAL

Incorporate the following to the TR 33.929-8.

# 4	Illustration of LI for MMS

## 4.1	General

## 4.2	MMS background with LI scope

## 4.3	Scope of target identities

## 4.4	Scope of MMS messages reported

### 4.4.1	General

### 4.4.2	3GPP, OMA and MMS LI messages

### 4.4.3	Topology views

#### 4.4.3.1	General

#### 4.4.3.2	MMBox specific operations

#### 4.4.3.3	Originating MMS domain

#### 4.4.3.4	Terminating MMS domain

#### 4.4.3.5	Redirecting MMS domain

#### 4.4.3.6	Single MMS Relay/Server - non-redirection case

#### 4.4.3.7	Single MMS Relay/Server - redirection case

##### 4.4.3.7.1	General

##### 4.4.3.7.2	Overview

##### 4.4.3.7.3	MM message retrieval by the redirected-to MMS User Agent

***** New text ****

##### 4.4.3.7.4	Deletion of MM message by the redirected-to MMS User Agent

###### 4.4.3.7.4.1	General

In the illustrations single MMS Relay/Server serves all the three MMS User Agents. In the illustration, the redirected-to MMS User Agent (C) upon receiving the notification deletes the MM message without a prior retrieval. Three scenarios are illustrated.

-	MMS User Agent (A) is the target.

-	MMS User Agent (B) is the target.

-	MMS User Agent (C) is the target.

Other than the status of the MM message in the read-report sent to the redirecting MMS user Agent, the MMS LI has no impact due to the deletion of the MM message by MMS User Agent (C) when MMS User Agent (A) or MMS User Agent B) are the targets.

###### 4.4.3.7.4.2	Originating MMS User Agent (A) is the target

In the figure 4.4.3.7.4.2-1, the originating MMS User Agent (A) is the target and sends an MM message to MMS User Agent (B) who in turn forwards the same to the MM User Agent (C). MMS User Agent (C) deletes the MM message without a prior retrieval. In this illustration, MMS user Agent (A) is the target.

Figure 4.4.3.7.4.2-1: Single MMS Relay/Server – MMS User Agent (A) is the target (delete)

When MM message is sent by the originating MMS User Agent, if requested so, a copy of the MM message is stored in the MMBox. Therefore, the storing of the MM message at the MMBox does not happen if the store indication is not sent in the m-send-req message.

Even though the trigger for the generation of xIRI MMSSend is the m-send-conf message that the MMS-Relay/Server sends to the MMS User Agent (A), the xIRI includes information from the m-send-req as well.

Since the sending of the m-delivery-ind (forwarded) is conditional, the generation of xIRIs MMSDeliveryReport is also conditional.

The deletion of MM message before retrieval has no impact on the MMS LI when MMS User Agent (A) is the target.

###### 4.4.3.7.4.3	Redirecting MMS User Agent (B) is the target

In the figure 4.4.3.7.4.3-1, the originating MMS User Agent (A) is the target and sends an MM message to MMS User Agent (B) who in turn forwards the same to the MM User Agent (C). MMS User Agent (C) deletes the MM message without a prior retrieval. In this illustration, the MMS User Agent (B) is the target.

Figure 4.4.3.7.4.3-1: Single MMS Relay/Server – MMS User Agent (B) is the target (delete)

When MM message is sent by the originating MMS User Agent, if requested so, a copy of the MM message is stored in the MMBox. Therefore, the storing of the MM message at the MMBox does not happen if the store indication is not sent in the m-send-req message.

Even though the trigger for the generation of xIRI MMSForward is the m-forward-conf message that the MMS-Relay/Server sends to the MMS User Agent (B), the xIRI includes information from retrieved MM message as well

If a read report is requested by the redirecting MMS User Agent (B), then the MMS User Agent (C) may send a m-read-rec-ind with "deleted without being read". The xIRI MMSReadReport is generated when the MMS Relay/Server sends the m-read-orig-ind message to the MMS User Agent (B), the target.

Since the reception of m-read-rec-ind from MMS User Agent (C) is conditional, the sending of m-read-orig-ind to MMS User Agent (B) is also conditional. Therefore, the generation of MMSReadReport is also conditional.

###### 4.4.3.7.4.4	Redirected-to MMS User Agent (C) is the target

In the figure 4.4.3.7.4.4-1, the originating MMS User Agent (A) is the target and sends an MM message to MMS User Agent (B) who in turn forwards the same to the MM User Agent (C). MMS User Agent (C) deletes the MM message without a prior retrieval. In this illustration, the MMS User Agent (C) is the target.

Figure 4.4.3.7.4.4-1: Single MMS Relay/Server – MMS User Agent (C) is the target (delete)

When MM message is sent by the originating MMS User Agent, if requested so, a copy of the MM message is stored in the MMBox. Therefore, the storing of the MM message at the MMBox does not happen if the store indication is not sent in the m-send-req message.

The MMSDeliveryAck is sent when the whe MMS Relay/Server sends the notifyresp-ind to the target MMS User Agent (C).

If a read report is requested by the redirecting MMS User Agent (B), then the MMS User Agent (C) may send a m-read-rec-ind with "deleted without being read". The xIRI MMSReadReport is generated when the MMS Relay/Server receives the m-read-rec-ind message.

Since the reception of m-read-rec-ind from MMS User Agent (C) is conditional, the generation of MMSReadReport is also conditional.

***** End of the new text ****