SA3#95-LI	 S3i240672

Las Vegas; October 29-November 1, 2024

Source:	Nokia

Title:	ID Association Caching – discussion around the GPSI

Document for:	Agreement

Agenda Item:	9

Work Item / Release:	LI19

Abstract of the contribution: This document discusses a few cases around the inclusion of GPSI in the LI_HILR response from ICF to LEA.

Recap

The ID Association Caching capability, in its initial days,  was allowing the LEAs to request and get the SUPI associated with a 5G-GUTI (or SUCI) and vice versa. Later, the capability was enhanced to include the PEI (when known), and recently, to include the GPSI (when known).

The concept used for the ID Association Caching is based on a specific function referred to as IEF in the AMF which provides the associations among different IDs whenever a new 5G-GUTI is assigned to the UE. The IEF sends the association records (known as IEF Association Record) to another specific function referred to as ICF.

A diagram taken from TR 33.929-5 depicting the concept is shown below

(since this is meant to be recap, details are presumed to be self-explanatory):

Figure 1: From 3GPP TR 33.929 (figure 4.3.4.1-1)

As per the initial version of the ID Association Caching, the following information was mandatory in the IEF Association Record:

| 5G-GUTI | SUPI | Time-stamp | TAI | N-CGI | N-CGI Time |
| --- | --- | --- | --- | --- | --- |

And the following information are optional for inclusion in an IEF Association Record message:

| SUCI | PEI | 5G TAI List |
| --- | --- | --- |

Few of the caching scenarios in support of the above are illustrated in the following diagram taken from TR 33.929-5:

Figure 2: Few of the caching scenarios from TR 33.929-5 (figure 4.3.6.1-1)

Initially, when the UE registers with the AMF with a SUCI, the AMF gets the SUPI from the UDM for that SUCI and then the AMF gets the PEI from the UE. The IEF that way is able to provide the PEI, SUPI, SUCI and the 5G-GUTI among other things to the ICF.

When a new 5G-GUTI is assigned (which may be for another registration request from the UE but using a 5G-GUTI, not SUCI), the AMF does not request (and get) the PEI from the UE and accordingly, the IEF Association Record does not include the PEI. However, the new record in the ICF would include the PEI as well copied from the previous record. The assumption here is that the PEI remains the same. The registration request would have used the SUCI instead of 5G-GUTI for a new PEI.

Now, CR (S3i210878, SA3LI#83-e-b, 1-5 November of 2021) that introduced the GPSI to the ICF records did so with the following changes:

Where an IEFAssociationRecord contains a PEI, GPSI or a TAI list, the ICF shall store the received values and associate them both the current received SUPI to 5G-GUTI association and any future association until a subsequent IEFAssociationRecord is received which updates the PEI, GPSI or TAI list values.

Also, there is a NOTE (see below) next to the table that provides the IEF Association Record payload:

NOTE 1:	Shall be provided in first association record to ICF after PEI or GPSI is available and following any change of PEI or GPSI.

Basically, the CR added the GPSI to the IEF Association Record with a NOTE saying that the GPSI shall be provided first time after it is available and following any change of GPSI (the text also mentions PEI, but left out of this discussion).

The figure 2 (of this contribution) modified to include GPSI will look as shown below:

Figure 3: Few of the caching scenarios from TR 33.929-5 (figure 4.3.6.1-1), modified to include GPSI

It is assumed that the AMF would get the Access and Mobility (AM) Subscription Data from the UDM which would include the GPSI among other AM subscription data. It is also assumed that the AMF would subscribe to UDM for any AM Data subscription data change notifications.

Discussion

The discussion focusses on the GPSI part.

Another diagram of TR 33.929-5 is modified to show the flow with GPSI included:

Figure 4: Another TR 33.929-5 diagram (redrawn 4.5.4.2-1), is modified to include GPSI

As in figure 3, the figure 4 presumes that during the registration, the AMF gets the AM subscription data for the UE which among other things includes the GPSI (shown as GPSI-1). Accordingly, the IEF Association Record and ICF would contain the GPSI-1 in the cached record.

The figure 4 also presumes that the AMF subscribes to the AM subscriber data change notifications.

At time T1, the GPSI for the SUPI-1 is changed to GPSI-2.

NOTE:	Occurrence of such an event can be a rarity, never-the-less, the illustration shows a possible consequence of such a change on ID Association Caching, if such a change ever happens.

Another IEF Association Record, however, is sent to the ICF only when the next time a new 5G-GUTI is assigned to the UE. Therefore, any queries from LEAs (shown as (1)) before the 5G-GUTI is changed again would get them the GPSI-1 even though the GPSI-2 is associated with the UE at the observed time indicated by the LEA in the request.

So, a query could give an incorrect GPSI to the LEAs.

Discussion points

Should this be fixed in the sense make changes to ensure the LEAs would get the current GPSI to their queries?

OR

Should the TS be amended to have a NOTE to indicate that the GPSI returned in response to a query may not be the current GPSI associated with the UE?

If (in case) the AMF does not subscribe to the AM subscriber data change notifications, then the GPSI will remain out of sync.

NOTE:	In most deployments, the AMF subscribes to AM subscriber data change notifications, however, since such a subscription is mandatory according to the standards text, at least considerations will have to be made for the case where AMF does not subscribe.

Feasible solutions

Method 1:

When the AMF receives the AM subscriber data change notifications, the IEF could send a new Association Record to the ICF that would contain the new GPSI-2. But, unlike other situations, the record is updated rather than creating a new record within the ICF since the 5G-GUTI is not changed. The concept may have to be modified for the ICF to de-cache the previous record if and only if the 5G-GUTI is changed.

This method is illustrated in figure 5 below:

Figure 5: A feasible solution (method 1) – new IEF Association Record

The query from LEA (shown as (1)) would get them the GPSI-2 among other things in the response.

Method 2:

ICF could subscribe to the subscriber data notifications for the SUPI. Such notifications are done for all UEs, perhaps, detectability concern can be dismissed.

The figure 6 represents method.

Figure 6: Feasible solution (method 2) - ICF subscribing  to AM Subscriber Data change notification

The query from LEA (shown as (1)) would get them the GPSI-2 among other things in the response.

In this method, since such a subscription results in notifications for all AM data changes, too many notification events from UDM to ICF may occur. That is a negative consequence of this method even though the ID association caching can be happy.

Method 3:

Having an IEF-sort of function in the UDM that would update the record in the ICF whenever the GPSI associated with the AM subscriber data changes.

The figure 7 represents idea.

Figure 7: Feasible solution (method 3) – IEF in UDM

Summary and recommendations

Without having answers to the issues raised in this contribution, TR 33.929-5 with changes to accommodate GPSI in the record and HIQR responses will fail to provide the appropriate guidance with the understanding that a guidance that says "do not trust what you get" will not be an acceptable guidance.

So, this contribution is seeking guidance on the changes to made to TR 33.929-5.

Recommend making changes to the normative LI specifications in support of feasible solution (method 1).  If this is agreed, no further guidance is needed to update the TR 33.929-5.