3GPP TSG-SA3 Meeting #95-LI	s3i240611

# Las Vegas, US, 29th Oct 2024 – 1st Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0674 | rev |  | Current version: | 19.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding TLS 1.3 IRI for AKMA LI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (NDRE) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI19 |  |  |  |  |  | Date: |  |  | 2024-10-11 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The TS 33.128 attachments include ASN.1 specifications of IRI applicable to interception of TLS 1.2 encrypted traffic when the encryption is being enabled by the CSP's AKMA service (TS 33.535). Since TLS 1.2 is starting to get replaced by TLS 1.3 (which is much more complex in terms of the needed IRIs), there is a need for adding ASN.1 details also for TLS 1.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds ASN.1 details of TLS 1.3 IRI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Risk of interoperabiliy problems or complete lack of interception capability for TLS 1.3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Attachments TS33128Payloads.asn |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is associated with the following changes in the Forge: Merge request: https://forge.3gpp.org/rep/sa3/li/-/merge_requests/276 Commit hash: https://forge.3gpp.org/rep/sa3/li/-/commit/ded2980b3a3cee766d29e6efd91eebca12772231. |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** START OF CHANGES (ATTACHMENTS) ****

START OF CHANGE 1

---a/33128/r19/TS33128Payloads.asn
+++b/33128/r19/TS33128Payloads.asn

@@ -1245,7 +1245,8 @@ AKMAAFID ::= SEQUENCE

1245	1245		UAStarParams ::= CHOICE

1246	1246		{

1247	1247		    tls12                 [1] TLS12UAStarParams,

1248		-	    generic               [2] GenericUAStarParams

1248	+	    generic               [2] GenericUAStarParams,

1249	+	    tls13                 [3] TLS13UAStarParams

1249	1250		}

1250	1251

1251	1252		GenericUAStarParams ::= SEQUENCE

@@ -1301,6 +1302,264 @@ TLS12UAStarParams ::= SEQUENCE

1301	1302		    tLSClientExtensions   [19] OCTET STRING (SIZE(0..65535)) OPTIONAL

1302	1303		}

1303	1304

1305	+	-- ==================================================

1306	+	-- Specific UaStarParams for TLS 1.3 (RFC8446)

1307	+	-- Inline references below are to clauses of RFC8446.

1308	+	-- ==================================================

1309	+

1310	+	-- RFC8846, clause 4.2: Available handshake extension types.

1311	+

1312	+	TLS13ExtensionType ::= ENUMERATED

1313	+	{

1314	+	        serverName(1),

1315	+	        maxFragmentLength(2),

1316	+	        statusRequest(3),

1317	+	        supportedGroups(4),

1318	+	        signatureAlgorithms(5),

1319	+	        useSRTP(6),

1320	+	        heartbeat(7),

1321	+	        applicationLayerProtocolNegotiation(8),

1322	+	        signedCertificateTimestamp(9),

1323	+	        clientCertificateType(10),

1324	+	        serverCertificateType(11),

1325	+	        padding(12),

1326	+	        presharedKey(13),

1327	+	        earlyData(14),

1328	+	        supportedVersions(15),

1329	+	        cookie(16),

1330	+	        pSKKeyExchangeModes(17),

1331	+	        certificateAuthorities(18),

1332	+	        oIDFilters(19),

1333	+	        postHandshakeAuth(20),

1334	+	        signatureAlgorithmsCert(21),

1335	+	        keyShare(22)

1336	+	}

1337	+

1338	+	-- RFC8846, clause 4.2: Extensions.

1339	+	-- Carried as extension to the client's and server's

1340	+	-- Hello, or, in the server's EncryptedExtensions message.

1341	+	TLS13Extension ::= SEQUENCE

1342	+	{

1343	+	    extensionType [1] TLS13ExtensionType,

1344	+	    extensionData [2] OCTET STRING (SIZE(0..65535))

1345	+	}

1346	+

1347	+

1348	+	-- RFC8446, clauses 4.1, 4.2, 7.1, and B.4: Cryptographic transforms

1349	+

1350	+	-- RFC8446, clauses 4.1.1 and 4.1.2: Ciphersuite

1351	+	TLS13CipherSuite ::= CHOICE

1352	+	{

1353	+	    rfc8446               [1] TLSCipherSuite

1354	+	    -- Extensible for possible future purposes

1355	+	}

1356	+

1357	+	-- RFC8446, clause 7.1 and B.4: Key derivation function.

1358	+	TLS13KDFAlgorithm ::= ENUMERATED

1359	+	{

1360	+	    rfc5869(1)     -- HKDF

1361	+	}

1362	+

1363	+	-- RFC8846, clause 4.2.11 and B.4: Hash algorithm.

1364	+	TLS13PSKHashAlgorithm ::= ENUMERATED

1365	+	{

1366	+	    sha256(1),

1367	+	    sha384(2),

1368	+	    sha512(3)

1369	+	}

1370	+

1371	+

1372	+	-- RFC8446, clause 4.4.2: Certificates.

1373	+

1374	+	TLS13CertificateType ::= ENUMERATED

1375	+	{

1376	+	    x509(1),

1377	+	    rawPublicKey(2)

1378	+	}

1379	+

1380	+	TLS13CerificateEntry ::= SEQUENCE

1381	+	{

1382	+	    tLSCertificateType [1] TLS13CertificateType,

1383	+	    tLSCertificateData [2] OCTET STRING (SIZE(1..16777215)),

1384	+	    extensions         [3] SEQUENCE OF TLS13Extension OPTIONAL

1385	+	}

1386	+

1387	+	-- This will typically correspond to a certificate chain.

1388	+	TLS13Certificate ::= SEQUENCE

1389	+	{

1390	+	    tLSCertificateRequestContext [1] OCTET STRING (SIZE(0..255)),

1391	+	    tLSCertificateList           [2] SEQUENCE OF TLS13CerificateEntry

1392	+	}

1393	+

1394	+

1395	+	-- RFC 8446, clause 4.2.9: Preshared key exchange mode.

1396	+	TLS13PSKKeyExchangeMode ::= ENUMERATED

1397	+	{

1398	+	    pSKKE(1),

1399	+	    pSKDHEKE(2)

1400	+	}

1401	+

1402	+	-- RFC8446, clause 4.2.11: Preshared keys (PSKs).

1403	+	-- PSKs obtained from AKMA use a specific prefix as part

1404	+	-- of the identity: 3GPP-AKMA, see TS 33.535 and 33.222.

1405	+	TLS13PSKIdentity ::= SEQUENCE

1406	+	{

1407	+	    identity            [1] OCTET STRING (SIZE(1..65535)),

1408	+	    obfuscatedTicketAge [2] INTEGER (0..4294967295)

1409	+	}

1410	+

1411	+	TLS13OfferedPSK ::= SEQUENCE

1412	+	{

1413	+	    identity [1] TLS13PSKIdentity,

1414	+	    binder   [2] OCTET STRING (SIZE(32..255))

1415	+	}

1416	+

1417	+	-- Actual TLS PSK, either explicitly provisioned or derived from

1418	+	-- an earlier issued TLS ticket and a resumptionMasterSecret.

1419	+	-- For AKMA, the PSK is the AKMA key K_AF.

1420	+	TLS13PSKInfo ::= SEQUENCE

1421	+	{

1422	+	    pSKIdentity        [1] TLS13PSKIdentity,

1423	+	    pSKValue           [2] OCTET STRING,

1424	+	    pSKKeyExchangeMode [3] TLS13PSKKeyExchangeMode,

1425	+	    hashAlgorithm      [4] TLS13PSKHashAlgorithm

1426	+	}

1427	+

1428	+

1429	+

1430	+	-- RFC 8446, clause 4.6.1: Tickets/resumption info.

1431	+

1432	+	TLS13NewSessionTicket ::= SEQUENCE

1433	+	{

1434	+	    ticketLifeTime [1] INTEGER (0..4294967295),

1435	+	    ticketAgeAdd   [2] INTEGER (0..4294967295),

1436	+	    ticketNonce    [3] OCTET STRING (SIZE(0..255)),

1437	+	    ticket         [4] OCTET STRING (SIZE(1..65535)),

1438	+	    extensions     [5] SEQUENCE OF TLS13Extension OPTIONAL

1439	+	}

1440	+

1441	+

1442	+

1443	+	-- RFC8446, clause 7.1 and 4.2.8: TLS-internal keys

1444	+	-- These keys/secrets are never directly used to encrypt

1445	+	-- data, and are therefore optional to provide as part of

1446	+	-- TLS13UAStarParams.

1447	+

1448	+	-- RFC8446, clause 4.2.8

1449	+	-- Shared Diffie-Hellman secret: derived from client's

1450	+	-- and server's key shares (as included in extensions).

1451	+	-- Can later be extended by adding PQC-based secrets,

1452	+	-- see e.g. draft-ietf-tls-hybrid-design-10.

1453	+	TLS13EstablishedSecrets ::= SEQUENCE

1454	+	{

1455	+	    dHE           [1] OCTET STRING (SIZE(1..65535)) OPTIONAL

1456	+	}

1457	+

1458	+

1459	+	-- RFC8446, clause 7.1

1460	+	-- The actual encryption keys are derived from the respective secret

1461	+	-- values below and the provided hashed handshake-message-transcript.

1462	+	TLS13EarlySecretInfo ::= SEQUENCE

1463	+	{

1464	+	    earlySecretValue [1] OCTET STRING (SIZE(1..65535)),

1465	+	    transcriptHash   [2] OCTET STRING (SIZE(1..65535))

1466	+	    -- Hash(ClientHello)

1467	+	}

1468	+

1469	+	TLS13HandshakeSecretInfo ::= SEQUENCE

1470	+	{

1471	+	    handshakeSecretValue [1] OCTET STRING (SIZE(1..65535)),

1472	+	    transcriptHash       [2] OCTET STRING (SIZE(1..65535))

1473	+	    -- Hash(ClientHello..ServerHello)

1474	+	}

1475	+

1476	+	TLS13MasterSecretInfo ::= SEQUENCE

1477	+	{

1478	+	    masterSecretValue [1] OCTET STRING (SIZE(1..65535)),

1479	+	    transcriptHashS   [2] OCTET STRING (SIZE(1..65535)),

1480	+	    -- Hash(ClientHello..server Finished)

1481	+	    transcriptHashC   [3] OCTET STRING (SIZE(1..65535))

1482	+	    -- Hash(ClientHello..client Finished)

1483	+	}

1484	+

1485	+

1486	+	-- Complete TLS 1.3 cryptographic context.

1487	+

1488	+	TLS13UAStarParams ::= SEQUENCE

1489	+	{

1490	+	    -- Basic cryptographic information.

1491	+	    --

1492	+	    -- The offeredPSKs, offeredPSKModes and selectedPSK

1493	+	    -- parameters are also provided as part of client and

1494	+	    -- server extensions below.

1495	+	    --

1496	+	    -- PSKs and key exchange modes offered by the client, see

1497	+	    -- clause 4.2.9, 4.2.11. When AKMA (or other CSP-provided)

1498	+	    -- keys are used, at least one offered PSK and key exchange mode

1499	+	    -- will always be present in the client extensions

1500	+	    offeredPSKs                      [1] SEQUENCE OF TLS13OfferedPSK,

1501	+	    offeredPSKKeyExchangeModes       [2] SEQUENCE OF TLS13PSKKeyExchangeMode,

1502	+	    -- The server-selected PSK, clause 4.2.11

1503	+	    -- selectedPSKIdentity = j means the server selects j:th offered PSK.

1504	+	    selectedPSKIdentity              [3] INTEGER (0..65535),

1505	+	    -- The actual TLS PSK. When AKMA is used, this key will typically also

1506	+	    -- be provided as an associated AFKeyInfo IRI, see below.

1507	+	    selectedPSK                      [4] TLS13PSKInfo,

1508	+	    -- Cryptographic state: transforms and sequence numbers.

1509	+	    kDFAlgorithm                     [5] TLS13KDFAlgorithm, -- clause 7.1.

1510	+	    clientSupportedCipherSuites      [6] SEQUENCE OF TLS13CipherSuite OPTIONAL,

1511	+	    selectedCipherSuite              [7] TLS13CipherSuite, -- clause 4.2 and B.4.

1512	+	    clientSequenceNumber             [8] INTEGER, -- clause 5.3

1513	+	    serverSequenceNumber             [9] INTEGER, -- clause 5.3

1514	+	    -- Handshake parameters, clause 4.1.2 and 4.1.3.

1515	+	    clientRandom                    [10] OCTET STRING (SIZE(32)),

1516	+	    serverRandom                    [11] OCTET STRING (SIZE(32)),

1517	+	    legacySessionID                 [12] OCTET STRING (SIZE(0..32)),

1518	+	    -- Complete contents of handshake extensions, clause 4.2.

1519	+	    tLSClientHelloExtensions        [13] SEQUENCE OF TLS13Extension,

1520	+	    tLSServerHelloExtensions        [14] SEQUENCE OF TLS13Extension,

1521	+	    -- Encrypted server extensions, clause 4.3.1 (here provided in plaintext).

1522	+	    tLSPlaintextEncryptedExtensions [15] SEQUENCE OF TLS13Extension OPTIONAL,

1523	+	    -- Authentication related parameters, clause 4.4

1524	+	    -- Certificates are encrypted between client and server,

1525	+	    -- these are the unencrypted certificate(s).

1526	+	    tLSPlaintextClientCertificate   [16] TLS13Certificate OPTIONAL,

1527	+	    tLSPlaintextServerCertificate   [17] TLS13Certificate OPTIONAL,

1528	+	    -- Session key information fields, clause 7.1

1529	+	    earlySecret                     [18] TLS13EarlySecretInfo OPTIONAL,

1530	+	    binderKey                       [19] OCTET STRING (SIZE(1..65535)) OPTIONAL,

1531	+	    -- The key used by the cipher suite to encrypt early data is

1532	+	    -- derived from clientEarlyTrafficSecret as described in clause 7.3.

1533	+	    clientEarlyTrafficSecret        [20] OCTET STRING (SIZE(1..65535)),

1534	+	    earlyExporterMasterSecret       [21] OCTET STRING (SIZE(1..65535)),

1535	+	    -- Established shared key (from Diffie-Hellman or similar).

1536	+	    sharedKey                       [22] TLS13EstablishedSecrets OPTIONAL,

1537	+	    handshakeSecret                 [23] TLS13HandshakeSecretInfo OPTIONAL,

1538	+	    -- The keys used by the cipher suite to encrypt the handshake are

1539	+	    -- derived from client- and serverHandshakeTrafficSecret as

1540	+	    -- described in clause 7.3.

1541	+	    clientHandshakeTrafficSecret    [24] OCTET STRING (SIZE(1..65535)),

1542	+	    serverHandshakeTrafficSecret    [25] OCTET STRING (SIZE(1..65535)),

1543	+	    masterSecret                    [26] TLS13MasterSecretInfo OPTIONAL,

1544	+	    -- The client and server application traffic secrets

1545	+	    -- can change value during an ongoing session due to

1546	+	    -- KeyUdpateRequest messages. The values to be provided below

1547	+	    -- refer to the currently valid ones when the IRI message is

1548	+	    -- generated.

1549	+	    -- The keys used by the cipher suite to encrypt the application

1550	+	    -- traffic are derived from client- and serverApplicationTrafficSecret

1551	+	    -- as described in clause 7.3.

1552	+	    clientApplicationTrafficSecret  [27] OCTET STRING (SIZE(1..65535)),

1553	+	    serverApplicationTrafficSecret  [28] OCTET STRING (SIZE(1..65535)),

1554	+	    -- Other session-specific keys.

1555	+	    exporterMasterSecret            [29] OCTET STRING (SIZE(1..65535)),

1556	+	    resumptionMasterSecret          [30] OCTET STRING (SIZE(1..65535)),

1557	+	    -- Tickets for resumption (issued during current session), clause 4.6.2.

1558	+	    tLSResumptionTickets            [31] SEQUENCE OF TLS13NewSessionTicket OPTIONAL

1559	+	}

1560	+

1561	+

1562	+

1304	1563		KAF ::= OCTET STRING

1305	1564

1306	1565		KAKMA ::= OCTET STRING

END OF CHANGE 1

**** END OF ALL CHANGES ****