3GPP SA3#95-LI	s3i240613

# 29 October – 01 November 2024, Las Vegas (US)

Title:	Reply LS on UE-Satellite-UE Communication Architectures

Response to:	LS (S2-2409442) on "Reply LS on KI#3 UE-Satellite-UE Communication Architectures"

Release:	Release 19

Work Item:	5GSAT_Ph3_ARCH

Source:	SA3-LI

To:	SA2

Cc:	SA3

Contact Person:

#### Name:	Nagaraja Rao

E-mail Address:	nagaraja.rao@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA3-LI thanks SA2 for their LS and the clarification in pursuing the UE-Satellite-UE communication architectures.

Point number 1

In reference to point number 1 (where the IMS-AGW would be on-board the satellite), SA3-LI would like to inform that if such an architecture is pursued, then the LI specifications for IMS LI will have to be updated to accommodate the possibility of a change in the IMS-AGW during an active IMS Session.

Within the IMS LI, the media interception is performed by an LI specific function (known as CC-POI) within the IMS-AGW triggered by an LI specific function (known as CC-TF) present in the P-CSCF. If the IMS-AGW changes mid-session, then the CC-TF will have to trigger the CC-POI present in the new IMS-AGW to continue with the media interception.

SA3 LI believes that such a trigger can be sent as long as P-CSCF is aware of the IMS-AGW change in real time. SA3-LI further believes no LI specific considerations have to be taken for this architecture by SA2.

Point number 2

In reference to point number 2 (where an intermediate UPF (I-UPF) is inserted to the media path with no IMS-AGW for a UE-Satellite-UE communication) the LI specifications for IMS LI will have to be enhanced to support an LI specific interface (known as LI_T3) from the CC-TF in P-CSCF to the CC-POI in the I-UPF onboard the satellite. In addition, the trigger should be able to help the CC-POI in the I-UPF to identify the packets that need to be intercepted. SA3-LI believes to support such an architecture, the following non-LI related changes will have to be made for all IMS sessions that require the media interception to be done at such an I-UPF:

- P-CSCF is able to identify the I-UPF that has the CC-POI. For this I-UPF address (e.g. IP address) should be made available to the P-CSCF.

- P-CSCF is able to identify the tunnel used for the IMS media flow. For this, the tunnel ID used at the I-UPF should be made available to the P-CSCF. If a point-to-point tunnel is used (e.g. on N6), then the UDP port number along with the IP address of the I-UPF shall be made available to the P-CSCF.

- P-CSCF is able to identify the UDP port number (RTP) used for the media packets. Port numbers from the access side (N3) and the core-side (N6) should be available.

- P-CSCF is able to identify the  QoS flow. For this, the QFI associated with QoS flow should be made available to the P-CSCF.

SA3-LI believes that non-LI related enhancements may have to be made to the RX (P-CSCF – PCF) and N7 (PCF – SMF) interfaces, and to the SMF and the PCF to provide the above information to the P-CSCF. If a separate tunnel is used for each QoS flow, then tunnel ID along with its associated interface indication (N6 Vs N3) may be sufficient.

In addition, SA3-LI would like to point out that scenarios such as multi-party calls, conferencing, call forwarding may have further implications with I-UPF based media interception.

SA3-LI believes that the following depict the UE-Satellite-UE communication architecture where the media interception would be made at the I-UPF.

Signaling only (before the SDP offer and answer):

The blue colored boxes indicate the LI related functions are present in the respective NFs. The LI capabilities are defined in TS 33.127 and TS 33.128. In the diagram, both UE (A) and UE (B) are the targets of interception.

Media interception (after the SDP offer and answer)

The blue colored boxes indicate the LI related functions are present in the respective NFs. Non-LI related enhancements will have to be made to N7, Rx, SMF, PCF,  and LI specific enhancements will have to be made to the P-CSCF to support the media interception at the I-UPF. In the diagram, both UE (A) and UE (B) are the targets of interception.

2. Actions:

To SA2:

ACTION: 	SA3 kindly asks SA2 to:

-	In reference to point 1, to inform SA3-LI that P-CSCF will be able to determine as and when the IMS-AGW is changed during a mid-session irrespective of LI.

- 	In reference to point 2, to inform SA3-LI that the suggested changes to RX, N7, SMF and PCF can be done for all IMS sessions irrespective LI.

3. Date of Next TSG-SA WG2 Meetings:

3GPP SA3#96-LI			28 - 31 January 2025			Sophia Antipolis, FR