SA3#95-LI	 S3i240620

Las Vegas; October 29-November 1, 2024

Source:	Nokia

Title:	Overview of MMS documents

Document for:	Agreement

Agenda Item:	9

Work Item / Release:	LI19

Abstract of the contribution: This document is for information and provides an overview of all the documents related to MMS LI submitted to this meeting.

RECAP

The pCRs generated make use of the previous analysis discussed in SA3-LI (S3i200217). The following generic diagram that depicts the MMS architecture is taken from TS 23.140:

Figure 1: From 3GPP TS 23.140

The TS 33.128 uses the name for the server: MMS Proxy-Relay. The pCRs (and TR 33.929-8) will use the name MMS Relay/Server as in TS 23.140.

Among the interfaces shown in figure 1, only MM1 and MM4 are referenced in TS 33.128 (note: to some extent MM3 may be referenced soon).

TS 33.128 uses the MM1 messages (with message names and other details from the OMA specification) to generate the xIRIs when the target ID is local to the MMS Relay/Server and MM4 messages (as defined in TS 23.140) to generate the xIRIs for the target non-local ID.

The table below compares the MM1/MM4 messages (TS 23.140) and MMSM messages (OMA) and the TS 33.128.

| MM1/MM4 messages (TS 23.140) | MMSM messages (OMA) | TS 33.128 references |
| --- | --- | --- |
| MM1_submit.REQ | m-send-req | m-send-req |
| MM1_submit.RES | m-send-conf | m-send-conf |
| MM1_notificaiton.REQ | m-notification-ind | m-notification-ind |
| MM1_notification.RES | m-notifyresp-ind | m-notifyresp-ind |
| MM1_retrieve.REQ | WEP/HTTP GET | WEP/HTTP GET |
| MM1_retrieve.RES | m-retrieve-conf | m-retrieve-conf |
| MM1_acknowledge.REQ | m-notifyresp.ind | m-notifyresp.ind |
| MM1_acknowledge.REQ | m-acknowledge-ind | m-acknowledge-ind |
| MM1_delivery_report.REQ | m-delivery-ind | m-delivery-ind |
| MM1_read_reply_receipient.REQ | m-read-rec-ind | m-read-rec-ind |
| MM1_read_reply_originator.REQ | m-read-orig-ind | m-read-orig-ind |
| MM1_forward.REQ | m-forward-ind | m-forward-ind |
| MM1_forward.RES | m-forward-conf | m-forward-conf |
| MM1_delete.REQ | m-delete-req | m-delete-req |
| MM1_delete.RES | m-delete-conf | m-delete-conf |
| MM1_cancel.REQ | m-cancel-req | m-cancel-req |
| MM1_cancel.RES | m-cancel-conf | m-cancel-conf |
| MM1_mmbox_store.REQ | m-mbox-store-req | m-mbox-store-req |
| MM1_mmbox_store.RES | m-mbox-store-conf | m-mbox-store-conf |
| MM1_mmbox_view.REQ | m-mbox-view-req | m-mbox-view-req |
| MM1_mmbox_view.RES | m-mbox-view-conf | m-mbox-view-conf |
| MM1_mmbox_upload.REQ | m-mbox-upload-req | m-mbox-upload-req |
| MM1_mmbox_upload.RES | m-mbox-upload-conf | m-mbox-upload-conf |
| MM1_mmbox_delete.REQ | m-mbox-delete-req | m-mbox-delete-req |
| MM1_mmbox_delete.RES | m-mbox-delete-conf | m-mbox-delete-conf |
| MM1_deleivery_report.REQ | m-delivery-ind | m-delivery-ind |
| MM1_read_reply_receipient.REQ | m-read-rec-ind | m-read-rec-ind |
| MM1_read_reply_originator.REQ | m-read-orig-ind | m-read-orig-ind |
| MM1_forward.REQ | m-forward-req | m-forward-req |
| MM1_forward.RES | m-forward-conf | m-forward-conf |
| MM4_forward.REQ | n/a | MM4_forward.REQ |
| MM4_forward.RES | n/a | MM4_forward.RES |
| MM4_delivery_report.REQ | n/a | MM4_delivery_report.REQ |
| MM4_delivery_report.RES | n/a | MM4_delivery_report.RES |
| MM4_read_reply_report.REQ | n/a | MM4_read_reply_report.REQ |
| MM4_read_reply_report.RES | n/a | MM4_read_reply_report.RES |

As discussed during SA3#92-LI (see the diagram below taken from S3i240056), the messages to or from the Email Address are not transferred over the MM4 interface.

Figure 1: diagram from S3i240056

During SA3#92-LI, it was agreed to include the Email Address as a target non-local ID. The TR 33.928 was updated (SA3#93-LI, S3i240280) to include Email Address as one of the target identifiers for MMS Proxy/Relay and an action was taken by OTD to draft the changes to the TS 33.128 for the same.

Figure 2: From TR 33.928

Even though the changes to the TS 33.128 are not made yet, the pCRs are generated to the TR 33.929-8 based on that understanding.

The pCRs do not talk much about the media interception.

Most of the contents for the pCRs are driven from a series of presentations made during the SA3 LI meetings held in 2020 (s3i200217 was the last among them) while the stage 3 text for MMS LI were incorporated.

Into the TR 33.929-8

To help the illustrations, a few terms and concepts are introduced (some of them are obvious).

- MMS User Agent.

- Email Client.

- MM Message.

- Phone number (any of MSISDN, GPSI (MSISDN, E.164 Number).

- Generic Email Address.

- MSISDN-based Email Address.

The application within MMS client's device is referred to as:

- MMS User Agent when the MM message originated from, or received at with, a phone number (e.g. MSISDN) as the identity.

- Email Client when the MM message originated from, or received at with, generic email address as the identity.

The MSISDN based Email Address is a specific case where the MSISDN (i.e. any of E.164 number, GPSI (MSISDN), or MSISDN) of the remote MMS User Agent is in the user-id part of that Email Address. The MSISDN based Email Address is used to address the remote MMS User Agent in an email format and allows an Email Client to send an MM message to an MMS User Agent and vice-versa. The domain name of the MSISDN based Email Address may belong to the CSP and may be helpful in finding the MMS Relay/Server that serves the MMS User Agent.

Overview of the pCRs:

- References and Abbreviations.

References to TS 24.140 and OMA spec are added.

Unused abbreviations are removed.

Background and LI scope

- MMS background and LI scope – Part I

The first general clause for the TR and then with some background information on MMS functional elements and two generic flow-diagrams. Nothing to do with LI yet.

- MMS background and LI scope – Part II

Complements the part I with a high level view of LI scope.

Scope of target identifiers

- Scope of target identifiers – Part I (without redirection, multiple MMS Relay/Servers)

The target identities can be a phone number or an email address. Several permutations are possible when messages are exchanged between two clients represented by those identities.

- Scope of target identifiers – Part II (without redirection, single MMS Relay/Server)

The MMS User Agents are served by the same MMS Relay/Server.

- Scope of target identifiers – Part III (with redirection, initial MSISDN to MSISDN, multiple MMS Relay/Servers)

- Scope of target identifiers – Part IV (with redirection, initial MSISDN to MSISDN based email address, multiple MMS Relay/Servers)

- Scope of target identifiers – Part V (with redirection, initial MSISDN to generic email address, multiple MMS Relay/Servers).

- Scope of target identifiers – Part VI (with redirection, initial generic email address to MSISDN based email address, multiple MMS Relay/Servers).

- Scope of target identifiers – Part VII (with redirection, initial generic email address to generic email address, multiple MMS Relay/Servers).

- Scope of target identifiers – Part VIII (with redirection, initial MSISDN to MSISDN, one MMS Relay/Server).

- Scope of target identifiers – Part IX (with redirection, initial MSISDN to MSISDN based email address, one MMS Relay/Server).

Scope of MMS messages

- Scope of MMS messages – Part I (introduction).

- Scope of MMS messages – Part II (MMBox specific).

- Scope of MMS messages – Part III (originating MMS domain, target is local to the MMS Relay/Server).

- Scope of MMS messages – Part IV (originating MMS domain, target non-local ID)

- Scope of MMS messages – Part V (terminating MMS domain, target is local to the MMS Relay/Server).

- Scope of MMS messages – Part VI (terminating MMS domain, target non-local ID)

- Scope of MMS messages – Part VII (redirecting scenarios, target is local to the redirecting MMS Relay/Server).

- Scope of MMS messages – Part VIII (redirecting scenarios, target non-local ID to the redirecting MMS Relay/Server).

- Scope of MMS messages – Part IX (single MMS Relay/Server case, general, originating target).

- Scope of MMS messages – Part X (single MMS Relay/Server case, terminating target).

- Scope of MMS messages – Part XI (single MMS Relay/Server case with redirection, redirected MMS user agent retrieves).

- Scope of MMS messages – Part XII (single MMS Relay/Server case with redirection, redirected MMS user agent deletes).

Most of the illustrations are in a topology view format and presumed to be easier to grasp the concepts by a reader.