SA3#95-LI	 S3i240626

Las Vegas; October 29-November 1, 2024

Source:	Nokia,

Title:	pCR to TR 33.929-8: MMS LI – scope of target identifiers – Part III (with redirection, initial MSISDN to MSISDN, multiple MMS Relay/Servers)

Document for:	Agreement

Agenda Item:	9

Work Item / Release:	LI19

Abstract of the contribution: This pCR provides input text to TR 33.929-8 LI for MMS. Scope of target identifiers with redirection are illustrated in a topology view format. Here, the initial MM message is from MSISDN to MSISDN. Multiple MMS Relay/Servers are used.

BACKGROUND

The TR 33.929-8 provides the implementation guidance for the LI aspects related to the MMS. The target identities can be a phone number or an email address. Several permutations are possible when messages exchanged between two clients represented by those identities are redirected (i.e. forwarded).

In the illustrations shown here, the initial MM message is from phone number to phone number. There are two cases depending on the addressing format.

- MSISDN to MSISDN.

- MSISDN to MSISDN based email address.

This pCR illustrates of the first of those scenarios. Multiple MMS Relay/Servers are used.

PROPOSAL

Incorporate the following to the TR 33.929-8.

# 4	Illustration of LI for MMS

## 4.1	General

## 4.2	MMS background with LI scope

## 4.3	Scope of target identities

### 4.3.1	General

### 4.3.2	Topology views - without redirection

*** New Text ***

### 4.3.3	Topology views - with redirection

#### 4.3.3.1	General

The redirection may be initiated by the MMS Relay/Server (i.e. redirecting the MM message) that serves the MMS User Agent or by the MMS User Agent (forwarding the MM message) upon receiving a notification of an incoming MM message (without a retrieval), in the former case, it may be through a prior setup at the MMS Relay/Server.

The MMS User Agent may also forward an incoming MM message after the retrieving the MM message, however, in this case, it would be same as if that MMS User Agent is sending the MM message, initially.

The Email Client may also forward the received MM message.

NOTE 1:	The term "forward" is used for the actions performed by the MMS User Agent or the Email Client.

NOTE 2:	The term "redirect" is used when the action is done by the MMS Relay/Server to avoid any confusion with the MMS Relay/Server's action of forwarding an MM message from input to output (e.g. MMS Relay/Server forwards the MM message from MM1 to MM3 or to MM4). The term "redirect" is also used to refer to the functional entity that performs the redirection/forwarding (e.g. redirecting MMS User Agent").

When the MMS User Agent or the Email Client forwards the MM message, the same steps as in the initial message transfer occur and hence, all the scenarios described in in clause 4.3.2 apply for a redirected MM message as well.

- The redirecting MMS User Agent can forward the MM message to an Email Address or to a phone number.

- The redirecting Email Client can forward the MM message to an Email Address or to a phone number.

All the three permutations as illustrated in clause 4.3.2.2.1 for MM message originations from a phone number (MSISDN, GPSI (MSISDN), E.164 number) are applicable when the redirection happens from a phone number (MSISDN, GPSI (MSISDN), E.164 number).

Both of the two permutations as illustrated in clause 4.3.2.2.1 for MM message originations from a generic Email Address are applicable when the redirection happens from a generic Email Address.

The MM messages redirected from the MMS Relay/Server are also handled in the same way except that in this case the initial MM1 message (between the redirecting MMS Relay/Server and the first terminating MMS User Agent) is not needed.

The subsequent clauses illustrate the user-forwarded cases of different identities that are applicable for MMS LI when separate MMS Relay Servers (when applicable) are involved in the MMS flow.

#### 4.3.3.2	Multiple MMS Relay/Servers

##### 4.3.3.2.1	General

The following cases are illustrated in the subsequent clauses (all are from the perspective of originating end):

- 	Initial MM message from MSISDN to MSISDN.

-	Initial MM message from MSISDN to MSISDN based Email Address.

-	Initial MM message from MSISDN to generic Email Address.

-	Initial MM message from generic Email Address to MSISDN based Email Address.

-	Initial MM message from generic Email Address to generic Email Address.

Multiple MMS Relay/Servers refers to the MMS Relay/Servers serving the MMS User Agents, when applicable, are different.

##### 4.3.3.2.2	Initial MM message from MSISDN to MSISDN

###### 4.3.3.2.2.1	Overview

MMS User Agent (A) initially sends an MM message using own phone number (MSISDN, GPSI (MSISDN), E.164 number) to MMS User Agent (B) (addressed with a phone number (MSISDN, GPSI (MSISDN), E.164 number) and the MMS User Agent (B) forwards the received MM message to destinations as illustrated.

In this illustration, it is presumed that MMS User Agent (A), MMS User Agent (B) and MMS User Agent (C) are served by different MMS Relay Servers.

Figure 4.3.3.2.2.1-1: Scope of target identities with redirection (multiple Relay/Servers – case 1)

The path of the MM message flow from MMS User Agent (A) to MMS User Agent (B) is same as in the scenario 1 of the illustrations in clause 4.3.2.2 (figure 4.3.2.2.1-1 and 4.3.2.2.2.2-1). For this part, the phone number A can be a target in the MMS Relay/Server (A) and a target non-local ID in the MMS Relay/Server (B). The phone number B can be a target in the MMS Relay/Server (B) and a target non-local ID in the MMS Relay/Server (A).

The redirection of the MM message may happen to a phone number or to an Email Address.

When the MMS User Agent (B) forwards the MM message to an MSISDN based Email Address (shown as C@mmsc-c), the redirecting MMS Relay/Server (B) adds the domain name for B (shown as mmsc-b) and the redirected-to MMS Relay/Server (C) would remove the domain from for C (i.e. removes the mmsc-c from C@mmsc-c).

The three scenarios of redirected MMS flow shown as R-1, R-2, and R-3 in figure 4.3.3.2.2.1-1 are further illustrated in figure 4.3.3.2.2.1-2.

Figure 4.3.3.2.2.1-2: Three scenarios for the redirected MM message for case 1

The three redirected scenarios are described in clause 4.3.3.2.2.2.

###### 4.3.3.2.2.2	Descriptions of MM message redirections from a phone number

4.3.3.2.2.2.1	Scenario R-1:

In this scenario, the redirecting MMS User Agent (B) forwards the received MM message to a phone number (MSISDN, GPSI (MSISDN), E.164 number). The MMS User Agent (C) is the recipient of that redirected MM message.

The phone number B can be a target in the MMS Relay/Server (B) for the redirected MM message.

The phone number B can be a target non-local ID in the MMS Relay/Server (C).

The phone number C can be a target in the MMS Relay/Server (C) and a target non-local ID in the MMS Relay/Server (B).

4.3.3.2.2.2.2	Scenario R-2:

In this scenario, the MMS User Agent (B) forwards the received MM message to a phone number (MSISDN, GPSI (MSISDN), E.164 number) by addressing the destination using an MSISDN based Email Address (shown as C@mmsc-c). The MMS User Agent (C) is the recipient of that redirected MM message.

The MMS Relay/Server (B) adds the domain name (shown as @mmsc-B) to the B's identity (shown as B@mmsc-b) before forwarding the MM message to the External Server serving the domain mmsc-c.

The MMS Relay/Server (C) would remove the domain name (shown as @mmsc-c) at the terminating end.

The phone number B can be target in the MMS Relay/Server (B) for the redirected MM message.

The MSISDN based Email Address B@mmsc-b can be a target non-local ID in the MMS Relay/Server (C).

The phone number C can be a target in the MMS Relay/Server (C) and C@mmsc-c can be a target non-local ID in the MMS Relay/Server (B).

4.3.3.2.2.2.3	Scenario R-3:

In this scenario, MMS User Agent (B) forwards the received MM message to a generic Email Address (shown as CC@email). This MM message goes from the external server (email) to the Email Client (CC).

The MMS Relay/Server (B) adds the domain name (shown as @mmsc-b) to the B's identity (shown as A@mmsc-b) before forwarding the MM message to the External Server.

The phone number B can be target in the MMS Relay/Server (B) for the redirected MM message.

The Email Address CC@email can be a target non-local ID in the MMS Relay/Server (B).

***** End of the new text ****