SA3#95-LI	 S3i240627

Las Vegas; October 29-November 1, 2024

Source:	Nokia,

Title:	pCR to TR 33.929-8: MMS LI – scope of target identifier– Part IV (with redirection, initial MSISDN to MSISDN based email, multiple MMS Relay Servers)

Document for:	Agreement

Agenda Item:	9

Work Item / Release:	LI19

Abstract of the contribution: This pCR provides input text to TR 33.929-8 LI for MMS. Scope of target identifiers with redirection are illustrated in a topology view format. Here, the initial MM message is from MSISDN to MSISDN based email. Multiple MMS Relay/Servers are used.

BACKGROUND

The TR 33.929-8 provides the implementation guidance for the LI aspects related to the MMS. The target identities can be a phone number or an email address. Several permutations are possible when messages exchanged between two clients represented by those identities are redirected (i.e. forwarded).

In the illustrations shown here, the initial MM message is from phone number to phone number. There are two cases depending on the addressing format.

- MSISDN to MSISDN.

- MSISDN to MSISDN based email address.

This pCR illustrates of the second of those scenarios. Multiple MMS Relay/Servers are used.

PROPOSAL

Incorporate the following to the TR 33.929-8.

# 4	Illustration of LI for MMS

## 4.1	General

## 4.2	MMS background with LI scope

## 4.3	Scope of target identities

### 4.3.1	General

### 4.3.2	Topology views - without redirection

### 4.3.3	Topology views - with redirection

#### 4.3.3.1	General

#### 4.3.3.2	Multiple MMS Relay/Servers

##### 4.3.3.2.1	General

##### 4.3.3.2.2	Initial MM message from MSISDN to MSISDN

***** New text ****

##### 4.3.3.2.3	Initial MM message from MSISDN to MSISDN based Email Address

MMS User Agent (A) initially sends an MM message using own phone number (MSISDN, GPSI (MSISDN), E.164 number) to MMS User Agent (B), addressed with an MSISDN based Email Address (shown as B@mmsc-b). The MMS User Agent (B) forwards the MM message to destinations as illustrated.

In this illustration, it is presumed that MMS User Agent (A), MMS User Agent (B) and MMS User Agent (C) are served by different MMS Relay Servers.

Figure 4.3.3.2.3-1: Scope of target identities with redirection (multiple Relay/Servers – case 2)

The path of the MM message from MMS User Agent (A) to MMS User Agent (B) is same as in the scenario 2 of the illustrations in clause 4.3.2.2 (figure 4.3.2.2.1-1 and 4.3.2.2.2.3-1). For this part, the phone number A can be a target in the MMS Relay/Server (A) and A@mmsc-a can be target non-local ID in the MMS Relay/Server (B). Likewise, The phone number B can be a target in the MMS Relay/Server (B) and B@mmsc-b can be a target non-local ID in the MMS Relay/Server (A).

The redirection of the MM message may happen to a phone number or to an Email Address.

When the MMS User Agent (B) forwards the MM message to an MSISDN based Email Address (shown as C@mmsc-c), the redirecting MMS Relay/Server (B) adds the domain name for B (shown as mmsc-b) and the redirected-to MMS Relay/Server (C) would remove the domain from for C (i.e. removes the mmsc-c from C@mmsc-c).

The three redirected scenarios of MMS flow, shown as R-1, R-2, R-3 in figure 4.3.3.2.3-1, are further illustrated in figure 4.3.3.2.3-2.

Figure 4.3.3.2.3-2: Three scenarios for the redirected MM message for case 2

The three redirected scenarios are described in clause 4.3.3.2.2.2.

***** End of the new text ****