DeletSA3#95-LI	 S3i240633

Las Vegas; October 29-November 1, 2024

Source:	Nokia,

Title:	pCR to TR 33.929-8: MMS LI – scope of MMS messages – Part I (introduction)

Document for:	Agreement

Agenda Item:	9

Work Item / Release:	LI19

Abstract of the contribution: This pCR provides input text to TR 33.929-8 LI for MMS. An overview of the scope of MMS messages reported as xIRIs.

BACKGROUND

The TR 33.929-8 provides the implementation guidance for the LI aspects related to the MMS.

PROPOSAL

Incorporate the following to the TR 33.929-8.

# 4	Illustration of LI for MMS

## 4.1	General

## 4.2	MMS background with LI scope

## 4.3	Scope of target identities

***** New text ****

## 4.4	Scope of MMS messages reported

### 4.4.1	General

As illustrated in clause 4.3, when the MMS User Agent, local to an MMS Relay/Server, is the target, the xIRIs generated by the IRI-POI in the MMS Relay/Server are based off of the MM1 messages as defined in the OMA specification [19].

For target non-local ID represented by a phone number (E.164 number, GPSI (MSISDN), MSISDN), the xIRIs generated by the IRI-POI in the MMS Relay/Server are based off of the MM4 messages as defined in TS 23.140 [18].

For target non-local ID represented by an Email Address, the xIRIs generated by the IRI-POI in the MMS Relay/Server are based off of the MM1 messages as defined in the OMA specification [19] transfer over or from the MM3 interface. MM3 messages by themselves are not referenced.

Figure 4.4.1-1: Referenced source specifications for MMS LI

The TS 33.127 [3] and TS 33.128 [4] use the MMS architecture defined in TS 23.140 [18] for MMS LI. However, the TS 23.140 [18] does not define the stage 3 details for the MM1 messages. For that reason, the TS 33.128 [4] uses the OMA spec [19] for the message details.

### 4.4.2	3GPP, OMA and MMS LI messages

#### 4.4.2.1	General

A comparison of the message-names used in the TS 23.140 [18] and the message names used in the OMA specification [19] along with associated xIRI messages defined in TS 33.128 [4] are given in this clause.

The MMBox related operations result in XIRIs only when the target that is local to the MMS Relay/Server.

#### 4.4.2.2	MM1 to xIRIs (originating side)

When the target is local to the MMS Relay/Server or when the target non-local ID is represented by an Email Address, the MM1 messages are used to generate the xIRIs.

Table 4.4.2.2-1: MMS LI from (MM1, MMSM) messages (target is local MMS Relay/Server)

| MM1 [18] | MMSM [19] | MM message direction | MMS LI (xIRI [4]) |
| --- | --- | --- | --- |
| MM1_submit.REQ | m-send-req | UA Relay/Server | MMSSend (NOTE 1) |
| MM1_submit.RES | m-send-conf | Relay/Server UA | MMSSend (NOTE 1) |
| MM1_delivery_report.REQ | m-delivery-ind | Relay/Server UA | MMSDeliveryReport |
| MM1_read_reply_originator.REQ | m-read-orig-ind | Relay/Server UA | MMSReadReport |
| MM1_mmbox_view.REQ | m-mbox-view-req | UA Relay/Server | MMSMBoxViewRequest |
| MM1_mmbox_view.RES | m-mbox-view-conf | Relay/Server UA | MMSMBoxViewResponse |
| MM1_mmbox_upload.REQ | m-mbox-upload-req | UA Relay/Server | MMSMBoxUpload (NOTE 2) |
| MM1_mmbox_upload_RES | m-mbox-upload-conf | Relay/Server UA | MMSMBoxUpload (NOTE 2) |
| MM1_mmbox_store_REQ | m-mbox-store-req | UA Relay/Server | MMSMBoxStore (NOTE 3) |
| MM1_mmbox_store.RES | m-mbox-store-conf | Relay/Server UA | MMSMBoxStore (NOTE 3) |
| MM1_mmbox_delete.REQ | m-mbox-delete-req | UA Relay/Server |  |
| MM1_mmbox_delete.RES | m-mbox-delete-conf | Relay/Server UA | MMSMBoxDelete |
| NOTE 1: The xIRI is constructed from the m-send-req, but the trigger for the xIRI is m-send-conf. NOTE 2: The xIRI is constructed from the m-mbox-upload-req but the trigger for the xIRI is m-mbox-upload-conf. NOTE 3: The xIRI is constructed from the m-mbox-store-req but the trigger for the xIRI is m-mbox-store-conf. Furthermore, at the originating end, this procedure is only used to update the state of an already stored MM message. |  |  |  |

As shown in table 4.4.2.2-1, most of the xIRIs are generated when a confirmation is sent to the MMS User Agent. An illustration of the message mapping shown in table 4.4.2.2-1 along with the directional aspects are illustrated in clause 4.4.3.

Table 4.4.2.2-2: MMS LI from (MM1, MMSM) messages (target is Email Address, non-local ID)

| MM1 [18] | MMSM [19] | MM message direction | MMS LI (xIRI [4]) |
| --- | --- | --- | --- |
| MM1_submit.REQ | m-send-req | UA Relay/Server |  |
| MM1_submit.RES | m-send-conf | Relay/Server UA |  |
| MM1_delivery_report.REQ | m-delivery-ind | Relay/Server UA |  |
| MM1_read_reply_originator.REQ | m-read-orig-ind | Relay/Server UA |  |

Editor's Note: The last column in the above tables may require a change once the CR to include the Email Address as a target identifier is approved.

The MMBox specific messages are not included in table 4.4.2.2-2 as such operations do not apply to target non-local ID.

An illustration of the message mapping shown in table 4.4.2.2-2 along with the directional aspects are illustrated in clause 4.4.3.

#### 4.4.2.3	MM1 to xIRIs (terminating side)

When the target is local to the MMS Relay/Server or when the target non-local ID is represented by an Email Address, the MM1 messages are used to generate the xIRIs.

Table 4.4.2.3-1: MMS LI from (MM1, MMSM) messages (target is local MMS Relay/Server)

| MM1 [18] | MMSM [19] | MM message direction | MMS LI (xIRI [4]) |
| --- | --- | --- | --- |
| MM1_notification.REQ | m-notification-ind | Relay/Server UA | MMSNotification |
| MM1_notification.RES | m-notifyresp-ind | UA Relay/Server | MMSNotificationResponse |
| MM1_retrieve.REQ | WEP/HTTP GET | UA Relay/Server |  |
| MM1_retrieve.RES | m-retrieve-conf | Relay/Server UA | MMSRetrieval |
| MM1_acknowledge.REQ | m-notifyresp-ind (NOTE 1) | UA Relay/Server | MMSDeliveryAck |
| MM1_acknowledge.REQ | m-acknowledge-ind (NOTE 2) | UA Relay/Server | MMSDeliveryAck |
| MM1_read_reply_receipient.REQ | m-read-rec-ind | UA Relay/Server | MMSReadReport |
| MM1_forward.REQ | m-forward-req | UA Relay/Server | MMSForward (NOTE 3) |
| MM1_forward.RES | m-forward-conf | Relay/Server UA | MMSForward (NOTE 3) |
| MM1_delete.REQ | m-delete-req | UA Relay/Server |  |
| MM1_delete.RES | m-delete-conf | Relay/Server UA | MMSDeleteFromRelay |
| MM1_cancel.REQ | m-cancel-req | Relay/Server UA | MMSCancel |
| MM1_cancel.RES | m-cancel-conf | UA Relay/Server |  |
| MM1_mmbox_store.REQ | m-mbox-store-req | UA Relay/Server | MMSMboxStore (NOTE 4) |
| MM1_mmbox_store.RES | m-mbox-store-conf | Relay/Server UA | MMSMboxStore (NOTE 4) |
| MM1_mmbox_view.REQ | m-mbox-view-req | UA Relay/Server | MMSMBoxViewRequest |
| MM1_mmbox_view.RES | m-mbox-view-conf | Relay/Server UA | MMSMBoxViewResponse |
| MM1_mmbox_delete.REQ | m-mbox-delete-req | UA Relay/Server |  |
| MM1_mmbox_delete.RES | m-mbox-delete-conf | Relay/Server UA | MMSMBoxDelete |
| NOTE 1: Immediate retrieval NOTE 2: Delayed retrieval NOTE 3: The xIRI is constructed from the m-forward-req, but the trigger for the xIRI is m-forward-conf. NOTE 4: The xIRI is constructed from the m-mbox-store, but the trigger for the xIRI is m-mbox-store-conf. |  |  |  |

An illustration of the message mapping shown in table 4.4.2.3-1 along with the directional aspects are illustrated in clause 4.4.3.

Table 4.4.2.3-1: MMS LI from (MM1, MMSM) messages (target is Email Address, non-local ID)

| MM1 [18] | MMSM [19] | MM message direction | MMS LI (xIRI [4]) |
| --- | --- | --- | --- |
| MM1_notification.REQ | m-notification-ind | Relay/Server UA |  |
| MM1_notification.RES | m-notifyresp-ind | UA Relay/Server |  |
| MM1_retrieve.REQ | WEP/HTTP GET | UA Relay/Server |  |
| MM1_retrieve.RES | m-retrieve-conf | Relay/Server UA |  |
| MM1_acknowledge.REQ | m-notifyresp-ind (NOTE 1) | UA Relay/Server |  |
| MM1_acknowledge.REQ | m-acknowledge-ind (NOTE 2) | UA Relay/Server |  |
| MM1_read_reply_receipient.REQ | m-read-rec-ind | UA Relay/Server |  |
| MM1_forward.REQ | m-forward-req | UA Relay/Server |  |
| MM1_forward.RES | m-forward-conf | Relay/Server UA |  |
| MM1_delete.REQ | m-delete-req | UA Relay/Server |  |
| MM1_delete.RES | m-delete-conf | Relay/Server UA |  |
| MM1_cancel.REQ | m-cancel-req | Relay/Server UA |  |
| MM1_cancel.RES | m-cancel-conf | UA Relay/Server |  |

Editor's Note: The last column in the above tables may require a change once the CR to include the Email Address as a target identifier is approved.

The MMBox specific messages are not included in table 4.4.2.2-2 as such operations do not apply to target non-local ID.

An illustration of the message mapping shown in table 4.4.2.2-2 along with the directional aspects are illustrated in clause 4.4.3.

#### 4.4.2.4	MM4 to XIRIs

When the target non-local ID is represented by a phone number, the MM4 messages as defined in TS 33.128 [4] are used for the xIRI message generation. The following tables map the MM4 messages [18] to the xIRI messages [4].

Table 4.4.2.4-1: MMS LI from MM4 messages (originating side)

| MM4 [18] | MM message direction | MMS LI (xIRI [4]) |
| --- | --- | --- |
| MM4_forward.REQ | Relay/Server (A) to Relay Server (B) | MMSToNonLocalTarget |
| MM4_forward.RES | Relay/Server (B) to Relay Server (A) |  |
| MM4_delivery_report.REQ | Relay/Server (B) to Relay Server (A) | MMSDeliveryReportNonLocalTarget |
| MM4_delivery_report.RES | Relay/Server (A) to Relay Server (B) |  |
| MM4_read_reply_report.REQ | Relay/Server (B) to Relay Server (A) | MMSReadReportNonLocalTarget |
| MM4_read_reply_report.RES | Relay/Server (A) to Relay Server (B) |  |

An illustration of the message mapping shown in table 4.4.2.4-1 along with the directional aspects are illustrated in clause 4.4.3.

Table 4.4.2.4-2: MMS LI from MM4 messages (terminating side)

| MM4 [18] | MM message direction | MMS LI (xIRI [4]) |
| --- | --- | --- |
| MM4_forward.REQ | Relay/Server (A) to Relay Server (B) | MMSSendByNonLocalTarget |
| MM4_forward.RES | Relay/Server (B) to Relay Server (A) |  |
| MM4_delivery_report.REQ | Relay/Server (B) to Relay Server (A) | MMSDeliveryReportNonLocalTarget |
| MM4_delivery_report.RES | Relay/Server (A) to Relay Server (B) |  |
| MM4_read_reply_report.REQ | Relay/Server (B) to Relay Server (A) | MMSReadReportNonLocalTarget |
| MM4_read_reply_report.RES | Relay/Server (A) to Relay Server (B) |  |

An illustration of the message mapping shown in table 4.4.2.4-2 along with the directional aspects are illustrated in clause 4.4.3.

***** End of the new text ****