DeletSA3#95-LI	 S3i240635

Las Vegas; October 29-November 1, 2024

Source:	Nokia,

Title:	pCR to TR 33.929-8: MMS LI – scope of MMS messages reported – Part III (originating MMS domain, target is local to MMS Relay/Server)

Document for:	Agreement

Agenda Item:	9

Work Item / Release:	LI19

Abstract of the contribution: This pCR provides input text to TR 33.929-8 LI for MMS. Scope of MMS messages reported as xIRIs in the originating MMS domain. The illustrations assume that the target is local to the MMS Relay/Server.

BACKGROUND

The TR 33.929-8 provides the implementation guidance for the LI aspects related to the MMS.

This pCR illustrates the cases from an originating MMS domain perspective, where the target is a local to MMS Relay/Server. The destination could be another MMS User Agent (addressed by a phone number or an MSISDN based Email Address) or an Email Client. From an originating MMS domain perspective, the destination when addressed in an email address format, MSISDN based or generic, are the same.

PROPOSAL

Incorporate the following to the TR 33.929-8.

# 4	Illustration of LI for MMS

## 4.1	General

## 4.2	MMS background with LI scope

## 4.3	Scope of target identities

## 4.4	Scope of MMS messages reported

### 4.4.1	General

### 4.4.2	3GPP, OMA and MMS LI messages

### 4.4.3	Topology views

#### 4.4.3.1	General

#### 4.4.3.2	MMBox specific operations

***** New text ****

#### 4.4.3.3	Originating MMS domain

##### 4.4.3.3.1	General

###### 4.4.3.3.1.1	Scope

Three cases are illustrated in the subsequent clauses:

- Target is local to the MMS Relay/Server, destination is an MMS User Agent is served by a different MMS Relay/Server.

- Target is local to the MMS Relay/Server, destination is an Email Client or an MMS User Agent with an MSISDN based Email Address.

- Target non-local ID.

In the case of target non-local ID identified with a phone number, two MMS Relay/Servers are always used. The target non-local ID is the destination of the MM message.

###### 4.4.3.3.1.2 	MMBox

When MM message is sent by the originating MMS User Agent, if requested so, a copy of the MM message is stored in the MMBox with the status showing "sent", if an MMBox is supported for MMS User Agent (A). Therefore, the storing of the MM message at the MMBox does not happen if the store indication is not sent in the m-send-req message, even if the MMBox is supported for the MMS User Agent (A).

When MMBox is not used, it is presumed that the MM messages are stored locally at the MMS/Relay Server.

When the MMBox is used, the MMS User Agent may also perform a few MMBox specific operations as illustrated in clause 4.4.3.2.2.

LI does not apply to such MMBox specific operations for target non-local ID.

###### 4.4.3.3.1.3	Delivery reports

The receipt of delivery report from the terminating MMS domain is conditional. The delivery report is received only if the terminating MMS Relay/Server sends such a report. The terminating MMS Relay/Server sends such a report if and only if the originating MMS User Agent has requested for such a report and the terminating MMS User Agent permits the sending of such a report.

The delivery report is not received if the destination is an Email Address.

###### 4.4.3.3.1.4	Read reports

The receipt of read report from the terminating MMS User Agent is conditional. The read report is received if the originating MMS User Agent has requested for such a report and the terminating MMS User Agent sends such a report.

The read report is not received, if the destination is an Email Address.

##### 4.4.3.3.2	Target is local to MMS Relay Server

###### 4.4.3.3.2.1	Overview

The figure 4.4.3.3.2.1-1 gives an overview of the MMS LI messages applicable to the originating MMS domain when the target is local to the MMS Relay/Server and the destination can be an Email Client or an MMS User Agent served by a different MMS Relay/Server with a phone number or an MSISDN based Email Address.

When the destination MMS User Agent uses an MSISDN based Email Address, from the MMS Relay/Server (A)'s perspective, the handling is similar to when the destination is an Email Client since the outbound MM messages are transferred over the MM3 interface in an email message format.

Figure 4.4.3.3.2.1-1: MMS LI messages – originating MMS domain (target is local)

The MM1 messages between the MMS User Agent (A), the target and the MMS Relay/Server (A) are used to generate the xIRIs.

The type of destination (i.e. Email Client Vs MMS User Agent) has no impact on the MMS LI when the originating MMS User Agent is a target and local to the MMS Relay/Server, except that the conditional delivery reports and the read reports are never received when the destination is an Email Client.

###### 4.4.3.3.2.2	MM messages to MMS LI messages

The figure 4.4.3.3.2.2-1 gives an overview of the MMS LI messages applicable to the originating MMS domain when the target is local to the MMS Relay/Server.

In this illustration, the destination is an MMS User Agent served (identifier by a phone number) by a different MMS Relay/Server (the other MMS Relay/Server is not shown in the figure).

Figure 4.4.3.3.2.2-1: MMS LI messages from MM messages – MMS Relay/Server (A) – case 1

When MM message is sent by the originating MMS User Agent, if requested so, a copy of the MM message is stored in the MMBox. Therefore, the storing of the MM message at the MMBox does not happen if the store indication is not sent in the m-send-req message.

Even though the trigger for the generation of xIRI MMSSend is the m-send-conf message that the MMS-Relay/Server (A) sends to the MMS User Agent (A), the xIRI includes information from the m-send-req as well.

As shown, there are two instances where the m-delivery-ind is sent to the MMS User Agent:

- When the MM message is forwarded to the destination (using MM4_forward,REQ).

- When a delivery report is received from the destination (using MM4_delivery-report.REQ).

Since the receipt of delivery report from the destination (MM4_delivery-report-REQ) is conditional, the sending of associated m-delivery-ind is also conditional. Therefore, the generation of the xIRI MMSDeliveryReport is conditional.

Since the receipt of the read-reply report from the destination (MM4_read-reply-report.REQ) is conditional, the sending of m-read-orig-ind is also conditional. Therefore, the generation of the xIRI MMSReadReport is conditional.

###### 4.4.3.3.2.3	Destination is an email address

The figure 4.4.3.3.2.3-1 gives an overview of the MMS LI messages applicable to the originating MMS domain when the target is local to the MMS Relay/Server and the destination is an Email Client or an MMS User Agent with an MSISDN based email address..

Figure 4.4.3.3.2.3-1: MMS LI messages from MM messages – MMS Relay/Server (A) - case 2

When MM message is sent by the originating MMS User Agent, if requested so, a copy of the MM message is stored in the MMBox. Therefore, the storing of the MM message at the MMBox does not happen if the store indication is not sent in the m-send-req message.

Even though the trigger for the generation of xIRI MMSSend is the m-send-conf message that the MMS-Relay/Server (A) sends to the MMS User Agent (A), the xIRI includes information from the m-send-req as well.

As shown, a m-delivery-ind is sent to the MMS User Agent when the MM message (in the email format) is forwarded to the destination (using MM3L SMTP Send).

As can be inferred, there are subsequent delivery reports or a read report from the destination end.

***** End of the new text ****