DeletSA3#95-LI	 S3i240637

Las Vegas; October 29-November 1, 2024

Source:	Nokia

Title:	pCR to TR 33.929-8: MMS LI – scope of MMS messages reported – Part V (terminating MMS domain, target is local to MMS Relay/Server)

Document for:	Agreement

Agenda Item:	9

Work Item / Release:	LI19

Abstract of the contribution: This pCR provides input text to TR 33.929-8 LI for MMS. Scope of MMS messages reported as xIRIs in the termination MMS domain without redirection are illustrated in a topology view format. The illustrations assume that the target is local to the MMS Relay/Server.

BACKGROUND

The TR 33.929-8 provides the implementation guidance for the LI aspects related to the MMS.

This pCR illustrates the cases from a terminating MMS domain perspective, where the target is local to MMS Relay/Server. The originating could be another MMS User Agent (addressed by a phone number or an MSISDN based Email Address) or an Email Client. From terminating MMS domain perspective, the origination when addressed in an email address format, MSISDN based or generic, are the same.

PROPOSAL

Incorporate the following to the TR 33.929-8.

# 4	Illustration of LI for MMS

## 4.1	General

## 4.2	MMS background with LI scope

## 4.3	Scope of target identities

## 4.4	Scope of MMS messages reported

### 4.4.1	General

### 4.4.2	3GPP, OMA and MMS LI messages

### 4.4.3	Topology views

#### 4.4.3.1	General

#### 4.4.3.2	MMBox specific operations

#### 4.4.3.3	Originating MMS domain

***** New text ****

#### 4.4.3.4	Terminating MMS domain

##### 4.4.3.4.1	General

###### 4.4.3.4.1.1	Scope

Three cases are illustrated in the subsequent clauses:

- Target is local to the MMS Relay/Server, origination is an MMS User Agent is served by a different MMS Relay/Server.

- Target is local to the MMS Relay/Server, origination is an Email Client or an MMS User Agent with an MSISDN based Email Address.

- Target non-local ID.

In the case of target non-local ID identified with a phone number, two MMS Relay/Servers are always used. The target non-local ID is the origination end of the MM message.

###### 4.4.3.4.1.2 	MMBox

When the MMBox is supported, the incoming MM message is automatically stored in the MMBox , if setup and configured so, by the terminating MMS User Agent. The status of thus stored MM message would be "New". In that case, MMS User Agent would point to the MM message stored in the MMBox for any subsequent actions (e.g. retrieval, deletion, forwarding). The status of the MM message changes according to the action taken (e.g. "Retrieved", "Forwarded").

If the MMBox is supported, but not configured to store the incoming MM message automatically, then the MMS User Agent (B) may request the MMS Relay/Server (B) to store the MM message in the MMBox using the m-mbox-store-req message as illustrated in clause 4.4.3.2.2.

If the MMBox is not supported or not setup by the MMS User Agent, then the MMBox related aspects shown in subsequent clauses are not applicable, and in that case, the MMS Relay/Server (B) stores the message locally and reference then would be to the internally stored message for any subsequent actions (e.g. Retrieve, Forward, Delete).

The MMS User Agent may also perform a few MMBox specific operations as illustrated in clause 4.4.3.2.2.

The LI does not apply to such MMBox specific operations for target non-local ID.

###### 4.4.3.4.1.3	Delivery reports

The sending of delivery report to the originating MMS domain is conditional. The delivery report is sent only if the originating MMS User Agent has requested such a report and the terminating MMS User Agent permits the sending of such a report.

The delivery report is not sent if the origination from an Email Address.

###### 4.4.3.4.1.4	Read reports

The sending of the read report to the originating MMS domain is conditional. The read report is sent only if the originating MMS User Agent has requested for such a report and the terminating MMS User Agent sends such a report.

The read report is not sent, if the origination is from an Email Address.

###### 4.4.3.4.1.5	Retrieval of MM message

The terminating MMS User Agent upon receiving a notification for an incoming MM message may retrieve the MM message either automatically (referred to as immediate retrieval) or manually (referred to as deferred retrieval).

###### 4.4.3.4.1.6	Deletion of MM message

The terminating MM user upon receiving a notification for an incoming MM message may delete the MM message without retrieval.

##### 4.4.3.4.2	Target is local to MMS Relay Server

###### 4.4.3.4.2.1	Overview

The figure 4.4.3.4.2.1-1 gives an overview of the MMS LI messages applicable to the terminating MMS domain when the target is local to the MMS Relay/Server. The MM message may have been originated from an MMS User Agent or an Email Client or can be an MMS User Agent with an MSISDN based Email Address.

When the origination MMS User Agent uses an MSISDN based Email Address, from the MMS Relay/Server (B)'s perspective, the handling is similar to when the origination is from an Email Client since the inbound MM messages are transferred over the MM3 interface in an email message format.

Figure 4.4.3.4.2.1-1: MMS LI messages – terminating MMS domain (target is local)

The MM1 messages between the MMS Relay Server (B) and the MMS User Agent (B), the target, are used to generate the xIRIs.

The type of MMS origination (i.e. Email Client Vs MMS User Agent) has no impact on the MMS LI when the terminating MMS User Agent is a target and local to the MMS Relay/Server.

###### 4.4.3.4.2.2	MM messages to MMS LI messages

4.4.3.4.2.2.1	Immediate retrieval

The figure 4.4.3.4.2.2.1-1 illustrates the generation of xIRIs from the MMS messages as the MMS User Agent (B) is notified and the MMS User Agent (B) automatically retrieves the MM message, referred to as immediate retrieval.

In this illustration, the origination is an MMS User Agent (identified with a phone number) served by a different MMS Relay/Server (the other MMS Relay/Server is not shown in the figure).

Figure 4.4.3.4.2.2.1-1: MMS LI messages from MM messages (immediate) – MMS Relay/Server (B)

The trigger for xIRI MMSDeliveryAck is MMS Relay/Server (B) receiving the m-notifyresp.ind message.

The trigger for the xIRIMMSReadReport is MMS Relay Server (B) receiving the m-read-rec-ind from the target MMS User Agent (B). However, MMS User Agent (B) sending of the m-read-rec-ind is conditional, and therefore, the generation of xIRI MMSReadReport is conditional.

The MMS Relay/Server (B) may cancel (shown in figure 4.4.3.4.2.2.1-1) the MM message retrieved by the MMS User Agent (B). This step is not part of the normal flow. However, if the MMS Relay/Server (B) cancels the retrieved MM message at the MMS User Agent (B), then the xIRI MMCancel is generated as shown.

4.4.3.4.2.2.2	Deferred retrieval

The figure 4.4.3.4.2.2.2-1 illustrates the generation of xIRIs from the MMS messages as the MMS User Agent (B) is notified and subsequently retrieves the MM message, referred to as deferred retrieval.

In this illustration, the origination is an MMS User Agent (identified with a phone number) served by a different MMS Relay/Server (the other MMS Relay/Server is not shown in the figure).

Figure 4.4.3.4.2.2.2-1: MMS LI messages from MM messages (deferred) – MMS Relay/Server (B)

The trigger for xIRI MMSDeliveryAck is MMS Relay/Server (B) receiving the m-acknowledge.ind message.

The trigger for the xIRIMMSReadReport is MMS Relay Server (B) receiving the m-read-rec-ind from the target MMS User Agent (B). However, MMS User Agent (B) sending of the m-read-rec-ind is conditional, and therefore, the generation of xIRI MMSReadReport is conditional.

The MMS Relay/Server (B) may cancel (shown in figure 4.4.3.4.2.2.2-1) the MM message retrieved by the MMS User Agent (B). This step is not part of the normal flow. However, if the MMS Relay/Server (B) cancels the retrieved MM message at the MMS User Agent (B), then the xIRI MMCancel is generated as shown.

4.4.3.4.2.2.3	Deletion of MM message

The MMS User Agent (B) may delete the MM message from the MMS Relay/Server instead of the retrieval.

The figure 4.4.3.4.2.2.3-1 illustrates the generation of xIRIs from the MMS messages as the MMS User Agent (B) is notified and the MMS User Agent (B) deletes the MM message from the MMS Relay/Server instead of retrieving. This may happen only with the deferred retrieval case.

In this illustration, the origination is an MMS User Agent (identified with a phone number) served by a different MMS Relay/Server (the other MMS Relay/Server is not shown in the figure).

Figure 4.4.3.4.2.2.3-1: MMS LI messages from MM messages (deletion) – MMS Relay/Server (B)

If a read report is requested by the originating MMS User Agent, then the MMS User Agent (B) may send a m-read-rec-ind with "deleted without being read". The xIRI MMSReadReport is generated when the MMS Relay/Server (B) receives the m-read-rec-ind message.

4.4.3.4.2.2.4	Origination is an email address

The figure 4.4.3.4.2.2.4-1 gives an overview of the MMS LI messages applicable to the terminating MMS domain when the target is local to the MMS Relay/Server and the origination is an Email Client or an MMS User Agent with an MSISDN based email address. The deferred retrieval case is illustrated below.

Figure 4.4.3.3.2.4-1: MMS LI messages from MM messages – MMS Relay/Server (from email address)

As can be inferred, there are no delivery report or a read report since the originating end is an Email Address.

.

***** End of the new text ****