DeletSA3#95-LI	 S3i240638

Las Vegas; October 29-November 1, 2024

Source:	Nokia

Title:	pCR to TR 33.929-8: MMS LI – scope of MMS messages reported – Part VI (terminating MMS domain, target non-local ID)

Document for:	Agreement

Agenda Item:	9

Work Item / Release:	LI19

Abstract of the contribution: This pCR provides input text to TR 33.929-8 LI for MMS. Scope of MMS messages reported as xIRIs in the termination MMS domain without redirection are illustrated in a topology view format. The illustrations assume that the target is non-local to the MMS Relay/Server. The case of Email Address as a target non-local ID is not considered here, since the details of stage 3 are yet to be specified in TS 33.128.

BACKGROUND

The TR 33.929-8 provides the implementation guidance for the LI aspects related to the MMS.

This pCR illustrates the cases from a terminating MMS domain perspective, where the target is non-local ID to MMS Relay/Server. The targe non-local ID is a phone number. An Email Addressed based target non-local ID will be covered later after the associated stage 3 details are specified.

PROPOSAL

Incorporate the following to the TR 33.929-8.

# 4	Illustration of LI for MMS

## 4.1	General

## 4.2	MMS background with LI scope

## 4.3	Scope of target identities

## 4.4	Scope of MMS messages reported

### 4.4.1	General

### 4.4.2	3GPP, OMA and MMS LI messages

### 4.4.3	Topology views

#### 4.4.3.1	General

#### 4.4.3.2	MMBox specific operations

#### 4.4.3.3	Originating MMS domain

#### 4.4.3.4	Terminating MMS domain

##### 4.4.3.4.1	General

##### 4.4.3.4.2	Target is local to MMS Relay Server

***** New text ****

##### 4.4.3.4.3	Target non-local ID

###### 4.4.3.4.3.1	Overview

The figure 4.4.3.4.3.1-1 gives an overview of the MMS LI messages applicable to the terminating MMS domain for the target non-local ID. Here, the identity of the target non-local ID is either an E.164 number or an MSISDN (i.e. another MMS user Agent).

Figure 4.4.3.4.3.1-1: MMS LI messages – originating MMS domain (target non-local ID)

As shown in figure 4.4.3.4.3.1-1, the MM4 messages are referenced for the MMS LI when the origination target non-local ID is an MMS User Agent.

The MMS LI for target non-local ID in the terminating MMS Relay/Server (B) is same for immediate retrieval and deferred retrieval cases. Only the immediate retrieval case and deletion before retrieval cases are illustrated.

###### 4.4.3.4.3.2	MM messages to MMS LI messages

4.4.3.4.3.2.1	Immediate retrieval

The figure 4.4.3.4.3.2.1-1 illustrates the generation of xIRIs from the MMS messages as the MMS User Agent (B) is notified and automatically retrieves the MM message sent from MMS User Agent (target non-local ID) present in a different network domain.

Figure 4.4.3.4.3.2.1-1: MMS LI messages from MM messages (immediate) – MMS Relay/Server (B)

The triggers for the xIRIs are sending or receiving the MM4 messages as shown in figure 4.4.3.4.3.2-1.

Since the sending of MM4_delivery-report.REQ is conditional, the generation of xIRI MMSDeliveryReportNonLocalTarget is also conditional.

Since the sending of MM4_ read-reply-report.REQ is conditional, the generation of xIRI MMSReadReportNonLocalTarget is also conditional.

4.4.3.4.3.2.2	Deferred retrieval

The figure 4.4.3.4.3.2.2-1 illustrates the generation of xIRIs from the MMS messages as the MMS User Agent (B) is notified and subsequently retrieves the MM message sent from MMS User Agent (target non-local ID) present in a different network domain.

Figure 4.4.3.4.3.2.2-1: MMS LI messages from MM messages (deferred) – MMS Relay/Server (B)

The triggers for the xIRIs are sending or receiving the MM4 messages as shown in figure 4.4.3.4.3.2-1.

Since the sending of MM4_delivery-report.REQ is conditional, the generation of xIRI MMSDeliveryReportNonLocalTarget is also conditional.

Since the sending of MM4_ read-reply-report.REQ is conditional, the generation of xIRI MMSReadReportNonLocalTarget is also conditional.

4.4.3.4.3.2.3	Deletion of MM message

The MMS User Agent (B) may delete the MM message from the MMS Relay/Server instead of the retrieval.

The figure 4.4.3.4.3.2.3-1 illustrates the generation of xIRIs from the MMS messages as the MMS User Agent (B) is notified and the MMS User Agent (B) deletes the MM message from the MMS Relay/Server instead of retrieving.

Figure 4.4.3.4.3.2.3-1: MMS LI messages from MM messages (deletion) – MMS Relay/Server (B)

If a read report is requested by the originating MMS User Agent, then the MMS User Agent (B) may send a m-read-rec-ind with "deleted without being read". Upon receiving such a m-read-rec-ind from the MMS User Agent (B), the MMS Relay/Server sends the MM4_read-reply-report.REQ to the originating MMS domain. The sending of MM4_read-reply-report.REQ triggers the sending of xIRI MMSReadReportNonLocalTarget.

The MMS User Agent (B) may also separately delete the MM message stored in the MMBox using the m-mbox-delete-req message as illustrated in clause 4.4.3.2.2. However, this has no impact on the MMS LI since the target is non-local ID.

***** End of the new text ****