DeletSA3#95-LI	 S3i240640

Las Vegas; October 29-November 1, 2024

Source:	Nokia,

Title:	pCR to TR 33.929-8: MMS LI – scope of MMS messages – Part VIII (redirection scenarios, target non-local ID to the redirecting MMS Relay/Server)

Document for:	Agreement

Agenda Item:	9

Work Item / Release:	LI19

Abstract of the contribution: This pCR provides input text to TR 33.929-8 LI for MMS. Scope of MMS messages reported as xIRIs for the redirecting case. The target is non-local ID to the redirecting MMS Relay/Server. In other words, either originating end or the terminating end are the target non-local ID. The case of Email Address as a target non-local ID is not considered here, since the details of stage 3 are yet to be specified in TS 33.128.

BACKGROUND

The TR 33.929-8 provides the implementation guidance for the LI aspects related to the MMS.

This pCR illustrates the cases from a redirecting MMS domain perspective, where either the originating target or the terminating target is non-local ID to redirecting MMS Relay/Server. The targe non-local ID is a phone number. An Email Addressed based target non-local ID will be covered later after the associated stage 3 details are specified.

PROPOSAL

Incorporate the following to the TR 33.929-8.

# 4	Illustration of LI for MMS

## 4.1	General

## 4.2	MMS background with LI scope

## 4.3	Scope of target identities

## 4.4	Scope of MMS messages reported

### 4.4.1	General

### 4.4.2	3GPP, OMA and MMS LI messages

### 4.4.3	Topology views

#### 4.4.3.1	General

#### 4.4.3.2	MMBox specific operations

#### 4.4.3.3	Originating MMS domain

#### 4.4.3.4	Terminating MMS domain

#### 4.4.3.5	Redirecting MMS domain

##### 4.4.3.5.1	General

##### 4.4.3.5.2	Target is local to MMS Relay Server

***** New text ****

##### 4.4.3.5.3	Target non-local ID

###### 4.4.3.5.3.1	Overview

The figure 4.4.3.5.3.1-1 gives an overview of the MMS LI messages applicable to the redirecting MMS domain when both originating and redirected-to MMS User Agents are target non-local IDs at the redirecting MMS Relay/Server. Note that either both (as shown) or one of them or none of them can be the target non-local IDs.

Figure 4.4.3.5.3.1-1: MMS LI messages – redirecting MMS domain (target non-local IDs)

As shown in figure 4.4.3.5.3.1-1, the incoming MM4 messages are referenced for the MMS LI for the originating target non-local ID, i.e. MMS User Agent (A) and outgoing MM4 messages are used for the MMS LI when the destination of the redirected MM message is a target non-local ID, i.e. MMS User Agent (C).

###### 4.4.3.5.3.2	MM messages to MMS LI messages – origination is target non-local ID – case 1

The figure 4.4.3.5.3.2-1 illustrates the scenario for the generation of xIRIs from the MMS messages as the MMS User Agent (B) is notified and subsequently forwards the MM message received from MMS User Agent A (target non-local ID) to MMS User Agent (C). Both MMS User Agent (A) and MMS User Agent (C) are different network domains.

Figure 4.4.3.5.3.2-1: MMS LI messages from MM messages (origin – target non-local ID) - case 1

The triggers for xIRIs are the MM4 messages sent to, or received from, the originating MMS domain.

The MMSSendByNonLocalTarget is generated MMS Relay Server (B) receives the MM4_forward.REQ message.

The trigger for MMSDeliveryReportNonLocalTarget is MMS Relay Server (B) sending of the MM4_delivery-report.REQ message. However, sending of MM4_delivery-report.REQ is conditional, and therefore, the generation of MMSDeliveryReportNonLocalTarget is also conditional.

###### 4.4.3.5.3.3	MM messages to MMS LI messages – origination is target non-local ID – case 2

The figure 4.4.3.5.3.3-1 illustrates the scenario for the generation of xIRIs from the MMS messages as the MMS User Agent (B) is notified and subsequently forwards the MM message received from MMS User Agent A (target non-local ID) to Email Client or to an MMS User Agent with MSISDN based Email Address.

Figure 4.4.3.5.3.3-1: MMS LI messages from MM messages (origin – target non-local ID) - case 2

The triggers for xIRIs are the MM4 messages sent to, or received from, the originating MMS domain.

The MMSSendByNonLocalTarget is generated MMS Relay Server (B) receives the MM4_forward.REQ message.

The trigger for MMSDeliveryReportNonLocalTarget is MMS Relay Server (B) sending of the MM4_delivery-report.REQ message. However, sending of MM4_delivery-report.REQ is conditional, and therefore, the generation of MMSDeliveryReportNonLocalTarget is also conditional.

4.4.3.5.3.4	MM messages to MMS LI messages (destination is target non-local ID) – case 1

The figure 4.4.3.5.3.4-1 illustrates the scenario for the generation of xIRIs from the MMS messages as the MMS User Agent (B) is notified and subsequently forwards the MM message received from MMS User Agent A to MMS User Agent C (target non-local ID). Both MMS User Agent (A) and MMS User Agent (C) are different network domains.

Figure 4.4.3.5.3.4-1: MMS LI messages from MM messages (destination – target non-local ID) - case 1

The triggers for xIRIs are the MM4 messages sent to, or received from, the destination MMS domain.

The MMSToNonLocalTarget is generated MMS Relay Server (B) sends the MM4_forward.REQ message toward the destination MMS domain.

The MM4_delivery_report.REQ received from the redirected-to end triggers the sending of xIRI MMSDeliveryReportNonLocalTarget. Since the receipt of MM4_delivery-report.REQ is conditional, the generation of xIRI MMSDeliveryReportNonLocalTarget is also conditional.

The MM4_read-reply-report.REQ received from the redirected-to end triggers the sending of xIRI MMSReadReportNonLocalTarget. Since the receipt of MM4_ read-reply-report.REQ is conditional, the generation of xIRI MMSReadReportNonLocalTarget is also conditional.

4.4.3.5.3.5	MM messages to MMS LI messages (destination is target non-local ID) – case 2

The figure 4.4.3.5.3.5-1 illustrates the scenario for the generation of xIRIs from the MMS messages as the MMS User Agent (B) is notified and subsequently forwards the MM message received from Email Client or MMS User Agent with an MSISDN based Email Address to MMS User Agent C (target non-local ID).

Figure 4.4.3.5.3.5-1: MMS LI messages from MM messages (destination – target non-local ID) - case 2

The triggers for xIRIs are the MM4 messages sent to, or received from, the destination MMS domain.

The MMSToNonLocalTarget is generated MMS Relay Server (B) sends the MM4_forward.REQ message toward the destination MMS domain.

The MM4_delivery_report.REQ received from the redirected-to end triggers the sending of xIRI MMSDeliveryReportNonLocalTarget. Since the receipt of MM4_delivery-report.REQ is conditional, the generation of xIRI MMSDeliveryReportNonLocalTarget is also conditional.

The MM4_read-reply-report.REQ received from the redirected-to end triggers the sending of xIRI MMSReadReportNonLocalTarget. Since the receipt of MM4_ read-reply-report.REQ is conditional, the generation of xIRI MMSReadReportNonLocalTarget is also conditional.

***** End of the new text ****