DeletSA3#95-LI	 S3i240641

Las Vegas; October 29-November 1, 2024

Source:	Nokia,

Title:	pCR to TR 33.929-8: MMS LI – scope of MMS messages – Part IX (single MMS Relay/Server case, general, originating target)

Document for:	Agreement

Agenda Item:	9

Work Item / Release:	LI19

Abstract of the contribution: This pCR provides input text to TR 33.929-8 LI for MMS. Scope of MMS messages reported as xIRIs when only one MMS Relay Server is used. Originating MMS User Agent being a target is illustrated here with common text that applies when the terminating MMS User Agent is a target. Since only one MMS Relay/Server is used, all users are presumed to be MMS User Agents identified with a phone number.

For the scenario where the MMS User Agent even though served by the same MMS Relay/Server but addressed using an MSISDN-based Email Address is not included here since in the process of MM messages going over the MM3 interface, it is treated as if in a different MMS Relay/Server.

BACKGROUND

The TR 33.929-8 provides the implementation guidance for the LI aspects related to the MMS.

In the illustrations shown in this pCRs, both originating and terminating MMS User Agents are served by the same MMS Relay/Server. This pCR covers the case when the originating MMS User Agent is the target.

The originating MMS User Agent or terminating MMS User Addressed using an MSISDN based Email Address are not illustrated here because from an MMS LI perspective, messages are transferred over MM3 interface.

The cases are same multiple MMS Relay/Server scenario illustrated before.

PROPOSAL

Incorporate the following to the TR 33.929-8.

# 4	Illustration of LI for MMS

## 4.1	General

## 4.2	MMS background with LI scope

## 4.3	Scope of target identities

## 4.4	Scope of MMS messages reported

### 4.4.1	General

### 4.4.2	3GPP, OMA and MMS LI messages

### 4.4.3	Topology views

#### 4.4.3.1	General

#### 4.4.3.2	MMBox specific operations

#### 4.4.3.3	Originating MMS domain

#### 4.4.3.4	Terminating MMS domain

#### 4.4.3.5	Redirecting MMS domain

***** New text ****

#### 4.4.3.6	Single MMS Relay/Server – non-redirection case

##### 4.4.3.6.1	General

###### 4.4.3.6.1.1	Scope

In the illustrations, both originating and terminating MMS User Agents are served by the same MMS Relay/Server. Six cases are illustrated in the subsequent clauses:

- Immediate retrieval case, originating MMS User Agent is the target.

- Immediate retrieval case, terminating MMS User Agent is the target.

- Deferred retrieval case, originating MMS User Agent is the target.

- Deferred retrieval case, terminating MMS User Agent is the target.

- Deletion case, originating MMS User Agent is the target.

- Deletion case, terminating MMS User Agent is the target.

The case of Email Client or MSISDN based Email Address or target non-local ID do not come into play in these illustrations, since both users are local to the MMS Relay Server.

###### 4.4.3.6.1.2	MMBox

When MM message is sent by the originating MMS User Agent, if requested so, a copy of the MM message is stored in the MMBox with the status showing "sent", if an MMBox is supported for the originating MMS User Agent. Therefore, the storing of the MM message at the MMBox does not happen if the store indication is not sent in the m-send-req message, even if the MMBox is supported for the originating MMS User Agent.

Likewise, when the MMBox is supported, the incoming MM message is automatically stored in the MMBox , if setup and configured so, by the terminating MMS User Agent. The status of thus stored MM message would be "New". In that case, the terminating MMS User Agent would point to the MM message stored in the MMBox for any subsequent actions (e.g. retrieval, deletion, forwarding). The status of the MM message changes according to the action taken (e.g. "Retrieved", "Forwarded").

When MMBox is not used, it is presumed that the MM messages are stored locally at the MMS/Relay Server.

When the MMBox is used, the MMS User Agent may also perform a few MMBox specific operations as illustrated in clause 4.4.3.2.2.

Each MMS User Agent has own inboxes in the MMBox. In the illustrations, separate MMBox shown for each MMS User Agent to infer that individuality. Either both, or one, or none may have the MMBox.

###### 4.4.3.6.1.3	Delivery reports

The delivery report is conditional. The MMS Relay/Server sends a delivery report to the originating MMS User Agent if and only if the originating MMS User Agent has requested for such a report and the terminating MMS User Agent permits the delivery of such a report.

###### 4.4.3.6.1.4	Read reports

The read report from the terminating MMS User Agent is conditional. The terminating MMS User Agent may send a read-report if the originating MMS User Agent requests for such a report and the terminating MMS User chooses to send such a report.

##### 4.4.3.6.2	Overview

Figure 4.4.3.6.2-1 gives an overview where both originating and terminating MMS User Agents are served by the same MMS Relay/Server. In the illustration, both originating and terminating MMS User Agents are the targets.

Figure 4.4.3.6.2-1: Single MMS Relay/Server serving originating and terminating targets

The MM1 messages that flow between the MMS Relay/Server and the target MMS User Agent are used to generate the corresponding xIRIs.

Use of the MMBox is optional on a per MMS User Agent level. In other words, either one or both, or neither of the two originating MMS User Agents may have the MMBox. When used, the MMBox specific operations (which either of the two MMS User Agents may perform in a mutually exclusive manner) along with the associated LI aspects are illustrated in 4.4.3.2.2.

##### 4.4.3.6.3	Originating MMS User Agent is the target

###### 4.4.3.6.3.1	General

In all the illustrations, when MM message is sent by the originating MMS User Agent, if requested so, a copy of the MM message is stored in the MMBox. Therefore, the storing of the MM message at the MMBox does not happen if the store indication is not sent in the m-send-req message.

Even though the trigger for the generation of xIRI MMSSend is the m-send-conf message that the MMS-Relay/Server

Since the sending of the m-delivery-ind and m-read-orig-ind are conditional, the generation of xIRIs MMSDeliveryReport and MMSReadReport are also conditional.

###### 4.4.3.6.3.1	Case 1 - Immediate retrieval

The figure 4.4.3.6.3.1-1 gives an overview of the MMS LI messages applicable when single MMS Relay/Server is serving both originating and terminating MMS User Agent with originating MMS User Agent being the target.

The MMS User Agent (B) upon receiving the notification automatically retrieves the MM message (immediate retrieval).

Figure 4.4.3.6.3.1-1: Single MMS Relay/Server – MMS User Agent (A) is the target (immediate retrieval)

In this illustration, the terminating MMS User Agent automatically retrieves the MM message.

###### 4.4.3.6.3.2	Originating MMS User Agent is the target (deferred retrieval)

The figure 4.4.3.6.3.2-1 gives an overview of the MMS LI messages applicable when single MMS Relay/Server is serving both originating and terminating MMS User Agent with originating MMS User Agent being the target.

The MMS User Agent (B) receives the notification and subsequently retrieves the MM message (deferred retrieval).

Figure 4.4.3.6.3.2-1: Single MMS Relay/Server – MMS User Agent (A) is the target (deferred retrieval)

In this illustration, the terminating MMS User Agent upon receiving a notification of the incoming MM message retrieves the MM message with a time-gap, referred to as deferred retrieval.

###### 4.4.3.6.3.3	Originating MMS User Agent is the target (deletion)

The figure 4.4.3.6.3.3-1 gives an overview of the MMS LI messages applicable when single MMS Relay/Server is serving both originating and terminating MMS User Agent with originating MMS User Agent being the target.

The MMS User Agent (B) upon receiving the notification and deletes the MM message. This case is applicable only for a deferred retrieval case.

Figure 4.4.3.6.3.3-1: Single MMS Relay/Server – MMS User Agent (A) is the target (deletion)

In this illustration, the terminating MMS User Agent, upon receiving a notification of the incoming MM message deletes the MM message without a retrieval.

***** End of the new text ****