3GPP TSG-SA3 Meeting #95-LI	s3i240650

# Las Vegas, United States, 29th Oct 2024 - 1st Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.127 | CR | 0253 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | * |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LI for 5G LAN parameter provisionning (VN Group) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3LI (Ministère Economie et Finances) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI19 |  |  |  |  |  | Date: |  |  | 2024-10-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Lack of VN Group information related to a specific target by NEF PoI |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add 5G LAN Parameter provisioning (VN Group) and related events for an LI system |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Regulatory issues |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.9.X; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | * | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | * | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | * | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF FIRST CHANGE

### 7.9.X	LI for 5G LAN parameter provisioning

#### 7.9.X.1	Background

5G LAN Parameter provisioning is a provisioning capability exposed by NEF to AF (see TS 23.502 [4] clause 4.15.6 and TS 29.522 [31] clause 4.4.15). The AF can use this capability to create, update, delete and query 5G VN group and its information. The AF can then use the group identifier in service procedures invoked by AF towards the NEF instead of using each UE identity and invoking the service procedures per UE of the group. The AF provides the 5G LAN parameter provisioning data specified in TS 29.522 [31] clause 5.7.2 to NEF and NEF updates the 5G VN group information towards UDM as described in figure 7.9.X.1-1.

Figure 7.9.X.1-1: 5GS architecture for 5G LAN parameter provisioning

#### 7.9.X.2	Architecture

Figure 7.9-1 without the CC-POI in NEF provides the architecture for LI for 5G LAN parameter provisioning.

#### 7.9.X.3	Target identities

The LIPF present in the ADMF provisions the intercept information associated with the following target identities to the IRI-POI present in the NEF:

-	GPSI.

#### 7.9.X.4	IRI events

The IRI-POI present in the NEF shall generate xIRI, when it detects the following specific events or information related to 5G LAN parameter provisioning:

-	5G VN group creation.

-	5G VN group update.

-	5G VN group deletion.

-	5G VN group query.

The 5G VN group creation xIRI is generated when the IRI-POI present in the NEF detects that an AF sent a request to create a 5G VN group the target UE belongs to.

The 5G VN group update xIRI is generated when the IRI-POI present in the NEF detects that an AF sent a request to update a 5G VN group the target UE belongs to.

The 5G VN group deletion xIRI is generated when the IRI-POI present in the NEF detects that an AF sent a request to delete a 5G VN group the target UE belongs to.

The 5G VN group query xIRI is generated when the IRI-POI present in the NEF detects that an AF sent a request to query a 5G VN group the target UE belongs to.

END OF FIRST CHANGE

*** End of last change ***